3GPP TSG-RAN WG2 Meeting #120	R2-2212797

Toulouse, France, 14th Nov. – 18th Nov., 2022

Agenda Item:	8.15.2

Source: 	ITL

Title: 	Discussion on sidelink un-licensed

Document for:	Discussion and decision

Introduction

In RAN2#119bis-e [1], the agreements related to this contribution are as follows:

Agreement on consistent LBT failure:

1: 	SL-specific LBT failure indication from PHY is needed for SL-specific consistent LBT failure detection in the MAC. How/whether it is used for other purposes can be further discussed.

2:	Support SL-specific consistent LBT failure detection and recovery procedure in the MAC for SL-U. Details of recovery to be further worked on granularity of (consistent) LBT failure.

3:	Send LS to RAN1 asking “When an SL-specific LBT failure indication is notified for an SL transmission by the PHY, in which resource granularity the SL-specific LBT failure can be considered as being detected (e.g. per Resource Pool, per RB set, per SL BWP, etc.)?

- Detailed wording can be discussed during the email discussion. Some background information (e.g. why/what we (actually) ask) can be also provided.

4:	As the general principle, reuse the consistent LBT failure detection procedure in NR-U as the baseline for SL-specific consistent LBT failure detection in SL-U.

5:	As in NR-U, introduce the following parameters and variables for the SL-specific consistent LBT failure detection in SL-U as the baseline:

- An SL-specific LBT failure indication counter (e.g. SL_LBT_COUNTER);

- An SL-specific maximum LBT failure instance count threshold (e.g. sl-LBT-FailureInstanceMaxCount);

- An SL-specific LBT failure detection timer (e.g. sl-LBT-FailureDetectionTimer).

6:	Reuse the following MAC behaviors on TIMER/COUNTER handling in NR-U for SL-specific consistent LBT failure detection procedure in SL-U as the baseline:

- As in NR-U, if an SL-specific LBT failure indication is received from the lower layer, the SL-specific LBT failure indication counter (e.g. SL_LBT_COUNTER) is incremented by one.

- As in NR-U, if an SL-specific LBT failure indication is received from the lower layer, start or restart the SL-specific LBT failure detection timer (e.g. sl-LBT-FailureDetectionTimer)

- As in NR-U, if the SL-specific LBT failure indication counter value is equal to or larger than the SL-specific maximum LBT failure instance count threshold (e.g. sl-LBT-FailureInstanceMaxCount), consistent LBT failure is triggered/declared by the MAC entity.

- As in NR-U, if the SL-specific LBT failure detection timer (e.g. sl-LBT-FailureDetectionTimer) expires, the SL-specific LBT failure indication counter (e.g. SL_LBT_COUNTER) is reset to 0.

- As in NR-U, if the maximum LBT failure instance count threshold (e.g. sl-LBT-FailureInstanceMaxCount) or SL-specific LBT failure detection timer (e.g. sl-LBT-FailureDetectionTimer) is reconfigured, SL-specific LBT failure indication counter (e.g. SL_LBT_COUNTER) is reset to 0.

7:	Support the mechanism that a mode-1 UE can indicate the SL-specific consistent LBT failure to the gNB. FFS on a mode-2 UE in RRC_CONNECTED.

In this contribution, we discuss sidelink un-licensed operation (SL-U) on RAN2 scope.

Discussion

In RAN1#109-e [2], the existing SL resource pool and LBT bandwidth (RB set) in NR-U have agreed to reuse for SL-U. Sidelink transmission on multiple LBT Bandwidths (RB sets) agreed to support high data rate application.

Agreement SL BWP, SL resource pool in R16/R17 NR SL and RB set in R16 NR-U are reused for SL-U as baseline - Only one SL BWP is (pre-)configured within a carrier - The SL BWP is (pre-)configured to include one or multiple SL resource pools - At least support that one SL resource pool can be (pre-)configured to include integer number of RB sets - FFS: whether/how to support one SL resource pool can include sub-set of PRBs of one RB set - FFS: the applicable resource pool - FFS: the impact on sub-channel size and number of sub-channels in a resource pool if sub-channel is supported - PRBs within intra-cell guard band of two adjacent RB sets belong to a resource pool if the resource pool includes the two adjacent RB sets - FFS details, e.g., how such PRBs are used, the applicable resource pool, etc. - FFS: whether R16/R17 NR SL S-SSB slots and/or new S-SSB slots (if supported) are excluded from resource pool - FFS: which slots belong to resource pool, e.g., how to set the value of bitmap, whether to consider SL-U/NR-U operating in the same carrier and whether TDD configuration are considered, etc. - FFS: the impact of PSCCH/PSSCH mapping to frequency resources on resource pool configuration, on sub-channel definition if sub-channel is supported, etc. Agreement Channel access procedures for transmission(s) on multiple channels are supported for NR sidelink operation as defined by TS37.213 for NR-U (wherever applicable) - FFS whether the downlink, uplink and/or semi-static multiple channel access procedure(s) (if supported) from NR-U should be used as a baseline and whether/how they are applied in SL mode 1 and mode 2 operation

For sidelink mode 1 and 2, the existing RA (Resource allocation) modes are supported as a baseline in RAN1#109-e [2]. Mode 1 is referred as gNB provides dynamic grant and configured grant sidelink grants to UEs for SL transmission and mode 2 is referred as UE performs resource selection and reservation based on a full/partial sensing, random selection.

Agreement - The existing sidelink mode 1 RA including dynamic grant, Type 1 and Type 2 configured grants are supported as a baseline for sidelink operation in a shared carrier, subject to applicable regional regulations. At least in dynamic channel access, SL UE performs Type 1 or one of the Type 2 LBTs before SL transmission using the allocated resource(s), in compliance with transmission gap and LBT sensing idle time requirements specified in TS37.213. - FFS whether/how mode 1 resource allocation procedure needs to be updated / enhanced due to shared spectrum channel access - The existing sidelink mode 2 RA schemes are supported as a baseline for sidelink operation in a shared carrier, subject to applicable regional regulations. At least in dynamic channel access, SL UE performs Type 1 or one of the Type 2 LBTs before SL transmission using the selected and/or reserved resources, in compliance with transmission gap and LBT sensing idle time requirements specified in TS37.213. - FFS whether/how mode 2 resource selection procedure needs to be updated / enhanced due to shared spectrum channel access - FFS whether/how multi-consecutive slots transmission can be supported for NR sidelink operation in unlicensed spectrum, including the following aspects - channel access, resource allocation and PHY channel design - FFS whether/how enhancement is needed between the end of the LBT procedure and the start of the SL transmission to retain channel access - RAN1 to strive for a common solution for channel access for Mode 1 and Mode 2

Consistent LBT failure

Figure 1. Consistent LBT failure in NR-U

Figure 1 shows the consistent LBT failure detection based on the LBT counter and LBT failure detection timer. In Figure 1 (a), the LBT counter is initialized to 0 because the LBT failure detection timer expires. In Figure 1 (b), the consistent LBT failure is triggered when the LBT counter is equal to or greater than the LBT MAX counter.

In RAN2#119bis-e [1], RAN2 has agreed to detect SL-specific consistent LBT failure based on LBT counter and LBT failure detection timer like NR-U. For example, when the LBT failure indication is received from PHY layer increase the LBT counter. If the LBT counter reaches the LBT MAX counter, the consistent LBT failure is triggered on the granularity of consistent LBT failure.

In NR-U, consistent LBT failure is detected per UL BWP. However, in SL-U, since only one SL BWP is supported within the SL carrier, RAN2 needs to define the granularity of consistent LBT failure. Therefore, RAN2 sent the LS to RAN1 to request feedback on the granularity of consistent LBT failure (per SL BWP, per SL resource pool, per RB set, etc) [3].

Figure 2. LBT failure MAC CE

Figure 2 shows the LBT failure MAC CE used for consistent LBT failure recovery. If the consistent LBT failure is triggered and not canceled, UE reports the LBT failure MAC CE to the gNB. The Ci field indicates the consistent LBT failure on serving cell i.

In RAN2#119bis-e [1], RAN2 has agreed to support the mechanism that a mode 1 UE can indicate the SL specific consistent LBT failure to the gNB. However, how to indicate the SL specific consistent still needs to be discussed. We think that at least MAC CE based SL specific consistent is supported for SL specific consistent LBT failure recovery like NR-U. Also, the content of MAC CE should be based on the granularity of SL specific consistent LBT failure. For example, if the granularity of SL specific consistent LBT failure is detected per resource pool, the serving cell index i should be replaced with the resource pool index. This enables the gNB to allocate SL grant resources except for the resource pool or RB set where SL specific consistent LBT failure is triggered.

Proposal 1: Introduce SL LBT failure MAC CE for SL specific consistent LBT failure recovery

Proposal 2: The content of SL LBT failure MAC CE should be based on the granularity of SL specific consistent LBT failure

In mode 2, we show our opinion on each granularity of consistent LBT failure considering the autonomous SL specific consistent LBT failure recovery as follows:

- Per SL BWP: Since RAN1 has already agreed to support only one SL BWP in a SL-U carrier, it is hard to define how to operate consistent LBT failure recovery.

- Per SL resource pool: Resource pool re-selection could be considered for consistent LBT failure recovery. Consistent LBT failure should be considered for resource pool (re-)selection.

  - Resource pool (re-)selection: select any resource pool among the configured resource pool where consistent LBT failure is not triggered.

- Per RB set: Resource pool re-selection and resource (re-)selection could be considered for consistent LBT failure recovery.

  - Resource pool (re-)selection: If selected resource pool has one RB set and consistent LBT failure is triggered, select any resource pool among the configured resource pool where consistent LBT failure is not triggered on at least 1 RB set.

  - Resource (re-)selection: If the selected resource pool has more than two RB sets and consistent LBT failure is not triggered at least one RB set, select the time and frequency resources for transmission opportunity within RB set(s) where consistent LBT failure is not triggered from the resource pool.

We think that both resource pool (re-)selection enhancement and resource (re-)selection enhancement could be used for an autonomous SL specific consistent LBT failure recovery for a mode 2 UE.

Proposal 3: we support consistent LBT failure is detected per resource pool or per RB set

Proposal 4: Introduce autonomous SL specific consistent LBT failure recovery for a mode 2 UE

- Resource pool (re-)selection

- Resource (re-)selection

Proposal 5: If P4 is agreed, introduce the consistent LBT failure result based resource/resource pool (re-)selection

DRX (Discontinuous reception)

In RAN1#109e, RAN1#110 and RAN1#110bis-e [2, 4, 5], agreements related to PSFCH are as followings:

Agreement For PSFCH and SL-HARQ in SL-U: - At least R16 NR SL PSFCH format 0 is supported - FFS whether to introduce new PSFCH format - FFS: how to meet OCB and PSD requirement for PSFCH transmission, e.g., using interlaced RB transmission, whether/how to avoid too small PSFCH capacity, etc. - FFS: the locations of PSFCH resources, e.g., (pre-)configured, dynamically indicated, etc. - FFS: whether/how to address PSFCH transmission dropping due to LBT failure, e.g., whether to have multiple PSFCH occasions for a PSSCH and the related PSSCH-PSFCH mapping relationship, impact on SL HARQ-ACK reporting to the gNB for Mode 1, etc. - FFS: whether/how to address PSFCH and related PSSCH in different COTs Agreement Regarding PSFCH transmission, at least the followings alternatives can be further studied - Alt 1: each PSFCH transmission occupies a common interlace and zero or one or more dedicated PRB(s) - Alt 2: each PSFCH transmission occupies an interlace, and may or may not further apply code domain enhancement (e.g., OCC, PRB-level cyclic shifts) - Alt 3: each PSFCH transmission occupies some dedicated PRBs and some common PRBs - FFS details of above alternatives Agreement If RAN1 decides that LBT is performed for PSFCH transmission, for the time and frequency domain locations of PSFCH resources, at least the followings alternatives can be further studied - Alt 1: PSFCH resources are (pre-)configured - Alt 2: PSFCH resources are dynamically indicated - Combination of above alternatives are not precluded - FFS details of above alternatives Agreement At least there is 1 PSFCH occasion per PSCCH/PSSCH transmission, FFS details Agreement To address PSFCH transmission dropping due to LBT failure, the followings are to be studied: - Alt 1: Support more than 1 PSFCH occasion per PSCCH/PSSCH transmission - Alt 2: PSFCH resources are dynamically indicated - Alt 3: Convey SL-HARQ feedback information in PSCCH/PSSCH, e.g., new SCI or new MAC-CE - Alt 4: drop PSFCH transmission - Alt 5: Support trigger based HARQ feedback reporting for non-numerical HARQ FB and one shot HARQ FB - Combination of above alternatives are not precluded - FFS details of above alternatives

In Rel-17 Sidelink enhancements, SL DRX was introduced for power saving. The DRX timers (e.g., sl-drx-HARQ-RTT-Timer, sl-drx-RetransmissionTimer) are defined differently depending on the presence of PSFCH resource within resource pool, the presence of retransmission resources in SCI for PC5 interface. And, the DRX timers (e.g., drx-HARQ-RTT-TimerSL, drx-RetransmissionTimerSL) are defined differently depending on the presence of PUCCH resource for Uu interface.

To solve the case where PSFCH transmission cannot be performed due to LBT failure and MCOT (Maximum channel occupancy time), a dedicated PSFCH resource configuration, dynamic PSFCH resource indication and one or more HARQ Feedback timings are being discussed in RAN1. Thus, RAN2 should discuss how to operate the DRX timer considering the un-licensed characteristics of MCOT and COT sharing as well as the HARQ feedback methods discussed in RAN1.

Proposal 6: RAN2 needs to discuss how to operate the DRX timer considering the HARQ feedback method (e.g., more than 1 PSFCH occasion per PSCCH/PSSCH, dynamic PSFCH indication) discussed by RAN1

Conclusion

In this contribution, we discussed sidelink operation in un-licensed. According to discussion in section 2, we have the following proposals:

Proposal 1: Introduce SL LBT failure MAC CE for SL specific consistent LBT failure recovery

Proposal 2: The content of SL LBT failure MAC CE should be based on the granularity of SL specific consistent LBT failure

Proposal 3: we support consistent LBT failure is detected per resource pool or per RB set

Proposal 4: Introduce autonomous SL specific consistent LBT failure recovery for a mode 2 UE

- Resource pool (re-)selection

- Resource (re-)selection

Proposal 5: If P4 is agreed, introduce the consistent LBT failure result based resource/resource pool (re-)selection

Proposal 6: RAN2 needs to discuss how to operate the DRX timer considering the HARQ feedback method (e.g., more than 1 PSFCH occasion per PSCCH/PSSCH, dynamic PSFCH indication) discussed by RAN1

Reference

RAN2#119bis-e Meeting report

RAN1#109e Meeting report

R2-2210936, LS on LBT failure indication and consistent LBT failure

RAN1#110 Meeting report

RAN1#110bis-e Meeting report