3GPP TSG-RAN WG2 Meeting #120	R2-2212799

Toulouse, France, 14th Nov. – 18th Nov., 2022

Agenda Item:	8.7.4.1

Source: 	ITL

Title: 	Discussion on NTN-TN and NTN-NTN cell re-selection

Document for:	Discussion and decision

Introduction

In Rel-18 WID of NTN enhancement [1], the objective related to this contribution is as follows:

- NTN-TN and NTN-NTN mobility and service continuity enhancements This work considers existing methods from NR TN as well as outcome of Rel-17 NR NTN WI outcome as baseline for NTN-TN mobility. - Specify NTN-TN and NTN-NTN measurement/mobility and service continuity enhancements [RAN2,RAN3,RAN4] - For NTN-NTN mobility, specify cell reselection enhancements for earth moving cell, the timing based and location-based cell reselection for quasi-earth fixed cell in Rel-17 can be considered as the starting point. [RAN2, RAN3, RAN4] - Specify NTN-NTN handover enhancement for RRC_CONNECTED UEs in the quasi-earth-fixed cell and earth-moving cell to reduce the signalling overhead. [RAN2, RAN3] - Specify cell reselection enhancements for RRC_IDLE/INACTIVE UEs to reduce UE power consumption (NTN-TN mobility is prioritized). [RAN2, RAN3, RAN4] - Study and, if needed, specify enhancement to Xn[/NG] signalling to support feeder link switch-over, CHO, e.g. exchange of necessary information between gNBs. [RAN3]

In RAN2#119-e and RAN2#119bis-e [2-3], the agreements related to this contribution are as follows:

Agreements: 1. RAN2 to work on a solution so that measurements for TN’s coverage are performed only when relevant (FFS what relevant means). 2. RAN2 to work on assistance information that can be provided to NTN UEs for the above. 3. Cell reselection enhancements (for both NTN-NTN and NTN-TN mobility) are considered for both Earth-moving and (quasi-)Earth-fixed scenarios, at least via the use of system information for broadcasting necessary parameters (dedicated signalling is not precluded). FFS whether the same or different solutions are used for Earth-moving and (quasi-)Earth-fixed scenarios Agreements: 1. For NTN-NTN cell reselection with earth moving cell, RAN2 will consider providing parameters of serving cell to UE, for UE to estimate when the serving cell stops providing coverage at the present UE location (FFS whether this will be an optional UE feature) (this does not exclude any time-based or location-based approach) (other solutions can also be considered) 2. To enhance NTN-TN cell reselection, means are defined for a UE to differentiate when camping in an area only covered by NTN network (earth-moving or earth-fixed) vs an area where TN network(s) is/are also available. Agreements: 1. System information is the basic means for providing necessary parameters to assist UE to estimate when the serving cell stops providing coverage at the present UE location. 2. UE is not required to perform neighbour cell measurements for TN neighbour cells in an area where there is no TN network coverage. 3. The method of detecting the transmission energy or SIB presence to determine the NTN coverage when a UE currently camps on a TN cell is not pursued. 4. In Earth-moving cell, the reference location and distance threshold of serving cell are provided by network for UE to estimate when the serving cell stops providing coverage at the present UE location. FFS how the reference location and/or distance threshold are provided to the UE

In this contribution, we focus on cell reselection enhancements for RRC IDLE/INACTIVE UEs to reduce UE power consumption and cell reselection enhancements for earth moving cell.

Discussion

NTN-TN and NTN-NTN mobility and service continuity enhancements have been discussed in Rel-17 NTN. During the discussion of Rel-17 NTN, timing and location based CHO (Conditional handover), cell re-selection for earth-fixed cell was mainly discussed because the fluctuation of received signal strength in NTN is smaller than in TN.

- Timing based cell re-selection: Network provides the time information (t-Service) that when a cell is going to stop serving the area to UE. Thus, UE measures neighbouring cells for cell re-selection before t-Service.

- Location based cell re-selection: UE measures the neighbour cells for cell re-selection when the distance between UE and reference location is greater than a distance threshold.

NTN-TN cell reselection for power saving

Figure 1. NTN-TN co-existence scenario

In Rel-17 NTN, RAN2 agreed that TN cells can be configured with a higher priority over NTN because TN cells provide more performance than NTN cells. In that, UE can cell reselect to TN cells in NTN-TN co-existence scenario as in Figure 1. In this case, UEs (UE 1, 2, 3) always measure neighbouring TN cells which are provided by system information even if there are no neighbour TN cells. This will cause continuous power consumption. And, power consumption of RRC Idle/Inactive is one of the important factors. Therefore, it was decided to discuss methods for reducing power consumption in Rel-18 NTN enhancements.

We think that an additional distance based cell measurement rule for cell re-selection is a way to reduce power consumption. For example, UE may not measure TN cells when the distance between UE and the reference location of a TN cell (which can be a cluster of TN cells or a specific area) is larger than a threshold. This method can be applied to both earth-moving and fixed cells because the reference location is fixed in terrestrial.

RAN2 has agreed to support assistance information based cell reselection enhancements. Some aspects of assistance information are as follows:

- Opt 1: Neighbour cell’s cell type (TN or NTN)

- Opt 2: Where TN network is available

  - The cell center and cell radius of TN neighbour cells, or in other terms, the reference location and a distance threshold of TN neighbour cells

  - The boundary line between TN area and NTN area

  - For quasi-earth fixed cells, TN coverage is described by a distance range from the cell center and an angle range based on a reference direction

  - An indication could be included in system information to indicate NTN cell’s coverage overlaps with terrestrial TN cell’s coverage

  - NTN cell can be divided to several virtual areas based on certain criteria. The virtual areas and the corresponding TN frequency information are broadcast as assistance information to help UE perform more accurate TN measurements.

  - Introduce a parameter using the polygon shape captured in TS 23.032 to describe the coverage area of a TN neighbour cell.

For the Opt 1, we think that the cell type of the neighbor cell indication (explicit or implicit way) is help UE which frequencies should be measured when the condition (distance) is met. As discussed in [3], UE can differentiate TN and NTN frequencies using the band number (implicit way). However, since there is overlap frequency bands between TN and NTN, indication of which frequency is TN (Explicit way) helps to UE.

Proposal 1: Introduce the cell type indication of neighboring cell (TN or NTN) by implicit or explicit way

For the Opt 2, assistance information of the reference location of TN cells and distance threshold is enough to use cell re-selection. Also, we think that UE has to know which frequencies are related to a reference location of TN cell(s) and distance threshold. This can help UE to reduce power consumption. For example, if NW provides the assistance information (reference location, distance threshold and frequencies) for each TN cell when there are multiple TN cells with far away, UE can select which frequencies should be measured.

Proposal 2: The reference location and a distance threshold of TN neighbour cells are used for UE to identify an area where TN network is available

Proposal 3: Introduce assistance information about TN frequencies per the reference location

NTN-NTN cell reselection for earth moving cell

Figure 3. Reference location update

As discussed in Rel-17 NTN and for WID of Rel-18, timing and location based cell reselection for earth moving cell can be considered as starting point. In Rel-17 NTN, timing and location based cell reselection for earth moving cell was not agreed due to satellite moving. It means that if the reference location is fixed on time [T1, T2] in Figure 3, then UE 2 may choose not to perform the measurement of NR inter-frequency because the distance between UE 2 and the reference location is smaller than the distance threshold. For this reason, we think that the reference location needs to be updated. i.e., UE applies different reference locations at different times.

Proposal 4: Introduce reference location update scheme for cell reselection enhancements for earth moving cell

Figure 4. UlSyncValdityDuration

We think that timer based reference location update could be applied easily. In Figure 4, the parameter (UlSyncValidityDuration) is introduced to update parameters related to the UL Synchronization (TA Common, Ephemeris) in Rel-17 NTN. UE starts the UlSyncValidityDuration from the Epochtime (SFN & Subframe) that is the uplink time synchronization reference point. UE should attempt to re-acquire SIB19 (NTN specific SIB) before the end of the UlSyncValidityDuration. Like this, the UlSyncValidityDuration can be re-used to update the reference location which is broadcasted by SIB19. Also, a new timer (e.g., Reference location update timer) can be considered to update the reference location.

We also support to provide reference location list based scheme. Earth moving cell provides the multiple reference locations and its time information (Epoch time) to the UE. In this way, the UE can clearly know when to update the reference location and no additional signaling is required. Table 1 shows an example of a reference location list. The epoch time can be expressed SFN & Subframe number as in Rel-17 NTN.

Table 1. Example of reference location list

| Epoch time (SFN & Subframe number) | Reference location (Ellipsoid-point) |
| --- | --- |
| 2022-06-29 02:00:00.000 | Reference location 1 |
| 2022-06-29 02:05:00.000 | Reference location 2 |
| 2022-06-29 02:10:00.000 | Reference location 3 |
| 2022-06-29 02:15:00.000 | Reference location 4 |
| 2022-06-29 02:20:00.000 | Reference location 5 |
| 2022-06-29 02:25:00.000 | Reference location 6 |
| 2022-06-29 02:30:00.000 | Reference location 7 |

Proposal 5: Introduce reference location update scheme one of the following options:

- Option 1: Timer based reference location update: reuse the UlSyncValidityDuration or define a new timer (e.g., Reference location update timer)

- Option 2: Multiple reference locations and its time information of Earth-moving cell

Conclusion

In this contribution, we discussed cell re-selection enhancement in idle/inactive mode. According to discussion in section 2, we have the following proposals:

Proposal 1: Introduce the cell type indication of neighbour cell (TN or NTN) by implicit or explicit way

Proposal 2: The reference location and a distance threshold of TN neighbour cells is used for UE to identify an area where TN network is available

Proposal 3: Introduce assistance information about TN frequencies per the reference location

Proposal 4: Introduce reference location update scheme for cell reselection enhancements for earth moving cell

Proposal 5: Introduce reference location update scheme one of the following options:

- Option 1: Timer based reference location update: reuse the UlSyncValidityDuration or define a new timer (e.g., Reference location update timer)

- Option 2: Multiple reference locations and its time information of Earth-moving cell

Reference

RP-221819, R18 WID NR_NTN_enh

RAN2#119e Meeting report

RAN2#119bis-e Meeting report