3GPP TSG-RAN WG2 Meeting #120	R2-2212805

# Toulouse, 14th– 18th Nov, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 3737 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on essential SIB19 for NR NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi, CAICT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions-Core |  |  |  |  |  | Date: |  |  | 2022-11-2 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In RAN2 #119e, it was agreed that: => RAN2 understands that SIB19 is essential for NTN cell, and it could be up to UE implementation if UE cannot acquire SIB19. FFS if a note is needed in the spec for this Currently, from the specification point of view, Regarding what is essential system information, we can only refer to 38.331 chapter 5.2.2.5 Essential system information missing. In this capture, only MIB and SIB1 is mentioned as essential system information. The term “essential system information” is used in both RRC establishment and reestablishment procedure. 5.3.3 RRC connection establishment 5.3.3.2 Initiation The UE shall ensure having valid and up to date essential system information as specified in clause 5.2.2.2 before initiating this procedure. 5.3.7 RRC connection re-establishment 5.3.7.3 Actions following cell selection while T311 is running Upon selecting a suitable NR cell, the UE shall: 1> ensure having valid and up to date essential system information as specified in clause 5.2.2.2; Thus, specification should make it clear that SIB19 is essential system information. In addition, current 38.331 specification has not captured capture a note about UE implementation is allowed when SIB19 is unable to acquire. If there is no such a note, UE cannot do cell reselection when SIB19 cannot be acquired, as this is currently forbidden from the specification, i.e. the current specification ask UE to stay in the best cell, UE cannot violate the best cell principle. Consequently, spec developer may end up following the best cell principle. Thus, such a misleading should be avoided. |  |  |  |  |  |  |  |  |
|  |  | th |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a note to clarify that SIB19 is essential and the behavior when not being able to acquire SIB19 when UE is in RRC_IDLE or in RRC_INACTIVE or in RRC_CONNECTED while T311 is running. Impact analysis Impacted functionality: System Information acquisition Inter-operability: If the network is implemented according to the CR while the UE is not, no inter-operability issue found. If the UE is implemented according to the CR while the network is not, no inter-operability issue found. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the CR is not approved, UE may not consider SIB19 as essential system information, and UE may continue camped on the current cell and thus cannot establish/reestablish/resume the connection. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.5; |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR … |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

5.2.2.5	Essential system information missing

The UE shall:

1>	if in RRC_IDLE or in RRC_INACTIVE or in RRC_CONNECTED while T311 is running:

2>	if the UE is unable to acquire the MIB:

3>	consider the cell as barred in accordance with TS 38.304 [20];

3>	perform barring as if intraFreqReselection is set to allowed;

2>	else if the UE is unable to acquire the SIB1:

3>	consider the cell as barred in accordance with TS 38.304 [20];

3>	if the UE is a RedCap UE:

4>	peform barring as if intraFreqReselectionRedCap is set to allowed;

3>	else:

4>	perform cell re-selection to other cells on the same frequency as the barred cell as specified in TS 38.304 [20].

NOTE:	SIB19 is considered as essential system information if UE is accessing NR via NTN access. For UE in RRC_IDLE or in RRC_INACTIVE or in RRC_CONNECTED while T311 is running, if UE is accessing NR via NTN access, and if UE is unable to acquire SIB19, it is up to UE implementation to decide whether to bar the cell and reselect to another cell.

END OF CHANGES