3GPP TSG RAN WG2 #120	R2-2212842

Toulouse, France, November 14th – 18th, 2022

Source: 	MediaTek Inc.

Title: 	Recommendations for network energy saving techniques

Agenda item:	8.3.6

Document for:	Discussion and Decision

Introduction

In this contribution, evalautions are conducted on the selected NWES schemes for different cell load conditions, including low load (0%-15% RU) and Light/Medium load (15% - 30%/30% - 50% RU). Recommendations are then provided based on the evaluated NW energy saving gains.

From the useful power saving schemes identified, there also reveals the need of efficient adaptation framework(s) to realize the benefit of network energy saving over different network load conditions, by considering UE-group/cell-wise adaptation and practical UE support, for technical direction considerations and decision in RAN2.

Network Energy Saving Techniques for Low Load Condition

In this section, we evaluate and reocommend NWES schemes for low load condition (0% - 15% RU).

Alignment of DRX offsets

- When the traffic load is low, e.g., mainly voice activity, it can be beneficial to aggregate BS transmission time intervals. Since UE expects data activity only within DRX on-durations, such BS transmission time aggregation implies alignment of UE DRX offsets, in a cell-specific or group-specific manner as illustrated in Figure 1. Note that, since DRX cycle, on-duration timer, and inactivity-timer settings are typically optimized per service QoS requirement, it will be more reasonable that NWES adaptation only adapts UE DRX offset value.

Figure 1: Illustration of aggregated BS activity by aligning DRX offsets across UEs in a cell

- To justify the benefit based on the latest agreed network power consumption model [5], the following evaluation assumptions are considered:

- Scenario: Urban Macro (ISD: 500m) in FR1 of 30kHz; single carrier case

  - Baseline: Random DRX offset per UE (5 ms granularity)

  - Improved: Aligned DRX offset to 0 for UEs of a group or a cell

- Traffic: VoIP with DRX (cycle, on-duration, inactivity timer) = (40, 4, 10) ms

  - Note: VoIP is considered to simulate low network loading condition while user activities are still frequent

- RU: Low load (0% - 15%)

- In the table below, there show the network energy saving gains for Cat 1 and Cat 2 types of BS’s. Accordingly, we also have the following:

Observation 1: For the case of low network load (0% - 15%), aligning UE DRX offsets for aggregated BS activity can achieve good NW energy saving gain, i.e., 30% for Cat 1 BS and 14% for Cat 2 BS, even with frequent UE data activities (e.g. VoIP with 40ms DRX cycle)

| NW load condition | Technique | NW energy saving gain | Evaluation Settings |
| --- | --- | --- | --- |
| Low (0% - 15% RU) | Alignment of DRX offsets | Cat1: 30% Cat2: 14% | Traffic: VoIP Baseline: Random DRX offsets (5 ms granularity) Improved: Cell-wise aligned DRX offset (= 0); no DRX cycle change |

Proposal 1: Aligning UE DRX offsets in a cell-specific manner is recommended for network energy saving in low load condition (0% - 15% RU).

Wake Up of BS triggered by UE UL Signal

- When network loading is low, it is possible for network to make some BS’s sleep to achieve energy saving. On the other hand, this can induce performance impact to UE data service, e.g., causing significant delay due to repeated retransmissions or link failure. In this regard, allowing UE to “wake up” gNB when a pre-defined quality metric (e.g. packet latency) is violated will be useful to balance network energy saving and user experience.

- To justify the benefit based on the latest agreed network power consumption model [5], the following evaluation assumptions are considered:

- Scenario: Urban Macro (ISD: 500m) in FR1 of 30kHz; single carrier case

  - Baseline: All BS’s are active

  - Improved: Keep 3 out of 7 BS’s active

  - Avoided: Keep only 1 out of 7 BS’s active

- Traffic: VoIP with DRX (cycle, on-duration, inactivity timer) = (40, 4, 10) ms

  - Note: VoIP is considered to simulate low network loading condition while user activities are still frequent

- RU: Low load (0% - 15%)

- In the following, there show the network energy saving gains for Cat 1 and Cat 2 types of BS’s. Accordingly, we also have the following:

Observation 2: For the case of low network load (0% - 15%), disabling BS with BS wake-up triggered by UE when a pre-configured metric is violated can achieve significant NW energy saving gain, i.e., 49% for Cat 1 BS and 52% for Cat 2 BS, while keeping good user experience.

| NW load condition | Technique | NW energy saving gain | Evaluation Settings |
| --- | --- | --- | --- |
| Low (0% - 15% RU) | Wake up of gNB triggered by UE wake up signal | Cat1: 49.3% Cat2: 51.9% | Traffic: VoIP Baseline: FR1 Urban Macro scenario (ISD: 500 m) Improved: NW turn off 4 BS’s out of 7 BS’s (Avoided: UE wakes up BS’s when NW turns off 6 BSs and 67% latency increment 67% detected) |

Proposal 2: Wake Up of BS triggered by UE UL Signal is recommended for balancing network energy saving and user experience in low load condition (0% - 15% RU).

Network Energy Saving Techniques for Light Load or Medium Load Condition

In this section, we evaluate and reocommend NWES schemes for low load condition (15% - 30% RU) or medium load condition (30% - 50% RU)

SSB/SIB1-less SCell

When a cell is used as a capacity booster rather than providing coverage, the cell could be defined as a secondary cell (SCell), and a gNB may turn off SSB or SIB1 transmission on the SCell for network energy savings.

For evaluating the potential benefits, the following settings are considered:

- Scenario: Urban Macro (ISD: 500m) in FR1 of 30kHz with PCell and one SCell; multi-carrier case (inter-band CA)

  - Baseline: 100M 2CCs with SSB and SIB1 in both CCs

  - Improved: 100M 2CCs with SSB and SIB1 disabled in Scell

- SSB: 20 RBs, 4 symbols, 2 SSBs per slot, and 4 consecutive slots at periodicity of 20 ms

- SIB1: 48 RBs, 8 consecutive slots at periodicity of 20 ms, multiplexing pattern 1 with SSB

- Traffic: Video traffic with DRX (cycle, on-duration, inactivity timer) = (160, 8, 100) ms

  - Note: Video traffic considered since CA is typically for scenarios with higher data activity

- RU: Light load (15% - 30%)

In the following table, there show the network energy saving gains for Cat 1 and Cat 2 types of BS’s, and we have

Observation 3: For CA use cases with higher data activity, disabling SSB and/or SIB1 for SCell achieves very limited energy saving gains, i.e., <8% for Cat 1 BS and < 1% for Cat 2 BS.

| NW load condition | Technique | NW energy saving gain | Evaluation Settings |
| --- | --- | --- | --- |
| Light (15% - 30% RU) | SSB/SIB1-less SCell | Cat1: 8% Cat2: 1% | Traffic: Video Baseline: 100M 2CCs with SSB and SIB1 in both CCs Improved: 100M 2CCs with SSB and SIB1 disabled in Scell |

Proposal 3: Disabling SSB and/or SIB1 for SCell is NOT pursued for network energy saving.

BWP-Based Joint Adaptation on PDSCH Power and Spatial Elements

When network load is light (15% - 30%) or medium (30% - 50%), reducing BS active time may not be practical due to the impact to data latency. In this regard, reducing PDSCH power and the number of TxRU while keeping data transmission can be utilized to achieve better trade-off in network energy saving and data latency. To characterize the trade-off, the following evaluation settings are considered:

- Scenario: Urban Macro (ISD: 500m) in FR1 of 30kHz; single carrier setting; same PSD per TxRU

  - Baseline: PDSCH offset = 0 dB and 64 TXRU

  - Improved: PDSCH offset = -3 dB and 32 TXRU

  - Note: Adaptation by BWP switch or reconfiguration

- Traffic: Video traffic with DRX (cycle, on-duration, inactivity timer) = (160, 8, 100) ms

  - Note: Video traffic with more UEs utilized to simulate higher network load conditions

- RU: Light load (15% - 30%) and medium load (30% - 50%)

In the following table, there show the comparison of BS power consumption and data latency with reduced #TxRU for light load and medium load cases, respectively. Accordingly, one can check:

Observation 4: For the NW scenario with light load (15% - 30%) to medium load, reducing PDSCH power by 3dB and #TxRU from 64 to 32 can bring 16% to 27% and 17% to 30% network energy saving gain, respectively, for Cat 1 BS and Cat 2 BS.

| NW load condition | Technique | NW energy saving gain | Evaluation Settings |
| --- | --- | --- | --- |
| Light – Medium (15% - 50% RU) | BWP-based joint adaptation on PDSCH power and spatial elements | Cat1: 16% - 27% Cat2: 17% - 30% | Traffic: Video Baseline: PDSCH offset = 0 dB and 64 TXRU Improved: PDSCH offset = -3 dB and 32 TXRU Note: Adaptation by BWP switch or reconfiguration |

Proposal 4: BWP-based joint adaptation on PDSCH power offset and spatial elements is recommended for network energy saving.

Furthermore, BWP adaptation framework has been developed in Rel-15 and widely supported by UEs. Since BWP adaptation trigger is UE specific, there will be concern on the signaling overhead and the corresponding BS power consumption that can reduce the benefit of network energy saving. By the above the following proposals are suggested:

Proposal 5: Enhancements to enable UE group-common or cell-specific BWP configuration and /or switching is recommended for network energy saving.

Conclusion

By the above evaluations and discussion, the following table summarizes our recommendations for NWES schemes for further specification in work phase:

| NW load condition | Technique | NW energy saving gain | Evaluation Settings |
| --- | --- | --- | --- |
| Recommended for NWES work phase |  |  |  |
| Low (0% - 15% RU) | Wake up of gNB triggered by UE wake up signal | Cat1: 49.3% Cat2: 51.9% | Traffic: VoIP Baseline: FR1 Urban Macro scenario (ISD: 500 m) Improved: NW turn off 4 BS’s out of 7 BS’s; UE wakes up BS’s when NW turns off 6 BSs and 67% latency increment 67% detected |
| Low (0% - 15% RU) | Adaptation of DTX/DRX | Cat1: 30% Cat2: 14% | Traffic: VoIP Baseline: Random DRX offsets (5 ms granularity) Improved: Cell-wise aligned DRX offset (= 0); no DRX cycle change |
| Light – Medium (15% - 50% RU) | BWP-based joint adaptation on PDSCH power and spatial elements | Cat1: 16% - 27% Cat2: 17% - 30% | Traffic: Video Baseline: PDSCH offset = 0 dB and 64 TXRU Improved: PDSCH offset = -3 dB and 32 TXRU - Adaptation by BWP switch or reconfiguration |
| Not recommended for NWES work phase |  |  |  |
| Light (15% - 30% RU) | SSB/SIB1-less SCell | Cat1: 8% Cat2: 1% | Traffic: Video Baseline: 100M 2CCs with SSB and SIB1 in both CCs Improved: 100M 2CCs with SSB and SIB1 disabled in Scell |

Regarding the adaptation framework, the following is also suggested:

Proposal 5: Enhancements to enable UE group-common or cell-specific BWP configuration and /or switching is recommended for network energy saving.

Reference

RP-213554, 	“New SI: Study on network energy savings for NR”, RAN#94-e, Rapporteur: Huawei

RP-221443,	“SID revision: Study on network energy savings for NR”, RAN#96-e, Rapporteur: Huawei

RP-221442,	“Status report for WI: Study on network energy savings for NR”, RAN#96-e, Rapporteur: Huawei

RP-222546,	“Status report for SI Study on network energy savings for NR”, RAN#97-e, Rapporteur: Huawei

“Draft minimutes RAN1#110-bis-e”, online available @ https://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_110b-e/Report