3GPP TSG-RAN WG2 #120	R2-2212885

Toulouse, France, 2022-11-14 - 2022-11-18

Agenda Item:	8.5.4.1

Source:	Ericsson

Title:	Discussion on BSR enhancements

Document for:	Discussion, Decision

# 1	Introduction

In RAN plenary 94-e [1], a new SI for Rel-18 on extended reality (XR) was agreed, with objectives covering 1) XR-awareness in RAN, 2) XR-specific power saving, and 3) XR-specific capacity improvements.

In this contribution, we discuss possible study topics related to the third area, following the objectives in [1]:

“Objectives on XR-specific capacity improvements (RAN1, RAN2):

- Study mechanisms that provide more efficient resource allocation and scheduling for XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc…). Focus is on the following mechanisms:

  - SPS and CG enhancements;

  - Dynamic scheduling / grant enhancements.”

In RAN2#119-bis-e [2] the following agreement to further discuss solutions from the TR38.838 was made.

- 1: introduce new BS table(s) to reduce the quantisation errors (e.g. for high bit rates). FFS how new BSR tables are created and how they impact BSR formats (can be discussed in WI phase).

- Delay information consists of at least “remaining time”.

- 2: RAN2 considers a delay information is useful for XR. FFS if dynamic reporting from UE to network (e.g. via BSR) is needed, or whether PSDB is sufficient. If we have delay information, it needs to distinguish how much data is buffered for which delay value. Stage-3 details (e.g. what’s contained, how the triggering is done) can be discussed in the WI phase.

- If we have delay information reporting, RAN2 aims to define how the UE determines the “remaining time” in the delay information

In this paper we are focusing on the BSR enhancements related to Dynamic Scheduling.

# 2	Discussion on BSR enhancements

Capacity is an important area for XR. It has been established that few XR users will consume most of the network capacity. Thus, introducing mechanisms to stretch that capacity for other XR users and, even more importantly, to protect other services such as eMBB is essential.

## 2.1	Design of BS tables

The current BSR format reports an index which quantizes the buffer size. The index indicates that the UE buffer size in within two values, a maximum and a minimum value. Currently, the relationship between buffer sizes and indexes is provided in two distinct tables, one for the short BSR and a second table for the long BSR. The higher the index, the larger the quantification error – exponentially as shown in Figure 1 (i.e. the larger the difference between the minimum and maximum value the index represents).

Figure 1 Buffer sizes mapped to each index for the short BSR table.

3GPP has now agreed to introduce new BS table(s) to address the negative effect current tables have in XR application. The question is how to design these tables.

There are two main aspects to consider when designing the new BS tables:

- XR traffic is characterized by large and variable PDU set sizes.

- The size and variance of the PDU set sizes will depend on the encoder and other application algorithms; thus:

  - The size and variance of the PDU set may vary within one XR application

  - Different XR applications will show different PDU set sizes and variances

Taking these design aspects into account, it can be concluded that it is not possible to find one or two fixed BS tables which fit all XR applications. As shown in Figure 2, providing a precise buffer status report has the potential to improve the network capacity. Thus, to truly address the current problem, new BS tables need to be tailored to the traffic characteristics of each XR application.

Figure 2 Capacity improvements when using a BSR with a more precise in buffer size table.

There are mainly two options to design the BS tables:

- New pre-defined (fixed value) tables, or

- New BS tables based on NW configuration.

As argued above, it is not possible to build few tables having fixed values which can fit all XR applications, all kind of values, variations, ranges, and accuracies. Thus, the first approach, while it may seem simple in principle, it will be a technically bad choice for XR applications, which may ultimately result in no capacity enhancements.

- Pre-defined tables are poor solutions to handle the multiple variations of XR applications

On the other hand, creating specific tables using a configuration provided via RRC is a more effective and adaptative way to design BS tables. The configuration can be adjusted to the specific service/application and its traffic characteristics. RRC could, for example, include in each new BS table few basic parameters such as the minimum value, the maximum value, and the number of steps. With only few parameters, the tables can be constructed easily and customized for each specific XR application.

New BS tables are configurable and built based on NW configuration.

## 2.2	Delay information in BSR

In general, buffer or delay information are related to a PDU set. The buffer size could belong to 1 or more PDU sets, for example. Indicating the buffer size for each of the PDU sets potentially with its delay information is more useful for the scheduler as it allows the scheduler to decide on the resources based on the packets which need to be transmitted within the PDB considering the provided delay information.

Buffer status information and delay information are provided per PDU set

It has been demonstrated that more accurate buffer information can increase capacity even using a simple round robin scheduler. However, more gains can be achieved with more advanced scheduling algorithms which could, for example, use time information. Figure 3 shows that another 10% capacity gains are achievable when delay information for the PDU Set is known by the network. In this example, delay information is the time left until exceeding the PDB for the application packets in the UE buffer, and a delay-aware scheduling algorithm (Least Slack Time) is used. Delay information can be defined in multiple ways; however, it should be defined in a way that is useful for the scheduler. The PDB of the PDU set is not enough since it only points out the “latency left” when the PDU Set is first generated, and it does not take into account the queued/buffered time. Further it cannot be assumed that PDU sets are always generated in the same instant at the UE. Different devices will have different processing capabilities. Thus, it cannot be assumed all encoders will work the same and all devices will have the same capabilities. Applications contribute to the total jitter and IP packets within a PDU Set may arrive at the UE buffer at different time instances.

Delay information should, therefore, be the “latency left” i.e. the PDB of the PDU set minus the queued/buffered time, or it can also be defined as the “queued/buffered time”. This information is the most relevant timing information for the NW as it can be used directly in the scheduler to take next scheduling decisions for the PDU set.

Timing information is defined as the “latency left”, i.e. PDB of the PDU Set minus queued/buffered time, or the queued/buffered time of the PDU set.

Figure 3. Capacity improvement when delay information is included in BSR and used by a Least Slack Time scheduler. Note that BSR granularity enhancements are not considered in these results.

Similar as for the buffer status, delay information would need to be quantified. A similar approach as the BS tables can be followed here. Considering the packet delay budgets for XR as well as the TDD patters, only few buckets may be needed. Yet, as for the BS tables, these delay information tables can be built based on RRC configuration. As an example, the NW could provide the steps, 4 steps, and the min and max value, min=0 max=15. A table as shown below could be built by the UE.

This way to build tables allows more flexibility for the network and it allows to be adapted to the specific requirements of each XR application and their corresponding flows.

New timing information tables are configurable and built based on NW configuration.

## 2.3	Triggering of BSR

Another aspect to discuss is the inter-operation between legacy BSRs and the new BSR reporting. While these are aspects to be discussed during the Work Item phase, we think that the inter-operation between these 2 BSRs is simple. If the UE is configured with additional tables, the UE always would use the table which will provide a most accurate buffer status information and uses the associated BSR format.

At this stage, if it not foreseen that new triggering mechanisms are needed. The same mechanisms as for legacy BSR triggering are considered sufficient to meet the QoS requirements as notifying NW about buffer changes is the most critical when data arrives in an empty buffer, i.e. when there are no ongoing data transmissions. XR traffic with high periodicity of data may still benefit from frequent BSR transmissions but this can be accommodated by configuration of frequent periodic BSR. Nonetheless, this topic can also be reviewed during the Stage 3 given a new BSR is introduced.

Current BSR triggering conditions are the baseline conditions for any new BSR. Further conditions can be discussed in Stage 3.

## 2.4	New BSR formats

In RAN2#119-bis-e concerns about what BSR formats should be used were raised. Some companies claimed that only long BSR formats would be relevant as the tables would need to be large. However this is not true since a dynamically created table specifically constructed for the traffic in hand could potentially be sufficient even with a short table. Avoiding BSR overhead by utilizing short BSR may still be relevant, especially since frequent BSR transmission may be beneficial (see 2.3 for triggering discussions). Further on as is explained in this paper there are other information, e.g. delay information and PDU Set information, that may need to be fit in the BSR to cope with the needs of XR. Having buffer information for a whole logical channel group is not efficient any longer. However, having buffer and delay information per PDU set within a logical channel ID could be much more effective from a capacity point of view, so the NW can allocate resources accurately when taking into account the delay for the given PDU set. This would require new BSR formats to be introduced which should be decoupled to existing formats, e.g. short and long BSR formats.

New BSR format(s) are created.

Introduce in the TR the TP found in the annex.

# 3 Conclusion

In the previous sections we made the following observations:

Observation 1	Pre-defined tables are poor solutions to handle the multiple variations of XR applications

Based on the discussion in the previous sections we propose the following:

Proposal 1	New BS tables are configurable and built based on NW configuration.

Proposal 2	Buffer status information and delay information are provided per PDU set

Proposal 3	Timing information is defined as the “latency left”, i.e. PDB of the PDU Set minus queued/buffered time, or the queued/buffered time of the PDU set.

Proposal 4	New timing information tables are configurable and built based on NW configuration.

Proposal 5	Current BSR triggering conditions are the baseline conditions for any new BSR. Further conditions can be discussed in Stage 3.

Proposal 6	New BSR format(s) are created.

Proposal 7	Introduce in the TR the TP found in the annex.

# 4 References

- RP-220285, “Study on XR Enhancements for NR”

- “Report of 3GPP TSG RAN WG2 meeting #119bis-e, Online”, 10 – 19 October, 2022.

# Annex

## 5.2.2.x 	BSR enhancements

XR is characterized by large and variable PDU set sizes. When a PDU set arrives to the UE buffer, the UE will typically generate a BSR. This BSR reports an index which quantizes the buffer size. The relationship between buffer sizes and indexes is provided in two distinct tables, one for the short BSR and a second table for the long BSR. The index only indicates that the UE buffer size in within two values, a maximum and a minimum value. The higher the index, the larger the quantification error (i.e. the larger the difference between the minimum and maximum value the index represents).

New buffer size (BS) tables would be needed to address the inaccuracy when the indexes increase. Either new hardcoded tables are introduced in the standard, or tables are dynamically configured and built. With the first approach, many tables need to be introduced to be able to cope with different ranges and different accuracies. Thus, pre-defining tables will be a sub-optimal and not future proof solution. On the other hand, creating tables dynamically, adapted to the specific service/application and its traffic characteristics, is a more forward and efficient and simpler method. The NW could provide, for example, a minimum value, a maximum value, and the number of steps. These values would depend on what the network has learned about the traffic characteristics and/or the information provided by the application. With only few parameters, the tables can be constructed easily.

In addition to buffer information, delay information per each PDU set, e.g., the time left until exceeding the PDB for the application packets in the UE buffer, can be used in a delay-aware scheduling algorithm to better prioritize users. In this way, network resources can be used more effectively with another 10% gain in XR capacity.

New BSR formats would be needed to convey, at least, this new information i.e. buffer status and delay information per PDU set. The baseline triggers can be used for these new formats, though, new triggers specific for XR could be discussed.

RAN2 recommends: introducing new dynamic BS tables; including BS and delay information per PDU set; legacy triggering mechanisms are taken as baseline.