3GPP TSG-RAN WG2 #120	R2-2212887

Toulouse, France, 2022-11-14 - 2022-11-18

Agenda Item:	8.5.2.3

Source:	Ericsson

Title:	Discussion on PDU Discard

Document for:	Discussion, Decision

# 1	Introduction

In [1], a new Study Item identifying three main areas of study, i.e., XR-Awareness in RAN, XR specific power savings and XR specific capacity improvements, was agreed.

“Objectives on XR-awareness in RAN (RAN2):

- Study and identify the XR traffic (both UL and DL) characteristics, QoS metrics, and application layer attributes beneficial for the gNB to be aware of.

- Study how the above information aids XR-specific traffic handling.”

In RAN2#119-bis-e the following agreements related to study topic was made.

- 1. For UE transmitter, the PDCP discard should be performed per PDU set basis.

- 2. For UE transmitter, The PDCP discard is managed per SDU for PDU set, the PDCP entity discards all PDCP SDUs associated with the PDU set.

In this contribution we share our view on PDU Discard.

# 2	Discussion PDU Discard

## 2.1	PDU Set dropping

The view of SA4 is that network should aim to deliver all packets since any packet losses will impact the user experience somehow [2]. Furthermore, applications behave differently and use encoders with different types of dependency and thus it is impossible to know exactly how the impact on QoE would be in each specific case of losses.

Even if packet dropping in theory should make it faster to transmit the following packets it is not certain that a scheme dropping packets from one user will increase the number of successfully delivered packets for that user. If later arriving packets are not dependent on the transmission of earlier packets (i.e. independent PDU Sets) then those later packets will likely have a much larger size and it will be more problematic to meet the requirements, i.e. if there was a struggle to transmit the smaller packets, to the point that they were lost, there will be even larger struggle with the larger packets under the same radio conditions. If radio conditions improve significantly, so that meeting the requirements of a large frame is possible, then transmitting the small frames will create a low overhead and the impact on the larger frame will be insignificant. In conclusion dropping packets will generally not improve the QoE of the user itself, but it can still help freeing up resources for other users. When looking at the metric of satisfied users dropping is in general only beneficial if a user is anyway already unsatisfied. If a user is not satisfied but requires a lot of resources it will impact all other users in the cell negatively since there is a lack of available resources and dropping packets from that user can then be a possible solution to increase the overall satisfaction in the cell.

Dropping solutions should be used to improve the performance of users not performing the dropping

In a congested network scenario, the scheduler would need to identify the users which could be suitable for dropping. Dropping need to be done on a PDU Set level, as reception of individual IP packets are generally not important and all the IP packets belonging to a PDU set need to be delivered on time (or close to that time) for a good user experience. For this the scheduler may utilize the PDB of the PDU Sets and the PDU Set size to estimate if the PDU Set can be delivered within its latency requirement and if deemed not to be the case the scheduler may decide to drop the entire PDU Set to free up radio resources. These resources can then be assigned to other users or data transmissions which can still meet the PDB [3].

Two basic mechanisms can be used to trigger PDU set dropping. A first and most common mechanism in 3GPP is to define a timer, “PDU set dropping timer”. After its expiration, some action is taken. In this case, the timer would start when the first IP packet is received in the buffer and, when/if it expires, all IP packets associated to the PDU set would be discarded. This seems to be a very simple mechanism; however, this simplicity may lead to no gains. If the timer expires and only a small fraction of the packets remains to be transmitted, dropping them will not lead to any noticeable capacity increase. In other words, the latter the timer expires, the lower the capacity gains are obtained. If dropping is needed, the decision should be taken as early as possible, preferably even before the transmission starts.

A second aspect which has also been brought up in 3GPP is that current PDCP discard timer is sufficient since all the UL IP packets in a PDU set will arrive at the UE buffer at the same instance. This cannot be assumed since, similarly as in 3GPP with all device types/categories and capabilities, all devices will not have powerful processing capabilities and high-end encoders. This wide spectrum will also result in different performances and behaviour. Thus, current PDCP discard timer cannot be used as is today.

Current PDCP discard timer for UL cannot be used.

The later the dropping decision is taken, the lower the capacity improvements are.

Another mechanism to decide PDU set dropping is to evaluate the dropping decision based on the bit rate, the PDU set size, and packet delay budget or latency left (PDB – queued time). The network can evaluate using BSR information and timing indications (if provided, or else estimated), whether it can provide grants to complete the transmission in the indicated time. When that it not the case, the NW can indicate to the UE to drop the PDU set. It could be argued that the PDU set size, packet delay budget (or latency left), and statistical bit rate can also be known at the UE. Additional configuration parameters could be provided to the UE. Then, the UE could decide when to drop a PDU set. The main drawback from this UE-based of solution is that the UE buffer size would be changing without the NW knowing about it which could cause the network to issue larger grants than needed by the UE. This could negatively affect the network capacity.

Figure 1 shows that the second option is far better than using a PDU set dropping timer. Good and early decisions can provide capacity enhancements of 10%. The green line exemplifies the second mechanism, while the red line exemplifies the timer solution.

With respect to the NG-RAN architecture, the PDCP layer entity is located in the gNB-CU while the RLC entity is located in the gNB-DU. Therefore, there are impacts that affect F1 interface between CU and DU. RAN3 needs to be informed of this impact so that information from the PDCP layer in the CU is passed to the RLC layer in the DU to indicate which RLC SDUs packets need to be delivered in the same time budget or else, dropped.

Figure 1 Capacity improvement when PDU Set dropping mechanisms are used. Two solutions are compared, when dropping based on timers or prediction.

RAN2 should specify mechanism for the UE and signalling for NW to support PDU Set dropping solutions assuming application awareness, e.g. information about the PDU Set size and the PDU Set delay budget

UL PDU Set dropping mechanisms should be designed so that they are either 1) the NW indicates the UE to drop UL PDU sets, or 2) the NW configures the UE with adequate rules and thresholds

## 2.1	FEC based dropping

In RAN2#119-bis-e companies argued that there could be gains with dropping solutions that drop parts of a PDU Set based on that the whole PDU Set is not needed by the receiver to be useful, e.g. because of Forward Error Correction (FEC) based coding. It has been described as if a loss of some PDUs is detected then the rest of the PDUs in the PDU Set could be dropped. However this is misconception as we have already observed in 2.1 that dropping only parts of a PDU Set generally give low gains in capacity. When a user performs poor, e.g. to the point of losing packets, in the majority of cases it can’t successfully deliver within the latency requirements. A poorly performing user will often have much buffered data since new PDU Sets arrive to the buffer before it can finish transmission of earlier arrived data, thus dropping only parts of a PDU Set doesn’t reduce the resource usage and in turn does not improve the network performance. It should also be noted that the whole point of dropping based on detecting a loss in most cases will be too late to react upon, e.g. most of the data will have been transmitted already before detecting the loss and in many cases the deadline for the whole PDU Set may have already passed.

Dropping based on detecting losses in PDU Set transmission will not improve capacity to any meaningful extent

### 2.2 	PDU Set Dependence

In ongoing SA2 FS_XRM and RAN2#119 multiple companies have argued for the need of dependence information related to the PDU Set be conveyed to RAN in multiple ways, e.g., GTP-U and 5QI. In this section we share our general view on this concept.

Existing RAN scheduler already considers various requirements such as bitrate, PDB, PER, priority, ARP etc. It is unclear how adding this PDU Set dependence/importance information to the scheduler will increase the system capacity or give gain in power saving.

The general view seems to be that PDU Set dependence can be used to discard dependent packets/PDU Set’s in certain scenarios, e.g. loss of an I-frame/slice renders the following dependent P frame/slice meaningless to the application layer. However, it can be questioned if such simplified scenarios apply for XR traffic. The LS reply from SA4 [2] to SA2, says “In particular, low latency XR and cloud gaming services such as Split-Rendering or Cloud Gaming typically would not use the traditional coding structure with a fixed Group of-Picture (GOP)” and “…the handling of dependent PDU Sets once a leading PDU Set is lost is not universally defined and depends on the operation of the application”. Since there is no universal way of dependence handling, considering the multiple ways of handling PDU Set dependency in the application and RAN for the sole purpose of discarding PDU Sets it will be very complex to make such solutions work. Further it can also be investigated how such dependence would impact the XR capacity.

The handling of dependent PDU Sets once a leading PDU Set is lost is not universally defined and depends on the operation of the application and likely will create complex solutions

We have done such investigations by introducing the GOP dependence structure in our evaluations. In Figure 2 and 3 we provide simulations results based on the traffic models in TR 38.838. The simulations compares system capacity of a Cloud Gaming (CG) scenario assuming either the baseline traffic model or the GoP model [4] along with several scenarios for different PER and PDB of I and P frame as outlined in [5]. In Figure 2 we show the system capacity when frame dependence is not considered, i.e. when a reference frame is lost the scheduler does not drop the dependent P-frame. Despite not giving a penalty for the dependence in this case the different GoP based scenarios still show a small loss in capacity compared to the baseline traffic model. Shown in Figure 3 is the capacity when all dependent P-frames are marked as lost when a reference I-frame or P-frame is lost. Both figures are strikingly similar which indicate that no further users become unsatisfied, i.e. it is only the users that are already unsatisfied that experience any impactful number of frames losses. Since the goal of the dependence is about omitting to transmit unneeded packets, i.e. discarding the dependent PDU Sets, this can be done in a simpler way as explained in previous section 2.1, i.e. by only looking at how a user performs. If a user is in very bad radio situation, need to transmit large PDU Sets with a tight deadline, then scheduler may deem it impossible to meet the requirements and drop packets in favour of letting other users utilize those resources.

Figure 3 Evaluation for frame dependence. Here, if a frame is missing, then all dependent frames are marked as missing as well. Notice the similarities with the baseline in Figure 1.

Figure 2 Baseline case for frame dependence, i.e., no penalty for missing frames ([PER_I,PER_R, PDB_I, PDB_P])

Introducing frame/PDU Set dependence show no impact on user satisfaction and doesn’t change the selection of which users packets that are subject to dropping

Dropping packets based on dependence or priority is not beneficial.

RAN2 should not consider PDU Set dependence information

Capture the draft TP in annex about PDU discard in TR 38.835

# Annex: Text proposals for TR 38.835

### 5.3.2.X PDU set discarding

When looking at the metric of satisfied users dropping is in general only beneficial if a user is anyway already unsatisfied. If a user is not satisfied but requires a lot of resources it will impact all other users in the cell negatively since there is a lack of available resources and dropping packets from that user can then be a possible solution to increase the overall satisfaction in the cell. Dropping needs to be done on a PDU Set level, as reception of individual IP packets are anyway not important and all the IP packets belonging to a PDU set need to be delivered on time (or close to that time) for a good user experience.

# 3 Conclusion

In the previous sections we made the following observations:

Observation 1	Dropping solutions should be used to improve the performance of users not performing the dropping

Observation 2	Current PDCP discard timer for UL cannot be used.

Observation 3	The later the dropping decision is taken, the lower the capacity improvements are.

Observation 4	Dropping based on detecting losses in PDU Set transmission will not improve capacity to any meaningful extent

Observation 5	The handling of dependent PDU Sets once a leading PDU Set is lost is not universally defined and depends on the operation of the application and likely will create complex solutions

Observation 6	Introducing frame/PDU Set dependence show no impact on user satisfaction and doesn’t change the selection of which users packets that are subject to dropping

Observation 7	Dropping packets based on dependence or priority is not beneficial.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 should specify mechanism for the UE and signalling for NW to support PDU Set dropping solutions assuming application awareness, e.g. information about the PDU Set size and the PDU Set delay budget

Proposal 2	UL PDU Set dropping mechanisms should be designed so that they are either 1) the NW indicates the UE to drop UL PDU sets, or 2) the NW configures the UE with adequate rules and thresholds

Proposal 3	RAN2 should not consider PDU Set dependence information

Proposal 4	Capture the draft TP in annex about PDU discard in TR 38.835

# 4 References

RP-213587, Study on XR Enhancements for NR, Nokia, RAN#94e, December 2021.

S4-220492, “LS Reply on QoS support with PDU Set granularity” SA4 Meeting #118E e-meeting, April 2022

R2-2208676, “Discussion on capacity enhancements for XR”, Ericsson, RAN#119-e, August 2022

3GPP TR 38.838, “Study on XR (Extended Reality) Evaluations for NR (Release 17)”, V17.0.0, Dec. 2021.

R1-2110811, “Performance evaluation results for XR and Cloud Gaming”, Huawei, HiSilicon, November 2021