3GPP TSG-RAN WG2 #120	R2-2212943

Electronic meeting, 2022-11-14 - 2022-11-18

Agenda Item:	7.2.2

Source:	Ericsson

Title:	R17 IoT NTN User Plane issues

Document for:	Discussion, Decision

# 1	Introduction

In this contribution we discuss some remaining issues for the user plane:

- UE-eNB RTT definition and using “subframes” after UE-eNB RTT

- The start of drx-InactivityTimer after UL HARQ RTT Timer expiry for NB-IoT

- Transmissions using PUR

- The HARQ RTT Timer for eMTC

# 2	UE-eNB RTT and Koffset terminology

## 2.1	UE-eNB RTT definition

In 3.1 we have

UE-eNB RTT: For non-terrestrial networks, the sum of the UE's Timing Advance value (see TS 36.211 [7], clause 8.1) and k_Mac in units of subframe, not rounded or truncated toward an integer number of subframes.

Where k_Mac shall refer to an RRC parameter, however, in RRC the name of the parameter is k-Mac:

k-Mac-r17						INTEGER (1..512) 				OPTIONAL,	-- Need OP

Change “k_Mac” in UE-eNB RTT definition to “k-Mac” in the MAC section 3.1.

## 2.2	UE-eNB RTT “subframes”

As the definition of UE-eNB RTT is “in units of subframe”, when UE-eNB RTT is used throughout the MAC spec, putting subframes after “UE-eNB RTT” is a redundant repetition, but often “subframe” is needed for the other part that UE-eNB RTT is added to. Further in 7.7 the addition of subframe(s) in the setting of (UL) HARQ RTT Timer values is not consistent.

In MAC spec, when UE-eNB RTT is used, there is no need to add “subframes” after.

Introduce consistent use of “subframes” when UE-eNB RTT is used in MAC spec.

For P1 and P2, consider the text proposal below.

In 5.1.4:

If the UE is a BL UE or a UE in enhanced coverage:

-	if the random access preamble was transmitted in a non-terrestrial network:

-	RA Response window starts at the subframe that contains the end of the last preamble repetition plus 3 subframes plus UE-eNB RTT and has length ra-ResponseWindowSize for the corresponding enhanced coverage level;

-	else:

-	RA Response window starts at the subframe that contains the end of the last preamble repetition plus three subframes and has length ra-ResponseWindowSize for the corresponding enhanced coverage level.

If the UE is an NB-IoT UE:

-	if the random access preamble was transmitted in a non-terrestrial network:

-	RA Response window starts at the subframe that contains the end of the last preamble repetition plus X subframes plus UE-eNB RTT and has length ra-ResponseWindowSize for the corresponding enhanced coverage level, where value X is determined from Table 5.1.4-1 based on the used preamble format and the number of NPRACH repetitions;

In 5.1.5:

Once Msg3 is transmitted, the MAC entity shall:

-	if the UE is an NB-IoT UE, a BL UE or a UE in enhanced coverage:

-	if Msg3 is transmitted on a non-terrestrial network:

-	if, for EDT, edt-SmallTBS-Enabled is set to TRUE for the corresponding PRACH resource:

-	start mac-ContentionResolutionTimer and restart mac-ContentionResolutionTimer at each HARQ retransmission of the bundle in the subframe corresponding to the last subframe of a PUSCH transmission corresponding to the largest TBS indicated by the UL grant plus UE-eNB RTT.

-	else:

-	start mac-ContentionResolutionTimer and restart mac-ContentionResolutionTimer at each HARQ retransmission of the bundle in the subframe containing the last repetition of the corresponding PUSCH transmission plus UE-eNB RTT.

In 5.4.7.1:

After transmission using PUR, the MAC entity shall monitor PDCCH identified by PUR-RNTI in the PUR response window using timer pur-ResponseWindowTimer:

-	if PUR was transmitted in a non-terrestrial network:

-	the MAC entity shall start pur-ResponseWindowTimer at the subframe that contains the end of the corresponding PUSCH transmission plus 4 subframes plus UE-eNB RTT.

In 7.7:

For BL UEs and UEs in enhanced coverage, when single TB is scheduled by PDCCH the HARQ RTT Timer corresponds to 7 + N subframes plus RTToffset, where N is the used PUCCH repetition factor, where only valid (configured) UL subframes as configured by upper layers in fdd-UplinkSubframeBitmapBR are counted for N. In case of TDD, HARQ RTT Timer corresponds to 3 + k + N subframes plus RTToffset, where k is the interval between the last repetition of downlink transmission and the first repetition of the transmission of associated HARQ feedback, and N is the used PUCCH repetition factor, where only valid UL subframes are counted for N as indicated in clauses 10.1 and 10.2 of TS 36.213 [2].

For BL UEs and UEs in enhanced coverage, when multiple TBs are scheduled by PDCCH and HARQ-ACK bundling is not configured, the HARQ RTT Timer corresponds to 7 + m * N subframes plus RTToffset, where N is the used PUCCH repetition factor and m is the number of scheduled TBs as indicated in PDCCH, where only valid (configured) UL subframes as configured by upper layers in fdd-UplinkSubframeBitmapBR are counted for m * N.

For BL UEs and UEs in enhanced coverage, when multiple TBs are scheduled by PDCCH and HARQ-ACK bundling is configured the HARQ RTT Timer corresponds to 7 + M * N subframes plus RTToffset, where N is the used PUCCH repetition factor and M is the number of TB bundles as specified in clause 7.3 of TS 36.213 [2], where only valid (configured) UL subframes as configured by upper layers in fdd-UplinkSubframeBitmapBR are counted for M * N.

For NB-IoT, when single TB is scheduled by PDCCH or when multiple TBs are scheduled for the interleaved case when HARQ-ACK bundling is configured the HARQ RTT Timer is set to k+3+N subframes plus RTToffset + deltaPDCCH, where k is the interval between the last subframe of the downlink transmission and the first subframe of the associated HARQ feedback transmission and N is the transmission duration in subframes of the associated HARQ feedback, and deltaPDCCH is the interval starting from the subframe following the last subframe of the associated HARQ feedback transmission plus 3 subframes plus RTToffset to the first subframe of the next PDCCH occasion.

For NB-IoT, when multiple TBs are scheduled by PDCCH for the non-interleaved case or for the interleaved case when HARQ-ACK bundling is not configured, the HARQ RTT Timer is set to k+2*N+1 subframes plus RTToffset + deltaPDCCH where k is the interval between the last subframe of the downlink transmission and the first subframe of the first HARQ feedback transmission and N is the transmission duration in subframes of the associated HARQ feedback, and deltaPDCCH is the interval starting from the subframe following the last subframe of the last HARQ feedback transmission plus 1 subframe plus RTToffset to the first subframe of the next PDCCH occasion.

Except for NB-IoT and for HARQ processes scheduled using Short Processing Time and for short TTI, UL HARQ RTT Timer length is set to 4 subframes plus RTToffset for FDD and Frame Structure Type 3, and set to kULHARQRTT plus RTToffset for TDD, where kULHARQRTT equals to the kPHICH value indicated in Table 9.1.2-1 of TS 36.213 [2] if the UE is not configured with upper layer parameter symPUSCH-UpPts for the serving cell, otherwise the kPHICH value is indicated in Table 9.1.2-3.

For NB-IoT, when single TB is scheduled by PDCCH the UL HARQ RTT timer length is set to 4 subframes plus RTToffset + deltaPDCCH, where deltaPDCCH is the interval starting from the subframe following the last subframe of the PUSCH transmission plus 3 subframes plus RTToffset to the first subframe of the next PDCCH occasion.

For NB-IoT, when multiple TBs are scheduled by PDCCH the UL HARQ RTT timer length is set to 1 subframe plus RTToffset + deltaPDCCH, where deltaPDCCH is the interval starting from the subframe following the last subframe of the PUSCH transmission plus 1 subframe plus RTToffset to the first subframe of the next PDCCH occasion.

# 3	DRX for UL NB-IoT transmissions

For NB-IoT, the UE shall start drx-InactivityTimer when UL HARQ RTT Timer expires. However, in section 5.7 there seems to be a small copy-paste mistake in the description for when multiple TBs were scheduled.

Correct the NB-IoT start of drx-InactivityTimer when UL HARQ RTT Timer expires for multiple scheduled TBs, see text proposal below.

In 5.7:

-	if an UL HARQ RTT Timer expires in this subframe:

-	start the drx-ULRetransmissionTimer or drx-ULRetransmissionTimerShortTTI for the corresponding HARQ process.

-	if NB-IoT:

-	if lower layers had indicated multiple TBs were scheduled for the associated expired UL HARQ RTT Timer:

-	start or restart drx-InactivityTimer when all UL HARQ RTT Timer have expired;

-	else:

-	start or restart the drx-InactivityTimer.

# 4	Transmissions using PUR

When PUR is used, the UE may receive PDCCH addressed to the PUR-RNTI, and then restart the response window timer. However, for retransmissions the UE-eNB RTT have not been considered.

Further, when pur-ResponseWindowTimer expires, and the UE has a transmission scheduled through PDCCH addressed to the PUR-RNTI – then the UE shall not consider PUR has failed (like what happens when mac-ContentionResolutionTimer expires).

Correct the PUR retransmission start of PUR response window timer and the expiry of PUR response window timer, see text proposal below.

In 5.4.7.1:

While pur-ResponseWindowTimer is running, the MAC entity shall:

-	if the PDCCH transmission is addressed to the PUR-RNTI and contains an UL grant for a retransmission:

-	if PUR was transmitted in a non-terrestrial network:

-	restart pur-ResponseWindowTimer at the last subframe of a PUSCH transmission corresponding to the retransmission indicated by the UL grant plus 4 subframes plus UE-eNB RTT.

-	else:

-	restart pur-ResponseWindowTimer at the last subframe of a PUSCH transmission corresponding to the retransmission indicated by the UL grant plus 4 subframes.

…

-	if the pur-ResponseWindowTimer expires:

-	if PUR was transmitted in a non-terrestrial network:

-	if no notification of a reception of a PDCCH transmission addressed to the PUR-RNTI containing an UL grant for a retransmission was received after the start of pur-ResponseWindowTimer:

-	indicate to upper layers the transmission using PUR has failed;

-	discard the PUR-RNTI.

-	else:

-	indicate to upper layers the transmission using PUR has failed;

-	discard the PUR-RNTI.

# 5	HARQ RTT Timer for eMTC

During the RAN2#119bis-e meeting the following was agreed:

Agreements online:

- RAN2 agrees to align the behaviour of eMTC HARQ RTT Timer to NR NTN, either by changing the offset added (from RTToffset to Koffset + k-Mac, where Koffset is the UE specific Koffset defined in 36.213 section 4.2 and k-Mac is a RRC configured parameter) or by changing the start of the HARQ RTT timer (to be based on UL feedback transmission timing, as for NR NTN). Come back in next meeting on the exact solution.

Thus, RAN2 shall align the eMTC “HARQ RTT Timer” to how the drx-HARQ-RTT-TimerDl/drx-RetransmissionTimerDL work in NR NTN by considering the two options:

- Changing the offset added to legacy “HARQ RTT Timer” from “RTToffset” to “Koffset + k-Mac” where Koffset is the UE specific Koffset defined in 36.213 section 4.2 and k-Mac is a RRC configured parameter, or

- Change the start of “HARQ RTT Timer” to be based on UL feedback transmission timing

Here we analyse the two options.

For A, we only need to change the length of the HARQ RTT Timer to be legacy “HARQ RTT Timer” plus “Koffset + k-Mac”

For B, the time from when the HARQ feedback is sent in the UL until the time until the first PDCCH occasion (that may be used by the eNB to send an assignment based on the HARQ feedback) will be the same time as between when the UE sends a PUSCH transmission and the first PDCCH occasion (that may contain a new grant based on the received PUSCH transmission), that is the length of the “UL HARQ RTT Timer”.

The option B may be a little complex to model in the MAC spec. We may either

B1) use a method like in NR NTN and update the start the “HARQ RTT Timer” (to be after HARQ feedback is sent in UL) and update the length of the “HARQ RTT Timer” (to be 4 + RTToffset) or we can us

B2) a method similar to NB-IoT where start of “HARQ RTT Timer” is the same as before, but the length of “HARQ RTT Timer” is updated (for example for one TB in NB-IoT: HARQ RTT Timer = k + 3 + N + RTToffset + deltaPDCCH subframes)

We illustrate with a simple example of the A HARQ RTT Timer and B1 HARQ RTT Timer in the figure 1 below.

We assume a 600 km LEO constellation, BL UE, single TB, no repetitions (N=1), propagation RTT=8.5 ms, TA = 4.5 ms, Koffset = TA + 1.5, UL/DL aligned at the reference point (Kmac = 4), and time between PDSCH and HARQ feedback transmission of 4 subframes (denoted B in the figure). For these parameters, the legacy HARQ RTT Timer will be 7 + 1 = 8 subframes. Further, the eMTC “UL HARQ RTT Timer” was at last meeting updated to be 4+UE-eNB RTT, which in this case is 4+TA+Kmac = 4+4.5+4 = 12.5 subframes.

Notation:

Nw Tx		Timing at NW when transmitting the DL

RP DL	Timing of the DL at the reference point (where UL and DL are aligned)

UE Rx	Timing at UE when receiving the DL

UE Tx		Timing at UE when transmitting the UL

RP UL	Timing of the UL at the reference point

Nw Rx	Timing at NW when receiving the UL

C		Legacy time between PDSCH and HARQ feedback transmission, 4 subframes

A0		assignment for HARQ process ID 0

D0		PDSCH transmission for HARQ process ID 0

F0		HARQ feedback transmission for HARQ process ID 0

TA		the timing advance, assumed to be 4.5 ms

Koffset	The UE specific Koffset, assumed to be TA+1.5 = 6 ms

Kmac		The broadcasted Kmac, assumed to be 4 ms

Figure 1: eMTC HARQ RTT Timer options

## 5.1	Spec impact of the options A, B1 and B2

The start of HARQ RTT Timer is specified in MAC spec section 5.7:

-	if the PDCCH indicates a DL transmission or if a DL assignment has been configured for this subframe:

-	if the UE is an NB-IoT UE, a BL UE or a UE in enhanced coverage:

-	if lower layers have indicated scheduling of transmission of multiple TBs:

-	start the HARQ RTT Timers for all HARQ processes corresponding to the scheduled TBs in the subframe containing the last repetition of the PDSCH corresponding to the last scheduled TB;

-	else:

-	start the HARQ RTT Timer for the corresponding HARQ process in the subframe containing the last repetition of the corresponding PDSCH reception;

-	else:

-	start the HARQ RTT Timer for the corresponding HARQ process;

The length of the “HARQ RTT Timer” for eMTC is specified in section 7.7 of the MAC spec:

For BL UEs and UEs in enhanced coverage, when single TB is scheduled by PDCCH the HARQ RTT Timer corresponds to 7 + N + RTToffset, where N is the used PUCCH repetition factor, where only valid (configured) UL subframes as configured by upper layers in fdd-UplinkSubframeBitmapBR are counted for N. In case of TDD, HARQ RTT Timer corresponds to 3 + k + N + RTToffset, where k is the interval between the last repetition of downlink transmission and the first repetition of the transmission of associated HARQ feedback, and N is the used PUCCH repetition factor, where only valid UL subframes are counted for N as indicated in clauses 10.1 and 10.2 of TS 36.213 [2].

For BL UEs and UEs in enhanced coverage, when multiple TBs are scheduled by PDCCH and HARQ-ACK bundling is not configured, the HARQ RTT Timer corresponds to 7 + m * N + RTToffset, where N is the used PUCCH repetition factor and m is the number of scheduled TBs as indicated in PDCCH, where only valid (configured) UL subframes as configured by upper layers in fdd-UplinkSubframeBitmapBR are counted for m * N.

For BL UEs and UEs in enhanced coverage, when multiple TBs are scheduled by PDCCH and HARQ-ACK bundling is configured the HARQ RTT Timer corresponds to 7 + M * N + RTToffset, where N is the used PUCCH repetition factor and M is the number of TB bundles as specified in clause 7.3 of TS 36.213 [2], where only valid (configured) UL subframes as configured by upper layers in fdd-UplinkSubframeBitmapBR are counted for M * N.

### 5.1.1	A: HARQ RTT Timer = Koffset + k-Mac

Option A is straight forward to implement in the spec

- In 7.7 RTToffset is exchanged for a new offset in HARQ RTT Timer for eMTC, for example DLoffset

- In 7.7 DLoffset is defined similar as RTToffset in a NOTE at the end:

  - “NOTE: DLoffset = 0 in terrestrial networks and DLoffset = Koffset + k-Mac where Koffset is the UE specific Koffset defined in TS 36.213 section 4.2 in Non-terrestrial networks”

For example:

For BL UEs and UEs in enhanced coverage, when single TB is scheduled by PDCCH the HARQ RTT Timer corresponds to 7 + N + DLoffset, where N is the used PUCCH repetition factor, where only valid (configured) UL subframes as configured by upper layers in fdd-UplinkSubframeBitmapBR are counted for N. In case of TDD, HARQ RTT Timer corresponds to 3 + k + N + RTToffset, where k is the interval between the last repetition of downlink transmission and the first repetition of the transmission of associated HARQ feedback, and N is the used PUCCH repetition factor, where only valid UL subframes are counted for N as indicated in clauses 10.1 and 10.2 of TS 36.213 [2].

For BL UEs and UEs in enhanced coverage, when multiple TBs are scheduled by PDCCH and HARQ-ACK bundling is not configured, the HARQ RTT Timer corresponds to 7 + m * N + DLoffset, where N is the used PUCCH repetition factor and m is the number of scheduled TBs as indicated in PDCCH, where only valid (configured) UL subframes as configured by upper layers in fdd-UplinkSubframeBitmapBR are counted for m * N.

For BL UEs and UEs in enhanced coverage, when multiple TBs are scheduled by PDCCH and HARQ-ACK bundling is configured the HARQ RTT Timer corresponds to 7 + M * N + DLoffset, where N is the used PUCCH repetition factor and M is the number of TB bundles as specified in clause 7.3 of TS 36.213 [2], where only valid (configured) UL subframes as configured by upper layers in fdd-UplinkSubframeBitmapBR are counted for M * N.

…

NOTE 1:	RTToffset = 0 in terrestrial networks and RTToffset = UE-eNB RTT in Non-terrestrial networks.

NOTE 2:	DLoffset = 0 in terrestrial networks and DLoffset = Koffset + k-Mac in Non-terrestrial networks where Koffset is the UE specific Koffset defined in TS 36.213 [2] section 4.2.

### 5.1.2	B1: Start HARQ RTT Timer based on HARQ feedback

B1) use a method like in NR NTN and update the start the “HARQ RTT Timer” (to be after HARQ feedback is sent in UL) and update the length of the “HARQ RTT Timer” (to be 4 + RTToffset)

For B1 we need change in 5.7 and 7.7:

- In 5.7 change the start of HARQ RTT Timer  or eMTC to be after HARQ feedback transmission

- In 7.7 change length of HARQ RTT Timer for eMTC to be 4 + RTToffset

For example, in 5.7:

-	if the PDCCH indicates a DL transmission or if a DL assignment has been configured for this subframe:

-	if the UE is an NB-IoT UE, a BL UE or a UE in enhanced coverage:

-	if lower layers have indicated scheduling of transmission of multiple TBs:

-	if the UE is a BL UE or a UE in enhanced coverage and this is a Non-terrestrial network:

-	start the HARQ RTT Timer for all HARQ processes corresponding to the scheduled TBs in the subframe containing the last repetition of the DL HARQ feedback corresponding to the last scheduled TB;

-	else:

-	start the HARQ RTT Timer for all HARQ processes corresponding to the scheduled TBs in the subframe containing the last repetition of the PDSCH corresponding to the last scheduled TB;

-	else:

-	if the UE is a BL UE or a UE in enhanced coverage and this is a Non-terrestrial network:

-	start the HARQ RTT Timer for the corresponding HARQ process in the subframe containing the last repetition of the corresponding DL HARQ feedback;

-	else:

-	start the HARQ RTT Timer for the corresponding HARQ process in the subframe containing the last repetition of the corresponding PDSCH reception;

-	else:

-	start the HARQ RTT Timer for the corresponding HARQ process;

And in 7.7:

For BL UEs and UEs in enhanced coverage, when single TB is scheduled by PDCCH the HARQ RTT Timer corresponds to:

-	if this is a Non-terrestrial network:

-	4 + RTToffset;

-	else:

-	7 + N;

where N is the used PUCCH repetition factor, where only valid (configured) UL subframes as configured by upper layers in fdd-UplinkSubframeBitmapBR are counted for N. In case of TDD, HARQ RTT Timer corresponds to

-	if this is a Non-terrestrial network:

-	4 + RTToffset;

-	else:

-	3 + k + N;

where k is the interval between the last repetition of downlink transmission and the first repetition of the transmission of associated HARQ feedback, and N is the used PUCCH repetition factor, where only valid UL subframes are counted for N as indicated in clauses 10.1 and 10.2 of TS 36.213 [2].

For BL UEs and UEs in enhanced coverage, when multiple TBs are scheduled by PDCCH and HARQ-ACK bundling is not configured, the HARQ RTT Timer corresponds to

-	if this is a Non-terrestrial network:

-	4 + RTToffset;

-	else:

-	7 + m * N;

where N is the used PUCCH repetition factor and m is the number of scheduled TBs as indicated in PDCCH, where only valid (configured) UL subframes as configured by upper layers in fdd-UplinkSubframeBitmapBR are counted for m * N.

For BL UEs and UEs in enhanced coverage, when multiple TBs are scheduled by PDCCH and HARQ-ACK bundling is configured the HARQ RTT Timer corresponds to

-	if this is a Non-terrestrial network:

-	4 + RTToffset;

-	else:

-	7 + M * N;

where N is the used PUCCH repetition factor and M is the number of TB bundles as specified in clause 7.3 of TS 36.213 [2], where only valid (configured) UL subframes as configured by upper layers in fdd-UplinkSubframeBitmapBR are counted for M * N.

### 5.1.3	B2: Set HARQ RTT Timer based on HARQ feedback

Here we look at spec impact from B2:

B2) use a method similar to NB-IoT where start of “HARQ RTT Timer” is the same as before, but the length of “HARQ RTT Timer” is updated (for example for one TB in NB-IoT: HARQ RTT Timer = k + 3 + N + RTToffset + deltaPDCCH subframes)

The 7 that occurs in the formulas has three parts 1) the last subframe of PDSCH reception (1 subframe), 2) the processing time in the UE after PDSCH reception until HARQ feedback transmission (3 subframes), and 3) eNB processing time between reception of HARQ feedback until new assignment transmission on MPDCCH (3 subframes)

If we introduce k as “the interval between the last repetition of downlink transmission and the first repetition of the transmission of associated HARQ feedback”, then k covers the part 1) and 2) but not part 3). Therefore, we need to replace the 7 with k + 3.

- In 7.7 change the length of HARQ RTT Timer for eMTC to be

For BL UEs and UEs in enhanced coverage, when single TB is scheduled by PDCCH the HARQ RTT Timer corresponds to 3 + k + N + RTToffset, where k is the interval between the last repetition of downlink transmission and the first repetition of the transmission of associated HARQ feedback, and N is the used PUCCH repetition factor, where only valid (configured) UL subframes as configured by upper layers in fdd-UplinkSubframeBitmapBR are counted for N. In case of TDD, HARQ RTT Timer corresponds to 3 + k + N + RTToffset, where k is the interval between the last repetition of downlink transmission and the first repetition of the transmission of associated HARQ feedback, and N is the used PUCCH repetition factor, where only valid UL subframes are counted for N as indicated in clauses 10.1 and 10.2 of TS 36.213 [2].

For BL UEs and UEs in enhanced coverage, when multiple TBs are scheduled by PDCCH and HARQ-ACK bundling is not configured, the HARQ RTT Timer corresponds to 3 + k + m * N + RTToffset, where k is the interval between the last repetition of downlink transmission and the first repetition of the transmission of associated HARQ feedback, and N is the used PUCCH repetition factor and m is the number of scheduled TBs as indicated in PDCCH, where only valid (configured) UL subframes as configured by upper layers in fdd-UplinkSubframeBitmapBR are counted for m * N.

For BL UEs and UEs in enhanced coverage, when multiple TBs are scheduled by PDCCH and HARQ-ACK bundling is configured the HARQ RTT Timer corresponds to 3 + k + M * N + RTToffset, where k is the interval between the last repetition of downlink transmission and the first repetition of the transmission of associated HARQ feedback, and N is the used PUCCH repetition factor and M is the number of TB bundles as specified in clause 7.3 of TS 36.213 [2], where only valid (configured) UL subframes as configured by upper layers in fdd-UplinkSubframeBitmapBR are counted for M * N.

But of course, B2 and A will result in the same HARQ RTT Timer length…

## 5.2	Summary

We think that changing the start part of HARQ RTT Timer in 5.7 is unnecessary as the spec impacts in both 5.7 and 7.7 are quite large, therefore, only A and B2 shall be considered.

Further, A seems like a simpler change for eMTC compared to B2, however B2 makes eMTC formulas for FDD and TDD the same… We propose:

In NTNs for BL UEs and UEs in enhanced coverage, the offset added to the formula used for calculating the HARQ RTT timer shall be Koffset+Kmac instead of RTToffset, where Koffset is the UE specific Koffset defined in 36.213 section 4.2 and k-Mac is a RRC configured parameter.

# 6	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Change “k_Mac” in UE-eNB RTT definition to “k-Mac” in the MAC section 3.1.

Proposal 2	In MAC spec, when UE-eNB RTT is used, there is no need to add “subframes” after.

Proposal 3	Introduce consistent use of “subframes” when UE-eNB RTT is used in MAC spec.

Proposal 4	For P1 and P2, consider the text proposal below.

Proposal 5	Correct the NB-IoT start of drx-InactivityTimer when UL HARQ RTT Timer expires for multiple scheduled TBs, see text proposal below.

Proposal 6	Correct the PUR retransmission start of PUR response window timer and the expiry of PUR response window timer, see text proposal below.

Proposal 7	In NTNs for BL UEs and UEs in enhanced coverage, the offset added to the formula used for calculating the HARQ RTT timer shall be Koffset+Kmac instead of RTToffset, where Koffset is the UE specific Koffset defined in 36.213 section 4.2 and k-Mac is a RRC configured parameter.

All changes are incorporated in the accompanying CR [1].

# References

R2-22xxxxx, 36.321 CRxxxx, “Correction for IoT NTN”, Ericsson, RAN2#120, Nov. 2022