3GPP TSG-RAN WG2 #120	R2-2212947

Toulouse, France, 14-18 November 2022

Agenda Item:	6.10.4.2

Source:	Ericsson

Title:	Discussion on epoch time, validity and T430 start/end description

Document for:	Discussion

# Introduction

Here we discuss some remaining issues related to epoch time, validity of ephemeris & Common TA and the start/end description for T430.

# Discussion on epoch time and validity

At the last RAN2 meeting there were many discussions about the RAN1 agreements that for most cases the epoch time is in the future [1]. However, here we bring the RAN2 issues which need to be discussed.

The only case when epoch is not in the future is when epoch is signalled explicitly in a subframe of a system frame and the epoch subframe is in an earlier subframe of the same system frame – this is a scenario that is unwanted because it requires tight interaction between RRC (creating the RRC message with explicit epoch time) and MAC (scheduling the RRC message). For SIB with implicit epoch time, epoch is always in the future.

The epoch time can only be indicated in the future.

We note that an epoch time in the future leads to increased latency when UEs are doing:

Access from RRC_IDLE/RRC_INACTIVE

Handover, or conditional handover to cells when UE need to acquire SIB19

RRC reestablishment to cells when UE need to acquire SIB19

The epoch time in the future will increase latency for access from RRC_IDLE/RRC_INACTIVE and for Handovers or RRC Reestablishment if UE needs to acquire SIB19.

The 1) increased latency has the indirect effect that 2) handover success rate and 3) RRC reestablishment success rate will decrease.

These three things have a negative effect on the following Key Performance Indicators (KPIs, see 28.554):

PDU session establishment time

PDU session establishment success rate

Registration success rate

NG-RAN handover success rate

Mean time of inter-gNB handover execution

Success rate of mobility registration update

NG-RAN handover success rate for all handover types

DRB Retainability

Tuning the SI window parameters can only mitigate the problem of epoch time in the future. It is not a viable path, because SI window will need to be adapted for other reasons (long window may be needed to allow sufficient repetitions of SI for good coverage and to allowing sufficiently large TBSs to minimize delay for Public Warning Systems). Further, it requires a tight interaction between the function providing ephemeris and epoch time and the gNBs SIB broadcasting functions.

Tuning SI window parameters to decrease time between SI broadcast and end of SI window (implicit epoch time) can only mitigate latency issues and require tight interaction between gNBs and the function providing the ephemeris to the gNB.

RAN2 needs to address the latency created by epoch time in the future.

Two methods were discussed that will remove the latency of the epoch time in the future:

Support backwards propagation of ephemeris

Allow epoch time in the past

In both cases, ephemeris and Common TA are valid from the moment the UE receives them and there is no latency added for waiting until the epoch time.

Two solutions remove latency from epoch time in the future 1) support of backwards propagation or 2) allow epoch time in the past.

Some companies argue that backwards (BW) propagation has not been properly evaluated, especially for the Common TA part and what to do with the validity timer.

However, backwards propagation and Common TA estimation have been thoroughly evaluated in [2], which we summarize here:

- Errors in service link RTT for forward and backward propagation of ephemeris is the same for the same prediction length from the epoch time.

- The service link RTT error at 10.24 s (which is the maximum size of the SI window) from the epoch time is negligible (less than 3*10-8 s which is many times smaller than the cyclic prefix).

Figure 1: Prediction accuracy of forward and backward propagation of satellite position.

- Common TA is used to predict the RTT between the satellite and the reference point (RP). The RTT prediction error is only affected by the length of the period predicted (T) and the error is not affected by the choice of epoch time.

- The satellite to RP (when RP is at gNB) RTT error versus UE SIB19 reading frequency during the connection time to one satellite where epoch is a) exactly at the time reading the SIB19 (the best case for UE) (black dashed line), or b) 10.24 s into the future (except when update frequency is 5 s then epoch is 5 s into the future, the worst case for the UE) (red line), we have here:

Figure 2: Approximation error of common TA.

As can be seen the choice of epoch time has no effect (a and b are overlapping) and the maximum prediction period T is about 24 s, depending on how large satellite to RP RTT error is acceptable.

- One possible configuration is to have the RP in the satellite and thus RTT error is only from the service link (that is, the Common TA is zero). When there is only service link RTT, BW propagagtion gives the most gain.

- If service link RTT error is much smaller than the error in satellite to RP RTT prediction (as in the examples of point 2 and 4 above): 
The prediction calculated from the reception of SIB19 at t1 has low enough error until [t1+T] (T = the maximum prediction period) and setting epochTime and ntn-UlSyncValidityDuration must fulfil [t1+ T] ≥ [epochTime+ntn-UlSyncValidityDuration].

Figure 3: Duration of UL synchronization.

Regarding what to do with the validity timer T430 in the UE when backwards propagation is supported, the easiest solution is to start it at epochTime and let it expire after ntn-UlSyncValidityDuration.

To introduce backwards propagation in RAN1 spec is simple, no change is needed. To introduce backwards propagation in the RAN2 spec will require some small changes in RRC procedures (no ASN.1 change!):

##### 5.2.2.4.21	Actions upon reception of SIB19

Upon receiving SIB19, the UE shall:

1>	start or restart T430 for serving cell with the timer value set to ntn-UlSyncValidityDuration for the serving cell from the subframe indicated by epochTime for the serving cell;

1>	inform lower layers that UL synchronization is obtained;

NOTE:	UE should attempt to re-acquire SIB19 before the end of the duration indicated by ntn-UlSyncValidityDuration and epochTime by UE implementation.

#### 5.2.2.6	T430 expiry

The UE shall:

1>	if T430 for serving cell expires and if in RRC_CONNECTED:

2>	inform lower layers that UL synchronisation is lost;

2>	acquire SIB19 as defined in clause 5.2.2.3.2;

ntn-UlSyncValidityDuration

A validity duration configured by the network for assistance information (i.e. Serving and/or neighbour satellite ephemeris and Common TA parameters) which indicates the maximum time duration after the epochTime during which the UE can apply assistance information without having acquired new assistance information.

The unit of ntn-UlSyncValidityDuration is second. Value s5 corresponds to 5 s, value s10 indicate 10 s and so on. This parameter applies to both connected and idle mode UEs. If this field is absent in ntn-Config provided via NTN-NeighCellConfig, the UE uses validity duration from the serving cell assistance information. This field is excluded when determining changes in system information, i.e. changes of ntn-UlSyncValidityDuration should neither result in system information change notifications nor in a modification of valueTag in SIB1. ntn-UlSyncValidityDuration is only updated when at least one of epochTime, ta-Info, ephemerisInfo is updated.

To support epoch time in the past, no change is needed in RAN1 is needed but RAN2 specs may need some small changes in the signalling for epoch time or the field descriptions.

Both support of backwards propagation and support of epoch time in the past are feasible and require small or no changes in the specifications.

Backwards propagation will support longer validity time and longer intervals between when the UE needs to read SIB, compared to epoch time in the past, and thus potentially has lower UE energy consumption, especially for when reference point is in the satellite.

RAN2 to discuss and request RAN1 to either a) support backwards propagation, or 2) allow indication of epoch time in the past.

# 3	epochTime field description

The IPA CR in R2-2211018 captures the following field description for the field epochTime.

epochTime Indicate the epoch time for the NTN assistance information. When explicitly provided through SIB, or through dedicated signaling, EpochTime is the starting time of a DL sub-frame, indicated by a SFN and a sub-frame number signaled together with the assistance information. Denoted by f0 the frame where the message indicating the epochTime is received and by f1 the frame containing the DL sub-frame defining the epochTime. For serving cell, the UE considers f1 to be f0 if the indicated SFN equals the SFN of f0, or the next frame with the indicated SFN if the indicated SFN differs from the SFN of f0. For neighbor cell, the UE considers f1 to be the frame, with the indicated SFN, that is nearest to f0. The reference point for epoch time of the serving satellite ephemeris and Common TA parameters is the uplink time synchronization reference point. If this field is absent, the epoch time is the end of SI window where this SIB19 is scheduled. This field is mandatory present when provided in dedicated configuration. If this field is absent in ntn-Config provided via NTN-NeighCellConfig the UE uses epoch time from the serving satellite ephemeris, otherwise the field is based on the timing of the serving cell, i.e. the SFN and sub-frame number indicated in this field refers to the SFN and sub-frame of the serving cell. In case of handover, this field is based on the timing of the target cell, i.e. the SFN and sub-frame number indicated in this field refers to the SFN and sub-frame of the target cell. The UE considers the target cell epoch time, indicated by the SFN and sub-frame number in this field, to be the frame nearest to the frame in which the message indicating the epoch time is received. This field is excluded when determining changes in system information, i.e. changes to epochTime should neither result in system information change notifications nor in a modification of valueTag in SIB1.

Issus with this field description:

- The Language in 3GPP spec shall be UK English; thus, it is spelled “neighbour”.

- Further, it shall be “The epochTime” not “EpochTime” and “satellite” shall be replaced by “NTN payload”.

- Using italics for f0 and f1 seems to indicate that they are fields in RRC, which is incorrect – we prefer to not use italics for f0 and f1.

- For target cell, the HO/CHO command may be received from source in a source slot that happens to overlap two frames of target cell, but that can be handled by the target gNB setting the epochTime with a margin (that is, epoch is not too far from current frame). However, it must be made clear that it is the frame in the target that is referred to.

In field “epochTime”, replace “neighbor” with “neighbour”, replace “EpochTime” with “The epochTime”, and replace “satellite” with “NTN payload”.

In field “epochTime”, do not use italics for f0 and f1.

In field “epochTime”, replace “The UE considers the target cell epoch time, indicated by the SFN and sub-frame number in this field, to be the frame nearest to the frame in which the message indicating the epoch time is received.” with “For target cell, the UE considers the epoch time, indicated by the SFN and sub-frame number in this field, to be in the target cell frame nearest to the target cell frame in which the message indicating the epoch time is received.”

# 4	On T430

The discussions in RAN2#119bis did not conclude on start and end description for timer T430.

The timer text for T430 need to be updated, suggestion is as follows:

| T430 | Start or restart from the subframe indicated by epochTime upon reception of SIB19, or upon reception of RRCReconfiguration message including reconfigurationWithSync, or upon conditional reconfiguration execution i.e. when applying a stored RRCReconfiguration message including reconfigurationWithSync | Upon reception of RRCReconfiguration message including reconfigurationWithSync , or upon conditional reconfiguration execution i.e. when applying a stored RRCReconfiguration message including reconfigurationWithSync | Perform the actions as specified in 5.2.2.6. |
| --- | --- | --- | --- |

Adopt the above description to T430

# 5	Conclusion

In the previous sections, we made the following observations:

Observation 1	The epoch time can only be indicated in the future.

Observation 2	The epoch time in the future will increase latency for access from RRC_IDLE/RRC_INACTIVE and for Handovers or RRC Reestablishment if UE needs to acquire SIB19.

Observation 3	Tuning SI window parameters to decrease time between SI broadcast and end of SI window (implicit epoch time) can only mitigate latency issues and require tight interaction between gNBs and the function providing the ephemeris to the gNB.

Observation 4	Two solutions remove latency from epoch time in the future 1) support of backwards propagation or 2) allow epoch time in the past.

Observation 5	Both support of backwards propagation and support of epoch time in the past are feasible and require small or no changes in the specifications.

Observation 6	Backwards propagation will support longer validity time and longer intervals between when the UE needs to read SIB, compared to epoch time in the past, and thus potentially has lower UE energy consumption, especially for when reference point is in the satellite.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 needs to address the latency created by epoch time in the future.

Proposal 2	RAN2 to discuss and request RAN1 to either a) support backwards propagation, or 2) allow indication of epoch time in the past.

Proposal 3	In field “epochTime”, replace “neighbor” with “neighbour”, replace “EpochTime” with “The epochTime”, and replace “satellite” with “NTN payload”.

Proposal 4	In field “epochTime”, do not use italics for f0 and f1.

Proposal 5	In field “epochTime”, replace “The UE considers the target cell epoch time, indicated by the SFN and sub-frame number in this field, to be the frame nearest to the frame in which the message indicating the epoch time is received.” with “For target cell, the UE considers the epoch time, indicated by the SFN and sub-frame number in this field, to be in the target cell frame nearest to the target cell frame in which the message indicating the epoch time is received.”

Proposal 6	Adopt the above description to T430

# References

Email discussions “[POST119bis-e][114][NR NTN] LS on validity of assistance information (OPPO)” and “[AT119bis-e][114][NR NTN] Validity of assistance information (OPPO)”, RAN2#119bis-e, October, 2022.

R1-2212313, “On epoch time and validity of assistance information for R17 NR NTN”, Ericsson, RAN1#111, November 2022.