3GPP TSG-RAN WG2 Meeting #120	R2-2212953

Toulouse, France, 2022-11-14 – 2022-11-18

Agenda Item:	7.2.3

Source:	Ericsson

Title:	Neighbour cell information in IoT NTN

Document for:	Discussion, Decision

# 1	Introduction

In this paper, we provide some considerations regarding the introduction of neighbour cell ephemeris information in IoT NTN for measurement and mobility purposes.

# 2	Discussion

In RAN2#119-e and RAN2#119bis-e, the topic of broadcasting neighbour cell ephemeris in System Information was brought up again during offline discussions [1] and [2], respectively. The same Neighbour cell ephemeris information and common TA parameters were introduced in NR NTN for measurement and mobility purposes. This assistance information helps to determine the timing and frequency of a neighbour cell and perform measurements in a more efficient manner. For instance, this can be relevant for idle mode mobility in all IoT NTN UEs or connected mode measurements (e.g., to perform handover) for eMTC UEs.

Neighbour cell information helps to perform measurements more efficiently.

In this regard, RAN2 has received an LS from RAN4 [3] with the following content:

----

In LTE_NBIOT_eMTC_NTN_req WI, RAN4 is specifying RRM requirements for NB-IoT and eMTC operation over NTN functionality defined in the Rel-17 LTE_NBIOT_eMTC_NTN work item.

In RAN4#104-e, the following agreement [1] is reached on information for neighbor/target cell in IoT NTN

#### Issue 1-3-2: information for the neighbor/target cell - Similar as NR NTN, the mobility and measurement requirements for IoT NTN apply provided that valid information for the neighbour/target cell is made available to the UE.

RAN4 respectfully asks RAN2 to take the above agreement into consideration in their work.

----

During the online discussion, as captured in the chair notes [4], it was stated that RAN4 may have indicated that this information is essential for IoT NTN. However, the agreement above does not have such connotations. It merely states that certain requirements (mobility and measurements) are applicable if neighbour cell information is available to the UE. Hence, this should not be taken as an imperative indication from RAN4 that neighbour cell information needs to be adopted. In fact, the feasibility of introducing this new feature should be evaluated from a RAN2 technical standpoint.

Introducing neighbour cell information in IoT NTN needs to be evaluated from a RAN2 technical standpoint.

This is not a new topic and was already discussed during RAN2#115-e [5]. At that time, the majority considered that providing this information in System Information would be an optimization, i.e., not critical for operation. In addition, there was a considerable support to postpone this optimization to Release 18. Hence, the following agreement was made:

FFS if Satellite assistance information for neighbour cell(s) is provided to UE for cell selection/reselection (justification would be needed).

Finally, the size of SIB in eMTC and NB-IoT is quite limited compared to NR. [6] provides a detailed calculation for both technologies and concluded that the maximum number of neighbour satellites for which assistance information can be provided is 2 for eMTC and 1 for NB-IoT. Therefore, even if RAN2 agreed to provide this information is provided in SIB31, the benefit would be limited.

The maximum number of satellites for which assistance information can be provided in one SI is 3 for eMTC and 2 for NB-IoT, including serving cell satellite.

Considering the arguments stated above, there is not a strong technical reason to introduce this new feature in Release 17 during maintenance phase. The discussion should continue during Release 18 to agree on an acceptable format and associated UE capabilities.

Neighbour cell ephemeris information is not broadcast in Rel-17 IoT NTN.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	Neighbour cell information helps to perform measurements more efficiently.

Observation 2	Introducing neighbour cell information in IoT NTN needs to be evaluated from a RAN2 technical standpoint.

Observation 3	The maximum number of satellites for which assistance information can be provided in one SI is 3 for eMTC and 2 for NB-IoT, including serving cell satellite.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Neighbour cell ephemeris information is not broadcast in Rel-17 IoT NTN.

# References

R2-2208755, Report of [AT119-e][105][IoT-NTN] RRC corrections (Huawei), Huawei, HiSilicon, RAN2·119-e, August 2022.

Report of [Offline-107][IoT NTN] RRC corrections, Huawei, RAN2#119bis-e, October 2022.

R2-2211171, LS on information for neighbor/target cell in IoT NTN, RAN4#104bis-e, October 2022.

R2-2208701, Report from Break-out session on NR-NTN, IoT-NTN, REDCAP and CE, Vice Chairman (ZTE Corporation), RAN2#119-e, October 2022.

R2-2109176, Summary of AI 9.2.4.1 “TA and Mobility related” (Ericsson) - Ph2, Ericsson, RAN2#115-e, August 2021.

R2-2207150, Discussion on neighbour cell ephemeris, Huawei, HiSilicon, RAN2#119-e, August 2022.