3GPP TSG RAN WG2 Meeting #120			R2-2213141

Toulouse, France, 14th – 18th Nov, 2022

Source: 	CATT

Title: 	Text Proposals of TR 38.859 for Expanded and Improved NR Positioning

Agenda item:	8.2.1

Document for: 	Discussion

Introduction

This is to provide text proposal of TR 38.859 for Expanded and Improved NR Positioning from RAN2 based on the previous agreement reached in RAN2#119-e, #119bis-e and #120.

Text Proposal

/****Start of the changes****/

- 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 38.857: "Study on NR positioning enhancements".

[3]	3GPP TR 38.845: "Study on scenarios and requirements of in-coverage, partial coverage, and out-of-coverage NR positioning use cases".

[4]	3GPP TS 22.261: "Service requirements for the 5G system".

[5]	3GPP TR 22.855: "Study on ranging-based services".

[6]	3GPP TS 22.104: "Service requirements for cyber-physical control applications in vertical domains".

[7]	RP-213588: "New SID on Study on expanded and improved NR positioning".

[8]	3GPP TR 37.885: "Study on evaluation methodology of new Vehicle-to-Everything (V2X) use cases for LTE and NR".

[9]	3GPP TR 36.885: "Study on LTE-based V2X Services".

[10]	3GPP TR 36.843: "Study on LTE Device to Device Proximity Services".

[11]	3GPP TR 38.901: "Study on channel model for frequencies from 0.5 to 100 GHz".

[12]	3GPP TR 38.855: "Study on NR positioning support".

[13]	3GPP TR 38.840: "Study on User Equipment (UE) power saving in NR".

[14]	3GPP TR 38.802: "Study on New Radio Access Technology - Physical Layer Aspects".

[15]	3GPP TR 38.830: "Study on NR coverage enhancements".

[1x]	3GPP TR 23.700-86: "Study on Architecture Enhancement to support Ranging based services and sidelink positioning".

Definitions of terms, symbols and abbreviations

Terms

For the purposes of the present document, the terms given in TR 21.905 [6] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [6].

/**unchanged Skipped**/

SLPP: Protocol for Sidelink positioning procedures between UEs.

/**unchanged Skipped**/

- 5	Sidelink Positioning

/**unchanged Skipped**/

5.2.2	Potential Architecture and Signalling Procedures for Sidelink Positioning

#### 5.2.2.x	Potential Architecture for SL Positioning

Sidelink positioning in-coverage, partial coverage and out-of-coverage scenarios may be supported. In partial coverage scenarios, either of UEs including target UE and one or multiple anchor UEs may be OOC, but with at least one UE being in coverage.

The architecture and signaling procedures are studied to support below operation scenarios:

-	Operation Scenario 1: PC5-only-based positioning.

-	Operation Scenario 2: Combination of Uu- and PC5-based positioning.

Note: How to enable the procedures/signaling for supporting SL positioning in partial coverage will be further discussed in normative work.

RAN2 follow SA2 on the architecture, including the possibility of a UE as a location server.  RAT-independent SL positioning is not considered in this release. RAN2 waits for SA2 on the triggering of the positioning procedures from upper layers.

#### 5.2.2.y	Sidelink Positioning Protocol(SLPP)

With regards to the sidelink positioning procedures between UEs, SLPP is introduced to support at least the following functionalities:

-		SL Positioning Capability Transfer

-		SL Positioning Assistance Data exchange

-		SL Location Information Transfer

-		Error handling

-		Abort

The cast type for SLPP signalling is studied, including unicast, groupcast and broadcast.

Unicast/one-to-one operation is assumed as baseline for exchange of SLPP signaling between UEs. Unicast SLPP session-based operation is supported. At least “centralized” operation is supported, i.e., operation where one UE performs range and/or position calculations based on measurement/location information relating to itself and/or other UEs.It is feasible to send at least the following positioning signaling for groupcast/broadcast (in addition to unicast) from RAN2’s perspective:

-		SL positioning capability

-		SL positioning assistance data

Location information is not excluded and can be further considered in normative work.

RAN2 will further discuss in normative work:

-    The security issues (e.g., requirements for ciphering and/or integrity) on specific information of SL positioning capability and assistance data in groupcast/broadcast.

-    	The use cases for applying groupcast/broadcast.

Both session-based and session-less operation for sidelink positioning signalling are studied.

Sidelink positioning supports a session-based concept in SLPP, in which signalling messages within a session can be associated with one another by the involved UEs. The relationship to upper-layer designs from SA2 can be discussed during normative work.

FFS if there is also session-less operation and what aspects of session-based operation would not be included.

At least in the case that positioning methods are supported that do not require a mutual exchange of SLPP messages associated with one another among UEs, SLPP session-less operation can be supported. FFS if session-less operation can be operated with security.

If it is determined to support group positioning, it is feasible to perform at least ranging with the estimate calculation at multiple UEs.

SLPP is a separate ASN.1 module from LPP (this does not necessarily imply whether it is included in 37.355).

For the transport layer of SLPP, RAN2 agrees to down select between PDCP and PC5-U.

#### 5.2.2.z	Singaling between UE and LMF

The potential impact to LPP for the support of sidelink positioning procedures between UE and LMF is studied.  Protocol between UE and LMF for hybrid PC5+Uu positioning and PC5-only positioning in-coverage will be down-selected from the following options during normative work.

-    	Extension of LPP, whereby new signaling shall be defined to support hybrid Uu and PC5 based positioning, i.e. extend the existing LPP to support sidelink based positioning between UE and LMF

-    	Enhancement of LPP whereby SLPP/RSPP signaling can be transported within LPP transparently, i.e. use the newly defined SLPP/RSPP to support sidelink based positioning and use the existing LPP to support Uu based positioning; and the SLPP/RSPP is carried as a container in LPP

-    	Use of SLPP/RSPP between the UE and the LMF

The details of functionalities of LMF for supporting SL positioning will be discussed in normative work.

/**unchanged Skipped**/

- 5.4	Potential specification impact for Sidelink Positioning

RAN2 studied that there is potential specification impact for Sidelink Positioning including:

-    Introduce a new protocol for sidelink positioning procedures between UEs, FFS where it is specified.

-   The new protocol is a separate ASN.1 module from LPP (this does not necessarily imply whether it is included in 37.355).

-    Singaling between LMF and UE will be down-selected during normative work.

- 6	Positioning Enhancements for Improved Integrity, accuracy, and power efficiency

/**unchanged Skipped**/

## 6.1	Integrity for RAT-Dependent Positioning Techniques

### 6.1.1	Identification of error sources

### 6.1.2	Methodologies, procedures and signalling for determination of positioning integrity

#### 6.1.2.x	Integrity Principle of Operation

Reuse the concepts and principles developed for RAT-Independent GNSS positioning integrity for RAT-Dependent positioning integrity.

Both UE-based and LMF-based integrity for RAT-dependent cases are studied and supported in integrity for RAT-dependent positioning.

Use DNU flag for RAT-dependent integrity, with the meaning that the concerned assistance data cannot be used for integrity calculation but may be usable for positioning.  Signalling details and relation to error sources can be determined in normative work.  FFS which positioning methods are affected based on the progress in RAN1.

#### 6.1.2.y	Signalling for UE-Based positioning integrity

R17 UE-based integrity mode signaling can be used as baseline at least with the following aspects:

UE sends capability info to LMF on integrity for UE-based mode using LPP capability transfer procedure.

-    LMF sends the assistance data for integrity calculation to UE. LMF provides, in assistance data, the information of error sources (e.g., originated from RAN node) to UE for integrity in UE-based mode.

-    LMF sends integrity requirement e.g., TIR to UE in LPP request location information message for integrity of UE-based mode.

-    UE sends integrity result to LMF using LPP location information Transfer message.

#### 6.1.2.z	Signalling for LMF-Based positioning integrity

LMF-based integrity mode signaling is studied with the following aspects:

#### Editor note: the conclusion of offline [420] will be captured here.

### 6.1.3	Summary of Evaluation Results for Integrity for RAT-Dependent Positioning Techniques

### 6.1.4	Potential Specification Impact for Integrity for RAT-Dependent Positioning Techniques

Signalling design of both UE based and LMF based integrity can be supported.

/**unchanged Skipped**/

- 6.4	Low Power High Accuracy Positioning

/**unchanged Skipped**/

6.4.1	Target use cases and requirements for Low Power High Accuracy Positioning

Use case 6 defined in TS 22.104 [6] is the single representative use case for the study of LPHAP.

For LPHAP, the main objective of the evaluations from the perspective of lower layers is on UE power consumption.

At least relative power unit is adopted as the performance metric to evaluate the power consumption of the Rel-17 RRC_INACTIVE state positioning and potential enhancements.

A reference device (e.g., a mobile phone) with reference traffic type, reference battery capability, and reference battery life is defined for the purpose of identification of the performance gap that achieved by the Rel-17 RRC_INACTIVE state positioning baseline and the target battery life of LPHAP use case 6.

For the service type, at least the ‘Low Power Periodic and Triggered 5GC-MT-LR Procedures’ in TS 23.273 is supported.

6.4.x	Potential Enhancements for Low Power High Accuracy Positioning

#### 6.4.x.y Physical Layer Aspects

/**unchanged Skipped**/

#### 6.4.x.z Higher Layer Aspects

The potential enhancements for Low Power High Accuracy Positioning in higher layer aspect are studied as below:

a.	Enhancements on SRS configuration

Higher layer studied the following candidate enhancements on SRS configuration.

-	Validity area mechanism

SRS positioning validity area for UL positioning in RRC_INACTIVE can avoid reconfiguration of SRS configuration upon cell reselection and is recommended for normative work from RAN2’s perspective if feasible from RAN1’s perspective.

The solution should not require the gNB to monitor multiple SRS configuration simultaneously for a UE.

-	SRS configuration request

SRS configuration request can be discussed during normative work from RAN2 perspective. Scenarios requiring SRS configuration request include:

Scenario1: During the UL positioning procedure, when the SRS configuration turns invalid, e.g., when the UE moves out of the SRS positioning validity area.

Scenario2: At the initiation of UL positioning procedure when an event is detected.

Detailed solution for the SRS update, e.g., with RRC message, UL MAC, or NG-AP message can be discussed in the WI phase.

-	Pre-configure multiple SRS

Pre-configure multiple SRS, which could include broadcast transmission of configurations with UE-specific determination along with the network of a configuration.

Pre-configuration of multiple SRS configurations (e.g., for multiple SRS positioning validity areas) is feasible from RAN2 perspective and can be discussed in normative work.

The pre-configuration of multiple SRS configurations can be delivered to the UE either by dedicated signalling or SI broadcast.

Editor’s NOTE: The concerns on interference issues with SRS configurations across multiple cells and about the validity of SRS parameters, such as interference, timing alignment, spatial relation, and which SRS parameters can be valid across multiple cells, need to wait for RAN1’s response.

b.	Enhance DL-PRS configuration

Editor’s NOTE: the conclusion of offline [421] will be captured here.

c.	Exposure of LPHAP information to the gNB and/or LMF

Exposure of LPHAP information to the gNB and/or LMF (e.g., as a UE capability) can be discussed in normative work if any enhancement for LPHAP is agreed, taking into account any guidance from SA2.

d.	Positioning in RRC_IDLE state

Editor’s NOTE: the conclusion of offline [421] will be captured here.

Two candidate solutions on how to report measurements taken in RRC_IDLE is further studied:

Alt1: measurement is performed in IDLE and reported in CONNECTED, including the concerns:

-	Whether the mechanism of measurement in IDLE and report in CONNECTED is more beneficial for power saving than legacy mechanism, i.e. RRC_INACTIVE positioning.

-	Whether the CN can handle the measurement reports from the UE in RRC_CONNECTED, while the positioning was performed in RRC_IDLE.

Alt2: measurement is performed in IDLE and report is carried with initial access messages, including the concern:

-	Is there AS context/security issue on sending the measurements to LMF?

### 6.4.3	Potential Specification Impact for Low Power High Accuracy Positioning

LPHAP is recommended for normative work, including:

a.	Enhancements on SRS configuration

-	SRS positioning validity area for UL positioning in RRC_INACTIVE is recommended for normative work from R2’s perspective if feasible from R1’s perspective.

-	SRS configuration request can be discussed during normative work from RAN2 perspective.

-	Pre-configuration of multiple SRS configurations (e.g., for multiple SRS positioning validity areas) is feasible from RAN2 perspective and can be discussed in normative work.

- 6.5	Potential Enhancements for RedCap Positioning

/**unchanged Skipped**/

### 6.5.3	Potential Specification Impact for Positioning for RedCap UEs

The potential specification impact for the RedCap positioning are studied in high layer, and agreed that the decision on RedCap positioning recommendation is left to RAN1. No recommendation is needed from RAN2.

/**unchanged Skipped**/

- 7	Conclusions

- 7.w Sidelink Positioning

Sidelink positioning is recommended for normative work, including:

-    Sidelink positioning in-coverage, partial coverage and out-of-coverage scenarios may be supported.

-    How to enable the procedures/signaling for supporting SL positioning in in-coverage, partial coverage and out-of-coverage scenarios will be further discussed in normative work.

-    Protocols between UE and UE

-    RAN2 will enable the support of SL-PRS configuration in normative work based on the progress in RAN1.

-

RAN2 will design Protocol for SL positioning procedures between UEs (SLPP) in normative work.

-   RAN2 will enable the support of sidelink positioning procedures at least for unicast, and how to enable the procedures/signaling for groupcast/broadcast will be further discussed in normative work.

-   Protocols between LMF and UE

-    RAN2 will discuss the details of functionalities of LMF for supporting SL positioning in normative work.

-    RAN2 will discuss the protocol details to support sidelink positioning procedures between UE and LMF in normative work”.

- 7.x Integrity for RAT-Dependent Positioning Techniques

Both UE based and LMF based integrity are recommended for normative work.

- 7.y Low Power High Accuracy Positioning

LPHAP is recommended for normative work, including:

a.	Enhancements on SRS configuration

-    	SRS positioning validity area for UL positioning in RRC_INACTIVE is recommended for normative work from R2’s perspective if feasible from R1’s perspective.

-    	SRS configuration request can be discussed during normative work from RAN2 perspective.

-    	Pre-configuration of multiple SRS configurations (e.g., for multiple SRS positioning validity areas) is feasible from RAN2 perspective and can be discussed in normative work.

- 7.z RedCap Positioning

The potential specification impact for the RedCap positioning are studied in high layer, and agreed that the decision on RedCap positioning recommendation is left to RAN1. No recommendation is needed from RAN2.

- References

[1] RP-213588, Revised SID on Study on expanded and improved NR positioning, Intel, December 2021.

[2] R1-2204805, Study on expanded and improved NR positioning, Intel, CATT, Ericsson, RAN1 #109-e, May 2022.

[3] R2-2210804, Report from session on positioning and sidelink relay, RAN2 #119bis-e, Oct 2022.

[4] R2-2213148, Summary of [AT120][418][POS] TP to TR 38.859 (CATT).