3GPP TSG RAN WG2#120	R2-2213169

# Toulouse, 14th – 18th November, 2022

# Title:	LS to RAN1 on CAPC for SL-U

# Response to:	-

# Release:	Release 18

# Work Item:	NR_SL_enh2

Source:	RAN2

To:	RAN1

Cc:	-

Contact Person:

Name:	Martino Freda

E-mail Address:	martino.freda@interdigital.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

RAN2 discussed CAPC determination for SL-U and confirmed the working assumption from previous RAN2 meeting that “PQI is used to determine the CAPC mapping as in NR-U” as a baseline.  Regarding the PQI to CAPC mapping, the following working assumptions were made:

- Working assumption: mapping PQI 90/91/92/93/21/22/23/55/56/57/58 to CAPC priority class 1. FFS on other SL CAPC mapping criterion.

- Working assumption: mapping PQI 59/61 to CAPC priority class 3.

- Working assumption: mapping PQI 25 to CAPC priority class 2.

- Working assumption: mapping PQI 24/26/60 to CAPC priority class 1.

Regarding the multiplexing of SDUs with different CAPC, the following was agreed at RAN2 at this meeting and at RAN2#119bis-e:

- Similar to NR-U, to determine the CAPC of the SL TB when the CAPC is not indicated in the DCI:

- If only SL MAC CE(s) are included in the SL TB, the highest priority SL CAPC is used;

- If SCCH SDU(s) are included in the SL TB, the highest priority SL CAPC is used;

- Working assumption: The lowest priority CAPC of the logical channel(s) with MAC SDU multiplexed in the TB is used regardless of whether the TB also contains SL MAC CEs in addition to MAC SDUs.

Regarding the CAPC for SBCCH SDU transmitted in SL-SSB and for PSFCH, the following were agreed:

- The highest priority SL CAPC is used for SBCCH SDU transmission (if SL CAPC is applied to SBCCH SDU).

- For PSFCH, we leave it to RAN1 to decide the CAPC to use

2. Actions:

To RAN1 group.

ACTION: 	RAN2 kindly asks RAN1 to take the above information into consideration and provide feedback, if needed.

3. Date of Next RAN2 Meetings:

RAN2#121	February 27th – March 3rd, 2023 	Athens, GR

RAN2#121-bis-e	April 17th – 26th, 2023		Online