3GPP TSG-RAN WG2 Meeting #120	R2-2213172

# Electronic Meeting, November 14 – 18, 2022	was R3-226150

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | 0602 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 38.300 on IAB-topology definition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | R3 (Qualcomm) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB_enh-Core |  |  |  |  |  |  | Date: |  | 2022-09-27 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) |  |
| Reason for change: |  | TS 38.300 defines the IAB topology as “The unison of all IAB-nodes and IAB-donor-DUs that are interconnected via BH links and terminate F1 and/or RRC at the same IAB-donor-CU.” This definition makes sense when the boundary node is dual-connected. In case the boundary node is single connected (e.g., after partial migration), it will only belong to the IAB topology of the non-F1-terminating IAB-donor, and as a consequence, the descendent nodes are not part of any IAB topology: - The descendent nodes are not part of the F1-terminating IAB-donor’s topology since they do not have BH links with IAB-nodes and/or IAB-donor-DUs of that topology. - The descendent nodes are not part of the non-F1-terminating IAB-donor’s topology since they do not have RRC and/or F1 connectivity with the non-F1-terminating IAB-donor. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The defintion of the IAB-topology is changed so that the topology assignment of the single connected boundary node is matched to that of the dual-connected boundary node. This means that both single-connected and dual-connected boundary nodes belong to both IAB topologies. Also, the descendent nodes should belong to the IAB topology of the F1-terminating IAB-donor for both cases, i.e., the single-connected or the dual-connected boundary node. The IAB topology definition is changed to: “The unison of all IAB-nodes and IAB-donor-DUs whose F1 and/or RRC connections are terminated at the same IAB-donor-CU.” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | All Rel-17 IAB stage-3 configurations that explicitly refer to the “non-F1-terminating IAB-donor’s topology” remain undefined for descendent nodes of single-connected boundary nodes. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR's revision history: | This CR was endorsed by RAN3 in R3-226150. |
| --- | --- |

Begin of Changes

## 3.2	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1], in TS 36.300 [2] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1] and TS 36.300 [2].

>>> skip

IAB topology: The unison of all IAB-nodes and IAB-donor-DUs whose F1 and/or RRC connections are terminated at the same IAB-donor-CU.

>>> skip

End of changes