3GPP TSG-RAN WG2 Meeting #120	R2-2213116

Toulouse, FR, November 14 – 18, 2022

Agenda item:	5.3

Source: 	Qualcomm Incorporated (Rapporteur)

Title: 	Summary of Rel-15 and Rel-16 NR Positioning Support AIs 5.3.1 and 5.3.3

Document for:		Discussion

# 1.	Introduction

This document summarizes the following contributions submitted for Agenda Item 5.3.1 (NR Rel-15 and Rel-16 – Stage 2) and 5.3.3 (NR Rel-15 and Rel-16 – LPP).

AI 5.3.1 – General and Stage 2 corrections:

[1]	R2-2211150, "LS on DL PRS search window (R1-2210618; contact: Qualcomm)", RAN1, LS in.

[2]	R2-2212516, "Update Stage 2 SSR Phase Bias description to include yaw", Swift Navigation, Mitsubishi Electric Corporation, Ericsson.

[3]	R2-2212518, "Update Stage 2 SSR Phase Bias description to include yaw", Swift Navigation, Mitsubishi Electric Corporation, Ericsson.

[4]	R2-2212535, "Update Stage 2 SSR Phase Bias description to include yaw", Swift Navigation, Mitsubishi Electric Corporation, Ericsson.

[5]	R2-2212536, "Update Stage 2 SSR Phase Bias description to include yaw", Swift Navigation, Mitsubishi Electric Corporation, Ericsson.

[6]	R2-2212544, "Discussion and TP on Yaw Angle and Antenna Phase Center corrections for SSR assistance data", Swift Navigation, Mitsubishi Electric Corporation, Ericsson.

AI 5.3.3 – LPP corrections:

[7]	R2-2211420, "Corrections of LPP capabilities on DL-RPS"	, CATT.

[8]	R2-2211421, "Corrections of LPP capabilities on DL-RPS", CATT.

[9]	R2-2212229, "Correction to DL-PRS Search Window calculation", Qualcomm Incorporated.

[10]	R2-2212231, "Correction to DL-PRS Search Window calculation", Qualcomm Incorporated	.

[11]	R2-2212347, "Correction of NR DL-PRS BeamInfo attribute associated-DL-PRS-ID field description", Ericsson.

[12]	R2-2212348, "Correction of NR DL-PRS BeamInfo attribute associated-DL-PRS-ID field description", Ericsson.

[13]	R2-2212349, "Adding missing and correcting GNSS Types in GNSS-SSR-OrbitCorrections", 	Ericsson, u-blox, Swift Navigation.

[14]	R2-2212350, "Adding missing and correcting GNSS Types in GNSS-SSR-OrbitCorrections",	Ericsson, u-blox, Swift Navigation.

[15]	R2-2212351, "Clarifying the meaning of GNSS IOD SSR to avoid different interpretations",	Ericsson, u-blox, Swift Navigation.

[16]	R2-2212352, "Clarifying the meaning of GNSS IOD SSR to avoid different interpretations", 	Ericsson, u-blox, Swift Navigation.

[17]	R2-2212353, "Correcting field description and definition of GNSS-SSR-URA", Ericsson, u-blox, Swift Navigation.

[18]	R2-2212354, "Correcting field description and definition of GNSS-SSR-URA	", Ericsson, u-blox, Swift Navigation.

[19]	R2-2212507, "Addition of missing yaw angle and rate in SSR Phase Bias message (TS 37.355)",	Swift Navigation, Mitsubishi Electric Corporation, Ericsson	.

[20]	R2-2212511, "Addition of missing yaw angle and rate in SSR Phase Bias message (TS 37.355)",	Swift Navigation, Mitsubishi Electric Corporation, Ericsson.

References:

[21]	R2-2208119, "Issues with DL-PRS Search Window Definitions", Qualcomm Incorporated.

[22]	R2-2208962, "Summary of [AT119-e][407][POS] Rel-15/16 LPP (Qualcomm)", Qualcomm Incorporated.

[23]	R1-2210342, "Summary #1 of PRS Search Window Issue", Moderator (Qualcomm).

[24]	R1-2202763, "Updated RAN1 UE features list for Rel-16 NR after RAN1#108-e", Moderator (NTT DOCOMO, INC.).

[25]	R2-2208962, "Report from session on positioning and sidelink relay", RAN2#119-e, Session Chair (MediaTek).

[26]	RP-200217, "Revised WID: NR Positioning Support".

# 2.	Essential Corrections to NR Positioning Technologies

## 2.1	DL-PRS Search Window

| [1] | R2-2211150 | LS on DL PRS search window (R1-2210618; contact: Qualcomm) | RAN1 |
| --- | --- | --- | --- |
| [9] | R2-2212229 | Correction to DL-PRS Search Window calculation | Qualcomm Incorporated |
| [10] | R2-2212231 | Correction to DL-PRS Search Window calculation | Qualcomm Incorporated |

At RAN1#110bis-e, RAN1 discussed the "DL-PRS Search Window Issue" originally brought up in RAN2 at meeting #119-e in tdoc R2-2208119 [21] and discussed in [AT119-e][407][POS] [22].

RAN1 discussed the issue in [23] and provided their conclusion in the LS [1] to RAN2 as follows:

RAN1 would like to inform RAN2 about the following: - It is RAN1 understanding that sfn-Offset and integerSubframeOffset in NR-DL-PRS-SFN0-Offset is not required to be specified as round-down or round-to-the-closest, but should be set a value by the network to ensure that the UE assumption is correct when to determine the search window centre. The definition of sfn-Offset and integerSubframeOffset could be changed to reflect that intention. - An example of potential change in TS 37.355 is shown below: nr-DL-PRS-SFN0-Offset This field specifies the time offset of the SFN#0 slot#0 for the given TRP with respect to SFN#0 slot#0 of the assistance data reference TRP and comprises the following subfields: - sfn-Offset specifies the SFN offset at the TRP antenna location between the assistance data reference TRP and this neighbour TRP. The offset corresponds to the number of full radio frames counted from the beginning of a radio frame #0 of the assistance data reference TRP to the beginning of the closest subsequentradio frame #0 of this neighbour TRP. - integerSubframeOffset specifies the frame boundary offset at the TRP antenna location between the assistance data reference TRP and this neighbour TRP counted in full subframes. The offset corresponds to the number of full subframes counted from the beginning of a subframe #0 of the assistance data reference TRP to the beginning of the closest subsequent subframe #0 of this neighbour TRP. NOTE: Network shall set the value in accordance with the defined search window for the target device using nr-DL-PRS-ExpectedRSTD and nr-DL-PRS-ExpectedRSTD-Uncertainty. nr-DL-PRS-ExpectedRSTD This field indicates the RSTD value that the target device is expected to measure between this TRP and the assistance data reference TRP. The nr-DL-PRS-ExpectedRSTD field takes into account the expected propagation time difference as well as transmit time difference of PRS positioning occasions between the two TRPs. The resolution is 4Ts, with Ts=1/(15000*2048) seconds.. nr-DL-PRS-ExpectedRSTD-Uncertainty This field indicates the uncertainty in nr-DL-PRS-ExpectedRSTD value. The uncertainty is related to the location server′s a-priori estimate of the target device location. The nr-DL-PRS-ExpectedRSTD and nr-DL-PRS-ExpectedRSTD-Uncertainty together define the search window for the target device. The resolution R is - Ts if all PRS resources are in frequency range 2, - 4Ts otherwise, with Ts=1/(15000*2048) seconds. The target device may assume that the beginning of the subframe for the PRS of this TRP is received within the search window of size - [-nr-DL-PRS-ExpectedRSTD-UncertaintyR ; nr-DL-PRS-ExpectedRSTD-UncertaintyR] centred at TREF+1 millisecondN+nr-DL-PRS-ExpectedRSTD4Ts, where TREF is the reception time of the beginning of the subframe for the PRS of the assistance data reference TRP at the target device antenna connector, and N can be calculated based on - nr-DL-PRS-SFN0-Offset - dl-PRS-Periodicity-and-ResourceSetSlotOffset - dl-PRS-ResourceSlotOffset..

Rapporteur's Comment:

-	The LPP CRs in [9] (Rel-16, Cat F) and [10] (Rel-17, Cat A) follow the text proposal from RAN1 in the LS [1].

Proposal 1:	The CRs in 
R2-2212229, Correction to DL-PRS Search Window calculation, Qualcomm Incorporated, CR Rel-16	37.355	16.8.0 0391 - F NR_pos-Core
R2-2212231, Correction to DL-PRS Search Window calculation, Qualcomm Incorporated, CR Rel-17	37.355	17.2.0 0392 – A NR_pos-Core
are essential corrections.

## 2.2	DL-PRS Capability

| [7] | R2-2211420 | Corrections of LPP capabilities on DL-RPS | CATT |
| --- | --- | --- | --- |
| [8] | R2-2211421 | Corrections of LPP capabilities on DL-RPS | CATT |

The following DL-PRS capabilities as implemented in LPP are not in agreement with the RAN1 feature list in R1-2202763 [24]:

| 13-1 | Common DL PRS Processing Capability | - Maximum DL PRS bandwidth in MHz, which is supported and reported by UE. a) FR1 bands: {5, 10, 20, 40, 50, 80, 100} b) FR2 bands: {50, 100, 200, 400} - DL PRS buffering capability: Type 1 or Type 2 - Type 1 – sub-slot/symbol level buffering - Type 2 – slot level buffering - Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. - T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms - N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms - Max number of DL PRS resources that UE can process in a slot under it - FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz - FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz Note: The above parameters are reported assuming a configured measurement gap and a maximum ratio of measurement gap length (MGL) / measurement gap repetition period (MGRP) of no more than 30%. |
| --- | --- | --- |
| 13-2 | DL PRS Resources for DL AoD | - Max number of DL PRS Resource Sets per TRP per frequency layer supported by UE. Values = {1, 2} - Max number of TRPs across all positioning frequency layers per UE. Values = {4, 6, 12, 16, 24, 32, 64, 128, 256} - Max number of positioning frequency layers UE supports Values = {1, 2, 3, 4} |

The yellow highlighted items above are missing in LPP IE NR-DL-PRS-ProcessingCapability and IE NR-DL-PRS-ResourcesCapability.

Rapporteur's Comment:

-	The missing values for 6 and 12 MHz are added to maxNumOfDL-PRS-ResProcessedPerSlot-r16 in IE NR-DL-PRS-ProcessingCapability in the LPP CRs [7][8].

-	The missing value 24 TRPs is added to maxNrOfTRP-AcrossFreqs-r16 in IE NR-DL-PRS-ResourcesCapability in the LPP CRs [7][8].

-	Therefore, the CRs in [7][8] are in agreement with the RAN1 feature list in [24] and essential corrections.

-	CR Cover sheet issues:

-	Inter-operability: "If the UE implement the CR and the LMF does not, there is interoperability issue that LMF won't decode the message correctly."
There are no issues with 'decoding of the message'. The proposed correction in [7][8] is backwards compatible from ASN point of view. However, if a UE would signal the missing values a legacy LMF would not be able to receive the complete UE capability.

Proposal 2:	The CRs in 
R2-2211420, Corrections of LPP capabilities on DL-RPS, CATT, CR Rel-16 37.355 16.8.0 0388 -	 F	NR_pos-Core
R2-2211421, Corrections of LPP capabilities on DL-RPS, CATT, CR Rel-17 37.355 17.2.0 0389 - A	NR_pos-Core
are essential corrections. Update the inter-operability statement on the Cover Sheet.

## 2.3	associated-DL-PRS-ID in IE NR-DL-PRS-BeamInfo

| [11] | R2-2212347 | Correction of NR DL-PRS BeamInfo attribute associated-DL-PRS-ID field description | Ericsson |
| --- | --- | --- | --- |
| [12] | R2-2212348 | Correction of NR DL-PRS BeamInfo attribute associated-DL-PRS-ID field description | Ericsson |

According to [11],[12], "the field description of NR DL-PRS BeamInfo attribute associated-DL-PRS-ID is incorrect, disallowing the combination of associated-DL-PRS-ID and lcs-GCS-TranslationParameter which is needed to realize the function" with the consequences if not approved: "Incorrect specification in relation to the intended and agreed function of the feature when agreed. Assistance data for DL-AoD can become prohibitively large and make it impossible to support DL-AoD via posSIB due to the extreme DL-AoD DL-PRS-BeamInfo IE size."

The issue has been described and discussed in [22], section 2.3 and is not repeated here. Please see [22].

Rapporteur's Comment:

-	The proposed field description changes in [11][12] are not in full agreement with the corresponding field description in Rel-17 IE NR-TRP-BeamAntennaInfo.

-	There seems no additional information provided to support the claims/Reason for Change compared to what has been discussed before (e.g., [22]).

-	Although, Rapporteur shares the concern raised in [11][12], there needs to be full consensus for a (very late) change to a frozen release. Given that there are no additional/new information provided to support the CR, the Proposal from previous discussion [22][25] is repeated below.

Proposal 3:	RAN2 to discuss and decide whether the CRs in 
R2-2212347, Correction of NR DL-PRS BeamInfo attribute associated-DL-PRS-ID field description, Ericsson, CR Rel-16 37.355 16.8.0 0393 – F NR_pos-Core
R2-2212348, Correction of NR DL-PRS BeamInfo attribute associated-DL-PRS-ID field description, 	Ericsson, CR Rel-17 37.355 17.2.0 0394 – A NR_pos-Core
are essential corrections or not.

# 3.	Essential Corrections to GNSS SSR Assistance Data

## 3.1	Missing GNSS Types in GNSS-SSR-OrbitCorrections

| [13] | R2-2212349 | Adding missing and correcting GNSS Types in GNSS-SSR-OrbitCorrections | Ericsson, u-blox, Swift Navigation |
| --- | --- | --- | --- |
| [14] | R2-2212350 | Adding missing and correcting GNSS Types in GNSS-SSR-OrbitCorrections | Ericsson, u-blox, Swift Navigation |

According to [13],[14], "the SSR orbit corrections are relative to broadcasted Ephemeris and should be provided for GNSS Types GLONASS, Galileo, BeiDou. However, it is only specified for GPS and QZSS" and the Note 2 for GNSS-SSR-OrbitCorrections is proposed to be updated as follows:

NOTE 2: In the cases that gnss-ID indicates 'gps', 'qzss', galileo, glonass, bds, the iod refers to the specific broadcast ephemeris (GPS L1 C/A, QZSS L1 C/A, Galileo I/NAV, GLONASS L1 C/A, BDS B1 I, respectively, in table GNSS to iod Bit String(11) relation in IE GNSS-NavigationModel).

Rapporteur's Comment:

-	Rapporteur believes that the CRs in [13],[14] are not correct.

-	As specified in LPP before each SSR IE, the SSR parameters are used as specified in RTCM Standard 10403.3 and apply to all GNSSs.

-	The proposed changes in NOTE 2 "QZSS L1 C/A, Galileo I/NAV, GLONASS L1 C/A, BDS B1 I" are not entries in Table "GNSS to iod Bit String(11) relation" in IE GNSS-NavigationModel:

GNSS to iod Bit String(11) relation

| GNSS | iod Bit String(11) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit 1 (MSB) | Bit 2 | Bit 3 | Bit 4 | Bit 5 | Bit 6 | Bit 7 | Bit 8 | Bit 9 | Bit 10 | Bit 11 (LSB) |
| GPS L1/CA | '0' | Issue of Data, Clock [4] |  |  |  |  |  |  |  |  |  |
| Modernized GPS | toe (seconds, scale factor 300, range 0 – 604500) [4,5,6] |  |  |  |  |  |  |  |  |  |  |
| SBAS | '0' | '0' | '0' | Issue of Data ([10], Message Type 9) |  |  |  |  |  |  |  |
| QZSS QZS-L1 | '0' | Issue of Data, Clock [7] |  |  |  |  |  |  |  |  |  |
| QZSS QZS-L1C/L2C/L5 | toe (seconds, scale factor 300, range 0 – 604500) [7] |  |  |  |  |  |  |  |  |  |  |
| GLONASS | '0' | '0' | '0' | '0' | tb (minutes, scale factor 15) [9] |  |  |  |  |  |  |
| Galileo | '0' | IODnav [8] |  |  |  |  |  |  |  |  |  |
| BDS B1I/B3I | 11 MSB bits of toe (seconds, scale factor 512, range 0 – 604672) [23], [50] |  |  |  |  |  |  |  |  |  |  |
| BDS B1C/B2a | '0' | Issue of Data, Clock [39], [49] |  |  |  |  |  |  |  |  |  |
| NavIC | 11 MSB bits of toe (seconds, scale factor 512) [38] |  |  |  |  |  |  |  |  |  |  |

-	The reason why NOTE 2 is needed is because for GNSS-ID 'gps' and 'qzss', there are two rows in table "GNSS to iod Bit String(11) relation" and it needs to be clarified which row the iod field refers to. There is no ambiguity for the other GNSSs.

iod This field specifies the IOD value of the broadcast ephemeris for which the orbit corrections are valid (see IE GNSS-NavigationModel). NOTE 2.

-	However, with the introduction of BDS-B1C/B2a/B3I, there is now the same ambiguity as for 'gps' and 'qzss'. However, this requires a different CR with different Reason for Change (and probably different WI code).

Proposal 4:	The CRs in 
R2-2212349, Adding missing and correcting GNSS Types in GNSS-SSR-OrbitCorrections, Ericsson, u-blox, Swift Navigation, CR Rel-16 37.355 16.8.0 0395 – F NR_pos-Core
R2-2212350, Adding missing and correcting GNSS Types in GNSS-SSR-OrbitCorrections, Ericsson, u-blox, Swift Navigation, CR Rel-17 37.355 17.2.0 0396 – A NR_pos-Core
are not essential corrections.

## 3.2	Meaning of GNSS IOD SSR

| [15] | R2-2212351 | Clarifying the meaning of GNSS IOD SSR to avoid different interpretations | Ericsson, u-blox, Swift Navigation |
| --- | --- | --- | --- |
| [16] | R2-2212352 | Clarifying the meaning of GNSS IOD SSR to avoid different interpretations | Ericsson, u-blox, Swift Navigation |

According to [15][16], "the GNSS IOD SSR field description is not clear to ensure the UE handling on whether to combine or refrain to combine the different SSR IEs" and the following change is proposed (at multiple places in LPP):

iod-ssr This field specifies the Issue of Data number for the SSR data. A change of iod-ssr is used to indicate a change in the SSR generating configuration. A set of SSR IEs with the same IOD SSR is consistent and may be used together by the target device, while the device shall refrain from combining SSR IEs with different IOD SSR.

Rapporteur's Comment:

-	The changes in [15][16] seem to introduce a new UE requirement to a frozen Release ("shall").

-	The LPP SSR IEs are used as specified in RTCM Standard 10403.3 and IS-QZSS-L6-001. For example, IS-QZSS-L6-001:

The similar text can be found in RTCM Standard 10403.3, DF413 and seems consistent with LPP.

-	Rapporteur has not found the proposed changes/text in RTCM Standard 10403.3 or IS-QZSS-L6-001.

-	If the proposed change in [15][16] is common understanding, an informative Note may be more appropriate (probably sufficient for Rel-17 with "magic sentence").

Proposal 5:	RAN2 to discuss and decide whether the CRs in 
R2-2212351, Clarifying the meaning of GNSS IOD SSR to avoid different interpretations, Ericsson, u-blox, Swift Navigation, CR Rel-16 37.355 16.8.0 0397 – F NR_pos-Core
R2-2212352, Clarifying the meaning of GNSS IOD SSR to avoid different interpretations, Ericsson, u-blox, Swift Navigation, CR Rel-17 37.355 17.2.0 0398 – A NR_pos-Core
are essential corrections or not.

## 3.3	Definition of GNSS-SSR-URA

| [17] | R2-2212353 | Correcting field description and definition of GNSS-SSR-URA | Ericsson, u-blox, Swift Navigation |
| --- | --- | --- | --- |
| [18] | R2-2212354 | Correcting field description and definition of GNSS-SSR-URA | Ericsson, u-blox, Swift Navigation |

According to [17][18], "the SSR URA formula is incorrect and needs to be corrected. The formula

SSR URA  mm≤3CLASS1+VALUE4-1    [mm]

is incorrect. The "-1" should be outside the parenthesis".

Rapporteur's Comment:

-	According to RTCM Standard 10403.3:

Data Field Notes

SSR User Range Accuracy (URA) (1 sigma) for a range correction computed from complete SSR set as disseminated by RTCM SSR messages. The URA is represented by a combination of URA_CLASS and URA_VALUE. The 3 MSB define the URA_CLASS with a range of 0-7 and the 3 LSB define the URA_VALUE with a range of 0-7. ### The URA is computed by: URA mm≤3URA_CLASS1+URA_VALUE4-1 [mm] Special cases are: all bits zero “000 000” =URA undefined/unknown all bits set “111 111” =URA > 5466.5 mm

-	Cover sheet issues:

-	The Rel-17 CR should be Cat A.

Proposal 6:	The CRs in 
R2-2212353, Correcting field description and definition of GNSS-SSR-URA, Ericsson, u-blox, Swift Navigation, CR Rel-16 37.355 16.8.0 0399 – F NR_pos-Core
R2-2212354, Correcting field description and definition of GNSS-SSR-URA, Ericsson, u-blox, Swift Navigation, CR Rel-17 37.355 17.2.0 0400 – A NR_pos-Core
are essential corrections. Correct the Rel-17 CR Category on the Cover Sheet.

## 3.4	Satellite Yaw Angle

36.305 CRs:

| [2] | R2-2212516 | Update Stage 2 SSR Phase Bias description to include yaw 36.305 Rel-16 | Swift Navigation, Mitsubishi Electric Corporation, Ericsson |
| --- | --- | --- | --- |
| [3] | R2-2212518 | Update Stage 2 SSR Phase Bias description to include yaw 36.305 Rel-17 | Swift Navigation, Mitsubishi Electric Corporation, Ericsson |

38.305 CRs:

| [4] | R2-2212535 | Update Stage 2 SSR Phase Bias description to include yaw 38.305 Rel-16 | Swift Navigation, Mitsubishi Electric Corporation, Ericsson |
| --- | --- | --- | --- |
| [5] | R2-2212536 | Update Stage 2 SSR Phase Bias description to include yaw 38.305 Rel-17 | Swift Navigation, Mitsubishi Electric Corporation, Ericsson |

37.355 CRs:

| [19] | R2-2212507 | Addition of missing yaw angle and rate in SSR Phase Bias message (TS 37.355) | Swift Navigation, Mitsubishi Electric Corporation, Ericsson |
| --- | --- | --- | --- |
| [20] | R2-2212511 | Addition of missing yaw angle and rate in SSR Phase Bias message (TS 37.355) | Swift Navigation, Mitsubishi Electric Corporation, Ericsson |

According to [2]-[5], [19],[20]:

"Yaw information is currently missing in the SSR assistance data but is needed to compute the phase wind-up. Presently the specification implicitly assumes that the Network will only send zero yaw angle SSR corrections which is not true in practice for different correction providers. This change enables the Network to send non-zero yaw angle SSR corrections while also clarifying explicitly the UE behaviour if no yaw angle is set; more detail in discussion paper: R2-2212544"

and the following Stage 2 changes are proposed:

##### 8.1.2.1.24 SSR Phase Bias SSR Phase Bias provides the GNSS receiver with the GNSS signal phase bias that are added to the carrier phase measurements of the corresponding signal to get corrected phase ranges, including yaw information to correct for the phase wind-up. An indicator used to count events when phase bias is discontinuous is provided. An optional indicator is also provided to indicate whether fixed, widelane fixed or float PPP-RTK positioning modes are supported on a per signal basis. NOTE 1: On the UE side, phase bias corrections of appropriate type are needed to restore the integer nature of the phase ambiguities in PPP-RTK. Their absence will affect the quality of the positioning solution and prevent a fast convergence time. NOTE 2: PPP-RTK Fixed position mode corresponds to the UE fixing the carrier phase ambiguity to an integer value. The PPP-RTK Widelane Fixed positioning mode corresponds to forming the widelane combination of carrier phase measurements and fixing the resulting ambiguity as an integer value. In PPP-RTK Float positioning mode the carrier phase ambiguity is not treated as an integer value. NOTE 3: If yaw angle field is not set then the network must send SSR corrections that are consistent with a zero yaw angle.

The corresponding proposed LPP changes are as follows.

Rel-16:

SSR-PhaseBiasSignalElement-r16 ::= SEQUENCE { signal-and-tracking-mode-ID-r16 GNSS-SignalID, phaseBias-r16 INTEGER (-16384..16383), phaseDiscontinuityIndicator-r16 INTEGER (0..3), phaseBiasIntegerIndicator-r16 INTEGER (0..3) OPTIONAL, -- Need OP ..., [[ yawAngle-r16 INTEGER (0..511) OPTIONAL, -- Cond yawRate yawRate-r16 INTEGER (-128..127) OPTIONAL -- Need OP ]] }

Rel-17:

SSR-PhaseBiasSignalElement-r16 ::= SEQUENCE { signal-and-tracking-mode-ID-r16 GNSS-SignalID, phaseBias-r16 INTEGER (-16384..16383), phaseDiscontinuityIndicator-r16 INTEGER (0..3), phaseBiasIntegerIndicator-r16 INTEGER (0..3) OPTIONAL, -- Need OP ..., [[ yawAngle-r16 INTEGER (0..511) OPTIONAL, -- Cond yawRate yawRate-r16 INTEGER (-128..127) OPTIONAL -- Need OP ]], [[ ssr-IntegrityPhaseBiasBounds-r17 SSR-IntegrityPhaseBiasBounds-r17 OPTIONAL -- Need OR ]] }

yawAngle Yaw angle used for computation of phase wind-up correction and partial orientation for use with satellite antenna phase center data. The yaw angle is defined as the rotation angle around the satellites z-axis which is pointing towards the center of the earth. The reference direction is the yaw origin, a unit vector to form an orthogonal basis for the orbit plane and is in the general direction of the satellite velocity vector. Units of 1/256 semi-circles. If yawAngle field is not present, then it is interpreted as having value 0.

yawRate Rate of change of the yaw angle. Units of 1/8192 semi-circles/second. If yawRate field is not present, then it is interpreted as having value 0.

Additional background information is provided in [6].

Rapporteur's Comment:

-	The proposed changes are not backwards compatible, both, from functional point of view and from ASN.1.

-	According to [19],[20], 
		"If yawAngle field is not present, then it is interpreted as having value 0." 
However, a legacy UE will not comprehend the yawAngle extension and will always see the yawAngle field being not present. A legacy UE will always assume the yawAngle field is zero, independent on how the value is set by an updated LMF (see also below).

-	Since the SSR-PhaseBiasSignalElement-r16 has already been extended in Rel-17, the Rel-16 extension results in non-backwards compatible ASN.1 for Rel-17.

-	The Rel-16 SSR additions in LPP are defined according to the "Compact SSR" messages specified in IS-QZSS-L6-001, according to the Rel-16 WI objective [26]:

"Define extensions of LPP protocol to support GNSS SSR (PPP-RTK support) based on the "Compact SSR" definitions specified for QZSS for both LTE and NR [RAN2]

Note: Both, PPP and RTK, are already supported in Rel-15 LPP. The extensions are for PPP-RTK which requires additional info in Rel-16."

-	The "compact SSR" assume a satellite yaw angle of zero; see IS-QZSS-L6-001, section 5.5.6:

"The estimated compact SSR parameters of CLAS are free of reference station site displacements , phase wind up effect, receiver antenna PCV , inter system bias , quarter cycle shifts and etc. Users have to apply corresponding user dependent corrections when using compact SSR parameters in the positioning calculation process .

[…]

Note that in the phase wind up computation, a satellite yaw angle used for the computation is always zero. It is not necessary to use a satellite attitude model."

Therefore, legacy UEs always assume the satellite yaw has been considered when providing the SSR assistance data. I.e., an LMF always provides SSR corrections that are consistent with a zero-yaw angle.

-	Non-compact SSR specifications may not require the zero-yaw assumption, as noted in [6]. E.g., the RTCM SSR standard under development includes the satellite yaw in the assistance data (note, the "compact SSR" (as supported in LPP) are also under discussion in RTCM) . However, when the "compact SSR" assistance data have been added in Rel-16, Rapporteur believes that the common understanding was that additional SSR assistance data may be defined in LPP once an updated RTCM standard is available. This may then also require a dedicated WI, dependent on the foreseen amount of changes required.

-	To make the proposal in [2]-[5], [19],[20] backwards compatible, Rapporteur believes a new IE for the SSR Phase Bias with non-zero yaw assumption would be required, together with UE capabilities, posSIB, etc. (as usual). In this way, UEs may support the Rel-16 "compact SSR" version and/or the new SSR version, which would then be fully backwards compatible. However, this does not look like an essential correction to Rel-16.

Proposal 7:	RAN2 to discuss and decide whether the CRs in 
R2-2212516, Update Stage 2 SSR Phase Bias description to include yaw, Swift Navigation, Mitsubishi Electric Corporation, Ericsson, CR Rel-16 36.305 16.4.0 0111 – F NR_pos-Core
R2-2212518, Update Stage 2 SSR Phase Bias description to include yaw, Swift Navigation, Mitsubishi Electric Corporation, Ericsson, CR Rel-17 36.305 17.2.0 0112 – A NR_pos-Core
R2-2212535, Update Stage 2 SSR Phase Bias description to include yaw, Swift Navigation, Mitsubishi Electric Corporation, Ericsson, CR Rel-16 38.305 16.8.0 0113 – F NR_pos-Core
R2-2212536, Update Stage 2 SSR Phase Bias description to include yaw, Swift Navigation, Mitsubishi Electric Corporation, Ericsson, CR Rel-17 38.305 17.2.0 0114 – A NR_pos-Core 
R2-2212507, Addition of missing yaw angle and rate in SSR Phase Bias message (TS 37.355), Swift Navigation, Mitsubishi Electric Corporation, Ericsson, CR Rel-16 37.355 16.8.0 0401 – F NR_pos-Core
R2-2212511, Addition of missing yaw angle and rate in SSR Phase Bias message (TS 37.355),Swift Navigation, Mitsubishi Electric Corporation, Ericsson, CR Rel-17 37.355 17.2.0 0402 – A NR_pos-Core
are essential corrections or not.

## 3.5	Satellite Antenna Phase Centre Corrections (APC)

| [6] | R2-2212544 | Discussion and TP on Yaw Angle and Antenna Phase Center corrections for SSR assistance data | Swift Navigation, Mitsubishi Electric Corporation, Ericsson |
| --- | --- | --- | --- |

The contribution [6] is related to the CRs discussed in section 3.4 above and proposes in addition the specification of APC assistance data. [6] makes the following observations:

Observation 1:	The UE needs a way of ensuring the measured GNSS signals (at the UE) and received corrections (from the Network) can both be referenced to a common APC with respect to the satellite.

Observation 2:	For GNSS, the satellite APC is currently hardcoded or manually updated at the UE which is not scalable and can impact interoperability and precision. No Yaw angle is specified in LPP currently.

Observation 3:	Satellite APCs are important for absolute positioning (e.g. GNSS SSR) to ensure there is interoperability when using Network-provided SSR corrections.

Observation 4:	Incorrect handling of APC can lead to lower fix rates, incorrect fixing and can impact position error.

Observation 5:	This proposal only defines the generic message for sending APC and Yaw information; it does not attempt to specify the GNSS satellite APC models in the specification itself (these models are derived externally).

Observation 6:	Yaw angle and Phase Bias are heavily interrelated and by convention the yaw angle is sent as part of the Phase Bias message in other SSR standards.

Accordingly, the following Proposals are made in [6]:

Proposal 1:	Agree to capture the satellite yaw fields in TS 36.305/38.305 and TS 37.355.
Refer to CRs R2-2212516 (36.305 v16.4) and R2-2212518 (36.305 v17.2)
Refer to CRs R2-2212535 (38.305 v16.8) and R2-2212536 (38.305 v17.2)
Refer to CRs R2-2212507 (37.355 v16.8) and R2-2212511 (37.355 v17.2)

Proposal 2:	Discuss and agree to add the satellite Antenna Phase Centre message in the SSR assistance data.
LS to RAN3 to agree on new posSibType2-xy for GNSS-SSR-SatelliteAPC in TS 36.455/38.455.
Agree to develop the corresponding CRs for TS 37.355, TS 36.305/38.305, TS 36.331/38.331 and TS 36.455/38.455

Rapporteur's Comment:

-	Proposal 1 is discussed in section 3.4 above. As mentioned in section 3.4, to make the CRs in Proposal 1 backwards compatible, the same specification effort as in Proposal 2 would be required (new IE, new posSIB, etc.).

-	Antenna Phase Centre assistance data are not part of the current SSR messages in RTCM Standard 10403.3 and IS-QZSS-L6-001, which both are supported in LPP.

-	Future SSR standards (e.g., RTCM, IGS) may include APC correction information.

-	Similar to the comments made in section 3.4 above, additional SSR assistance data may be useful/sensible. However, this may require a separate WI and probably a standard baseline (e.g., RTCM, IGS). Therefore, the Proposals in [6] seem not essential corrections.

Proposal 7:	RAN2 to discuss and decide whether the Proposal 2 in
R2-2212544, Discussion and TP on Yaw Angle and Antenna Phase Center corrections for SSR assistance data, Swift Navigation, Mitsubishi Electric Corporation, Ericsson discussion Rel-16	NR_pos-Core, is an essential corrections or not:

"Proposal 2: Discuss and agree to add the satellite Antenna Phase Center message in the SSR assistance data.
LS to RAN3 to agree on new posSibType2-xy for GNSS-SSR-SatelliteAPC in TS 36.455/38.455.
Agree to develop the corresponding CRs for TS 37.355, TS 36.305/38.305, TS 36.331/38.331 and TS 36.455/38.455".

# 4.	Summary

[DL-PRS Search Window]

Proposal 1:	The CRs in 
R2-2212229, Correction to DL-PRS Search Window calculation, Qualcomm Incorporated, CR Rel-16	37.355	16.8.0 0391 - F NR_pos-Core
R2-2212231, Correction to DL-PRS Search Window calculation, Qualcomm Incorporated, CR Rel-17	37.355	17.2.0 0392 – A NR_pos-Core
are essential corrections.

[DL-PRS Capability]

Proposal 2:	The CRs in 
R2-2211420, Corrections of LPP capabilities on DL-RPS, CATT, CR Rel-16 37.355 16.8.0 0388 -	 F	NR_pos-Core
R2-2211421, Corrections of LPP capabilities on DL-RPS, CATT, CR Rel-17 37.355 17.2.0 0389 - A	NR_pos-Core
are essential corrections. Update the inter-operability statement on the Cover Sheet.

[associated-DL-PRS-ID in IE NR-DL-PRS-BeamInfo]

Proposal 3:	RAN2 to discuss and decide whether the CRs in 
R2-2212347, Correction of NR DL-PRS BeamInfo attribute associated-DL-PRS-ID field description, Ericsson, CR Rel-16 37.355 16.8.0 0393 – F NR_pos-Core
R2-2212348, Correction of NR DL-PRS BeamInfo attribute associated-DL-PRS-ID field description, 	Ericsson, CR Rel-17 37.355 17.2.0 0394 – A NR_pos-Core
are essential corrections or not.

[Missing GNSS Types in GNSS-SSR-OrbitCorrections]

Proposal 4:	The CRs in 
R2-2212349, Adding missing and correcting GNSS Types in GNSS-SSR-OrbitCorrections, Ericsson, u-blox, Swift Navigation, CR Rel-16 37.355 16.8.0 0395 – F NR_pos-Core
R2-2212350, Adding missing and correcting GNSS Types in GNSS-SSR-OrbitCorrections, Ericsson, u-blox, Swift Navigation, CR Rel-17 37.355 17.2.0 0396 – A NR_pos-Core
are not essential corrections.

[Meaning of GNSS IOD SSR]

Proposal 5:	RAN2 to discuss and decide whether the CRs in 
R2-2212351, Clarifying the meaning of GNSS IOD SSR to avoid different interpretations, Ericsson, u-blox, Swift Navigation, CR Rel-16 37.355 16.8.0 0397 – F NR_pos-Core
R2-2212352, Clarifying the meaning of GNSS IOD SSR to avoid different interpretations, Ericsson, u-blox, Swift Navigation, CR Rel-17 37.355 17.2.0 0398 – A NR_pos-Core
are essential corrections or not.

[Definition of GNSS-SSR-URA]

Proposal 6:	The CRs in 
R2-2212353, Correcting field description and definition of GNSS-SSR-URA, Ericsson, u-blox, Swift Navigation, CR Rel-16 37.355 16.8.0 0399 – F NR_pos-Core
R2-2212354, Correcting field description and definition of GNSS-SSR-URA, Ericsson, u-blox, Swift Navigation, CR Rel-17 37.355 17.2.0 0400 – A NR_pos-Core
are essential corrections. Correct the Rel-17 CR Category on the Cover Sheet.

[Satellite Yaw Angle]

Proposal 7:	RAN2 to discuss and decide whether the CRs in 
R2-2212516, Update Stage 2 SSR Phase Bias description to include yaw, Swift Navigation, Mitsubishi Electric Corporation, Ericsson, CR Rel-16 36.305 16.4.0 0111 – F NR_pos-Core
R2-2212518, Update Stage 2 SSR Phase Bias description to include yaw, Swift Navigation, Mitsubishi Electric Corporation, Ericsson, CR Rel-17 36.305 17.2.0 0112 – A NR_pos-Core
R2-2212535, Update Stage 2 SSR Phase Bias description to include yaw, Swift Navigation, Mitsubishi Electric Corporation, Ericsson, CR Rel-16 38.305 16.8.0 0113 – F NR_pos-Core
R2-2212536, Update Stage 2 SSR Phase Bias description to include yaw, Swift Navigation, Mitsubishi Electric Corporation, Ericsson, CR Rel-17 38.305 17.2.0 0114 – A NR_pos-Core 
R2-2212507, Addition of missing yaw angle and rate in SSR Phase Bias message (TS 37.355), Swift Navigation, Mitsubishi Electric Corporation, Ericsson, CR Rel-16 37.355 16.8.0 0401 – F NR_pos-Core
R2-2212511, Addition of missing yaw angle and rate in SSR Phase Bias message (TS 37.355),Swift Navigation, Mitsubishi Electric Corporation, Ericsson, CR Rel-17 37.355 17.2.0 0402 – A NR_pos-Core
are essential corrections or not.

[Satellite Antenna Phase Centre Corrections]

Proposal 7:	RAN2 to discuss and decide whether the Proposal 2 in
R2-2212544, Discussion and TP on Yaw Angle and Antenna Phase Center corrections for SSR assistance data, Swift Navigation, Mitsubishi Electric Corporation, Ericsson discussion Rel-16	NR_pos-Core, is an essential corrections or not:

"Proposal 2: Discuss and agree to add the satellite Antenna Phase Center message in the SSR assistance data.
LS to RAN3 to agree on new posSibType2-xy for GNSS-SSR-SatelliteAPC in TS 36.455/38.455.
Agree to develop the corresponding CRs for TS 37.355, TS 36.305/38.305, TS 36.331/38.331 and TS 36.455/38.455".