3GPP TSG RAN WG2#120	R2-2211115

Toulouse, 14th - 18th November, 2022

3GPP TSG RAN WG4 Meeting #104-e	R4-2214484

Electronic Meeting, 15th - 26th August 2022

Title:	Reply LS on configuring margin for 1 Rx RedCap UEs

Response to:	Reply LS on configuring margin for 1 Rx RedCap UEs

Release:	Rel-17

Work Item:	NR_redcap-Core

Source:	RAN4

To:	RAN2

Cc:

Contact person:	Muhammad Kazmi

Muhammad.kazmi@ericsson.com

Send any reply LS to:    3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: None

# 1	Overall description

RAN4 would like to thank RAN2 for their reply LS on configuring margin for 1 Rx RedCap UEs in R2-2206504/R4-2211523. RAN4 has discussed RAN2 questions and reached the following conclusions:

- A RedCap UE with 1 Rx branch applies the offset to the following broadcasted cell-specific RSRP thresholds related to L3 measuremets in RRC_IDLE/INACTIVE state:

  - rsrp-ThresholdSSB,

  - msgA-RSRP-ThresholdSSB,

  - msgA-RSRP-Threshold,

  - absThreshSS-BlocksConsolidation,

  - sdt-RSRP-Threshold.

  - the following cell-specific RSRP thresholds used for Rel-16 low mobility and/or not at cell edge conditions, and Rel-17 stationary and not at cell edge conditions for RRC IDLE/INACTIVE state:

    - s-SearchDeltaP-r16,

    - s-SearchDeltaP-Stationary-r17

    - s-SearchThresholdP-r16,

    - s-SearchThresholdQ-r16,

    - s-SearchThresholdP2-r17,

    - s-SearchThresholdQ2-r17.

  - the following cell (re)selection thresholds and ranking:

    - Qrxlevmin and Qqualmin

- The offset is a fixed value in dB in the above cell-specific RSRP thresholds and will be specified in TS 38.133. The magnitude of the exact offset value is [1] dB. It is not expected to introduce new RAN2 signalling.

RAN4 also would like to clarify that based on the latest agreement as communicated in this LS, the offset does not apply to rsrp-ThresholdBFR and rsrp-ThresholdCSI-RS.

Note: RAN2 is also invited to inform if any further cell-specific RSRP threshold used in RRC IDLE/INACTIVE state should be included in the list or in case any of the thresholds listed in this LS is not cell specific.

# 2	Actions

To RAN2

ACTION: 	RAN4 would like RAN2 to take into account the above RAN4 replies/feedback in their future work on Redcap UEs.

# 3	Dates of next TSG-RAN WG RAN4 meetings

RAN4#104bis-e	10th - 19th October 2022, Electronic Meeting

RAN4#105	14th - 18th November 2022, Europe