3GPP TSG RAN WG2#121	R2-2300040

Athens, Greece, 27th February - 3rd March, 2023

3GPP TSG-RAN WG4 Meeting # 104-e	R4-2214421

Electronic Meeting, 15 – 26 August 2022

Title:	LS on Pemax,c of S-SSB transmission when multiple resource pool is configured in a carrier

Response to:

Release:	Rel-16

Work Item:	5G_V2X_NRSL-Core

Source:	RAN4

To:	RAN1, RAN2

Cc:

Contact Person:

Name: 	Shuai Zhou

E-mail Address: 	shuai.zhou@vivo.com

Attachments:	N/A

1. Overall Description:

In RAN4#101-e meeting, an LS [1] was sent to RAN1 and RAN2, relating to the parameter of Pemax in the configured transmitted power for PSSCH/ PSCCH. RAN4 would like to thank RAN1 and RAN2 for the reply LS[2][3] on the Pemax issue for PSSCH/PSCCH.

However, RAN4 was also discussing the Pemax parameters for S-SSB transmission. The following question was captured as follows:

Question 1: RAN4 agreed not to define PEMAX,c for S-SSB configured Tx Power in TS38.101-1 as highlighted yellow below and would like to know if this can have any impact to RAN1/2, for example, Pcompensation defined in idle mode procedure in TS 38.304.

6.2E.4          Configured transmitted power for V2X

6.2E.4.1    General

The NR V2X UE is allowed to set its configured maximum output power PCMAX,f,c for carrier f of serving cell c in each slot. The configured maximum output power PCMAX,f,c is set within the following bounds:

PCMAX_L,f,c ≤  PCMAX,f,c  ≤  PCMAX_H,f,c with

PCMAX_L,f, c = MIN {PEMAX,c,  PPowerClass, V2X – MAX(MAX(MPRc , A-MPRc) + DTIB,c , P-MPRc), PRegulatory,c }

PCMAX_H,f, c = MIN {PEMAX,c, PPowerClass, V2X,  PRegulatory,c }

where

-  PCMAX,f,c is configured for PSSCH\PSCCH, S-SSB and PSFCH, respectively;

-  For the total transmitted power PCMAX,PSSCH/PSCCH, PEMAX,c is the value given by IE sl-maxTransPower, defined by TS 38.331.

-  For the total transmitted power PCMAX,S-SSB, the PCMAX_L,f,c and PCMAX_H,f,c are defined as follows:

PCMAX_L,f,c = MIN {PPowerClass, V2X – MAX(MAX(MPRc , A-MPRc) + DTIB,c , P-MPRc), PRegulatory,c}

PCMAX_H,f,c = MIN {PPowerClass, V2X,  PRegulatory,c}

RAN4 would like to know if there is an impact on RAN1 and RAN2 about the agreement RAN4 made on S-SSB.

2. Actions:

To RAN1, RAN2:

ACTION: RAN4 respectfully asks RAN1 and RAN2 to provide feedback for the above question.

3. Date of Next TSG WG RAN4 Meetings:

TSG-RAN4 Meeting#104-bis-e	10-19 October 2022	E-meeting

TSG-RAN4 Meeting#105	14-18 November 2022	Canada, CA

# References

[1] R4-2120047, Draft LS on PEMAX for NR-V2X, Huawei, CATT, RAN4#101E.

[2] R4-2206157(R1-2202816), Reply LS to RAN4 on PEMAX for NR-V2X, Huawei, RAN4#102-e.

[3] R4-2206158(R2-2203687), Reply LS on Pemax for NR-V2X, Huawei, CATT, RAN4#102-e.