3GPP TSG RAN WG2#121	R2-2300078

Athens, Greece, 27th February - 3rd March, 2023

3GPP TSG-SA WG2 Meeting #154AH-e	S2-2301467

E-meeting, January 16-20 2023	(was S2-2301157r11)

# Title:	LS on Partially allowed/rejected NSSAI

# Release:	Rel-18

# Work Item:	eNS_Ph3

Source:	SA2

To:	RAN3

Cc:	RAN2

Contact Person:

Name:	Peter Hedman

E-mail Address:	peter (dot) hedman (at) ericsson (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments: none

1. Overall Description:

SA2 is progressing the normative work on network slicing enhancements to Rel-18.

SA2 agreed to enhance the system with the possibility that a Registration Area can include TAs with different S-NSSAI support. For this purpose SA2 agreed a new NSSAI defined as:

Partially Allowed NSSAI: Includes the S-NSSAI values, each one associated with a subset of TAs, within the Registration Area where the S-NSSAI is supported.

SA2 proposes to make the Partially Allowed NSSAI available to NG-RAN over NGAP, i.e. sent at the same time as sending the Allowed NSSAI e.g. as to allow NG-RAN to optimize RRM logic (e.g. so the RAN can steer the UE considering  also these S-NSSAIs in the Partially Allowed NSSAI).

Question 1: SA2 asks RAN3 if the Partially Allowed NSSAI is useful fr NG-RAN to get over NG-AP in all messages where the Allowed NSSAI is sent?

Question 2: if the answer to Question 1 is "yes", whether it is needed to send also the Partially Allowed NSSAI with or without the associated TA-list for each S-NSSAI in the Partially Allowed NSSAI

Question 3: is it possible and feasible in rel-18 in the RAN to leverage the received Partially Allowed NSSAI to e.g. deactivate the PDU session, or trigger reporting of entering or exiting a S-NSSAI area of support to AMF?

SA2 agreed also to introduce a new rejected S-NSSAI with cause code "partially in the RA" which is associated with a list of TAs where this S-NSSAI is supported in the TA. Provided to the UE such that the UE can request the S-NSSAI when entering a TA within the RA.

Question 4: Should the S-NSSAIs of the Rejected S-NSSAI for part of the RA be made available to NG-RAN for RRM purposes e.g. so that the RAN can consider this information for RRM purposes e.g. to steer the UE to bands supporting also this S-NSSAI?

2. Actions:

To RAN3

ACTION: 	SA2 kindly asks RAN3 to take the above into account and answer the questions.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #155	Feb 20 – 24, 2023	Athens, Greece

TSG-SA2 Meeting #156	April 17 - 21, 2023	E-meeting