3GPP TSG-RAN WG2 Meeting #121                                              	R2-2300103

Athens, Greece, 27th Feb – 3rd Mar, 2023

Source:	CATT

Title:	Consideration on Service Continuity Enhancements for L2 U2N Relay

Agenda Item:	8.9.3

Document for:	Discussion and Decision

Introduction

In last RAN2 meeting, regarding to service continuity enhancements for L2 U2N relay, only one agreement is achieved [1]:

Agreement:

RAN2 will investigate whether providing lossless delivery in DL and UL in the inter-gNB service continuity cases is feasible using Rel-17 mechanisms.

Many issues were discussed in last meeting but have no convergence. In this contribution, we continue to discuss the leftover issues.

Discussion

  - RRM measurement

Measurement event(s)

In RAN2#119bis meeting, only one measurement event Z1 that Serving L2 U2N Relay UE becomes worse than threshold1 and Candidate L2 U2N Relay UE becomes better than threshold2 was agreed. But event Z2 is FFS. To discuss the feasibility of event Z2, two questions needs to be clarified:

- Question 1: If the remote UE already has a connection with a good quality relay UE, whether it needs to find another candidate relay UE?

- Question 2: Whether and how to compare the SL-RSRP of serving U2N relay UE and SD-RSRP of candidate U2N relay UE?

For question 1, the legacy event A3 is to make comparison between the serving cell and the candidate cell. Therefore, following the same principle, it is possible to change to a relay which has better channel condition than the current serving relay. It can be benefit for the PC5 link reliability and stability.

And for question 2, since the serving relay UE may not always be active exchange of data, the SD-RSRP can be used for the serving side; but the SL-RSRP cannot be measured for the candidate side since it is a corner case that the candidate relay has a direct sidelink with the remote UE. Therefore, since it is also not reasonable to compare between the SL-RSRP and the SD-RSRP, the event Z2 can be used if the serving and the candidate have the same measured quantity, e.g. SD-RSRP.

Proposal 1: Introduce the measurement event Z2 (Candidate L2 U2N Relay UE becomes an offset better than serving L2 U2N Relay UE) for i2i path switching, at least for the case that same measured quantity (SD-RSRP) is used for both the serving relay and the candidate relay.

To evaluate a single target node, the legacy Event Y2 introduced in R17 could be reused for indirect-to-indirect path switching: Candidate L2 U2N Relay UE becomes better than threshold.

Proposal 2: The legacy Event Y2 (Candidate L2 U2N Relay UE becomes better than threshold) can also be used for i2i path switching.

  - Which node to decide the target U2N relay UE

The legacy HO procedure is as below:

- In XnAP, the HANDOVER REQUEST message will be sent to the target NG-RAN node and carry Target Cell Global ID of the target cell;

- In the inter-node message of HandoverPreparationInformation imbedded in the HANDOVER REQUEST message, a list of recommended best candidate cell(s) on each frequency of this NG-RAN node for which measurement information was available can be optionally included, together with the measurement result of each cell;

- Based on all the information received from the source NG-RAN node and the information got in the target node itself, e.g. the load of different cells, the target NG-RAN node could choose any cell under the node to include it in the HO command.

Figure 1 Legacy inter-gNB handover procedures

For the L2 U2N relay scenario, RAN3 has agreed that:

RAN3#117e: Source gNB decides to trigger path switching for the L2 U2N remote UE. RAN3#117bis-e: WA: Source gNB selects the target path type (direct or indirect)

Based on the above agreements, when discussing the inter-gNB handover procedure for relay, two issues should be made clear step by step:

- Whether the target gNB can change the path type (direct or indirect), e.g. based on the target gNB’s final decision?

- Which information should be sent from the source gNB to the target gNB?

For 1), all measurement results which can be measured by the UE are reported to the source gNB. The measurement object of frequency can be tied to only one report configuration which includes only one measurement event, therefore the source gNB could deduce the best candidate target cell/relay UE reported by the UE. If more than one measured event or a periodical measurement is report to the source gNB, it is reasonable the source gNB decide the target gNB and send the HANDOVER REQUEST message to the target gNB. The agreement provided by RAN3 said that “Source gNB selects the target path type (direct or indirect)” [2]. Therefore, the source gNB is responsible to choose the target gNB(s) and a best target cell or a best target relay UE (which means the target path type) belongs to the gNB. The best target cell or the best target relay UE belongs to the gNB can be transmitted via XnAP which is RAN3 scope.

Meanwhile, from RAN2 point of view, to align with the legacy procedure, if the target path type is indirect, a list of candidate relay UEs should be introduced for signaling enhancement, together with the legacy candidate cells included in the inter-node message of HandoverPreparationInformation.

After the target gNB received the HANDOVER REQUEST message, we think the target gNB could choose another cell or another relay UE based on e.g. load information count by the target gNB, the RRC state of the target relay, and some other information else. Therefore it is reasonable for the source gNB to convey the candidate list including the identities of cell(s) and/or relay(s), and for the target gNB to final decide whether a cell or a relay UE under the cell is more appropriate for the new path of the UE.

Proposal 3: It is allowed for the target gNB to change the path type based on the target gNB’s decision.

For 2), based on the proposal above, the transmitted information between gNBs can be enhanced by:

- In XnAP, a target relay UE ID can be included, together with the legacy Target Cell Global ID;

- In the inter-node message of HandoverPreparationInformation, a set of candidate relay UE ID(s) and the related measurement result can be included, together with the legacy field of candidateCellInfoList.

Proposal 4: The candidate relay UE ID(s) and the related measurement result(s) can be sent to the target gNB via inter-node message, besides the legacy candidate cell information.

  - Lossless data delivery

It is RAN2 agreement to investigate whether using Rel-17 mechanisms to provide lossless delivery in DL and UL in the inter-gNB service continuity cases is feasible or not. Hence, the R17 mechanism is recalled firstly. The agreements for lossless data delivery in RAN2#115/116 meeting during Rel-17 discussion are listed below:

Agreements:

Proposal 13	RAN2 to confirm that tThe DL/UL lossless delivery during the path switch is done according to the PDCP status report. FFS if there is spec impact.

Proposal 25: No spec impact is required for DL lossless transmission during path switch.

And in RAN2#116bis, a summary contribution [3] is provided, and two options are raised for UL PDCP lossless in indirect-to-direct path switch issue:

- Option 1: No spec impact, i.e., assume loss of UL PDCP PDUs is a corner case or can be addressed by network implementation,

- Option 2: Remote UE re-transmits all the PDCP SDUs for which the successful delivery of the corresponding PDCP Data PDU has not been confirmed by PDCP status report in the target side after path switch.

Finally option 1 had been chosen based on majority companies’ view. The agreement is:

Agreements:

No spec impact for ensuring UL PDCP lossless behaviour in indirect-to-direct path switch (assume it is a corner case or can be addressed by network implementation).

From the above information, it shows that even using PDCP status report for DL/UL lossless delivery had been agreed at the beginning, but at last for DL there is no spec impact and can be performed due to gNB implementation, for UL the PDCP packet loss due to “the successful delivery of the corresponding PDCP Data PDU has not been confirmed by PDCP status report” [4] is assumed as corner case and still have no spec impact. Therefore in R17 there is no enhanced PDCP status report solution used for lossless delivery actually, and there still have chance that the Remote UE’s UL data may loss during Relay UE’s Uu link change, e.g. Uu RLF, Uu HO.

Observation 1: In R17 it is agreed that “No spec impact for ensuring UL PDCP lossless behaviour in indirect-to-direct path switch (assume it is a corner case or can be addressed by network implementation)”.

In R18, inter-gNB scenario and i2i path switch case are introduced.

For inter-gNB d2i case, since there only have one hop in the source, the RLC acknowledged information is accurate, so the un-acknowledged data can be re-transmitted in the indirect target link to/from the target gNB as legacy procedure. There is no spec impact for inter-gNB direct-to-indirect case in R18.

Proposal 5: No spec impact is required for ensuring PDCP lossless delivery for inter-gNB d2i path switch case.

For inter-gNB i2i, inter-gNB i2d and intra-gNB i2i path switch cases, to clearer discuss whether the R17 agreements can be continued to use in R18, it is necessary to define two assumptions, one is to consider the data loss in the source link is a corner case, and the other is not.

Proposal 6: To discuss R18 new cases, it is necessary to define two assumptions for lossless data delivery:

- Assumption 1 (Assum1): The Remote UE’s UL/DL data loss during Relay UE’s Uu/PC5 link change is still considered as a corner case as R17 does, or

- Assumption 2 (Assum2): The Remote UE’s UL/DL data may be lost during Relay UE’s Uu/PC5 link change.

We can discuss the R18 new scenarios and cases based on DL/UL as below, and only the i2d and i2i cases are considered (the d2i case has already discussed in Proposal5):

Case 1: DL data in inter-gNB path switch/HO scenario

For inter-gNB path switch/HO scenario, DL data should be forwarded to the target gNB and be re-transmitted to the UE based on the rule in [5]:

Step 1: For DL data forwarding, the PDCP SDUs with SN number in the source NG-RAN node may be firstly forwarded to the target NG-RAN node if not acknowledged by the UE:

- for DRBs for which preservation of SN status applies, the source NG-RAN node may forward in order to the target NG-RAN node via the DRB DL forwarding tunnel all downlink PDCP SDUs with their SN corresponding to PDCP PDUs which have not been acknowledged by the UE.

Step 2: The target NG-RAN node will re-transmit the forwarded PDCP SDUs:

For RLC-AM bearers: - The target gNB re-transmits and prioritizes all downlink data forwarded by the source gNB (i.e. the target gNB should first send all forwarded PDCP SDUs with PDCP SNs, then all forwarded downlink PDCP SDUs without SNs before sending new data from 5GC), excluding PDCP SDUs for which the reception was acknowledged through PDCP SN based reporting by the UE.

If follows the legacy spec, for i2d/i2i case, the DL data has not been acknowledged by the relay RLC may be forwarded:

For Assum1, the forwarded data to the target gNB is exactly the PDCP SDUs which are not transmitted to the remote UE; For Assum2, the data acknowledged by Relay but lost in the source PC5 link cannot be forwarded to the target gNB, therefore it cannot be re-transmitted from the target gNB to the UE, no matter the PDCP status report is configured to be sent by the UE or not.

Observation 2: For DL data in inter-gNB i2d/i2i path switch/HO scenario, the source gNB may not forward all the un-successfully delivered PDCP SDUs to the target gNB, if assumed the DL data may loss during the source PC5 link change.

Case 2: UL data in inter-gNB path switch/HO scenario

For inter-gNB path switch/HO scenario, the UE should re-transmit the UL data from the oldest PDCP SN number which has not been acknowledged at RLC based on the rule in [5]:

For RLC-AM bearers: - The UE re-transmits in the target gNB all uplink PDCP SDUs starting from the oldest PDCP SDU that has not been acknowledged at RLC in the source, excluding PDCP SDUs for which the reception was acknowledged through PDCP SN based reporting by the target.

If follows the legacy spec, for i2d/i2i case, the UL data has been acknowledged by the relay RLC may not be re-transmitted by the UE to the target gNB:

For Assum1, the re-transmitted data to the target gNB is equal to or larger than the PDCP SDUs which has not been confirmed by the PDCP in network side; For Assum2, if the data is acknowledged by Relay but lost in the source Uu link, it may not be re-transmitted to the target gNB if its PDCP SN is small than the oldest PDCP SDU that has not been acknowledged at RLC, even the PDCP status report from the target gNB ask for the lost PDCP SDUs.

Observation 3: For UL data in inter-gNB i2d/i2i path switch/HO scenario, the remote UE may not re-transmit all the un-successfully delivered PDCP SDUs to the target gNB, even the PDCP status report from the target gNB ask for the lost PDCP SDUs, if assumed the UL data may loss during the source Uu link change, e.g. Uu RLF, Uu HO.

In all, if we continue to use the Assumption 1, there can be no any spec impact for the R18 enhancements of inter-gNB scenario and i2i path switch case since “the successful delivery of the corresponding PDCP Data PDU has not been confirmed by PDCP status report” are considered as corner case:

Proposal 7: If continue the R17 assumption that “the successful delivery of the corresponding PDCP Data PDU has not been confirmed by PDCP status report” as corner case, there is no enhancement or spec impact for the R18 inter-gNB scenario or for the i2i path switch case.

But for Assumption 2, since the data forwarding for DL data and the UE re-transmission for UL data are based on the Relay’s acknowledgement, there have chance that the PDCP SDUs may be lost. To solve the problem:

For UL, the option 2 in R17 summary [3] mentioned above can be used: Remote UE re-transmits all the PDCP SDUs for which the successful delivery of the corresponding PDCP Data PDU has not been confirmed by PDCP status report in the target gNB after path switch;

For DL, since the RLC ACK from Relay cannot represent the end-to-end transmission status, the data forwarding mechanism can be enhanced to forward all the PDCP SDUs in the buffer to the target gNB, and then similar as UL, the target gNB re-transmits all the PDCP SDUs for which the successful delivery of the corresponding PDCP Data PDU has not been confirmed by PDCP status report in the target gNB after path switch.

Proposal 8: If using the assumption that the data may be lost in the source sidelink:

- For UL, the remote UE should re-transmit all the PDCP SDUs for which the successful delivery of the corresponding PDCP Data PDU has not been confirmed by PDCP status report in the target gNB after path switch;

- For DL, the data forwarding mechanism should be enhanced for the inter-gNB path switching scenario to forward all the PDCP SDUs in the buffer to the target gNB, and the target gNB re-transmits all the PDCP SDUs for which the successful delivery of the corresponding PDCP Data PDU has not been confirmed by PDCP status report in the target gNB after path switch.

Conclusion

According to the analysis in section 2, we propose:

Proposal 1: Introduce the measurement event Z2 (Candidate L2 U2N Relay UE becomes an offset better than serving L2 U2N Relay UE) for i2i path switching, at least for the case that same measured quantity (SD-RSRP) is used for both the serving relay and the candidate relay.

Proposal 2: The legacy Event Y2 (Candidate L2 U2N Relay UE becomes better than threshold) can also be used for i2i path switching.

Proposal 3: It is allowed for the target gNB to change the path type based on the target gNB’s decision.

Proposal 4: The candidate relay UE ID(s) and the related measurement result(s) can be sent to the target gNB via inter-node message, besides the legacy candidate cell information.

Observation 1: In R17 it is agreed that “No spec impact for ensuring UL PDCP lossless behaviour in indirect-to-direct path switch (assume it is a corner case or can be addressed by network implementation)”.

Proposal 5: No spec impact is required for ensuring PDCP lossless delivery for inter-gNB d2i path switch case.

Proposal 6: To discuss R18 new cases, it is necessary to define two assumptions for lossless data delivery:

- Assumption 1 (Assum1): The Remote UE’s UL/DL data loss during Relay UE’s Uu/PC5 link change is still considered as a corner case as R17 does, or

- Assumption 2 (Assum2): The Remote UE’s UL/DL data may be lost during Relay UE’s Uu/PC5 link change.

Observation 2: For DL data in inter-gNB i2d/i2i path switch/HO scenario, the source gNB may not forward all the un-successfully delivered PDCP SDUs to the target gNB, if assumed the DL data may loss during the source PC5 link change.

Observation 3: For UL data in inter-gNB i2d/i2i path switch/HO scenario, the remote UE may not re-transmit all the un-successfully delivered PDCP SDUs to the target gNB, even the PDCP status report from the target gNB ask for the lost PDCP SDUs, if assumed the UL data may loss during the source Uu link change, e.g. Uu RLF, Uu HO.

Proposal 7: If continue the R17 assumption that “the successful delivery of the corresponding PDCP Data PDU has not been confirmed by PDCP status report” as corner case, there is no enhancement or spec impact for the R18 inter-gNB scenario or for the i2i path switch case.

Proposal 8: If using the assumption that the data may be lost in the source sidelink:

- For UL, the remote UE should re-transmit all the PDCP SDUs for which the successful delivery of the corresponding PDCP Data PDU has not been confirmed by PDCP status report in the target gNB after path switch;

- For DL, the data forwarding mechanism should be enhanced for the inter-gNB path switching scenario to forward all the PDCP SDUs in the buffer to the target gNB, and the target gNB re-transmits all the PDCP SDUs for which the successful delivery of the corresponding PDCP Data PDU has not been confirmed by PDCP status report in the target gNB after path switch.

Reference

- Draft_R2_120_meeting_report_v1.docx

- RAN3_118_agenda_20221118_EOM.doc

- R2-2200009	Summary of [Post116-e][604][Relay] Remaining issues on service continuity (Xiaomi)	Xiaomi

- R2-2109428	Remaining issues on service continuity of L2 U2N relay	Qualcomm Incorporated

- TS 38.300 NR and NG-RAN Overall Description; Stage 2