3GPP TSG-RAN2 Meeting #121	R2-2200112

# Athens, Greece, February 27th –March 3rd, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.355 | CR | 0405 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to UE capability for MG (de-)activation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-Core |  |  |  |  |  | Date: |  |  | 2023-02-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For the current LPP spec, the following UE capability has been included for mg-activation for DL-TDOA, DL-AoD and multi-RTT mg-ActivationRequest This field, if present, indicates that the target device supports low latency measurement gap activation request for DL-PRS measurements. The UE can include this field only if the UE supports mg-ActivationRequestPRS-Meas and mg-ActivationCommPRS-Meas defined in TS 38.331 [35].. From the description above, it reads that the capability is for indicating the UE’s support for UL MAC CE for MG activation/deactivation request. It has also been clarified that the UE should indicate the field only if the UE supports the RRC radio capability. While according to the current description in 38.305, there are two modes of MG activation/deactivation, either by UL MAC CE or NRPPa message. Coupling the support of the posMG and UL MAC CE for MG activation/deactivation is uncessary since the MG can also be activated/deactivated by NRPPa message. In addition, the support of UL MAC CE for MG activation/deactivation is the issue between UE and gNB and can be transpartent to LPP spec. |
| Summary of change: |  | 1/ Change the field name of mg-ActivationRequest to posMeasGap-supported 2/ Change the field description of the field, that it indicates the support of pre-configured MG for positioning |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong description on the capability indication, which couples the UL MAC CE with MG activation/deactivation. Impact analysis Impacted functionality: Pre-configured MG for positioning Inter-operability: - If the UE is implemented according to the CR while the network is not, there is no inter-operability issue - If the network is implemented according to the CR while the UE is not, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.10.6, 6.5.11.6, 6.5.12.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Ver0 in RAN2#121e: R2-2200112 |  |  |  |  |  |  |  |  |

==================================CHANGE BEGINS===================================

6.5.10.6	NR DL-TDOA Capability Information

–	NR-DL-TDOA-ProvideCapabilities

The IE NR-DL-TDOA-ProvideCapabilities is used by the target device to indicate its capability to support NR DL-TDOA and to provide its NR DL-TDOA positioning capabilities to the location server.

-- ASN1START

NR-DL-TDOA-ProvideCapabilities-r16 ::= SEQUENCE {

nr-DL-TDOA-Mode-r16						PositioningModes,

nr-DL-TDOA-PRS-Capability-r16			NR-DL-PRS-ResourcesCapability-r16,

nr-DL-TDOA-MeasurementCapability-r16	NR-DL-TDOA-MeasurementCapability-r16,

nr-DL-PRS-QCL-ProcessingCapability-r16	NR-DL-PRS-QCL-ProcessingCapability-r16,

nr-DL-PRS-ProcessingCapability-r16		NR-DL-PRS-ProcessingCapability-r16,

additionalPathsReport-r16				ENUMERATED { supported }					OPTIONAL,

periodicalReporting-r16					PositioningModes							OPTIONAL,

...,

[[

ten-ms-unit-ResponseTime-r17			PositioningModes							OPTIONAL,

nr-PosCalcAssistanceSupport-r17			BIT STRING {	trpLocSup		(0),

beamInfoSup		(1),

rtdInfoSup		(2),

trpTEG-InfoSup	(3)

}	(SIZE (1..8))				OPTIONAL,

nr-los-nlos-AssistanceDataSupport-r17	SEQUENCE {

type-r17		LOS-NLOS-IndicatorType2-r17,

granularity-r17	LOS-NLOS-IndicatorGranularity2-r17,

...

}											OPTIONAL,

nr-DL-PRS-ExpectedAoD-or-AoA-Sup-r17	BIT STRING {	eAoD		(0),

eAoA		(1)

}	(SIZE (1..8))				OPTIONAL,

nr-DL-TDOA-On-Demand-DL-PRS-Support-r17	NR-On-Demand-DL-PRS-Support-r17				OPTIONAL,

nr-los-nlos-IndicatorSupport-r17		SEQUENCE {

type-r17		LOS-NLOS-IndicatorType2-r17,

granularity-r17	LOS-NLOS-IndicatorGranularity2-r17,

...

}											OPTIONAL,

additionalPathsExtSupport-r17			ENUMERATED { n4, n6, n8 }					OPTIONAL,

scheduledLocationRequestSupported-r17	ScheduledLocationTimeSupportPerMode-r17		OPTIONAL,

nr-dl-prs-AssistanceDataValidity-r17	SEQUENCE {

area-validity-r17	INTEGER (1..maxNrOfAreas-r17)			OPTIONAL,								...

}															OPTIONAL,

multiMeasInSameMeasReport-r17			ENUMERATED { supported }					OPTIONAL,

mg-ActivationRequest-r17				ENUMERATED { supported }					OPTIONAL

]]

}

-- ASN1STOP

NR-DL-TDOA-ProvideCapabilities field descriptions

nr-DL-TDOA-Mode This field specifies the NR DL-TDOA mode(s) supported by the target device.

periodicalReporting This field, if present, specifies the positioning modes for which the target device supports periodicalReporting. This is represented by a bit string, with a one-value at the bit position means periodicalReporting for the positioning mode is supported; a zero-value means not supported. If this field is absent, the target device does not support periodicalReporting in CommonIEsRequestLocationInformation.

ten-ms-unit-ResponseTime This field, if present, specifies the positioning modes for which the target device supports the enumerated value 'ten-milli-seconds' in the IE ResponseTime in IE CommonIEsRequestLocationInformation. This is represented by a bit string, with a one-value at the bit position means 'ten-milli-seconds' response time unit for the positioning mode is supported; a zero-value means not supported. If this field is absent, the target device does not support 'ten-milli-seconds' response time unit in CommonIEsRequestLocationInformation.

nr-PosCalcAssistanceSupport This field indicates the Position Calculation Assistance Data supported by the target device for UE-based DL-TDOA. This is represented by a bit string, with a one-value at the bit position means the particular assistance data is supported; a zero-value means not supported. - bit 0 indicates whether the field nr-TRP-LocationInfo in IE NR-PositionCalculationAssistance is supported or not; - bit 1 indicates whether the field nr-DL-PRS-BeamInfo in IE NR-PositionCalculationAssistance is supported or not; - bit 2 indicates whether the field nr-RTD-Info in IE NR-PositionCalculationAssistance is supported or not; - bit 3 indicates whether the field nr-DL-PRS-TRP-TEG-Info in IE NR-PositionCalculationAssistance is supported or not. The UE can indicate this bit only if the UE supports prs-ProcessingCapabilityBandList and any of maxNrOfDL-PRS-ResourceSetPerTrpPerFrequencyLayer, maxNrOfTRP-AcrossFreqs, maxNrOfPosLayer, maxNrOfDL-PRS-ResourcesPerResourceSet and maxNrOfDL-PRS-ResourcesPerPositioningFrequencylayer. Otherwise, the UE does not include this field.

nr-los-nlos-AssistanceDataSupport This field, if present, indicates that the target device supports the NR-DL-PRS-ExpectedLOS-NLOS-Assistance in IE NR-PositionCalculationAssistance: - type indicates whether the target device supports 'hard' value or 'hard' and 'soft' value in LOS-NLOS-Indicator in IE NR-DL-PRS-ExpectedLOS-NLOS-Assistance. - granularity indicates whether the target device supports nr-los-nlos-indicator in IE NR-DL-PRS-ExpectedLOS-NLOS-Assistance 'per-trp', 'per-resource', or both. The UE can include this field only if the UE supports one of maxDL-PRS-RSRP-MeasurementFR1, maxDL-PRS-RSRP-MeasurementFR2, dl-RSTD-MeasurementPerPairOfTRP-FR1, dl-RSTD-MeasurementPerPairOfTRP-FR2, maxNrOfRx-TX-MeasFR1, maxNrOfRx-TX-MeasFR2, supportOfRSRP-MeasFR1 and supportOfRSRP-MeasFR2. Otherwise, the UE does not include this field.

nr-DL-PRS-ExpectedAoD-or-AoA-Sup This field, if present, indicates that the target device supports the NR-DL-PRS-ExpectedAoD-or-AoA in NR-DL-PRS-AssistanceData.

nr-DL-TDOA-On-Demand-DL-PRS-Support This field, if present, indicates that the target device supports on-demand DL-PRS requests.

nr-los-nlos-IndicatorSupport This field, if present, indicates that the target device supports nr-los-nlos-Indicator reporting in IE NR-DL-TDOA-SignalMeasurementInformation. - type indicates whether the target device supports 'hard' value or 'hard' and 'soft' value in IE LOS-NLOS-Indicator. - granularity indicates whether the target device supports LOS-NLOS-Indicator reporting per TRP, per DL-PRS Resource, or both. NOTE: A single value is reported when both Multi-RTT and DL-TDOA are supported.

additionalPathsExtSupport This field, if present, indicates that the target device supports the nr-AdditionalPathListExt reporting in IE NR-DL-TDOA-SignalMeasurementInformation. The enumerated value indicates the number of additional paths supported by the target device. NOTE: The supportOfDL-PRS-FirstPathRSRP in IE NR-DL-TDOA-MeasurementCapability also applies to the additional paths.

scheduledLocationRequestSupported This field, if present, specifies the positioning modes for which the target device supports scheduled location requests – i.e., supports the IE ScheduledLocationTime in IE CommonIEsRequestLocationInformation – and the time base(s) supported for the scheduled location time for each positioning mode. If this field is absent, the target device does not support scheduled location requests.

nr-dl-prs-AssistanceDataValidity This field, if present, indicates that the target device supports validity conditions for pre-configured assistance data and comprises the following subfields: - area-validity indicates that the target device supports pre-configured assistance data with area validity. The integer number indicates the maximum number of areas the target device supports.

multiMeasInSameMeasReport This field, if present, indicates that the target device supports multiple measurement instances in a single measurement report.

posMeasGap-supported This field, if present, indicates that the target device supports low latency pre-configured positioning measurement gap for DL-PRS measurements. The UE can include this field only if the UE supports mg-ActivationCommPRS-Meas defined in TS 38.331 [35].

====================================NEXT CHANGE ===================================

6.5.11.6	NR DL-AoD Capability Information

–	NR-DL-AoD-ProvideCapabilities

The IE NR-DL-AoD-ProvideCapabilities is used by the target device to indicate its capability to support NR DL-AoD and to provide its NR DL-AoD positioning capabilities to the location server.

-- ASN1START

NR-DL-AoD-ProvideCapabilities-r16 ::= SEQUENCE {

nr-DL-AoD-Mode-r16						PositioningModes,

nr-DL-AoD-PRS-Capability-r16			NR-DL-PRS-ResourcesCapability-r16,

nr-DL-AoD-MeasurementCapability-r16		NR-DL-AoD-MeasurementCapability-r16,

nr-DL-PRS-QCL-ProcessingCapability-r16	NR-DL-PRS-QCL-ProcessingCapability-r16,

nr-DL-PRS-ProcessingCapability-r16		NR-DL-PRS-ProcessingCapability-r16,

periodicalReporting-r16					PositioningModes							OPTIONAL,

...,

[[

ten-ms-unit-ResponseTime-r17			PositioningModes							OPTIONAL,

nr-PosCalcAssistanceSupport-r17			BIT STRING {	trpLocSup		(0),

beamInfoSup		(1),

rtdInfoSup		(2),

beamAntInfoSup	(3)

}	(SIZE (1..8))				OPTIONAL,

nr-los-nlos-AssistanceDataSupport-r17	SEQUENCE {

type-r17		LOS-NLOS-IndicatorType2-r17,

granularity-r17	LOS-NLOS-IndicatorGranularity2-r17,

...

}											OPTIONAL,

nr-DL-PRS-ExpectedAoD-or-AoA-Sup-r17	BIT STRING {	eAoD		(0),

eAoA		(1)

}	(SIZE (1..8))				OPTIONAL,

dl-PRS-ResourcePrioritySubset-Sup-r17	ENUMERATED { sameSet, differentSet, sameOrDifferentSet }

OPTIONAL,

nr-DL-PRS-BeamInfoSup-r17				ENUMERATED { supported }					OPTIONAL,

nr-DL-AoD-On-Demand-DL-PRS-Support-r17	NR-On-Demand-DL-PRS-Support-r17				OPTIONAL,

nr-los-nlos-IndicatorSupport-r17		SEQUENCE {

type-r17		LOS-NLOS-IndicatorType2-r17,

granularity-r17	LOS-NLOS-IndicatorGranularity2-r17,

...

}											OPTIONAL,

scheduledLocationRequestSupported-r17	ScheduledLocationTimeSupportPerMode-r17

OPTIONAL,

nr-dl-prs-AssistanceDataValidity-r17	SEQUENCE {

area-validity-r17	INTEGER (1..maxNrOfAreas-r17)			OPTIONAL,

...

}															OPTIONAL,

multiMeasInSameMeasReport-r17			ENUMERATED { supported }					OPTIONAL,

mg-ActivationRequest-r17				ENUMERATED { supported }					OPTIONAL

]]

}

-- ASN1STOP

| NR-DL-AoD-ProvideCapabilities field descriptions |
| --- |
| nr-DL-AoD-Mode This field specifies the NR DL-AoD mode(s) supported by the target device. |
| periodicalReporting This field, if present, specifies the positioning modes for which the target device supports periodicalReporting. This is represented by a bit string, with a one-value at the bit position means periodicalReporting for the positioning mode is supported; a zero-value means not supported. If this field is absent, the target device does not support periodicalReporting in CommonIEsRequestLocationInformation. |
| ten-ms-unit-ResponseTime This field, if present, specifies the positioning modes for which the target device supports the enumerated value 'ten-milli-seconds' in the IE ResponseTime in IE CommonIEsRequestLocationInformation. This is represented by a bit string, with a one-value at the bit position means 'ten-milli-seconds' response time unit for the positioning mode is supported; a zero-value means not supported. If this field is absent, the target device does not support 'ten-milli-seconds' response time unit in CommonIEsRequestLocationInformation. |
| nr-PosCalcAssistanceSupport This field indicates the Position Calculation Assistance Data supported by the target device for UE-based DL-AoD. This is represented by a bit string, with a one-value at the bit position means the particular assistance data is supported; a zero-value means not supported. - bit 0 indicates whether the field nr-TRP-LocationInfo in IE NR-PositionCalculationAssistance is supported or not; - bit 1 indicates whether the field nr-DL-PRS-BeamInfo in IE NR-PositionCalculationAssistance is supported or not; - bit 2 indicates whether the field nr-RTD-Info in IE NR-PositionCalculationAssistance is supported or not. The UE can indicate this bit only if the UE supports prs-ProcessingCapabilityBandList and any of maxNrOfDL-PRS-ResourceSetPerTrpPerFrequencyLayer, maxNrOfTRP-AcrossFreqs, maxNrOfPosLayer, maxNrOfDL-PRS-ResourcesPerResourceSet and maxNrOfDL-PRS-ResourcesPerPositioningFrequencylayer. Otherwise, the UE does not include this field; - bit 3 indicates whether the field nr-TRP-BeamAntennaInfo in IE NR-PositionCalculationAssistance is supported or not. |
| nr-los-nlos-AssistanceDataSupport This field, if present, indicates that the target device supports the NR-DL-PRS-ExpectedLOS-NLOS-Assistance in IE NR-PositionCalculationAssistance: - type indicates whether the target device supports 'hard' value or 'hard' and 'soft' value in LOS-NLOS-Indicator in IE NR-DL-PRS-ExpectedLOS-NLOS-Assistance. - granularity indicates whether the target device supports nr-los-nlos-indicator in IE NR-DL-PRS-ExpectedLOS-NLOS-Assistance 'per-trp', 'per-resource', or both. The UE can include this field only if the UE supports one of maxDL-PRS-RSRP-MeasurementFR1, maxDL-PRS-RSRP-MeasurementFR2,dl-RSTD-MeasurementPerPairOfTRP-FR1, dl-RSTD-MeasurementPerPairOfTRP-FR2, maxNrOfRx-TX-MeasFR1, maxNrOfRx-TX-MeasFR2, supportOfRSRP-MeasFR1 and supportOfRSRP-MeasFR2 . Otherwise, the UE does not include this field. |
| nr-DL-PRS-ExpectedAoD-or-AoA-Sup This field, if present, indicates that the target device supports the NR-DL-PRS-ExpectedAoD-or-AoA in NR-DL-PRS-AssistanceData. |
| dl-PRS-ResourcePrioritySubset-Sup This field, if present, indicates that the target device supports the DL-PRS-ResourcePrioritySubset in IE NR-DL-PRS-Info. Enumerated value indicates the supported resource set relationship for the target DL-PRS Resource and the associated subset. |  |
| nr-DL-PRS-BeamInfoSup This field, if present, indicates that the target device supports the NR-DL-PRS-BeamInfo in IE NR-DL-AoD-ProvideAssistanceData. |
| nr-DL-AoD-On-Demand-DL-PRS-Support This field, if present, indicates that the target device supports on-demand DL-PRS requests. |
| nr-los-nlos-IndicatorSupport This field, if present, indicates that the target device supports nr-los-nlos-Indicator reporting in IE NR-DL-AoD-SignalMeasurementInformation. - type indicates whether the target device supports 'hard' value or 'hard' and 'soft' value in IE LOS-NLOS-Indicator. - granularity indicates whether the target device supports LOS-NLOS-Indicator reporting per TRP, per DL-PRS Resource, or both. |
| scheduledLocationRequestSupported This field, if present, specifies the positioning modes for which the target device supports scheduled location requests – i.e., supports the IE ScheduledLocationTime in IE CommonIEsRequestLocationInformation – and the time base(s) supported for the scheduled location time for each positioning mode. If this field is absent, the target device does not support scheduled location requests. |
| nr-dl-prs-AssistanceDataValidity This field, if present, indicates that the target device supports validity conditions for pre-configured assistance data and comprises the following subfields: - area-validity indicates that the target device supports pre-configured assistance data with area validity. The integer number indicates the maximum number of areas the target device supports. |
| multiMeasInSameMeasReport This field, if present, indicates that the target device supports multiple measurement instances in a single measurement report. |
| posMeasGap-supported This field, if present, indicates that the target device supports low latency pre-configured positioning measurement gapfor DL-PRS measurements. The UE can include this field only if the UE supports mg-ActivationCommPRS-Meas defined in TS 38.331 [35]. |

====================================NEXT CHANGE ===================================

6.5.12.6	NR Multi-RTT Capability Information

–	NR-Multi-RTT-ProvideCapabilities

The IE NR-Multi-RTT-ProvideCapabilities is used by the target device to indicate its capability to support NR Multi-RTT and to provide its NR Multi-RTT positioning capabilities to the location server.

-- ASN1START

NR-Multi-RTT-ProvideCapabilities-r16 ::= SEQUENCE {

nr-Multi-RTT-PRS-Capability-r16			NR-DL-PRS-ResourcesCapability-r16,

nr-Multi-RTT-MeasurementCapability-r16	NR-Multi-RTT-MeasurementCapability-r16,

nr-DL-PRS-QCL-ProcessingCapability-r16	NR-DL-PRS-QCL-ProcessingCapability-r16,

nr-DL-PRS-ProcessingCapability-r16		NR-DL-PRS-ProcessingCapability-r16,

nr-UL-SRS-Capability-r16				NR-UL-SRS-Capability-r16,

additionalPathsReport-r16				ENUMERATED { supported }					OPTIONAL,

periodicalReporting-r16					ENUMERATED { supported }					OPTIONAL,

...,

[[

ten-ms-unit-ResponseTime-r17			ENUMERATED { supported }					OPTIONAL,

nr-DL-PRS-ExpectedAoD-or-AoA-Sup-r17	BIT STRING {	eAoD		(0),

eAoA		(1)

}	(SIZE (1..8))				OPTIONAL,	nr-Multi-RTT-On-Demand-DL-PRS-Support-r17

NR-On-Demand-DL-PRS-Support-r17				OPTIONAL,

nr-UE-RxTx-TEG-ID-ReportingSupport-r17	BIT STRING {	case1	(0),

case2	(1),

case3	(2)

}	(SIZE (1..8))				OPTIONAL,

nr-los-nlos-IndicatorSupport-r17	SEQUENCE {

type-r17			LOS-NLOS-IndicatorType2-r17,

granularity-r17		LOS-NLOS-IndicatorGranularity2-r17,

...

}												OPTIONAL,

additionalPathsExtSupport-r17			ENUMERATED { n4, n6, n8 }					OPTIONAL,

scheduledLocationRequestSupported-r17	ScheduledLocationTimeSupport-r17			OPTIONAL,

nr-dl-prs-AssistanceDataValidity-r17	SEQUENCE {

area-validity-r17	INTEGER (1..maxNrOfAreas-r17)		OPTIONAL,

...

}															OPTIONAL,

multiMeasInSameMeasReport-r17			ENUMERATED { supported }					OPTIONAL,

mg-ActivationRequest-r17				ENUMERATED { supported }					OPTIONAL

]]

}

-- ASN1STOP

NR-Multi-RTT-ProvideCapabilities field descriptions

ten-ms-unit-ResponseTime This field, if present, indicates that the target device supports the enumerated value 'ten-milli-seconds' in the IE ResponseTime in IE CommonIEsRequestLocationInformation.

nr-DL-PRS-ExpectedAoD-or-AoA-Sup This field, if present, indicates that the target device supports the NR-DL-PRS-ExpectedAoD-or-AoA in NR-DL-PRS-AssistanceData.

nr-Multi-RTT-On-Demand-DL-PRS-Support This field, if present, indicates that the target device supports on-demand DL-PRS requests.

nr-UE-RxTx-TEG-ID-ReportingSupport This field, if present, indicates that the target device supports nr-UE-RxTx-TEG-Info reporting in IE NR-Multi-RTT-SignalMeasurementInformation. This is represented by a bit string, with a one-value at the bit position means the particular case is supported; a zero-value means not supported: - bit 0 indicates that the target device supports the 'case1' choice in NR-UE-RxTx-TEG-Info. - bit 1 indicates that the target device supports the 'case2' choice in NR-UE-RxTx-TEG-Info. - bit 2 indicates that the target device supports the 'case3' choice in NR-UE-RxTx-TEG-Info.

nr-los-nlos-IndicatorSupport This field, if present, indicates that the target device supports nr-los-nlos-Indicator reporting in IE NR-Multi-RTT-SignalMeasurementInformation. - type indicates whether the target device supports 'hard' value or 'hard' and 'soft' value in IE LOS-NLOS-Indicator. - granularity indicates whether the target device supports LOS-NLOS-Indicator reporting per TRP, per DL-PRS Resource, or both. NOTE: A single value is reported when both Multi-RTT and DL-TDOA are supported.

additionalPathsExtSupport This field, if present, indicates that the target device supports the nr-AdditionalPathListExt reporting in IE NR-Multi-RTT-SignalMeasurementInformation. The enumerated value indicates the number of additional paths supported by the target device. NOTE: The supportOfDL-PRS-FirstPathRSRP in IE NR-Multi-RTT-MeasurementCapability also applies to the additional paths.

scheduledLocationRequestSupported This field, if present, indicates that the target device supports scheduled location requests – i.e., supports the IE ScheduledLocationTime in IE CommonIEsRequestLocationInformation – and the time base(s) supported for the scheduled location time.

nr-dl-prs-AssistanceDataValidity This field, if present, indicates that the target device supports validity conditions for pre-configured assistance data and comprises the following subfields: - area-validity indicates that the target device supports pre-configured assistance data with area validity. The integer number indicates the maximum number of areas the target device supports.

multiMeasInSameMeasReport This field, if present, indicates that the target device supports multiple measurement instances in a single measurement report.

posMeasGap-supported This field, if present, indicates that the target device supports low latency pre-configured positioning measurement gap for DL-PRS measurements. The UE can include this field only if the UE supports mg-ActivationCommPRS-Meas defined in TS 38.331 [35].

==================================END OF CHANGES====================================