3GPP TSG-RAN WG2 Meeting #121	R2-2300116

Athens, Greece, 27th February- 3rd March, 2022

Agenda Item:	8.2.3

Source:	Huawei, HiSilicon

Title:	Discussion on RAT-dependent integrity

Document for: Discussion

Introduction

During RANP#98, New WID on Expanded and Improved NR Positioning has been approved [1], where the objective for the integrity for RAT-dependent positioning methods is excerpted as follows.

- Specify the error modelling parameters, signalling, and procedures to support UE-based and LMF-based integrity of RAT-dependent positioning methods [RAN2, RAN3].

During the study phase, discussion on RAT-dependent integrity has been carried out and considerable agreements has been achieved, which can be categorized as follows.

- Principles

RAN2#119 Agreements:

Proposal 1: RAN2 to confirm the integrity principle of operation defined in the section 8.1.1a of TS38.305, including integrity definition (e.g., Error, Bound, Time to Alert, DNU, Residual Risk, irMinimum, irMaximum and Correlation Times; FFS if all parameters are needed in the RAT-dependent case), Equations for the GNSS integrity are reused for RAT dependent positioning methods.

Proposal 2 (modified): RAN2 may add the mapping between Integrity definition/Fields (Integrity Alerts, error bounds (mean, StdDev), Residual Risks, Integrity correlation times) and Error sources/assistance data for RAT-dependent positioning methods later once RAN1 identifies new error sources.

RAN2#119b Agreement:

Proposal 4. RAN2 will study the both UE-based and LMF-based integrity for RAT-dependent cases.

- DNU flag

RAN2#119b Agreement:

Proposal 1-2. RAN2 study the usage of DNU flag for the RAT-dependent positioning integrity (assuming RAN1 agree to leave it to RAN2) and conclude on whether to indicate the DNU presence in the integrity principle equation.

RAN2#120 Agreement:

Proposal 1 (modified): Use DNU flag for RAT-dependent integrity, with the meaning that the concerned assistance data cannot be used for integrity calculation but may be usable for positioning.  Signalling details and relation to error sources can be determined in normative work.  FFS which positioning methods are affected based on the progress in RAN1.

- Signaling aspects

RAN2#119b Agreement:

Proposal 7 (modified). RAN2 agree that R17 UE-based integrity mode signaling can be used as baseline with the following aspects:

-	UE sends capability info to LMF on integrity for UE-based mode using LPP capability transfer procedure

-	LMF sends the assistance data for integrity calculation to UE for integrity of UE-based mode

-	LMF sends integrity requirement e.g., TIR to UE in LPP request location information message for integrity of UE-based mode

-	UE sends integrity result to LMF using LPP location information Transfer message

RAN2#119b Agreement:

LMF provides, in assistance data, the information of error sources (e.g., originated from RAN node) to UE for integrity in UE-based mode.

Meanwhile, RAN1 made exhaustive discussion of RAT-dependent integrity including the error sources of different positioning methods for LMF-based and UE-based positioning mode, the error profile, etc. and quite a few agreements have been achieved as in [2].

During WI phase, we think several open issues should be handled to facilitate the progress, which include the application of DNU flag, how to capture RAN1 progress on error sources, and the signalling enhancements to support the LMF-based/UE-based integrity, etc.

Discussion

DNU flag for measurement error

There should be two aspects for the DNU flag according to the category of the error source: measurement or assistance data.

2.1.1	DNU flag for Assistance data

Recall that RAN2#120 has agreed to adopt the DNU flag for concerned assistance data of RAT-dependent integrity.

Agreement:

Proposal 1 (modified): Use DNU flag for RAT-dependent integrity, with the meaning that the concerned assistance data cannot be used for integrity calculation but may be usable for positioning.  Signalling details and relation to error sources can be determined in normative work.  FFS which positioning methods are affected based on the progress in RAN1.

Note that in Rel-17 GNSS integrity, ProvidingAssistanceData without the Integrity Service Alert IE or Real Time Integrity IEs is interpreted as a DNU=FALSE condition. Specifically, the indication of DNU flag is implicitly realized by the absence/presence of the IE GNSS-RealTimeIntegrity, which is specified in TS 37.355.

NOTE 1: If GNSS integrity assistance data are provided (i.e., any of GNSS-Integrity-ServiceParameters, GNSS-Integrity-ServiceAlert, ORBIT-IntegrityParameters, SSR-IntegrityOrbitBounds, CLOCK-IntegrityParameters, SSR-IntegrityClockBounds, SSR-IntegrityCodeBiasBounds, SSR-IntegrityPhaseBiasBounds, STEC-IntegrityParameters, STEC-IntegrityErrorBounds, SSR-GriddedCorrectionIntegrityParameters, TropoDelayIntegrityErrorBounds) the following interpretation of the IE GNSS-RealTimeIntegrity applies: - Absence of the IE GNSS-RealTimeIntegrity indicates DNU=FALSE according to the Integrity Principle of Operation specified in clause 8.1.1a of TS 38.305 [40] for all GNSS satellites for which integrity assistance data are provided. - Presence of the IE GNSS-RealTimeIntegrity for a GNSS satellite and signal combination indicates DNU=TRUE for this GNSS satellite and signal combination according to the Integrity Principle of Operation specified in clause 8.1.1a of TS 38.305 [40].

Obviously, the case is different for RAT-dependent integrity. First, the function of DNU for RAT-dependent integrity change to the indication of the concerned assistance data cannot be used for integrity calculation but may be usable for positioning. Second, there is no integrity related IE for RAT-dependent integrity. In this respect, RAN2 should consider the signaling design of DNU for RAT-dependent integrity. There may be two candidate solutions.

- Alt1. Along the lines of Rel-17 GNSS integrity, a new IE (the content could be further discussed) may be introduced for the RAT-dependent integrity with its absence/presence indicating the DNU value.

- Alt2. Introduce a DNU flag for each related integrity assistance data per RAT-dependent positioning method.

Considering that the integrity assistance data is defined per RAT-dependent positioning method, it is difficult for the new IE to capsulate the indication of every assistance data, so we think Alt2 is more reasonable.

- Introduce a DNU flag for the integrity assistance data for each error source per RAT-dependent positioning method.

2.2.2	DNU flag for Measurement

Another issue is whether the DNU flag can also be used to indicate whether the measurement results can be used for integrity calculation. It is natural to reuse the “DNU flag” for RAT-dependent integrity to indicate the validity of integrity related assistance data as Rel-17 GNSS integrity. We may notice that the difference between Rel-18 RAT-dependent integrity and Rel-17 GNSS integrity is that LMF-based integrity is also supported in addition to UE-based integrity. For LMF-based integrity, in addition to assistance data error sources (as summarized in Table 2.2-1), the error sources also include measurements error. Therefore, we need to discuss whether the "DNU flag" is also applicable to the measurements related errors sources.

From our perspective, the DNU flag can also be applied for the measurements related errors, e.g. to indicate whether the measurement results can be utilized for integrity calculation or how reliable the measurement results can be. For example, even if the measurement error (e.g., RSTD measurement) cannot be used for integrity calculation, the corresponding integrity related information (e.g., distribution, mean and/or standard deviation for integrity overbounding model, range) may still be helpful for position calculation. In this case, a DNU flag may be necessary.

However, it might be hard for R2 to decide whether such DNU flag for measurement might be useful for LMF. Hence, we propose the following

- Send an LS to R1 asking whether the DNU flag for measurement is needed for LMF-based integrity.

Error sources

During SI phase, considerable progress has been made on the study of error sources for LMF-based and UE-based positioning integrity modes.

RAN1#110bis Agreement:

The following distributions are identified as candidates for modeling of the distribution for inter-TRP synchronization error (e.g., NR-RTD-Info in TS 37.355)

- Uniform distribution

Note: this may already be consistent with the existing parameter NR-RTD-Info in TS 37.355

- Normal distribution

Note: it is up to RAN2 how to use the identified distributions

RAN1#110bis Agreement:

Timing measurement error can be modeled as Normal distribution.

Note: The timing measurement is applicable to RSTD, RTOA and UE/gNB Rx-Tx time difference measurement

Note: it is assumed that the timing measurement error is associated with the first path

Note: it is up to RAN2 how to use the identified distribution

RAN1#110bis Agreement:

The following distributions are identified as candidates for modeling of the distribution for TRP location (e.g., NR-TRP-LocationInfo in TS 37.355) error

- Uniform distribution

Note: this may already be consistent with the uncertainty related to NR-TRP-LocationInfo specified in TS 37.355

- Normal distribution

Note: it is up to RAN2 how to use the identified distributions

RAN1#111 Agreement:

For LMF-based positioning integrity mode, for DL-TDOA, DL-AoD, UL-TDOA, UL-AoA and multi-RTT, the following distributions are identified as candidates for modeling the distribution of TRP location (e.g., Geographical Coordinates in TS 38.455) error

- Uniform distribution

- Normal distribution

Note: it is up to RAN2 how to use the identified distributions

RAN1#111 Agreement:

For LMF-based positioning integrity mode, for UL-AoA, the following distributions are identified as candidates for modeling the distribution of ARP location (e.g., ARPLocationInformation in TS 38.455) error

- Uniform distribution

- Normal distribution

Note: it is up to RAN2 how to use the identified distribution

According to the progress of RAN1, we have the following observation that may be helpful to the discussion of that in RAN2.

Observation 1: RAN1 study the error sources from the perspective of different positioning modes (LMF-based and UE-based) as well as different positioning methods, respectively.

- For LMF-based positioning integrity, the error sources are categorized into (a) timing-related measurements; (b)angle-related measurements and (c) assistance data.

- For UE-based positioning integrity, the error sources only include assistance data.

To provide a whole picture of error sources for RAT-dependent integrity, we summarize the current progress on the study of error sources in RAN1 in Table 2.2-1.

Table 2.2-1 Summary of RAN1 progress on error sources for RAT-dependent integrity

| Positioning mode | Positioning method | Error sources | Category of error sources | Candidate(s) for distribution |
| --- | --- | --- | --- | --- |
| LMF-based positioning | DL-TDOA | - RSTD measurement | Timing related measurement | Gaussian distribution |
|  |  | - TRP location (e.g., Geographical Coordinates in TS 38.455) | Assistance data | - Uniform distribution - Gaussian distribution |
|  |  | - Inter-TRP synchronization (can be caused in part by errors in SFN initialization time) | Assistance data | - Uniform distribution (NOTE 1) - Gaussian distribution |
|  | UL-TDOA | - RTOA measurement | Timing related measurement | Gaussian distribution |
|  |  | - Inter-TRP synchronization (can be caused in part by errors in SFN initialization time) | Assistance data | - Uniform distribution (NOTE 1) - Gaussian distribution |
|  |  | - TRP location (e.g., Geographical Coordinates in TS 38.455) | Assistance data | - Uniform distribution - Gaussian distribution |
|  | Multi-RTT | - UE Rx-Tx time difference measurement - gNB Rx-Tx time difference measurement | Timing related measurement | Gaussian distribution |
|  |  | - TRP location (e.g., Geographical Coordinates in TS 38.455) | Assistance data | - Uniform distribution - Gaussian distribution |
|  | UL-AoA | - Angle of arrival measurement (e.g., AoA/ZoA) | Angle related measurement | Alt. 1: No conversion (e.g., the measurement error is expressed as error in AoA or ZoA in LCS/GCS) Alt. 2: conversion function (defined as function of AoA/ZoA in LCS). |
|  |  | - ARP location (e.g., ARPLocationInformation) error | Assistance data | - Uniform distribution - Gaussian distribution |
|  |  | - TRP location (e.g., Geographical Coordinates in TS 38.455) | Assistance data | - Uniform distribution - Gaussian distribution |
|  | DL-AoD | - TRP location (e.g., Geographical Coordinates in TS 38.455) error | Assistance data | - Uniform distribution - Gaussian distribution |
|  |  | - DL-PRS RSRPP of the first path or RSRP | Measurement | FFS |
| UE-based positioning | DL-TDOA | - TRP location (e.g., NR-TRP-LocationInfo) | Assistance data | - Uniform distribution (NOTE 2) - Gaussian distribution |
|  |  | - Inter-TRP synchronization (e.g., NR-RTD-Info) | Assistance data | - Uniform distribution (NOTE 1) - Gaussian distribution |
|  | DL-AoD | - TRP location (e.g., NR-TRP-LocationInfo) | Assistance data |  |
| NOTE 1: This may already be consistent with the uncertainty related to NR-RTD-Info in TS 37.355. NOTE 2: This may already be consistent with the uncertainty related to NR-TRP-LocationInfo in TS 37.355. NOTE 3: RAN1 could not reach consensus on whether beam information (NR-TRP-BeamAntennaInfo) and boresight direction of DL PRS (NR-DL-PRS-BeamInfo) are error sources or not as of now. |  |  |  |  |

- Capture the above table for summarizing the error sources for the integrity of RAT-dependent positioning.

In addition, during SI phase, RAN1 agreed that it’s up to RAN2 how to use the error distribution (e.g., inter-TRP synchronization error, Timing measurement error, TRP location error, ARP location error). Therefore, the details of the spec enhancement for reporting the error distribution should be handled in WI phase.

- Note that the spec impact of the integrity related positioning measurements (e.g., RTOA, gNB Rx-Tx time difference measurement) and assistance data (e.g., TRP location, ARP location) transfer between LMF and gNB is still FFS, which should refer to RAN3 expertise.

- LS to RAN3 to capture the integrity related error sources (i.e., integrity related positioning measurements and assistance data) transfer between LMF and gNB.

Signalling aspects

2.3.1	Signaling for UE-based integrity

In the newly approved WID, it has been agreed to study both UE-based and LMF-based integrity for RAT-dependent positioning. For UE-based integrity, Rel-17 GNSS UE-based integrity signaling can be used as baseline, where some potential enhancements related to errors in assistance data (e.g., related to inter-TRP synchronization error and TRP locations) should be considered and could be achieved by the enhancement of LPP ProvideAssistanceData.

- For UE-based integrity, support the following enhancements:

  - LPP signaling to deliver the error of related assistance data from LMF to UE, which at least includes:

    - TRP location error for DL-TDOA and DL-AoD

    - Inter-TRP synchronization error for DL-TDOA

  - NRPPa signaling to deliver the error of related integrity assistance data from gNBs to LMF, which at least includes:

    - TRP location (e.g., Geographical Coordinates) error

    - inter-TRP synchronization error

2.3.2	Signaling for LMF-based integrity

For LMF-based integrity, several enhancements could be considered based on the discussion of SI phase, which may include:

- LPP Signalling

  - Signalling for RAT-dependent positioning integrity capability, integrity KPIs and integrity results transfer for GNSS integrity can be reused, so no essential significant is required.

  - Signalling to deliver the integrity related positioning measurements from UE to LMF

    - RSTD measurement error for DL-TDOA

    - UE Rx-Tx time difference error for Multi-RTT

    - DL-PRS RSRPP error of the first path or RSRP

- NRPPa Signalling

  - Signalling to deliver the integrity related positioning measurements from gNBs to LMF

    - RTOA measurement error for UL-TDOA

    - gNB Rx-Tx time difference measurement error for Multi-RTT

    - Angle of arrival measurement (e.g., AoA/ZoA) error for UL-AoA

  - Signalling to deliver integrity assistance data from gNBs to LMF

    - TRP location (e.g., Geographical Coordinates) error

    - ARP location (e.g., ARPLocationInformation) error for UL-AoA

    - inter-TRP synchronization error for DL-TDOA and UL-TDOA

- For LMF-based integrity, support the following enhancements:

  - LPP signaling to deliver the error of related positioning measurements from UE to LMF, which at least includes:

    - RSTD measurement error for DL-TDOA

    - UE Rx-Tx time difference error for Multi-RTT

    - DL-PRS RSRPP error of the first path or RSRP

  - NRPPa signaling to deliver the error of related positioning measurements from gNBs to LMF, which at least includes:

    - RTOA measurement error for UL-TDOA

    - gNB Rx-Tx time difference measurement error for Multi-RTT

    - Angle of arrival measurement (e.g., AoA/ZoA) error for UL-AoA

  - NRPPa signaling to deliver the error of related integrity assistance data from gNBs to LMF, which at least includes:

    - TRP location (e.g., Geographical Coordinates) error

    - ARP location (e.g., ARPLocationInformation) error for UL-AoA

    - inter-TRP synchronization error for DL-TDOA and UL-TDOA

2.3.3	Signalling of request/response of RAT-dependent integrity

Request of RAT-dependent integrity measurements

In R17 GNSS integrity, only UE-based GNSS integrity calculation is supported. The UE reports the calculated PL to the LMF directly and does not need to send measurement report to the LMF. Hence, there is no such issue reporting integrity measurement for LMF-based approach in R17. However, in R18 RAT-dependent integrity, LMF-based approach has been defined that the UE can be requested to report measurement results related to integrity for LMF-based integrity. Based on this, we propose the following

- For the request of RAT-dependent integrity measurement, support the request of integrity related positioning measurements per positioning method for LMF-based RAT-dependent integrity.

Request for RAT-dependent integrity results

In addition to LMF-based approach, UE-based approach has also been supported for RAT-dependent integrity. In R17 GNSS integrity, the following change has been made for RequestLocationinformation

CommonIEsRequestLocationInformation ::= SEQUENCE { locationInformationType LocationInformationType, triggeredReporting TriggeredReportingCriteria OPTIONAL, -- Cond ECID periodicalReporting PeriodicalReportingCriteria OPTIONAL, -- Need ON additionalInformation AdditionalInformation OPTIONAL, -- Need ON qos QoS OPTIONAL, -- Need ON environment Environment OPTIONAL, -- Need ON locationCoordinateTypes LocationCoordinateTypes OPTIONAL, -- Need ON velocityTypes VelocityTypes OPTIONAL, -- Need ON ..., [[ messageSizeLimitNB-r14 MessageSizeLimitNB-r14 OPTIONAL -- Need ON ]], [[ segmentationInfo-r14 SegmentationInfo-r14 OPTIONAL -- Need ON ]], [[ scheduledLocationRequest-r17 ScheduledLocationRequest-r17 OPTIONAL, -- Need ON integrityInformationRequest-r17 IntegrityInformationRequest-r17 OPTIONAL -- Need ON ]] }

Considering integrity is enabled for multiple positioning methods for RAT-dependent positioning in Rel-18, while if a result is requested, the result should be consolidated between multiple positioning methods that have been provided assistance data for UE-based integrity. Hence, we propose the following.

- For the request for RAT-dependent integrity results, reuse the legacy signaling in commonIEsRequestLocationInformation. No spec change is needed.

Text Proposal for LPP

In the following, we provide a text proposal based on the discussion above for the LPP spec. And, we propose

- Consider the text proposal of LPP in the Annex A as a baseline for the support of RAT-dependent positioning integrity.

Conclusion

In this contribution, we provide our considerations on the open issues from the perspective of the DNU flag, error sources and signalling for RAT-dependent positioning integrity. Based on our analysis and discussion, we have the following observations and proposals:

Observation 1: RAN1 study the error sources from the perspective of different positioning modes (LMF-based and UE-based) as well as different positioning methods, respectively.

- For LMF-based positioning integrity, the error sources are categorized into (a) timing-related measurements; (b)angle-related measurements and (c) assistance data.

- For UE-based positioning integrity, the error sources only include assistance data.

DNU flag

- Introduce a DNU flag for the integrity assistance data for each error source per RAT-dependent positioning method.

- Send an LS to R1 asking whether the DNU flag for measurement is needed for LMF-based integrity.

Error sources

- Capture the table in Annex B for summarizing the error sources for the integrity of RAT-dependent positioning.

- LS to RAN3 to capture the integrity related error sources (i.e., integrity related positioning measurements and assistance data) transfer between LMF and gNB.

- Signaling aspects

- For UE-based integrity, support the following enhancements:

  - LPP signaling to deliver the error of related assistance data from LMF to UE, which at least includes:

    - TRP location error for DL-TDOA and DL-AoD

    - Inter-TRP synchronization error for DL-TDOA

  - NRPPa signaling to deliver the error of related integrity assistance data from gNBs to LMF, which at least includes:

    - TRP location (e.g., Geographical Coordinates) error

    - inter-TRP synchronization error

- For LMF-based integrity, support the following enhancements:

  - LPP signaling to deliver the error of related positioning measurements from UE to LMF, which at least includes:

    - RSTD measurement error for DL-TDOA

    - UE Rx-Tx time difference error for Multi-RTT

    - DL-PRS RSRPP error of the first path or RSRP

  - NRPPa signaling to deliver the error of related positioning measurements from gNBs to LMF, which at least includes:

    - RTOA measurement error for UL-TDOA

    - gNB Rx-Tx time difference measurement error for Multi-RTT

    - Angle of arrival measurement (e.g., AoA/ZoA) error for UL-AoA

  - NRPPa signaling to deliver the error of related integrity assistance data from gNBs to LMF, which at least includes:

    - TRP location (e.g., Geographical Coordinates) error

    - ARP location (e.g., ARPLocationInformation) error for UL-AoA

    - inter-TRP synchronization error for DL-TDOA and UL-TDOA

Signaling of request/response of RAT-dependent integrity

- For the request of RAT-dependent integrity measurement, support the request of integrity related positioning measurements per positioning method for LMF-based RAT-dependent integrity.

- For the request for RAT-dependent integrity results, reuse the legacy signaling in commonIEsRequestLocationInformation. No spec change is needed.

- Text Proposal

- Consider the text proposal of LPP in the Annex A as a baseline for the support of RAT-dependent positioning integrity.

References

- RP-223549, New WID on Expanded and Improved NR Positioning, 12th – 16th Dec., 2022.

- RP-222982, Status Report to TSG, RAN#98, e-Meeting, 12th – 16th Dec., 2022.

# Annex A: Text Proposal to TS 37.355 for RAT-dependent positioning integrity

============================START OF CHANGE===========================

### 6.4.3	Common NR Positioning Information Elements

#### –	NR-RTD-Info

The IE NR-RTD-Info is used by the location server to provide time synchronization information between a reference TRP and a list of neighbour TRPs.

-- ASN1START

NR-RTD-Info-r16 ::= SEQUENCE {

referenceTRP-RTD-Info-r16		ReferenceTRP-RTD-Info-r16,

rtd-InfoList-r16				RTD-InfoList-r16,

...

}

ReferenceTRP-RTD-Info-r16 ::= SEQUENCE {

dl-PRS-ID-Ref-r16				INTEGER (0..255),

nr-PhysCellID-Ref-r16			NR-PhysCellID-r16		OPTIONAL,	-- Need ON

nr-CellGlobalID-Ref-r16			NCGI-r15				OPTIONAL,	-- Need ON

nr-ARFCN-Ref-r16				ARFCN-ValueNR-r15		OPTIONAL,	-- Need ON

refTime-r16						CHOICE {

systemFrameNumber-r16		BIT STRING (SIZE (10)),

utc-r16						UTCTime,

...

},

rtd-RefQuality-r16				NR-TimingQuality-r16	OPTIONAL,	-- Need ON

...,

[[

rtd-RefError-r18				CHOICE {

nr-TimingQuality-r16		NR-TimingQuality-r16,

nr-TimingQualityNorm-r18	NR-TimingQualityNorm-r18

}														OPTIONAL  -- Cond Integrity1

]]

}

RTD-InfoList-r16 ::= SEQUENCE (SIZE (1..nrMaxFreqLayers-r16)) OF RTD-InfoListPerFreqLayer-r16

RTD-InfoListPerFreqLayer-r16 ::= SEQUENCE (SIZE(1..nrMaxTRPsPerFreq-r16)) OF RTD-InfoElement-r16

RTD-InfoElement-r16 ::= SEQUENCE {

dl-PRS-ID-r16					INTEGER (0..255),

nr-PhysCellID-r16				NR-PhysCellID-r16		OPTIONAL,	-- Need ON

nr-CellGlobalID-r16				NCGI-r15				OPTIONAL,	-- Need ON

nr-ARFCN-r16					ARFCN-ValueNR-r15		OPTIONAL,	-- Need ON

subframeOffset-r16				INTEGER (0..1966079),

rtd-Quality-r16					NR-TimingQuality-r16,

...,

[[

rtd-Error-r18				CHOICE {

nr-TimingQuality-r16		NR-TimingQuality-r16,

nr-TimingQualityNorm-r18	NR-TimingQualityNorm-r18

}														OPTIONAL  -- Cond Integrity1

]]

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| Integrity1 | The field is mandatory present if UE-based RAT-dependent integrity for DL-TDOA is supported; otherwise it is not present. |

NR-RTD-Info field descriptions

referenceTRP-RTD-Info This field defines the reference TRP for the RTD and comprises the following sub-fields: - dl-PRS-ID-Ref: This field is used along with a DL-PRS Resource Set ID and a DL-PRS Resources ID to uniquely identify a DL-PRS Resource, and is associated to the reference TRP. - nr-PhysCellId-Ref: This field specifies the physical cell identity of the reference TRP. - nr-CellGlobalId-Ref: This field specifies the NCGI, the globally unique identity of a cell in NR, of the reference TRP. - nr-ARFCN-Ref: This field specifies the NR-ARFCN of the TRP's CD-SSB (as defined in TS 38.300 [47]) corresponding to nr-PhysCellID. - refTime: This field specifies the reference time at which the rtd-InfoList is valid. The systemFrameNumber choice refers to the SFN of the reference TRP. - rtd-RefQuality: This field specifies the quality of the timing of reference TRP, used to determine the RTD values provided in rtd-InfoList.

dl-PRS-ID This field is used along with a DL-PRS Resource Set ID and a DL-PRS Resources ID to uniquely identify a DL-PRS Resource. This ID can be associated with multiple DL-PRS Resource Sets associated with a single TRP for which the RTD-InfoElement is applicable.

nr-PhysCellID This field specifies the physical cell identity of the associated TRP for which the RTD-InfoElement is applicable, as defined in TS 38.331 [35].

nr-CellGlobalID This field specifies the NCGI, the globally unique identity of a cell in NR, of the associated TRP for which the RTD-InfoElement is applicable, as defined in TS 38.331 [35]. The server should include this field if it considers that it is needed to resolve ambiguity in the TRP indicated by nr-PhysCellID.

nr-ARFCN This field specifies the NR-ARFCN of the TRP's CD-SSB (as defined in TS 38.300 [47]) corresponding to nr-PhysCellID for which the RTD-InfoElement is applicable.

subframeOffset This field specifies the subframe boundary offset at the TRP antenna location between the reference TRP and this neighbour TRP in time units where Δfmax=480∙103 Hz and (TS 38.211 [41]). The offset is counted from the beginning of a subframe #0 of the reference TRP to the beginning of the closest subsequent subframe of this neighbour TRP. Scale factor 1 Tc.

rtd-Quality This field specifies the quality of the RTD.

rtd-RefError This field specifies the RTD error for the reference TRP, which is used for integrity calculation.

rtd-Error This field specifies the RTD error for the associated TRP, which is used for integrity calculation.

#### –	RelativeLocation

The IE RelativeLocation provides a location relative to some known reference location.

-- ASN1START

RelativeLocation-r16 ::= SEQUENCE {

milli-arc-second-units-r16 	ENUMERATED { mas0-03, mas0-3, mas3, mas30, ...},

height-units-r16			ENUMERATED {mm, cm, m, ...},

delta-latitude-r16			Delta-Latitude-r16,

delta-longitude-r16			Delta-Longitude-r16,

delta-height-r16			Delta-Height-r16,

locationUNC-r16				LocationUncertainty-r16				OPTIONAL,		-- Need OP

...,

[[

locationError-r18				CHOICE {

locationUncertainty-r16			LocationUncertainty-r16,

locationUncertaintyNorm-r18		LocationUncertaintyNorm-r18

}														OPTIONAL          -- Cond Integrity1

]]

}

Delta-Latitude-r16 ::= SEQUENCE {

delta-Latitude-r16						INTEGER (-1024..1023),

coarse-delta-Latitude-r16				INTEGER (0..4095)		OPTIONAL,		-- Need OP

...

}

Delta-Longitude-r16 ::= SEQUENCE {

delta-Longitude-r16						INTEGER (-1024..1023),

coarse-delta-Longitude-r16				INTEGER (0..4095)		OPTIONAL,		-- Need OP

...

}

Delta-Height-r16 ::= SEQUENCE {

delta-Height-r16						INTEGER (-1024..1023),

coarse-delta-Height-r16					INTEGER (0..4095)		OPTIONAL,		-- Need OP

...

}

LocationUncertainty-r16 ::= SEQUENCE {

horizontalUncertainty-r16				INTEGER (0..255),

horizontalConfidence-r16				INTEGER (0..100),

verticalUncertainty-r16					INTEGER (0..255),

verticalConfidence-r16					INTEGER (0..100)

}

LocationUncertaintyNorm-r18 ::= SEQUENCE {

meanHorizontalUncertainty-r18		INTEGER (0..255),

stdHorizontalUncertainty-r18		INTEGER (0..255),

meanVerticalUncertainty-r18		INTEGER (0..255),

stdVerticalUncertainty-r18		INTEGER (0..255)

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| Integrity1 | The field is mandatory present if UE-based RAT-dependent integrity is supported; otherwise it is not present. |

RelativeLocation field descriptions

milli-arc-second-units This field provides the units and scale factor for the delta-latitude and delta-longitude fields. Enumerated values mas0-03, mas0-3, mas3, and mas30, correspond to 0.03, 0.3, 3, and 30 milliarcseconds, respectively.

height-units This field provides the units and scale factor for the delta-height field. Enumerated values mm, cm, and m correspond to 10-3 metre, 10-2 metre, and 1 metres, respectively.

delta-latitude This field specifies the delta value in latitude of the desired location, defined as "desired location" minus "reference point location" and comprises the following sub-fields: - delta-Latitude specifies the delta value in latitude in the unit provided in milli-arc-second-units field. - coarse-delta-Latitude specifies the delta value in latitude in 1024 times the size of the unit provided in milli-arc-second-units field and with the same sign as in the delta-Latitude field. If this field is absent, the value for coarse-delta-Latitude is zero. I.e., the full delta-latitude is given by: (delta-Latitude × milli-arc-second-units) ± (coarse-delta-Latitude × 1024 × milli-arc-second-units) [milli-arc-seconds]

delta-longitude This field specifies the delta value in longitude of the desired location, defined as "desired location" minus "reference point location" and comprises the following sub-fields: - delta-Longitude specifies the delta value in longitude in the unit provided in milli-arc-second-units field. - coarse-delta-Longitude specifies the delta value in longitude in 1024 times the size of the unit provided in milli-arc-second-units field and with the same sign as in the delta-Longitude field. If this field is absent, the value for coarse-delta-Longitude is zero. I.e., the full delta-longitude is given by: (delta-Longitude × milli-arc-second-units) ± (coarse-delta-Latitude × 1024 × milli-arc-second-units) [milli-arc-seconds]

delta-height This field specifies the delta value in ellipsoidal height of the desired location, defined as "desired location" minus "reference point location" and comprises the following sub-fields: - delta-Height specifies the delta value in ellipsoidal height in the unit provided in height-units field. - coarse-delta-Height specifies the delta value in ellipsoidal height in 1024 times the size of the unit provided in height-units field and with the same sign as in the delta-Height field. If this field is absent, the value for coarse-delta-Height is zero. I.e., the full delta-height is given by: (delta-Height × height-units) ± (coarse-delta-Height × 1024 × height-units) [metres]

locationUNC This field specifies the uncertainty of the location coordinates and comprises the following sub-fields: - horizontalUncertainty indicates the horizontal uncertainty of the ARP latitude/longitude. The ′horizontalUncertainty′ corresponds to the encoded high accuracy uncertainty as defined in TS 23.032 [15] and ′horizontalConfidence′ corresponds to confidence as defined in TS 23.032 [15]. - verticalUncertainty indicates the vertical uncertainty of the ARP altitude. The 'verticalUncertainty' corresponds to the encoded high accuracy uncertainty as defined in TS 23.032 [15] and 'verticalConfidence' corresponds to confidence as defined in TS 23.032 [15]. If this field is absent, the uncertainty is the same as for the associated reference point location.

location-Error This field specifies the uncertainty of the location coordinates in uniform distribution or normal distribution, which is used for integrity calculation as defined in TS 38.305 [40].

locationUncertaintyNorm This field specifies the uncertainty of the location coordinates in normal distribution and comprises the following sub-fields: - meanHorizontalUncertainty is the mean value for the horizontal uncertainty of the ARP latitude/longitude. - stdHorizontalUncertainty is standard deviation for the horizontal uncertainty of the ARP latitude/longitude. - meanVerticalUncertainty is the mean value for the vertical uncertainty of the ARP altitude. - stdVerticalUncertainty is standard deviation for the vertical uncertainty of the ARP latitude/longitude. If this field is absent, the uncertainty is the same as for the associated reference point location.

- –	NR-TimingQualityNorm

The IE NR-TimingQualityNorm defines the quality of a timing value (e.g., of a TOA measurement), which is given in the form of normal distribution and used for integrity calculation as defined in TS 38.305 [40].

-- ASN1START

NR-TimingQualityNorm-r18 ::= SEQUENCE {

stdTimingQualityNorm-r18		INTEGER (0..255),

...

}

-- ASN1STOP

NR-TimingError field descriptions

stdTimingQualityNorm This field provides the standard deviation of the uncertainty/error for the timing value.

============================NEXT CHANGE==============================

#### 6.5.10.4	NR DL-TDOA Location Information Elements

#### –	NR-DL-TDOA-SignalMeasurementInformation

The IE NR-DL-TDOA-SignalMeasurementInformation is used by the target device to provide NR DL-TDOA measurements to the location server.

NOTE 1:	The dl-PRS-ReferenceInfo defines the "RSTD reference" TRP. The nr-RSTD's and nr-RSTD-ResultDiff's in nr-DL-TDOA-MeasList are provided relative to the "RSTD reference" TRP.

NOTE 2:	The "RSTD reference" TRP may or may not be the same as the "assistance data reference" TRP provided by nr-DL-PRS-ReferenceInfo in IE NR-DL-PRS-AssistanceData.

NOTE 3:	The target device includes a value of zero for the nr-RSTD and nr-RSTD-ResultDiff of the "RSTD reference" TRP in nr-DL-TDOA-MeasList.

-- ASN1START

NR-DL-TDOA-SignalMeasurementInformation-r16 ::= SEQUENCE {

dl-PRS-ReferenceInfo-r16		DL-PRS-ID-Info-r16,

nr-DL-TDOA-MeasList-r16			NR-DL-TDOA-MeasList-r16,

...,

[[

nr-UE-RxTEG-TimingErrorMargin-r17	TEG-TimingErrorMargin-r17		OPTIONAL	-- Cond UERxTEG

]]

}

NR-DL-TDOA-MeasList-r16 ::= SEQUENCE (SIZE(1..nrMaxTRPs-r16)) OF NR-DL-TDOA-MeasElement-r16

NR-DL-TDOA-MeasElement-r16 ::= SEQUENCE {

dl-PRS-ID-r16					INTEGER (0..255),

nr-PhysCellID-r16				NR-PhysCellID-r16								OPTIONAL,

nr-CellGlobalID-r16				NCGI-r15										OPTIONAL,

nr-ARFCN-r16					ARFCN-ValueNR-r15								OPTIONAL,

nr-DL-PRS-ResourceID-r16		NR-DL-PRS-ResourceID-r16	 					OPTIONAL,

nr-DL-PRS-ResourceSetID-r16		NR-DL-PRS-ResourceSetID-r16						OPTIONAL,

nr-TimeStamp-r16				NR-TimeStamp-r16,

nr-RSTD-r16						CHOICE {

k0-r16						INTEGER (0..1970049),

k1-r16						INTEGER (0..985025),

k2-r16						INTEGER (0..492513),

k3-r16						INTEGER (0..246257),

k4-r16						INTEGER (0..123129),

k5-r16						INTEGER (0..61565),

...

},

nr-AdditionalPathList-r16		NR-AdditionalPathList-r16						OPTIONAL,

nr-TimingQuality-r16			NR-TimingQuality-r16,

nr-DL-PRS-RSRP-Result-r16		INTEGER (0..126)								OPTIONAL,

nr-DL-TDOA-AdditionalMeasurements-r16

NR-DL-TDOA-AdditionalMeasurements-r16			OPTIONAL,

...,

[[

nr-UE-Rx-TEG-ID-r17					INTEGER (0..maxNumOfRxTEGs-1-r17)			OPTIONAL,

nr-DL-PRS-FirstPathRSRP-Result-r17	INTEGER (0..126)							OPTIONAL,

nr-los-nlos-Indicator-r17			CHOICE {

perTRP-r17						LOS-NLOS-Indicator-r17,

perResource-r17					LOS-NLOS-Indicator-r17

}																				OPTIONAL,

nr-AdditionalPathListExt-r17		NR-AdditionalPathListExt-r17				OPTIONAL,

nr-DL-TDOA-AdditionalMeasurementsExt-r17

NR-DL-TDOA-AdditionalMeasurementsExt-r17	OPTIONAL

]],

[[

nr-RSTD-Error-R18				NR-TimingQualityNorm-r18		OPTIONAL

]]

}

NR-DL-TDOA-AdditionalMeasurements-r16 ::= SEQUENCE (SIZE (1..3)) OF

NR-DL-TDOA-AdditionalMeasurementElement-r16

NR-DL-TDOA-AdditionalMeasurementsExt-r17 ::= SEQUENCE (SIZE (1..maxAddMeasTDOA-r17)) OF

NR-DL-TDOA-AdditionalMeasurementElement-r16

NR-DL-TDOA-AdditionalMeasurementElement-r16 ::= SEQUENCE {

nr-DL-PRS-ResourceID-r16		NR-DL-PRS-ResourceID-r16	 					OPTIONAL,

nr-DL-PRS-ResourceSetID-r16		NR-DL-PRS-ResourceSetID-r16 					OPTIONAL,

nr-TimeStamp-r16				NR-TimeStamp-r16,

nr-RSTD-ResultDiff-r16			CHOICE {

k0-r16						INTEGER (0..8191),

k1-r16						INTEGER (0..4095),

k2-r16						INTEGER (0..2047),

k3-r16						INTEGER (0..1023),

k4-r16						INTEGER (0..511),

k5-r16						INTEGER (0..255),

...

},

nr-TimingQuality-r16			NR-TimingQuality-r16,

nr-DL-PRS-RSRP-ResultDiff-r16	INTEGER (0..61)									OPTIONAL,

nr-AdditionalPathList-r16		NR-AdditionalPathList-r16						OPTIONAL,

...,

[[

nr-UE-Rx-TEG-ID-r17				INTEGER (0..maxNumOfRxTEGs-1-r17)				OPTIONAL,

nr-DL-PRS-FirstPathRSRP-ResultDiff-r17

INTEGER (0..61)									OPTIONAL,

nr-los-nlos-IndicatorPerResource-r17

LOS-NLOS-Indicator-r17							OPTIONAL,

nr-AdditionalPathListExt-r17	NR-AdditionalPathListExt-r17					OPTIONAL

]],

[[

nr-RSTD-Error-R18				NR-TimingQualityNorm-r18		OPTIONAL

]]

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| UERxTEG | The field is optionally present, need OP, if the field nr-UE-Rx-TEG-ID is present; otherwise it is not present. |

NR-DL-TDOA-SignalMeasurementInformation field descriptions

nr-UE-RxTEG-TimingErrorMargin This field specifies the UE Rx TEG timing error margin value for all the UE Rx TEGs within one NR-DL-TDOA-SignalMeasurementInformation. If the nr-UE-Rx-TEG-ID is present and this field is absent, the receiver should consider the UE Rx TEG timing error margin value to be the maximum applicable value as defined in TS 38.133 [46].

dl-PRS-ID This field is used along with a DL-PRS Resource Set ID and a DL-PRS Resources ID to uniquely identify a DL-PRS Resource. This ID can be associated with multiple DL-PRS Resource Sets associated with a single TRP. Each TRP should only be associated with one such ID.

nr-PhysCellID This field specifies the physical cell identity of the associated TRP, as defined in TS 38.331 [35].

nr-CellGlobalID This field specifies the NCGI, the globally unique identity of a cell in NR, of the associated TRP, as defined in TS 38.331 [35].

nr-ARFCN This field specifies the NR-ARFCN of the TRP's CD-SSB (as defined in TS 38.300 [47]) corresponding to nr-PhysCellID.

nr-TimeStamp This field specifies the time instance at which the TOA and DL PRS-RSRP/RSRPP (if included) measurement is performed. The nr-SFN and nr-Slot in IE NR-TimeStamp correspond to the TRP provided in dl-PRS-ReferenceInfo as specified in TS 38.214 [45]. Note, the TOA measurement refers to the TOA of this neighbour TRP or the reference TRP, as applicable, used to determine the nr-RSTD or nr-RSTD-ResultDiff.

nr-RSTD This field specifies the relative timing difference between this neighbour TRP and the PRS reference TRP, as defined in TS 38.215 [36]. Mapping of the measured quantity is defined as in TS 38.133 [46].

nr-AdditionalPathList This field specifies one or more additional detected path timing values for the TRP or resource, relative to the path timing used for determining the nr-RSTD value. If this field was requested but is not included, it means the UE did not detect any additional path timing values. If this field is present, the field nr-AdditionalPathListExt shall be absent.

nr-TimingQuality This field specifies the target device′s best estimate of the quality of the TOA measurement. Note, the TOA measurement refers to the TOA of this neighbour TRP or the reference TRP, as applicable, used to determine the nr-RSTD or nr-RSTD-ResultDiff.

nr-DL-PRS-RSRP-Result This field specifies the NR DL-PRS reference signal received power (DL PRS-RSRP) measurement, as defined in TS 38.215 [36]. The mapping of the quantity is defined as in TS 38.133 [46].

nr-UE-Rx-TEG-ID This field provides the ID of the UE Rx TEG associated with the TOA measurement. Note, the TOA measurement refers to the TOA of this neighbour TRP or the reference TRP, as applicable, used to determine the nr-RSTD or nr-RSTD-ResultDiff. When different UE Rx TEGs for RSTD measurements are requested, the maximum number of reported RSTD measurements associated with different DL-PRS Resources per UE Rx TEG per target TRP is 4.

nr-DL-PRS-FirstPathRSRP-Result This field specifies the NR DL-PRS reference signal received path power (DL PRS-RSRPP) of the first detected path in time, as defined in TS 38.215 [36]. The mapping of the measured quantity is defined as in TS 38.133 [46].

nr-los-nlos-Indicator This field specifies the target device's best estimate of the LOS or NLOS of the TOA measurement for the TRP or resource. Note, the TOA measurement refers to the TOA of this neighbour TRP or the reference TRP, as applicable, used to determine the nr-RSTD or nr-RSTD-ResultDiff. NOTE: If the requested type or granularity in nr-los-nlos-IndicatorRequest is not possible, the target device may provide a different type and granularity for the estimated LOS-NLOS-Indicator.

nr-AdditionalPathListExt This field provides up to 8 additional detected path timing values for the TRP or resource, relative to the path timing used for determining the nr-RSTD value. If this field was requested but is not included, it means the UE did not detect any additional path timing values. If this field is present, the field nr-AdditionalPathList shall be absent.

nr-DL-TDOA-AdditionalMeasurementsExt This field, in addition to the measurements provided in NR-DL-TDOA-MeasElement, provides TOA measurements of up to 4 DL-PRS Resources of a TRP with different UE Rx TEGs. For a certain DL-PRS Resource, there can be up to 8 TOA measurement results with respect to different Rx TEGs. If this field is present, the field nr-DL-TDOA-AdditionalMeasurements should not be present.

nr-RSTD-ResultDiff This field provides the additional DL RSTD measurement result relative to nr-RSTD. The RSTD value of this measurement is obtained by adding the value of this field to the value of the nr-RSTD field. The mapping of the field is defined in TS 38.133 [46].

nr-DL-PRS-RSRP-ResultDiff This field provides the additional DL-PRS RSRP measurement result relative to nr-DL-PRS-RSRP-Result. The DL-PRS RSRP value of this measurement is obtained by adding the value of this field to the value of the nr-DL-PRS-RSRP-Result field. The mapping of the field is defined in TS 38.133 [46].

nr-DL-PRS-FirstPathRSRP-ResultDiff This field specifies the additional NR DL PRS reference signal received path power (DL PRS-RSRPP) of the first detected path in time relative to nr-DL-PRS-FirstPathRSRP-Result. The DL-PRS RSRPP of first path value of this measurement is obtained by adding the value of this field to the value of the nr-DL-PRS-FirstPathRSRP-Result field. The mapping of the field is defined in TS 38.133 [46].

nr-los-nlos-IndicatorPerResource This field specifies the target device's best estimate of the LOS or NLOS of the TOA measurement for the resource. Note, the TOA measurement refers to the TOA of this neighbour TRP or the reference TRP, as applicable, used to determine the nr-RSTD or nr-RSTD-ResultDiff. This field may only be present if the field nr-LOS-NLOS-Indicator choice indicates perResource.

nr-RSTD-Error This field provides the RSTD measurement error used for integrity calculation as in TS 38.355 [40].

============================NEXT CHANGE==============================

- 6.5.10.5	NR DL-TDOA Location Information Request

- –	NR-DL-TDOA-RequestLocationInformation

The IE NR-DL-TDOA-RequestLocationInformation is used by the location server to request NR DL-TDOA location measurements from a target device.

-- ASN1START

NR-DL-TDOA-RequestLocationInformation-r16 ::= SEQUENCE {

nr-DL-PRS-RstdMeasurementInfoRequest-r16	ENUMERATED { true }				OPTIONAL,-- Need ON

nr-RequestedMeasurements-r16				BIT STRING { prsrsrpReq (0),

firstPathRsrpReq-r17 (1)

} (SIZE(1..8)),

nr-AssistanceAvailability-r16				BOOLEAN,

nr-DL-TDOA-ReportConfig-r16					NR-DL-TDOA-ReportConfig-r16		OPTIONAL, -- Need ON

additionalPaths-r16							ENUMERATED { requested }		OPTIONAL, -- Need ON

...,

[[

nr-UE-RxTEG-Request-r17						ENUMERATED { requested }		OPTIONAL, -- Need ON

nr-los-nlos-IndicatorRequest-r17	SEQUENCE {

type-r17			LOS-NLOS-IndicatorType1-r17,

granularity-r17	LOS-NLOS-IndicatorGranularity1-r17,

...

}										OPTIONAL, -- Need ON

additionalPathsExt-r17						ENUMERATED { requested }		OPTIONAL, -- Need ON

additionalPathsDL-PRS-RSRP-Request-r17		ENUMERATED { requested }		OPTIONAL, -- Need ON

multiMeasInSameReport-r17					ENUMERATED { requested }		OPTIONAL  -- Need ON

]],

[[

nr-DL-PRS-RstdMeasurementErrorRequest-r18	ENUMERATED { true }				OPTIONAL-- Need ON

]]

}

NR-DL-TDOA-ReportConfig-r16 ::= SEQUENCE {

maxDL-PRS-RSTD-MeasurementsPerTRPPair-r16	INTEGER (1..4)					OPTIONAL, -- Need ON

timingReportingGranularityFactor-r16 		INTEGER (0..5)					OPTIONAL, -- Need ON

...,

[[

measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17

ENUMERATED { n0, n2, n3, n4, n6, n8, ... }

OPTIONAL, -- Need ON

reducedDL-PRS-ProcessingSamples-r17			ENUMERATED { requested, ... }	OPTIONAL, -- Need ON

lowerRxBeamSweepingFactor-FR2-r17			ENUMERATED { requested } 		OPTIONAL  -- Need ON

]]

}

-- ASN1STOP

NR-DL-TDOA-RequestLocationInformation field descriptions

nr-DL-PRS-RstdMeasurementInfoRequest This field indicates whether the target device is requested to report DL-PRS Resource ID(s) or DL-PRS Resource Set ID(s) used for determining the timing of each TRP in RSTD measurements.

nr-RequestedMeasurements This field specifies the NR DL-TDOA measurements requested. This is represented by a bit string, with a one-value at the bit position means the particular measurement is requested; a zero-value means not requested.

nr-AssistanceAvailability This field indicates whether the target device may request additional PRS assistance data from the server. TRUE means allowed and FALSE means not allowed.

additionalPaths This field, if present, indicates that the target device is requested to provide the nr-AdditionalPathList in IE NR-DL-TDOA-SignalMeasurementInformation. If this field is present, the field additionalPathsExt shall be absent.

nr-UE-RxTEG-Request This field, if present, indicates that the target device is requested to provide the nr-UE-Rx-TEG-ID in IE NR-DL-TDOA-SignalMeasurementInformation.

nr-los-nlos-IndicatorRequest This field, if present, indicates that the target device is requested to provide the indicated type and granularity of the estimated LOS-NLOS-Indicator in the NR-DL-TDOA-SignalMeasurementInformation.

additionalPathsExt This field, if present, indicates that the target device is requested to provide the nr-AdditionalPathListExt in IE NR-DL-TDOA-SignalMeasurementInformation. If this field is present, the field additionalPaths shall be absent.

additionalPathsDL-PRS-RSRP-Request This field, if present, indicates that the target device is requested to provide the nr-DL-PRS-RSRPP for the additional paths in IE NR-AdditionalPathList.

multiMeasInSameReport This field, if present, indicates that the target device is requested to provide multiple measurement instances in a single measurement report; i.e., include the nr-DL-TDOA-SignalMeasurementInstances (in the case of UE-assisted mode is requested) or nr-DL-TDOA-LocationInformationInstances (in the case of UE-based mode is requested) in IE NR-DL-TDOA-ProvideLocationInformation.

maxDL-PRS-RSTD-MeasurementsPerTRPPair This field specifies the maximum number of. DL-PRS RSTD measurements per pair of TRPs. The maximum number is defined across all Positioning Frequency Layers.

timingReportingGranularityFactor This field specifies the recommended reporting granularity for the DL RSTD measurements. Value (0..5) corresponds to (k0..k5) used for nr-RSTD and nr-RSTD-ResultDiff in NR-DL-TDOA-MeasElement. The UE may select a different granularity value for nr-RSTD and nr-RSTD-ResultDiff.

measureSameDL-PRS-ResourceWithDifferentRxTEGs This field, if present, indicates that the target device is requested to measure the same DL-PRS Resource of a TRP with N different UE Rx TEGs. Enumerated value 'n0' indicates that the number N of different UE Rx TEGs to measure the same DL PRS Resource can be determined by the target device, value 'n2' indicates that the target device is requested to measure the same DL-PRS Resource of a TRP with 2 different UE Rx TEGs, value 'n3' indicates that the target device is requested to measure the same DL-PRS Resource of a TRP with 3 different UE Rx TEGs, and so on. If this field is present, the field nr-UE-TxTEG-Request should also be present.

reducedDL-PRS-ProcessingSamples This field, if present and set to 'requested', indicates that the target device is requested to perform the requested measurements with reduced number of samples (M=1 or M=2) as specified in TS 38.133 [46].

lowerRxBeamSweepingFactor-FR2 This field, if present, indicates that the target device is requested to use a lower Rx beam sweeping factor than 8 for FR2 according to UE's capability.

nr-DL-PRS-RstdMeasurementErrorRequest This field indicates whether the target device is requested to report RSTD measurement error used for integrity calculation as in TS 38.305 [40].

============================NEXT CHANGE==============================

- 6.5.10.6	NR DL-TDOA Capability Information

- –	NR-DL-TDOA-ProvideCapabilities

The IE NR-DL-TDOA-ProvideCapabilities is used by the target device to indicate its capability to support NR DL-TDOA and to provide its NR DL-TDOA positioning capabilities to the location server.

-- ASN1START

NR-DL-TDOA-ProvideCapabilities-r16 ::= SEQUENCE {

nr-DL-TDOA-Mode-r16						PositioningModes,

nr-DL-TDOA-PRS-Capability-r16			NR-DL-PRS-ResourcesCapability-r16,

nr-DL-TDOA-MeasurementCapability-r16	NR-DL-TDOA-MeasurementCapability-r16,

nr-DL-PRS-QCL-ProcessingCapability-r16	NR-DL-PRS-QCL-ProcessingCapability-r16,

nr-DL-PRS-ProcessingCapability-r16		NR-DL-PRS-ProcessingCapability-r16,

additionalPathsReport-r16				ENUMERATED { supported }					OPTIONAL,

periodicalReporting-r16					PositioningModes							OPTIONAL,

...,

[[

ten-ms-unit-ResponseTime-r17			PositioningModes							OPTIONAL,

nr-PosCalcAssistanceSupport-r17			BIT STRING {	trpLocSup 		(0),

beamInfoSup		(1),

rtdInfoSup		(2),

trpTEG-InfoSup	(3)

}	(SIZE (1..8))				OPTIONAL,

nr-los-nlos-AssistanceDataSupport-r17	SEQUENCE {

type-r17		LOS-NLOS-IndicatorType2-r17,

granularity-r17	LOS-NLOS-IndicatorGranularity2-r17,

...

}											OPTIONAL,

nr-DL-PRS-ExpectedAoD-or-AoA-Sup-r17	BIT STRING {	eAoD 		(0),

eAoA		(1)

}	(SIZE (1..8))				OPTIONAL,

nr-DL-TDOA-On-Demand-DL-PRS-Support-r17	NR-On-Demand-DL-PRS-Support-r17				OPTIONAL,

nr-los-nlos-IndicatorSupport-r17		SEQUENCE {

type-r17		LOS-NLOS-IndicatorType2-r17,

granularity-r17	LOS-NLOS-IndicatorGranularity2-r17,

...

}											OPTIONAL,

additionalPathsExtSupport-r17			ENUMERATED { n4, n6, n8 }					OPTIONAL,

scheduledLocationRequestSupported-r17	ScheduledLocationTimeSupportPerMode-r17		OPTIONAL,

nr-dl-prs-AssistanceDataValidity-r17	SEQUENCE {

area-validity-r17	INTEGER (1..maxNrOfAreas-r17)			OPTIONAL,								...

}															OPTIONAL,

multiMeasInSameMeasReport-r17			ENUMERATED { supported }					OPTIONAL,

mg-ActivationRequest-r17				ENUMERATED { supported }					OPTIONAL

]],

[[

nr-IntegrityCalSupport-r18                 ENUMERATED { supported }	OPTIONAL 	-- Cond UEB

]]

}

-- ASN1STOP

NR-DL-TDOA-ProvideCapabilities field descriptions

nr-DL-TDOA-Mode This field specifies the NR DL-TDOA mode(s) supported by the target device.

periodicalReporting This field, if present, specifies the positioning modes for which the target device supports periodicalReporting. This is represented by a bit string, with a one-value at the bit position means periodicalReporting for the positioning mode is supported; a zero-value means not supported. If this field is absent, the target device does not support periodicalReporting in CommonIEsRequestLocationInformation.

ten-ms-unit-ResponseTime This field, if present, specifies the positioning modes for which the target device supports the enumerated value 'ten-milli-seconds' in the IE ResponseTime in IE CommonIEsRequestLocationInformation. This is represented by a bit string, with a one-value at the bit position means 'ten-milli-seconds' response time unit for the positioning mode is supported; a zero-value means not supported. If this field is absent, the target device does not support 'ten-milli-seconds' response time unit in CommonIEsRequestLocationInformation.

nr-PosCalcAssistanceSupport This field indicates the Position Calculation Assistance Data supported by the target device for UE-based DL-TDOA. This is represented by a bit string, with a one-value at the bit position means the particular assistance data is supported; a zero-value means not supported. - bit 0 indicates whether the field nr-TRP-LocationInfo in IE NR-PositionCalculationAssistance is supported or not; - bit 1 indicates whether the field nr-DL-PRS-BeamInfo in IE NR-PositionCalculationAssistance is supported or not; - bit 2 indicates whether the field nr-RTD-Info in IE NR-PositionCalculationAssistance is supported or not; - bit 3 indicates whether the field nr-DL-PRS-TRP-TEG-Info in IE NR-PositionCalculationAssistance is supported or not. The UE can indicate this bit only if the UE supports prs-ProcessingCapabilityBandList and any of maxNrOfDL-PRS-ResourceSetPerTrpPerFrequencyLayer, maxNrOfTRP-AcrossFreqs, maxNrOfPosLayer, maxNrOfDL-PRS-ResourcesPerResourceSet and maxNrOfDL-PRS-ResourcesPerPositioningFrequencylayer. Otherwise, the UE does not include this field.

nr-los-nlos-AssistanceDataSupport This field, if present, indicates that the target device supports the NR-DL-PRS-ExpectedLOS-NLOS-Assistance in IE NR-PositionCalculationAssistance: - type indicates whether the target device supports 'hard' value or 'hard' and 'soft' value in LOS-NLOS-Indicator in IE NR-DL-PRS-ExpectedLOS-NLOS-Assistance. - granularity indicates whether the target device supports nr-los-nlos-indicator in IE NR-DL-PRS-ExpectedLOS-NLOS-Assistance 'per-trp', 'per-resource', or both. The UE can include this field only if the UE supports one of maxDL-PRS-RSRP-MeasurementFR1, maxDL-PRS-RSRP-MeasurementFR2, dl-RSTD-MeasurementPerPairOfTRP-FR1, dl-RSTD-MeasurementPerPairOfTRP-FR2, maxNrOfRx-TX-MeasFR1, maxNrOfRx-TX-MeasFR2, supportOfRSRP-MeasFR1 and supportOfRSRP-MeasFR2. Otherwise, the UE does not include this field. NOTE: A single value is reported when both Multi-RTT and DL-TDOA are supported.

nr-DL-PRS-ExpectedAoD-or-AoA-Sup This field, if present, indicates that the target device supports the NR-DL-PRS-ExpectedAoD-or-AoA in NR-DL-PRS-AssistanceData.

nr-DL-TDOA-On-Demand-DL-PRS-Support This field, if present, indicates that the target device supports on-demand DL-PRS requests.

nr-los-nlos-IndicatorSupport This field, if present, indicates that the target device supports nr-los-nlos-Indicator reporting in IE NR-DL-TDOA-SignalMeasurementInformation. - type indicates whether the target device supports 'hard' value or 'hard' and 'soft' value in IE LOS-NLOS-Indicator. - granularity indicates whether the target device supports LOS-NLOS-Indicator reporting per TRP, per DL-PRS Resource, or both.

additionalPathsExtSupport This field, if present, indicates that the target device supports the nr-AdditionalPathListExt reporting in IE NR-DL-TDOA-SignalMeasurementInformation. The enumerated value indicates the number of additional paths supported by the target device. NOTE: The supportOfDL-PRS-FirstPathRSRP in IE NR-DL-TDOA-MeasurementCapability also applies to the additional paths.

scheduledLocationRequestSupported This field, if present, specifies the positioning modes for which the target device supports scheduled location requests – i.e., supports the IE ScheduledLocationTime in IE CommonIEsRequestLocationInformation – and the time base(s) supported for the scheduled location time for each positioning mode. If this field is absent, the target device does not support scheduled location requests.

nr-dl-prs-AssistanceDataValidity This field, if present, indicates that the target device supports validity conditions for pre-configured assistance data and comprises the following subfields: - area-validity indicates that the target device supports pre-configured assistance data with area validity. The integer number indicates the maximum number of areas the target device supports.

multiMeasInSameMeasReport This field, if present, indicates that the target device supports multiple measurement instances in a single measurement report.

mg-ActivationRequest This field, if present, indicates that the target device supports low latency measurement gap activation request for DL-PRS measurements. The UE can include this field only if the UE supports mg-ActivationRequestPRS-Meas and mg-ActivationCommPRS-Meas defined in TS 38.331 [35].

nr-IntegrityCalSupport This field, if present, indicates that the target device supports integrity calculation as defined in TS 38.305 [40].

============================NEXT CHANGE==============================

#### 6.5.11.4	NR DL-AoD Location Information Elements

#### –	NR-DL-AoD-SignalMeasurementInformation

The IE NR-DL-AoD-SignalMeasurementInformation is used by the target device to provide NR DL-AoD measurements to the location server.

-- ASN1START

NR-DL-AoD-SignalMeasurementInformation-r16 ::= SEQUENCE {

nr-DL-AoD-MeasList-r16			NR-DL-AoD-MeasList-r16,

...

}

NR-DL-AoD-MeasList-r16 ::= SEQUENCE (SIZE(1..nrMaxTRPs-r16)) OF NR-DL-AoD-MeasElement-r16

NR-DL-AoD-MeasElement-r16 ::= SEQUENCE {

dl-PRS-ID-r16					INTEGER (0..255),

nr-PhysCellID-r16				NR-PhysCellID-r16						OPTIONAL,

nr-CellGlobalID-r16				NCGI-r15								OPTIONAL,

nr-ARFCN-r16					ARFCN-ValueNR-r15						OPTIONAL,

nr-DL-PRS-ResourceID-r16		NR-DL-PRS-ResourceID-r16	 			OPTIONAL,

nr-DL-PRS-ResourceSetID-r16		NR-DL-PRS-ResourceSetID-r16 			OPTIONAL,

nr-TimeStamp-r16				NR-TimeStamp-r16,

nr-DL-PRS-RSRP-Result-r16		INTEGER (0..126),

nr-DL-PRS-RxBeamIndex-r16		INTEGER (1..8)							OPTIONAL,

nr-DL-AoD-AdditionalMeasurements-r16

NR-DL-AoD-AdditionalMeasurements-r16	OPTIONAL,

...,

[[

nr-DL-PRS-FirstPathRSRP-Result-r17

INTEGER (0..126)						OPTIONAL,

nr-los-nlos-Indicator-r17		CHOICE {

perTRP-r17					LOS-NLOS-Indicator-r17,

perResource-r17				LOS-NLOS-Indicator-r17

}																		OPTIONAL,

nr-DL-AoD-AdditionalMeasurementsExt-r17

NR-DL-AoD-AdditionalMeasurementsExt-r17	OPTIONAL

]],

[[

nr-DL-PRS-RSRP-Error-r18				CHOICE {

nr-DL-PRS-RSRP-Result-r16					INTEGER (0..126),

nr-DL-PRS-RSRP-ResultNorm-r18				NR-DL-PRS-RSRP-ErrorNorm-r18

}																		OPTIONAL,

nr-DL-PRS-FirstPathRSRP-Error-r18				CHOICE {

nr-DL-PRS-FirstPathRSRP-Result-r17			INTEGER (0..126),

nr-DL-PRS-FirstPathRSRP-ResultNorm-r18		NR-DL-PRS-RSRP-ErrorNorm-r18

}																		OPTIONAL

]]

}

NR-DL-AoD-AdditionalMeasurements-r16 ::= SEQUENCE (SIZE (1..7)) OF

NR-DL-AoD-AdditionalMeasurementElement-r16

NR-DL-AoD-AdditionalMeasurementsExt-r17 ::= SEQUENCE (SIZE (1..maxAddMeasAoD-r17)) OF

NR-DL-AoD-AdditionalMeasurementElement-r17

NR-DL-AoD-AdditionalMeasurementElement-r16 ::= SEQUENCE {

nr-DL-PRS-ResourceID-r16		NR-DL-PRS-ResourceID-r16	 			OPTIONAL,

nr-DL-PRS-ResourceSetID-r16		NR-DL-PRS-ResourceSetID-r16 			OPTIONAL,

nr-TimeStamp-r16				NR-TimeStamp-r16,

nr-DL-PRS-RSRP-ResultDiff-r16	INTEGER (0..30),

nr-DL-PRS-RxBeamIndex-r16		INTEGER (1..8)							OPTIONAL,

...

}

NR-DL-AoD-AdditionalMeasurementElement-r17 ::= SEQUENCE {

nr-DL-PRS-ResourceID-r17				NR-DL-PRS-ResourceID-r16		OPTIONAL,

nr-DL-PRS-ResourceSetID-r17				NR-DL-PRS-ResourceSetID-r16		OPTIONAL,

nr-TimeStamp-r17						NR-TimeStamp-r16,

nr-DL-PRS-RSRP-ResultDiff-r17			INTEGER (0..30)					OPTIONAL, -- Cond rsrp

nr-DL-PRS-RxBeamIndex-r17				INTEGER (1..8)					OPTIONAL,

nr-DL-PRS-FirstPathRSRP-ResultDiff-r17	INTEGER (0..61)					OPTIONAL, -- Cond rsrpp

nr-los-nlos-IndicatorPerResource-r17	LOS-NLOS-Indicator-r17			OPTIONAL,

...

}

NR-DL-PRS-RSRP-ErrorNorm-r18 ::= SEQUENCE {

mean-DL-PRS-RSRP-Error-r18		INTEGER (0..255),

std-DL-PRS-RSRP-Error-r18		INTEGER (0..255)

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| rsrp | The field is mandatory present if the field nr-DL-PRS-FirstPathRSRP-ResultDiff is absent; otherwise it is optionally present, need ON. |
| rsrpp | The field is mandatory present if the field nr-DL-PRS-RSRP-ResultDiff is absent; otherwise it is optionally present, need ON. |

NR-DL-AoD-SignalMeasurementInformation field descriptions

dl-PRS-ID This field is used along with a DL-PRS Resource Set ID and a DL-PRS Resources ID to uniquely identify a DL-PRS Resource. This ID can be associated with multiple DL-PRS Resource Sets associated with a single TRP. Each TRP should only be associated with one such ID.

nr-PhysCellID This field specifies the physical cell identity of the associated TRP, as defined in TS 38.331 [35].

nr-CellGlobalID This field specifies the NCGI, the globally unique identity of a cell in NR, of the associated TRP, as defined in TS 38.331 [35].

nr-ARFCN This field specifies the NR-ARFCN of the TRP's CD-SSB (as defined in TS 38.300 [47]) corresponding to nr-PhysCellID.

nr-TimeStamp This field specifies the time instance at which the measurement is performed.

nr-DL-PRS-RSRP-Result This field specifies the NR DL-PRS reference signal received power (DL PRS-RSRP) measurement, as defined in TS 38.215 [36]. The mapping of the measured quantity is defined as in TS 38.133 [46].

nr-DL-PRS-RxBeamIndex This field provides an index of the target device receive beam used for DL-PRS measurements. If the value of the receive beam index for two or more DL PRS measurements is the same, it indicates that the target device receive beam for the two or more DL PRS measurements were made with the same RX beam. The field is mandatory present if at least two DL-PRS RSRP measurements from the same DL-PRS Resource Set have been made with the same RX beam by the target device; otherwise it is not present.

nr-DL-PRS-FirstPathRSRP-Result This field specifies the NR DL PRS reference signal received path power (DL PRS-RSRPP) of the first detected path in time, as defined in TS 38.215 [36]. The mapping of the measured quantity is defined as in TS 38.133 [46].

nr-los-nlos-Indicator This field specifies the target device's best estimate of the LOS or NLOS of the RSRP or RSRPP of first path measurement for the TRP or resource. NOTE: If the requested type or granularity in nr-los-nlos-IndicatorRequest is not possible, the target device may provide a different type and granularity for the estimated LOS-NLOS-Indicator.

nr-DL-AoD-AdditionalMeasurementsExt This field specifies a list of additional PRS RSRP and/or RSRPP measurements of different DL-PRS resources for the same TRP. If this field is present, the field nr-DL-AoD-AdditionalMeasurements should not be present.

nr-DL-PRS-RSRP-ResultDiff This field provides the additional DL-PRS RSRP measurement result relative to nr-DL-PRS-RSRP-Result. The DL-PRS RSRP value of this measurement is obtained by adding the value of this field to the value of the nr-DL-PRS-RSRP-Result field. The mapping of the field is defined in TS 38.133 [46].

nr-DL-PRS-FirstPathRSRP-ResultDiff This field specifies the additional NR DL PRS reference signal received path power (DL PRS-RSRPP) of the first detected path in time relative to nr-DL-PRS-FirstPathRSRP-Result. The DL-PRS RSRPP of first path value of this measurement is obtained by adding the value of this field to the value of the nr-DL-PRS-FirstPathRSRP-Result field. The mapping of the field is defined in TS 38.133 [46].

nr-los-nlos-IndicatorPerResource This field specifies the target device's best estimate of the LOS or NLOS of the RSRP or RSRPP of first path measurement for the resource. This field may only be present if the field nr-LOS-NLOS-Indicator choice indicates perResource.

nr-DL-PRS-RSRP -Error This field provides the RSRP measurement error used for integrity calculation as defined in TS 38.355 [40].

nr-DL-PRS-FirstPathRSRP -Error This field provides the measurement error of the additional DL PRS-RSRPP of the first detected path in time relative to nr-DL-PRS-FirstPathRSRP-Result, which is used for integrity calculation as defined in TS 38.355 [40].

NR-DL-PRS-RSRP-ErrorNorm This field specifies the uncertainty/error of the RSRP/RSRPP measurement and comprises the following sub-fields: - mean-DL-PRS-RSRP-Error is the mean value of RSRP/RSRPP measurement error. - std-DL-PRS-RSRP-Error is the standard deviation of RSRP/RSRPP measurement error.

============================NEXT CHANGE==============================

#### 6.5.11.5	NR DL-AoD Location Information Request

#### –	NR-DL-AoD-RequestLocationInformation

The IE NR-DL-AoD-RequestLocationInformation is used by the location server to request NR DL-AoD location measurements from a target device.

-- ASN1START

NR-DL-AoD-RequestLocationInformation-r16 ::= SEQUENCE {

nr-AssistanceAvailability-r16				BOOLEAN,

nr-DL-AoD-ReportConfig-r16					NR-DL-AoD-ReportConfig-r16,

...,

[[

multiMeasInSameReport-r17					ENUMERATED { requested }	OPTIONAL  -- Need ON

]]

}

NR-DL-AoD-ReportConfig-r16 ::= SEQUENCE {

maxDL-PRS-RSRP-MeasurementsPerTRP-r16		INTEGER (1..8)				OPTIONAL, -- Need ON

...,

[[

maxDL-PRS-RSRP-MeasurementsPerTRP-r17		INTEGER (9..24)				OPTIONAL, -- Need ON

maxDL-PRS-RSRPP-MeasurementsPerTRP-r17		INTEGER (1..24)				OPTIONAL, -- Need ON

nr-los-nlos-IndicatorRequest-r17			SEQUENCE {

type-r17	LOS-NLOS-IndicatorType1-r17,

granularity-r17

LOS-NLOS-IndicatorGranularity1-r17,

...

}							OPTIONAL, -- Need ON

reducedDL-PRS-ProcessingSamples-r17			ENUMERATED { requested, ... }

OPTIONAL, -- Need ON

lowerRxBeamSweepingFactor-FR2-r17			ENUMERATED { requested }	OPTIONAL  -- Need ON

]],

[[

nr-DL-PRS-RSRP-MeasurementErrorRequest-r18	  			ENUMERATED { true }		OPTIONAL,-- Need ON

nr-DL-PRS-FirstPathRSRP-MeasurementErrorRequest-r18	  ENUMERATED { true }		OPTIONAL-- Need ON

]]

}

-- ASN1STOP

NR-DL-AoD-RequestLocationInformation field descriptions

nr-AssistanceAvailability This field indicates whether the target device may request additional PRS assistance data from the server. TRUE means allowed and FALSE means not allowed.

multiMeasInSameReport This field, if present, indicates that the target device is requested to provide multiple measurement instances in a single measurement report; i.e., include the nr-DL-AoD-SignalMeasurementInstances (in the case of UE-assisted mode is requested) or nr-DL-AoD-LocationInformationInstances (in the case of UE-based mode is requested) in IE NR-DL-TDOA-ProvideLocationInformation.

maxDL-PRS-RSRP-MeasurementsPerTRP This field specifies the maximum number of DL-PRS RSRP measurements on different DL-PRS Resources from the same TRP. If this field with -r17 suffix is present, the field with -r16 suffix should not be present.

maxDL-PRS-RSRPP-MeasurementsPerTRP This field specifies the maximum number of DL-PRS RSRPP measurements on different DL-PRS Resources from the same TRP.

nr-los-nlos-IndicatorRequest This field, if present, indicates that the target device is requested to provide the indicated type and granularity of the estimated LOS-NLOS-Indicator in the NR-DL-AoD-SignalMeasurementInformation.

reducedDL-PRS-ProcessingSamples This field, if present and set to 'requested', indicates that the target device is requested to perform the requested measurements with reduced number of samples (M=1 or M=2) as specified in TS 38.133 [46].

lowerRxBeamSweepingFactor-FR2 This field, if present, indicates that the target device is requested to use a lower Rx beam sweeping factor than 8 for FR2 according to UE's capability.

nr-DL-PRS-RSRP-MeasurementErrorRequest This field indicates whether the target device is requested to report RSRP measurement error used for integrity calculation as defined in TS 38.305 [40].

nr-DL-PRS-FirstPathRSRP-MeasurementErrorRequest This field indicates whether the target device is requested to report RSRP measurement error of the first path used for integrity calculation as defined in TS 38.305 [40].

============================NEXT CHANGE==============================

- 6.5.11.6	NR DL-AoD Capability Information

- –	NR-DL-AoD-ProvideCapabilities

The IE NR-DL-AoD-ProvideCapabilities is used by the target device to indicate its capability to support NR DL-AoD and to provide its NR DL-AoD positioning capabilities to the location server.

-- ASN1START

NR-DL-AoD-ProvideCapabilities-r16 ::= SEQUENCE {

nr-DL-AoD-Mode-r16						PositioningModes,

nr-DL-AoD-PRS-Capability-r16			NR-DL-PRS-ResourcesCapability-r16,

nr-DL-AoD-MeasurementCapability-r16		NR-DL-AoD-MeasurementCapability-r16,

nr-DL-PRS-QCL-ProcessingCapability-r16	NR-DL-PRS-QCL-ProcessingCapability-r16,

nr-DL-PRS-ProcessingCapability-r16		NR-DL-PRS-ProcessingCapability-r16,

periodicalReporting-r16					PositioningModes							OPTIONAL,

...,

[[

ten-ms-unit-ResponseTime-r17			PositioningModes							OPTIONAL,

nr-PosCalcAssistanceSupport-r17			BIT STRING {	trpLocSup 		(0),

beamInfoSup		(1),

rtdInfoSup		(2),

beamAntInfoSup	(3)

}	(SIZE (1..8))				OPTIONAL,

nr-los-nlos-AssistanceDataSupport-r17	SEQUENCE {

type-r17		LOS-NLOS-IndicatorType2-r17,

granularity-r17	LOS-NLOS-IndicatorGranularity2-r17,

...

}											OPTIONAL,

nr-DL-PRS-ExpectedAoD-or-AoA-Sup-r17	BIT STRING {	eAoD 		(0),

eAoA		(1)

}	(SIZE (1..8))				OPTIONAL,

nr-DL-PRS-BeamInfoSup-r17				ENUMERATED { sameSet, differentSet, sameOrDifferentSet }

OPTIONAL,

dl-PRS-ResourcePrioritySubset-Sup-r17	ENUMERATED { supported }					OPTIONAL,

nr-DL-AoD-On-Demand-DL-PRS-Support-r17	NR-On-Demand-DL-PRS-Support-r17				OPTIONAL,

nr-los-nlos-IndicatorSupport-r17		SEQUENCE {

type-r17		LOS-NLOS-IndicatorType2-r17,

granularity-r17	LOS-NLOS-IndicatorGranularity2-r17,

...

}											OPTIONAL,

scheduledLocationRequestSupported-r17	ScheduledLocationTimeSupportPerMode-r17

OPTIONAL,

nr-dl-prs-AssistanceDataValidity-r17	SEQUENCE {

area-validity-r17	INTEGER (1..maxNrOfAreas-r17)			OPTIONAL,

...

}															OPTIONAL,

multiMeasInSameMeasReport-r17			ENUMERATED { supported }					OPTIONAL,

mg-ActivationRequest-r17				ENUMERATED { supported }					OPTIONAL

]],

[[

nr-IntegrityCalSupport-r18                 ENUMERATED { supported }	OPTIONAL 	-- Cond UEB

]]

}

-- ASN1STOP

NR-DL-AoD-ProvideCapabilities field descriptions

nr-DL-AoD-Mode This field specifies the NR DL-AoD mode(s) supported by the target device.

periodicalReporting This field, if present, specifies the positioning modes for which the target device supports periodicalReporting. This is represented by a bit string, with a one-value at the bit position means periodicalReporting for the positioning mode is supported; a zero-value means not supported. If this field is absent, the target device does not support periodicalReporting in CommonIEsRequestLocationInformation.

ten-ms-unit-ResponseTime This field, if present, specifies the positioning modes for which the target device supports the enumerated value 'ten-milli-seconds' in the IE ResponseTime in IE CommonIEsRequestLocationInformation. This is represented by a bit string, with a one-value at the bit position means 'ten-milli-seconds' response time unit for the positioning mode is supported; a zero-value means not supported. If this field is absent, the target device does not support 'ten-milli-seconds' response time unit in CommonIEsRequestLocationInformation.

nr-PosCalcAssistanceSupport This field indicates the Position Calculation Assistance Data supported by the target device for UE-based DL-AoD. This is represented by a bit string, with a one-value at the bit position means the particular assistance data is supported; a zero-value means not supported. - bit 0 indicates whether the field nr-TRP-LocationInfo in IE NR-PositionCalculationAssistance is supported or not; - bit 1 indicates whether the field nr-DL-PRS-BeamInfo in IE NR-PositionCalculationAssistance is supported or not; - bit 2 indicates whether the field nr-RTD-Info in IE NR-PositionCalculationAssistance is supported or not. The UE can indicate this bit only if the UE supports prs-ProcessingCapabilityBandList and any of maxNrOfDL-PRS-ResourceSetPerTrpPerFrequencyLayer, maxNrOfTRP-AcrossFreqs, maxNrOfPosLayer, maxNrOfDL-PRS-ResourcesPerResourceSet and maxNrOfDL-PRS-ResourcesPerPositioningFrequencylayer. Otherwise, the UE does not include this field; - bit 3 indicates whether the field nr-TRP-BeamAntennaInfo in IE NR-PositionCalculationAssistance is supported or not.

nr-los-nlos-AssistanceDataSupport This field, if present, indicates that the target device supports the NR-DL-PRS-ExpectedLOS-NLOS-Assistance in IE NR-PositionCalculationAssistance: - type indicates whether the target device supports 'hard' value or 'hard' and 'soft' value in LOS-NLOS-Indicator in IE NR-DL-PRS-ExpectedLOS-NLOS-Assistance. - granularity indicates whether the target device supports nr-los-nlos-indicator in IE NR-DL-PRS-ExpectedLOS-NLOS-Assistance 'per-trp', 'per-resource', or both. The UE can include this field only if the UE supports one of maxDL-PRS-RSRP-MeasurementFR1, maxDL-PRS-RSRP-MeasurementFR2,dl-RSTD-MeasurementPerPairOfTRP-FR1, dl-RSTD-MeasurementPerPairOfTRP-FR2, maxNrOfRx-TX-MeasFR1, maxNrOfRx-TX-MeasFR2, supportOfRSRP-MeasFR1 and supportOfRSRP-MeasFR2 . Otherwise, the UE does not include this field. NOTE: A single value is reported when both Multi-RTT and DL-TDOA are supported.

nr-DL-PRS-ExpectedAoD-or-AoA-Sup This field, if present, indicates that the target device supports the NR-DL-PRS-ExpectedAoD-or-AoA in NR-DL-PRS-AssistanceData.

nr-DL-PRS-BeamInfoSup This field, if present, indicates that the target device supports the NR-DL-PRS-BeamInfo in IE NR-DL-AoD-ProvideAssistanceData.

dl-PRS-ResourcePrioritySubset-Sup This field, if present, indicates that the target device supports the DL-PRS-ResourcePrioritySubset in IE NR-DL-PRS-Info. Enumerated value indicates the supported resource set relationship for the target DL-PRS Resource and the associated subset.

nr-DL-AoD-On-Demand-DL-PRS-Support This field, if present, indicates that the target device supports on-demand DL-PRS requests.

nr-los-nlos-IndicatorSupport This field, if present, indicates that the target device supports nr-los-nlos-Indicator reporting in IE NR-DL-AoD-SignalMeasurementInformation. - type indicates whether the target device supports 'hard' value or 'hard' and 'soft' value in IE LOS-NLOS-Indicator. - granularity indicates whether the target device supports LOS-NLOS-Indicator reporting per TRP, per DL-PRS Resource, or both.

scheduledLocationRequestSupported This field, if present, specifies the positioning modes for which the target device supports scheduled location requests – i.e., supports the IE ScheduledLocationTime in IE CommonIEsRequestLocationInformation – and the time base(s) supported for the scheduled location time for each positioning mode. If this field is absent, the target device does not support scheduled location requests.

nr-dl-prs-AssistanceDataValidity This field, if present, indicates that the target device supports validity conditions for pre-configured assistance data and comprises the following subfields: - area-validity indicates that the target device supports pre-configured assistance data with area validity. The integer number indicates the maximum number of areas the target device supports.

multiMeasInSameMeasReport This field, if present, indicates that the target device supports multiple measurement instances in a single measurement report.

mg-ActivationRequest This field, if present, indicates that the target device supports low latency measurement gap activation request for DL-PRS measurements. The UE can include this field only if the UE supports mg-ActivationRequestPRS-Meas and mg-ActivationCommPRS-Meas defined in TS 38.331 [35].

nr-IntegrityCalSupport This field, if present, indicates that the target device supports integrity calculation as defined in TS 38.305 [40].

============================NEXT CHANGE==============================

#### 6.5.12.4	NR Multi-RTT Location Information Elements

#### –	NR-Multi-RTT-SignalMeasurementInformation

The IE NR-Multi-RTT-SignalMeasurementInformation is used by the target device to provide NR Multi-RTT measurements to the location server.

-- ASN1START

NR-Multi-RTT-SignalMeasurementInformation-r16 ::= SEQUENCE {

nr-Multi-RTT-MeasList-r16		NR-Multi-RTT-MeasList-r16,

nr-NTA-Offset-r16				ENUMERATED { nTA1, nTA2, nTA3, nTA4, ... }		OPTIONAL,

...,

[[

nr-SRS-TxTEG-Set-r17			SEQUENCE (SIZE(1..maxTxTEG-Sets-r17)) OF

NR-SRS-TxTEG-Element-r17					OPTIONAL

-- Cond Case2-3

]],

[[

nr-UE-RxTEG-TimingErrorMargin-r17	TEG-TimingErrorMargin-r17		OPTIONAL,-- Cond TEGCase3

nr-UE-TxTEG-TimingErrorMargin-r17	TEG-TimingErrorMargin-r17		OPTIONAL,-- Cond TEGCase2-3

nr-UE-RxTxTEG-TimingErrorMargin-r17	RxTxTEG-TimingErrorMargin-r17	OPTIONAL -- Cond TEGCase1-2

]]

}

NR-Multi-RTT-MeasList-r16 ::= SEQUENCE (SIZE(1..nrMaxTRPs-r16)) OF NR-Multi-RTT-MeasElement-r16

NR-Multi-RTT-MeasElement-r16 ::= SEQUENCE {

dl-PRS-ID-r16					INTEGER (0..255),

nr-PhysCellID-r16				NR-PhysCellID-r16								OPTIONAL,

nr-CellGlobalID-r16				NCGI-r15										OPTIONAL,

nr-ARFCN-r16					ARFCN-ValueNR-r15								OPTIONAL,

nr-DL-PRS-ResourceID-r16		NR-DL-PRS-ResourceID-r16						OPTIONAL,

nr-DL-PRS-ResourceSetID-r16		NR-DL-PRS-ResourceSetID-r16 					OPTIONAL,

nr-UE-RxTxTimeDiff-r16			CHOICE {

k0-r16						INTEGER (0..1970049),

k1-r16						INTEGER (0..985025),

k2-r16						INTEGER (0..492513),

k3-r16						INTEGER (0..246257),

k4-r16						INTEGER (0..123129),

k5-r16						INTEGER (0..61565),

...

},

nr-AdditionalPathList-r16		NR-AdditionalPathList-r16						OPTIONAL,

nr-TimeStamp-r16				NR-TimeStamp-r16,

nr-TimingQuality-r16			NR-TimingQuality-r16,

nr-DL-PRS-RSRP-Result-r16		INTEGER (0..126)								OPTIONAL,

nr-Multi-RTT-AdditionalMeasurements-r16

NR-Multi-RTT-AdditionalMeasurements-r16			OPTIONAL,

...,

[[

nr-UE-RxTx-TEG-Info-r17				NR-UE-RxTx-TEG-Info-r17						OPTIONAL,

nr-DL-PRS-FirstPathRSRP-Result-r17	INTEGER (0..126)							OPTIONAL,

nr-los-nlos-Indicator-r17			CHOICE {

perTRP-r17					LOS-NLOS-Indicator-r17,

perResource-r17				LOS-NLOS-Indicator-r17

}																				OPTIONAL,

nr-AdditionalPathListExt-r17		NR-AdditionalPathListExt-r17				OPTIONAL,

nr-Multi-RTT-AdditionalMeasurementsExt-r17

NR-Multi-RTT-AdditionalMeasurementsExt-r17	OPTIONAL

]],

[[

nr-UE-RxTxTimeDiff-Error-R18	NR-TimingQualityNorm-r18		OPTIONAL

]]

}

NR-Multi-RTT-AdditionalMeasurements-r16 ::= SEQUENCE (SIZE (1..3)) OF

NR-Multi-RTT-AdditionalMeasurementElement-r16

NR-Multi-RTT-AdditionalMeasurementsExt-r17 ::= SEQUENCE (SIZE (1..maxAddMeasRTT-r17)) OF

NR-Multi-RTT-AdditionalMeasurementElement-r16

NR-Multi-RTT-AdditionalMeasurementElement-r16 ::= SEQUENCE {

nr-DL-PRS-ResourceID-r16			NR-DL-PRS-ResourceID-r16					OPTIONAL,

nr-DL-PRS-ResourceSetID-r16			NR-DL-PRS-ResourceSetID-r16 				OPTIONAL,

nr-DL-PRS-RSRP-ResultDiff-r16		INTEGER (0..61)								OPTIONAL,

nr-UE-RxTxTimeDiffAdditional-r16	CHOICE {

k0-r16							INTEGER (0..8191),

k1-r16							INTEGER (0..4095),

k2-r16							INTEGER (0..2047),

k3-r16							INTEGER (0..1023),

k4-r16							INTEGER (0..511),

k5-r16							INTEGER (0..255),

...

},

nr-TimingQuality-r16				NR-TimingQuality-r16,

nr-AdditionalPathList-r16			NR-AdditionalPathList-r16					OPTIONAL,

nr-TimeStamp-r16					NR-TimeStamp-r16,

...,

[[

nr-UE-RxTx-TEG-Info-r17					NR-UE-RxTx-TEG-Info-r17			OPTIONAL,

nr-DL-PRS-FirstPathRSRP-ResultDiff-r17	INTEGER (0..61)					OPTIONAL,

nr-los-nlos-IndicatorPerResource-r17	LOS-NLOS-Indicator-r17			OPTIONAL,

nr-AdditionalPathListExt-r17			NR-AdditionalPathListExt-r17	OPTIONAL

]],

[[

nr-UE-RxTxTimeDiff-Error-R18		NR-TimingQualityNorm-r18	OPTIONAL

]]

}

NR-SRS-TxTEG-Element-r17 ::= SEQUENCE {

nr-TimeStamp-r17			NR-TimeStamp-r16							OPTIONAL,	-- Need OP

nr-UE-Tx-TEG-ID-r17			INTEGER (0..maxNumOfTxTEGs-1-r17),

carrierFreq-r17				SEQUENCE {

absoluteFrequencyPointA-r17		ARFCN-ValueNR-r15,

offsetToPointA-r17				INTEGER (0..2199)

}											OPTIONAL,

srs-PosResourceList-r17		SEQUENCE (SIZE (1..maxNumOfSRS-PosResources-r17)) OF

INTEGER (0..maxNumOfSRS-PosResources-1-r17),

...

}

NR-UE-RxTx-TEG-Info-r17 ::= CHOICE {

case1-r17				SEQUENCE {

nr-UE-RxTx-TEG-ID-r17	INTEGER (0..maxNumOfRxTxTEGs-1-r17)

},

case2-r17				SEQUENCE {

nr-UE-RxTx-TEG-ID-r17	INTEGER (0..maxNumOfRxTxTEGs-1-r17),

nr-UE-Tx-TEG-Index-r17	INTEGER (1..maxTxTEG-Sets-r17)

},

case3-r17				SEQUENCE {

nr-UE-Rx-TEG-ID-r17		INTEGER (0..maxNumOfRxTEGs-1-r17),

nr-UE-Tx-TEG-Index-r17	INTEGER (1..maxTxTEG-Sets-r17)

},

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| Case2-3 | The field is mandatory present if the IE NR-UE-RxTx-TEG-Info is provided for choice's case2 and case3. Otherwise it is not present. |
| TEGCase3 | The field is optionally present, need OP, if the IE NR-UE-RxTx-TEG-Info is provided for choice case3. Otherwise it is not present. |
| TEGCase2-3 | The field is optionally present, need OP, if the IE NR-UE-RxTx-TEG-Info is provided for choice's case2 and case3. Otherwise it is not present. |
| TEGCase1-2 | The field is optionally present, need OP, if the IE NR-UE-RxTx-TEG-Info is provided for choice's case1 and case2. Otherwise it is not present. |

NR-Multi-RTT-SignalMeasurementInformation field descriptions

nr-NTA-Offset This field provides the NTAoffset used by the target device as specified in TS 38.133 [46], Table 7.1.2-2. Enumerated values nTA1, nTA2, nTA3, and nTA4 correspond to NTAoffset of 25600 Tc, 0 Tc, 39936 Tc, and 13792 Tc, respectively.

nr-SRS-TxTEG-Set This field provides the SRS for Positioning Resources associated with a particular UE Tx TEG and comprises the following subfields: - nr-TimeStamp specifies the start time for which the NR-SRS-TxTEG-Element is valid. If this field is absent, the nr-TimeStamp of this instance of the NR-SRS-TxTEG-Element of the nr-SRS-TxTEG-Set is the same as the nr-TimeStamp of the previous instance of the NR-SRS-TxTEG-Element. If this field is also absent in the first NR-SRS-TxTEG-Element of the nr-SRS-TxTEG-Set, all NR-SRS-TxTEG-Element's provided are valid for the measurement period of the NR-Multi-RTT-SignalMeasurementInformation. - nr-UE-Tx-TEG-ID specifies the ID of this UE Tx TEG. - carrierFreq specifies the frequency of the SRS for positioning resources. - srs-PosResourceList specifies the SRS for Positioning Resources belonging to this UE Tx TEG. For each UE Tx TEG, there may be up to 8 changes (different nr-TimeStamp) of the TEG-SRS association information provided in nr-SRS-TxTEG-Set.

nr-UE-RxTEG-TimingErrorMargin This field specifies the UE Rx TEG timing error margin value for all the UE Rx TEGs within one NR-Multi-RTT-SignalMeasurementInformation. If the IE NR-UE-RxTx-TEG-Info is present with choice case3 and this field is absent, the receiver should consider the UE Rx TEG timing error margin value to be the maximum applicable value as defined in TS 38.133 [46].

nr-UE-TxTEG-TimingErrorMargin This field specifies the UE Tx TEG timing error margin value for all the UE Tx TEGs within one NR-Multi-RTT-SignalMeasurementInformation. If the IE NR-UE-RxTx-TEG-Info is present with choice case2 or case3 and this field is absent, the receiver should consider the UE Tx TEG timing error margin value to be the maximum value available in IE TEG-TimingErrorMargin.

nr-UE-RxTxTEG-TimingErrorMargin This field specifies the UE RxTx TEG timing error margin value for all the UE RxTx TEGs within one NR-Multi-RTT-SignalMeasurementInformation. If the IE NR-UE-RxTx-TEG-Info is present with choice case1 or case2 and this field is absent, the receiver should consider the UE RxTx TEG timing error margin value to be the maximum applicable value as defined in TS 38.133 [46].

dl-PRS-ID This field is used along with a DL-PRS Resource Set ID and a DL-PRS Resources ID to uniquely identify a DL-PRS Resource. This ID can be associated with multiple DL-PRS Resource Sets associated with a single TRP. Each TRP should only be associated with one such ID.

nr-PhysCellID This field specifies the physical cell identity of the associated TRP, as defined in TS 38.331 [35].

nr-CellGlobalID This field specifies the NCGI, the globally unique identity of a cell in NR, of the associated TRP, as defined in TS 38.331 [35].

nr-ARFCN This field specifies the NR-ARFCN of the TRP's CD-SSB (as defined in TS 38.300 [47]) corresponding to nr-PhysCellID.

nr-UE-RxTxTimeDiff This field specifies the UE Rx–Tx time difference measurement, as defined in TS 38.215 [36].

nr-AdditionalPathList This field specifies one or more additional detected path timing values for the TRP or resource, relative to the path timing used for determining the nr-UE-RxTxTimeDiff value. If this field was requested but is not included, it means the UE did not detect any additional path timing values. If this field is present, the field nr-AdditionalPathListExt shall be absent.

nr-TimeStamp This field specifies the time instance for which the measurement is performed.

nr-TimingQuality This field specifies the target device′s best estimate of the quality of the measurement.

nr-DL-PRS-RSRP-Result This field specifies the NR DL-PRS reference signal received power (DL PRS-RSRP) measurement, as defined in TS 38.215 [36]. The mapping of the quantity is defined as in TS 38.133 [46].

nr-UE-RxTx-TEG-Info This field provides the ID(s) of the UE TEG associated with the nr-UE-RxTxTimeDiff or nr-UE-RxTxTimeDiffAdditional measurement. One of the following combinations of TEG IDs can be provided: - case1 provides the UE RxTx TEG ID; - case2 provides the UE RxTx TEG ID together with the UE Tx TEG ID. The nr-UE-Tx-TEG-Index provides the index to the nr-SRS-TxTEG-Set field for the applicable UE Tx TEG ID, where value '1' indicates the first NR-SRS-TxTEG-Element in nr-SRS-TxTEG-Set, value '2' indicates the second NR-SRS-TxTEG-Element in nr-SRS-TxTEG-Set, and so on; - case3 provides the UE Rx TEG ID together with the UE Tx TEG ID. The nr-UE-Tx-TEG-Index provides the index to the nr-SRS-TxTEG-Set field for the applicable UE Tx TEG ID, where value '1' indicates the first NR-SRS-TxTEG-Element in nr-SRS-TxTEG-Set, value '2' indicates the second NR-SRS-TxTEG-Element in nr-SRS-TxTEG-Set, and so on.

nr-DL-PRS-FirstPathRSRP-Result This field specifies the NR DL PRS reference signal received path power (DL PRS-RSRPP) of the first detected path in time, as defined in TS 38.215 [36]. The mapping of the measured quantity is defined as in TS 38.133 [46].

nr-los-nlos-Indicator This field specifies the target device's best estimate of the LOS or NLOS of the UE Rx-Tx Time Difference, RSRP or RSRPP of first path measurement for the TRP or resource. NOTE: If the requested type or granularity in nr-los-nlos-IndicatorRequest is not possible, the target device may provide a different type and granularity for the estimated LOS-NLOS-Indicator.

nr-AdditionalPathListExt This field provides up to 8 additional detected path timing values for the TRP or resource, relative to the path timing used for determining the nr-UE-RxTxTimeDiff value. If this field was requested but is not included, it means the UE did not detect any additional path timing values. If this field is present, the field nr-AdditionalPathList shall be absent.

nr-Multi-RTT-AdditionalMeasurementsExt This field, in addition to the measurements provided in NR-Multi-RTT-MeasElement, provides UE Rx-Tx time difference measurements of up to 4 DL-PRS Resources of a TRP with different UE RxTx or UE Rx TEGs. For a certain DL-PRS Resource, there can be up to 8 measurement results with respect to different UE RxTx or UE Rx TEGs. If this field is present, the field nr-Multi-RTT-AdditionalMeasurements should not be present.

nr-DL-PRS-RSRP-ResultDiff This field provides the additional DL-PRS RSRP measurement result relative to nr-DL-PRS-RSRP-Result. The DL-PRS RSRP value of this measurement is obtained by adding the value of this field to the value of the nr-DL-PRS-RSRP-Result. The mapping of this field is defined as in TS 38.133 [46].

nr-UE-RxTxTimeDiffAdditional This field provides the additional UE Rx-Tx Difference measurement result relative to nr-UE-RxTxTimeDiff. The UE Rx-Tx Difference value of this measurement is obtained by adding the value of this field to the value of the nr-UE-RxTxTimeDiff field. The mapping of the field is defined in TS 38.133 [46].

nr-DL-PRS-FirstPathRSRP-ResultDiff This field specifies the additional NR DL-PRS reference signal received path power (DL PRS-RSRPP) of the first detected path in time relative to nr-DL-PRS-FirstPathRSRP-Result. The DL-PRS RSRPP of first path value of this measurement is obtained by adding the value of this field to the value of the nr-DL-PRS-FirstPathRSRP-Result field. The mapping of the field is defined in TS 38.133 [46].

nr-los-nlos-IndicatorPerResource This field specifies the target device's best estimate of the LOS or NLOS of the UE Rx-Tx Time Difference, RSRP or RSRPP of first path measurement for the resource. This field may only be present if the field nr-LOS-NLOS-Indicator choice indicates perResource.

nr-UE-RxTxTimeDiff -Error This field provides the UE Rx-Tx time difference error used for integrity calculation as in TS 38.355 [40].

============================NEXT CHANGE==============================

#### 6.5.12.5	NR Multi-RTT Location Information Request

#### –	NR-Multi-RTT-RequestLocationInformation

The IE NR-Multi-RTT-RequestLocationInformation is used by the location server to request NR Multi-RTT location measurements from a target device.

-- ASN1START

NR-Multi-RTT-RequestLocationInformation-r16 ::= SEQUENCE {

nr-UE-RxTxTimeDiffMeasurementInfoRequest-r16

ENUMERATED { true }					OPTIONAL, -- Need ON

nr-RequestedMeasurements-r16		BIT STRING { prsrsrpReq (0),

firstPathRsrpReq-r17 (1) } (SIZE(1..8)),

nr-AssistanceAvailability-r16		BOOLEAN,

nr-Multi-RTT-ReportConfig-r16		NR-Multi-RTT-ReportConfig-r16,

additionalPaths-r16					ENUMERATED { requested }			OPTIONAL, -- Need ON

...,

[[

nr-UE-RxTxTEG-Request-r17			ENUMERATED { case1, case2, case3, ... }

OPTIONAL, -- Need ON

measureSameDL-PRS-ResourceWithDifferentRxTxTEGs-r17

ENUMERATED { n0, n2, n3, n4, n6, n8, ... }

OPTIONAL, -- Need ON

measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17

ENUMERATED { n0, n2, n3, n4, n6, n8, ... }

OPTIONAL, -- Need ON

reducedDL-PRS-ProcessingSamples-r17

ENUMERATED { requested, ... }		OPTIONAL, -- Need ON

nr-los-nlos-IndicatorRequest-r17	SEQUENCE {

type-r17		LOS-NLOS-IndicatorType1-r17,

granularity-r17	LOS-NLOS-IndicatorGranularity1-r17,

...

}									OPTIONAL, -- Need ON

additionalPathsExt-r17				ENUMERATED { requested }			OPTIONAL, -- Need ON

additionalPathsDL-PRS-RSRP-Request-r17

ENUMERATED { requested }			OPTIONAL, -- Need ON

multiMeasInSameReport-r17			ENUMERATED { requested }			OPTIONAL, -- Need ON

lowerRxBeamSweepingFactor-FR2-r17	ENUMERATED { requested }			OPTIONAL  -- Need ON

]],

[[

nr-UE-RxTxTimeDiffMeasurementErrorRequest-r18	ENUMERATED { true }		OPTIONAL,-- Need ON

]]

}

NR-Multi-RTT-ReportConfig-r16 ::= SEQUENCE {

maxDL-PRS-RxTxTimeDiffMeasPerTRP-r16 	INTEGER (1..4)					OPTIONAL, -- Need ON

timingReportingGranularityFactor-r16 	INTEGER (0..5)					OPTIONAL  -- Need ON

}

-- ASN1STOP

NR-Multi-RTT-RequestLocationInformation field descriptions

nr-UE-RxTxTimeDiffMeasurementInfoRequest This field, if present, indicates that the target device is requested to report the DL-PRS Resource ID(s) or DL-PRS Resource Set ID(s) associated with the DL-PRS Resources(s) or the DL-PRS Resource Set(s) which are used in determining the UE Rx-Tx time difference measurements.

nr-AssistanceAvailability This field indicates whether the target device may request additional PRS assistance data from the server. TRUE means allowed and FALSE means not allowed.

maxDL-PRS-RxTxTimeDiffMeasPerTRP This field specifies the maximum number of UE-Rx-Tx time difference measurements for different DL-PRS Resources or DL-PRS Resource Sets per TRP.

timingReportingGranularityFactor This field specifies the recommended reporting granularity for the UE Rx-Tx time difference measurements. Value (0..5) corresponds to (k0..k5) used for nr-UE-RxTxTimeDiff and nr-UE-RxTxTimeDiffAdditional in NR-Multi-RTT-MeasElement. The UE may select a different granularity value for nr-UE-RxTxTimeDiff and nr-UE-RxTxTimeDiffAdditional.

additionalPaths This field, if present, indicates that the target device is requested to provide the nr-AdditionalPathList in IE NR-Multi-RTT-SignalMeasurementInformation. If this field is present, the field additionalPathsExt shall be absent.

nr-UE-RxTxTEG-Request This field, if present, indicates that the target device is requested to provide the NR-UE-RxTx-TEG-Info in IE NR-Multi-RTT-SignalMeasurementInformation. Enumerated value 'case1' indicates that the target device is requested to provide the case1 choice in NR-UE-RxTx-TEG-Info, enumerated value 'case2' indicates that the target device is requested to provide the case2 choice in NR-UE-RxTx-TEG-Info, and so on.

measureSameDL-PRS-ResourceWithDifferentRxTxTEGs This field, if present, indicates that the target device is requested to measure the same DL-PRS Resource of a TRP with N different UE RxTx TEGs and with the same UE Tx TEG. Enumerated value 'n0' indicates that the number N of different UE RxTx TEGs to measure the same DL PRS Resource can be determined by the target device, value 'n2' indicates that the target device is requested to measure the same DL-PRS Resource of a TRP with 2 different UE RxTx TEGs, value 'n3' indicates that the target device is requested to measure the same DL-PRS Resource of a TRP with 3 different UE RxTx TEGs, and so on. If this field is present, the field nr-UE-RxTxTEG-Request should also be present. If this field is present, the field measureSameDL-PRS-ResourceWithDifferentRxTEGs should not be present.

measureSameDL-PRS-ResourceWithDifferentRxTEGs This field, if present, indicates that the target device is requested to measure the same DL-PRS Resource of a TRP with N different UE Rx TEGs. Enumerated value 'n0' indicates that the number N of different UE Rx TEGs to measure the same DL PRS Resource can be determined by the target device, value 'n2' indicates that the target device is requested to measure the same DL-PRS Resource of a TRP with 2 different UE Rx TEGs, value 'n3' indicates that the target device is requested to measure the same DL-PRS Resource of a TRP with 3 different UE Rx TEGs, and so on. If this field is present, the field nr-UE-RxTxTEG-Request should also be present. If this field is present, the field measureSameDL-PRS-ResourceWithDifferentRxTxTEGs should not be present.

reducedDL-PRS-ProcessingSamples This field, if present and set to 'requested', indicates that the target device is requested to perform the requested measurements with reduced number of samples (M=1 or M=2) as specified in TS 38.133 [46].

nr-los-nlos-IndicatorRequest This field, if present, indicates that the target device is requested to provide the indicated type and granularity of the estimated LOS-NLOS-Indicator in the NR-Multi-RTT-SignalMeasurementInformation.

additionalPathsExt This field, if present, indicates that the target device is requested to provide the nr-AdditionalPathListExt in IE NR-Multi-RTT-SignalMeasurementInformation. If this field is present, the field additionalPaths shall be absent.

additionalPathsDL-PRS-RSRP-Request This field, if present, indicates that the target device is requested to provide the nr-DL-PRS-RSRP for the additional paths in IE NR-AdditionalPathList.

multiMeasInSameReport This field, if present, indicates that the target device is requested to provide multiple measurement instances in a single measurement report; i.e., include the nr-Multi-RTT-SignalMeasurementInstances in IE NR-Multi-RTT-ProvideLocationInformation.

lowerRxBeamSweepingFactor-FR2 This field, if present, indicates that the target device is requested to use a lower Rx beam sweeping factor than 8 for FR2 according to UE's capability.

nr-UE-RxTxTimeDiffMeasurementErrorRequest This field indicates whether the target device is requested to report UE Rx-Tx time difference error used for integrity calculation as in TS 38.305 [40].

================================END OF CHANGES========================

# Annex B: Stage2 text proposal for summary of the error sources

============================START OF CHANGE===========================

Table 2.2-1 Summary of RAN1 progress on error sources for RAT-dependent integrity

| Positioning mode | Positioning method | Error sources | Category of error sources | Candidate(s) for distribution |
| --- | --- | --- | --- | --- |
| LMF-based positioning | DL-TDOA | - RSTD measurement | Timing related measurement | Gaussian distribution |
|  |  | - TRP location (e.g., Geographical Coordinates in TS 38.455) | Assistance data | - Uniform distribution - Gaussian distribution |
|  |  | - Inter-TRP synchronization (can be caused in part by errors in SFN initialization time) | Assistance data | - Uniform distribution (NOTE 1) - Gaussian distribution |
|  | UL-TDOA | - RTOA measurement | Timing related measurement | Gaussian distribution |
|  |  | - Inter-TRP synchronization (can be caused in part by errors in SFN initialization time) | Assistance data | - Uniform distribution (NOTE 1) - Gaussian distribution |
|  |  | - TRP location (e.g., Geographical Coordinates in TS 38.455) | Assistance data | - Uniform distribution - Gaussian distribution |
|  | Multi-RTT | - UE Rx-Tx time difference measurement - gNB Rx-Tx time difference measurement | Timing related measurement | Gaussian distribution |
|  |  | - TRP location (e.g., Geographical Coordinates in TS 38.455) | Assistance data | - Uniform distribution - Gaussian distribution |
|  | UL-AoA | - Angle of arrival measurement (e.g., AoA/ZoA) | Angle related measurement | Alt. 1: No conversion (e.g., the measurement error is expressed as error in AoA or ZoA in LCS/GCS) Alt. 2: conversion function (defined as function of AoA/ZoA in LCS). |
|  |  | - ARP location (e.g., ARPLocationInformation) error | Assistance data | - Uniform distribution - Gaussian distribution |
|  |  | - TRP location (e.g., Geographical Coordinates in TS 38.455) | Assistance data | - Uniform distribution - Gaussian distribution |
|  | DL-AoD | - TRP location (e.g., Geographical Coordinates in TS 38.455) error | Assistance data | - Uniform distribution - Gaussian distribution |
|  |  | - DL-PRS RSRPP of the first path or RSRP | Measurement | FFS |
| UE-based positioning | DL-TDOA | - TRP location (e.g., NR-TRP-LocationInfo) | Assistance data | - Uniform distribution (NOTE 2) - Gaussian distribution |
|  |  | - Inter-TRP synchronization (e.g., NR-RTD-Info) | Assistance data | - Uniform distribution (NOTE 1) - Gaussian distribution |
|  | DL-AoD | - TRP location (e.g., NR-TRP-LocationInfo) | Assistance data |  |
| NOTE 1: This may already be consistent with the uncertainty related to NR-RTD-Info in TS 37.355. NOTE 2: This may already be consistent with the uncertainty related to NR-TRP-LocationInfo in TS 37.355. NOTE 3: RAN1 could not reach consensus on whether beam information (NR-TRP-BeamAntennaInfo) and boresight direction of DL PRS (NR-DL-PRS-BeamInfo) are error sources or not as of now. |  |  |  |  |

============================END OF CHANGES============================