3GPP TSG-RAN WG2 #121	R2-2300127

Athens, Greece, February-March 2023

Agenda Item:	8.15.3

Source:	OPPO

Title:	Discussion on SL-FR2 impact

Document for:	Discussion, Decision

Introduction

This paper will discuss the RAN2 work on the following objective on SL-FR2.

- Study enhanced sidelink operation on FR2 licensed spectrum [RAN1, RAN2] - Focus only on updating the evaluation methodology for commercial deployment scenario in 4Q 2022. [RAN1] - Study is limited to the support of sidelink beam management (including initial beam-pairing, beam maintenance, and beam failure recovery, etc) by reusing existing sidelink CSI framework and reusing Uu beam management concepts wherever possible. [RAN1, RAN2] - Beam management in FR2 licensed spectrum considers sidelink unicast communication only.

Discussion

The following table has listed the main RAN2 work in Uu beam management

Table 1 Summary of main RAN2 work in Uu beam management

|  | Uu work |
| --- | --- |
| Beamforming-based initial access (initial Beam-pairing) | Initial beam-pairing is done during RACH procedure. |
| Beam maintenance | DL beam measurement/report (L1): L1 RSRP/SINR of configured CSI-RS/SSB, CSI-RS/SSB is configured/activated by RRC+MAC CE DL beam indication: TCI state indication by RRC+MAC CE+DCI |
|  | UL beam indication: signal indication by RRC+ MAC CE |
| Beam failure detection and recovery | Beam failure detected for SpCell: RACH-based BFR; Beam failure detected for SCell: MAC CE based BFR report; |
| Beam-based RRM | Cell-level + beam-level L3 measurement report |

Initial Beam-pairing

In Uu, the initial beam-pairing is achieved during RACH, for the DL Tx beam,

- Firstly, the UE selects one ‘good’ SSB, and the DL Tx beam is selected accordingly;

- Then UE transmits the corresponding PRACH associated with the selected good SSB;

- gNB can be aware of the selected beam based on the association between SSB and the received PRACH.

In Uu, the initial beam-pairing is achieved during RACH, relying on the association between PRACH and SSB;

While in sidelink, there is no RACH procedure, the unicast link is established according to the following PC5-S signaling exchange[1]:

Figure 1: Layer-2 link establishment procedure[1]

In Sidelink, the link establishment procedure relies on discovery and PC5-S signalling exchange, without RACH-like procedure.

Therefore, the RACH-based beam access cannot be copied in sidelink, the initial beam-pairing during PC5 unicast link establishment needs to be further considered.

And in Uu, the initial beam selection is based on SSB measurement, while in sidelink, which signal/quantity should be measured and used seems boils down to RAN1 design on beam-based reference signal and measurement solution, so RAN2 should wait for RAN1’s progress on this issue first.

For SL initial beam-pairing, RAN2 wait for RAN1 progress first.

Beam management

The beam management after initial beam-pairing in Uu includes the following aspects:

- UE measures the Tx beam of the gNB and reports L1 SINR/RSRP based on NW configuration;

- gNB determines the Tx beam based on UE’s report and indicates DL beam to the UE.

By following Uu principle, similar mechanism can be considered in sidelink, e.g.,

- Rx UE performs beam measurement and reports the measurement results based on Tx UE configuration;

- Tx UE determines the Tx beam based on Rx UE’s report and indicates the Tx beam to the Rx UE.

However, in Uu beam management, the mechanism design was mainly done in RAN1 and RAN2 work was on some MAC CE (e.g., TCI state activation/deactivation MAC CE) content design based on RAN1 agreements.

In Uu beam management, the beam measurement (L1) and indication were designed by RAN1, while RAN2 work was on MAC CE design (e.g., TCI state activation/deactivation, etc.) as triggered by RAN1.

Therefore, RAN2 can wait for RAN1 progress on the beam measurement/report (L1) and indication now.

For SL beam management (including L1 measurement report and beam indication), RAN2 waits for RAN1 progress.

Beam-based RRM measurement

In Uu link, the RRM measurement is mainly used for mobility management. UE performs beam and cell measurement reports based on NW configuration, and network may use the measurement result to determine the mobility-related configuration.

While in SL, the RRM measurement is mainly used for power control, and whether the per-beam based RRM measurement report is needed or not is related to whether the per-beam based power control is needed. Power control is designed by RAN1, so whether the per-beam based RRM measurement report is needed is pending on RAN1 design on the power control in FR2.

For SL beam-based RRM measurement report, RAN2 waits for RAN1 progress.

Beam failure detection and recovery

In Uu, beam failure is detected in MAC by counting the beam failure instance indication from lower layers to MAC entity, and the UE declares beam failure when the number of beam failure instance indications from the physical layer reaches a configured threshold before a configured timer expires.

In Uu beam failure detection, both a timer and a counter are introduced, where the counter is reset when the timer expires and incremented when beam failure instance indication is received from the physical layer.

In SL-FR2, beam failure also needs to be detected and we can reuse the Uu beam failure detection mechanism, i.e., introduce both a timer and a counter in sidelink for beam failure detection.

For SL beam failure detection, RAN2 discuss to reuse the timer + counter-based mechanism as Uu for SL-FR2.

When beam failure is detected in Uu, serval solutions have been discussed to help the UE to recover from beam failure. For different carriers/cells, the UE behaviors upon consistent LBT failure are different,

- If beam failure happens on SpCell, UE performs RACH, and may trigger RLF if random access is not successful for the beam failure recovery;

- If beam failure happens on SCell, UE reports beam failure via MAC CE.

In Uu, the UE performs RACH upon beam failure on SpCell and triggers RLF if the random access is not successful.

In Uu, the UE reports BFR MAC CE upon beam failure on SCell.

When it comes to SL-FR2, we should also discuss the beam failure recovery on PC5 link, e.g., how to report the beam failure to the peer UE and whether SL-RLF is triggered if sidelink beam failure recovery is not successful.

For SL beam failure recovery/handling, RAN2 discuss the solution, e.g., report the beam failure to the peer UE, trigger RLF, etc.

Conclusion

We have the following observations:

Observation 1	In Uu, the initial beam-pairing is achieved during RACH, relying on the association between PRACH and SSB;

Observation 2	In Sidelink, the link establishment procedure relies on discovery and PC5-S signalling exchange, without RACH-like procedure.

Observation 3	In Uu beam management, the beam measurement (L1) and indication were designed by RAN1, while RAN2 work was on MAC CE design (e.g., TCI state activation/deactivation, etc.) as triggered by RAN1.

Observation 4	In Uu beam failure detection, both a timer and a counter are introduced, where the counter is reset when the timer expires and incremented when beam failure instance indication is received from the physical layer.

Observation 5	In Uu, the UE performs RACH upon beam failure on SpCell and triggers RLF if the random access is not successful.

Observation 6	In Uu, the UE reports BFR MAC CE upon beam failure on SCell.

We have the following proposals:

Proposal 1	For SL initial beam-pairing, RAN2 wait for RAN1 progress first.

Proposal 2	For SL beam management (including L1 measurement report and beam indication), RAN2 waits for RAN1 progress.

Proposal 3	For SL beam-based RRM measurement report, RAN2 waits for RAN1 progress.

Proposal 4	For SL beam failure detection, RAN2 discuss to reuse the timer + counter-based mechanism as Uu for SL-FR2.

Proposal 5	For SL beam failure recovery/handling, RAN2 discuss the solution, e.g., report the beam failure to the peer UE, trigger RLF, etc.

Reference

[1] TS 23.304 Proximity based Services (ProSe) in the 5G System