3GPP TSG-RAN WG2 #121	R2-2300137

Athens, Greece, February 2023

Agenda Item:	6.5.2

Source:	OPPO

Title:	Discussion on left issues for CP

Document for:	Discussion, Decision

Introduction

This is to discuss the left issue for the control plane.

Discussion

In R2#120, the following issue was discussed.

R2-2213117	[Pre120][402][Relay] Summary of agenda item 6.7.2 on relay control plane (Huawei)	Huawei, HiSilicon	discussion	Rel-17	NR_SL_relay-Core

Proposal 8: RAN2 can discuss the following if time allows:

–	B: OOC Remote UE in RRC_IDLE/INACTIVE can use preconfigured resource if the forwarded SIB12 doesn’t include normal pool and exception pool, from the moment the UE initiates RRC connection establishment or RRC connection resume, until receiving an RRCReconfiguration including sl-ConfigDedicatedNR, or receiving an RRCRelease or an RRCReject.

On P8B, Apple wonder if this is really a critical issue; a gNB would normally configure the pools, and they see this as more of a misconfiguration.  Ericsson think we cannot force the gNB to always provide the pools; it is up to gNB implementation, and this CR addresses the case where the gNB does not provide it.

Xiaomi understand that there is a view that the network can always provide the pools, but they see this as an unreasonable restriction on the network, and they think preconfiguration should be allowed in this case; they do not consider that this breaks the legacy principle of operation, since it only allows the UE OOC to use preconfiguration.

Huawei agree with Apple that this is not a very reasonable network behaviour.  On Xiaomi’s comment that the proposal is only applicable OOC, they do not agree since the IC case has the same situation and the remote UE may need to obtain the configuration from SIB12.  MediaTek and Qualcomm agree with Huawei.

OPPO have the same view as Huawei; they understand that previously we had the exceptional pool in SIB12, and this CR introduces a case where the UE would need to use preconfiguration IC.  If we allow this, they understand that the historical rule in this respect would be changed.

Ericsson clarify that the intention of the proposal is for IC UEs to go to connected mode to get the pools from dedicated signalling.

Apple want to understand why this proposal for IC UEs does not defeat the purpose of using a relay; they think it is still more correct for the network to configure the pools.

Xiaomi understand for the IC UE, the UE should reselect to the cell and not depend on the relay any more, but the OOC UE would be able to use preconfiguration.

Huawei think this proposal has been discussed several times on the reflector and pursuing it further would not be so helpful.

There are generally two aspects to this issue.

- Scenario-1: Remote UE is in-coverage, and no exceptional pool is provided in SIB, whether it can make use of pre-configuration or not.

- Scenario-2: Remote UE is out-of-coverage, and no exceptional pool is provided in SIB, whether it can make use of pre-configuration or not.

For Scenario-1, the answer is No.

Since LTE, pre-configuration is limited to out-of-coverage case, and in-coverage UE has to follow either SIB or dedicated-RRC configured resources. And in order to solve the issue that no sidelink resources to use before being RRC connected if there is no common Tx pool configured in SIB, exceptional pool is used as a solution.

Now in NR, if we would like to enable the usage of pre-configuration, we fail to see the advantage compared to exceptional pool based approach, since

- ‘exceptional pool based approach’ can achieve the effect that the ‘pre-configuration based approach’ can achieve (e.g., by configuring the e-pool in the same manner as in pre-configuration), and

- The usage of pre-configuration in in-coverage scenario (not limited to partial coverage any more) may cause interference to the reception of uplink and sidelink (allowed by dedicated-RRC), in which case the ‘pre-configuration based approach’ cannot work.

- For in-coverage RRC_IDLE/RRC_INACTIVE L2 remote UE, compared to the legacy ‘exceptional based approach’, the ‘preconfiguration-based approach’ does not provide advantages, but causes the concern on interference by limiting it the Tx pool configured in pre-configuration.

- R2 does not pursue further work to allow pre-configured Tx pool usage by in-coverage UE due to exceptional pool configuration being absent.

For Scenario-2, the answer is unclear.

At R2#114, it was agreed that

Proposal 3a (modified): RAN2 agree that for L2 remote UE which is out-of-coverage, but connected to network via a relay UE (i.e., either in RRC CONNECTED or RRC IDLE/INACTIVE), it should follow network configuration, i.e., SIB or dedicated signalling, if available.

Proposal 4a [easy]: RAN2 agree that for L2 remote UE which is out-of-coverage, but connected to network via a relay UE and in RRC IDLE/INACTIVE state, if the network configuration is not available, i.e., SIB, remote UE shall rely on pre-configuration to perform discovery.

It seems that

- If SIB is not available at all, pre-configuration shall be used, or

- If SIB is available with normal Tx pool configured, normal Tx pool shall be used; But

- If SIB is available, including the concerned PC5 frequency, but not including the normal Tx pool, it seems not super clear how the UE should do.

Then based on the current spec, in case c) above,

c1) if exceptional pool is configured, it can be used.

c2) if exceptional pool is not configured, it is not specified how the UE should behave

3>	else: (i.e., the remote UE, in RRC_IDLE/RRC_INACTIVE state, has acquired SIB (due to in-coverage, or due to forwarding by relay UE), and the concerned PC5 frequency is included in SIB)

4>	if the cell chosen for NR sidelink communication transmission provides SIB12:

5>	if SIB12 includes sl-TxPoolSelectedNormal for the concerned frequency, and a result of full/partial sensing, if selected and is allowed by sl-AllowedResourceSelectionConfig, on the resources configured in the sl-TxPoolSelectedNormal is available in accordance with TS 38.214 [19] or random selection, if allowed by sl-AllowedResourceSelectionConfig, is selected:

6>	configure lower layers to perform the sidelink resource allocation mode 2 based on resource selection operation according to sl-AllowedResourceSelectionConfig using the pools of resources indicated by sl-TxPoolSelectedNormal for the concerned frequency as defined in TS 38.321 [3];

5>	else if SIB12 includes sl-TxPoolExceptional for the concerned frequency:

6>	from the moment the UE initiates RRC connection establishment or RRC connection resume, until receiving an RRCReconfiguration including sl-ConfigDedicatedNR, or receiving an RRCRelease or an RRCReject; or

6>	if a result of full/partial sensing, if selected and is allowed by sl-AllowedResourceSelectionConfig, on the resources configured in sl-TxPoolSelectedNormal for the concerned frequency in SIB12 is not available in accordance with TS 38.214 [19]:

7>	configure lower layers to perform the sidelink resource allocation mode 2 based on random selection (as defined in TS 38.321 [3]) using the pool of resources indicated by sl-TxPoolExceptional for the concerned frequency;

So the final question is, how should we handle case c2) above.

But the question is whether c2) is valid case for L2 Relay. If that happens, it means the in-coverage UE would have no sidelink resource to use during RRC connection establishment, i.e., the network does not want to allow in-coverage RRC_IDLE / RRC_INACTIVE remote UE activity in this cell. If that is the intention, there seems no reason to allow out-of-coverage RRC_IDLE / RRC_INACTIVE remote UE activity.

In our view, at least there is no need to pursue normative change for this issue.

- For out-of-coverage RRC_IDLE/RRC_INACTIVE L2 remote UE, the current spec can already handle the cases where the SIB is unavailable, or the SIB is available with normal Tx pool and/or exceptional Tx pool are configured.

- For pre-configured Tx pool usage by out-of-coverage UE, RAN2 limit the discussion, if needed, to the case where SIB is available, including the concerned PC5 frequency, but including neither normal nor exceptional Tx pool. For this case, R2 at least does not puruse normative change.

Conclusion

We have the following proposals:

Proposal 1	R2 does not pursue further work to allow pre-configured Tx pool usage by in-coverage UE due to exceptional pool configuration being absent.

Proposal 2	For pre-configured Tx pool usage by out-of-coverage UE, RAN2 limit the discussion, if needed, to the case where SIB is available, including the concerned PC5 frequency, but including neither normal nor exceptional Tx pool. For this case, R2 at least does not puruse normative change.