3GPP TSG-RAN WG2 Meeting #121	R2-2300153

Athens, Greece, 27 February – 03 March 2023

Agenda item:	8.5.2.1

Source:	Nokia, Nokia Shanghai Bell

Title:	PDU set and data burst information

WID/SID:	FS_NR_XR_enh - Release 18

Document for:	Discussion and Decision

# 1	Introduction

In RAN2 #120 meeting, several companies raised some concern about the relationship between PDU Set and Data Burst, claiming that the definition of Data Burst was ambiguous and that this may lead to different interpretations of the relationship between PDU Set and Data Burst. In this contribution, we discuss the assumption and clarification made in SA2.

# 2	Discussion

SA2 has agreed to provide the following information dynamically [23.700-60]:

-	PDU Set Size in bytes;

-	End PDU of the PDU Set;

-	End of Data Burst (EOB) indication in the header of the last PDU of the Data Burst.

Additionally,  in their reply LS to RAN1 [S2-2301384], SA2 states that "During a Data Burst, the RAN should not assume periods of data transmission inactivity" and ““the period of time” [between consecutive Data Bursts] may vary for different Data Bursts and its duration is related to the data amount of the Data Burst and the reception time interval of each two successive packets of a Data Burst”. Therefore, the most appropriate indicator between EOB, End PDU of the PDU Set, and PDU Set size depends on the RAN functionality that use such information.

Solutions that exploit the periodicity of the traffic pattern, which is composed of a transmission period followed by a silent period, benefits from the use of EOB. These schemes include for example power saving schemes as the ones discussed during the RAN1 SI on XR enhancements, where UE sleep/inactive period is triggered once the last PDU of a frame has been successfully served. In this case the frame can correspond to a Data Burst and it is composed of multiple PDU Sets, which may for example correspond to the slices in which the frame has been split.

In contrast, solutions that benefit from the use of End PDU of the PDU Set and PDU Set size rather than EOB includes mechanisms that take advantage of the identification of PDUs as part a single entity, namely their PDU Set. For example, mechanism like discarding and prioritization based on PDU Set information belong to this category.

Observation: The most appropriate information between EOB indicator, End PDU of the PDU Set, PDU Set size, and other PDU Set information depends on the RAN functionality and its implementation.

Thus, both the notion of bursts and sets are useful, and we suggest to simply acknowledge the decisions made by SA2, without the need to mandate nor specify how these are used by the gNB.

Proposal: acknowledge the usefulness of both sets and bursts.

# 3	Conclusion

This contribution has observed the following:

Observation: The most appropriate information between EOB indicator, End PDU of the PDU Set, PDU Set size, and other PDU Set information depends on the RAN functionality and its implementation.

And proposed the following:

Proposal: acknowledge the usefulness of both sets and bursts.