3GPP TSG-RAN WG2 Meeting #121	R2-2300155

Athens, Greece, 27 February – 03 March 2023	R2-2211599

Agenda item:	8.5.2.3

Source:	Nokia, Nokia Shanghai Bell

Title:	PDU Discard for XR

WID/SID:	FS_NR_XR_enh - Release 18

Document for:	Discussion and Decision

# 1	Introduction

This contribution discuses PDU discard for XR services.

# 2	Need for Discard

An LS from SA4 was received in R2-2206969. Although the applicability of the IPB frames was generally questioned, the discarding of PDUs was still mentioned: "In some implementations (note that neither the video codec specifications, nor the IETF RFC, nor 3GPP specifications up to today provide any requirements or recommendation on implementations), the loss of one fragmentation packet of the NAL Unit may result in discarding the entire NAL unit and hence the second part of the PDU definition (which are of same importance requirement at application layer. All PDUs in a PDU Set are needed by the application layer to use the corresponding unit of information.) applies. In other implementations, receivers may use the data up to the first lost fragmentation unit to recover at least parts of the video data included in the NAL unit and apply error concealment afterward. In this case, the third part of the PDU Set definition (the application layer can still recover parts of the information unit, when some PDUs are missing) applies, but in this case the equal importance part of the PDU Set definition (which are of same importance requirement at application layer) may be misleading (Note that in this operation mode, as an example if the first packet of the PDU Set is lost, all other packets of the fragmentation units are useless, whereas of the last packet is lost, the decoder can use all packets except the last one."

This has been taken into account by SA2 in the PDU set definition [TR 23.700-60]: "A PDU Set is composed of one or more PDUs carrying the payload of one unit of information generated at the application level (e.g. a frame or video slice for XRM Services, as used in TR 26.926 [27]). In some implementations all PDUs in a PDU Set are needed by the application layer to use the corresponding unit of information. In other implementations, the

Observation 1: discarding of PDUs can be frequent for XR services.

# 3	Discarding PDUs

The discarding of PDUs is currently quite basic in PDCP [38.323] and RLC [38.322]:

-	PDCP reads the following:

When the discardTimer expires for a PDCP SDU, or the successful delivery of a PDCP SDU is confirmed by PDCP status report, the transmitting PDCP entity shall discard the PDCP SDU along with the corresponding PDCP Data PDU. If the corresponding PDCP Data PDU has already been submitted to lower layers, the discard is indicated to lower layers. For SRBs, when upper layers request a PDCP SDU discard, the PDCP entity shall discard all stored PDCP SDUs and PDCP PDUs. NOTE: Discarding a PDCP SDU already associated with a PDCP SN causes a SN gap in the transmitted PDCP Data PDUs, which increases PDCP reordering delay in the receiving PDCP entity. It is up to UE implementation how to minimize SN gap after SDU discard.

-	 While RLC reads the following:

When indicated from upper layer (i.e. PDCP) to discard a particular RLC SDU, the transmitting side of an AM RLC entity or the transmitting UM RLC entity shall discard the indicated RLC SDU, if neither the RLC SDU nor a segment thereof has been submitted to the lower layers. The transmitting side of an AM RLC entity shall not introduce an RLC SN gap when discarding an RLC SDU.

At PDCP, the current discard procedure can introduce SN gaps as it is left up to implementation. If outOfOrderDelivery is not configured, this will translate into reordering delays.

Observation 2: discarding data at PDCP can trigger reordering delays, unless outOfOrderDelivery is always configured.

At RLC, the current discard procedure is limited to the scenarios where neither the RLC SDU nor a segment thereof has been submitted to the lower layers. This limitation is aligned between RLC AM and RLC UM, even though in RLC UM there seems to be no strong reason against discarding a remaining tail segment. Besides, the current discarding cannot introduce any SN gap in case of RLC AM.

Observation 3: requesting RLC to discard SDUs does not always trigger an actual discard.

The mechanisms are quite basic because they were designed under the assumption that discard is more of an error case. For XR services, the assumptions are different: discard can be frequent. Without any optimisations, it would not always be possible to avoid transmitting data that is known to be useless for the receiver, and as a result, the capacity and power saving gains would be limited. Thus for the sake of capacity and power consumption, when discard is triggered for XR services, it should actually occur.

Furthermore, XR services have low latency requirements, and any discard procedure should not increase delays due to reordering.

Proposal 1: the discard procedures in PDCP and RLC should be enhanced to guarantee that discard will actually take place and without triggering additional delays.

# 4	PSIHI

The PDU Set Integrated Handling Indicator (PSIHI) determines whether all PDUs within a PDU set are required to be delivered so that the PDU set is counted as successfully received or not. This indication can have direct impact on the PDU discarding as if PSIHI indicator is set to true (all PDUs are required), then discarding a PDU may trigger discarding of the rest of the PDUs belonging to the same PDU set.

However, a careful consideration is required when discarding the rest of the PDUs as it may violate the Packet Error Rate (PER) requirement of the QoS flow that is carrying the PDU set. Therefore, PER calculations might require revision to consider such cases and do not count each discarded PDU as a single error.

Observation 2: when the PSIHI indicator is set, the loss of one of the PDUs of a set may be used to trigger discarding of the rest of the PDUs within that PDU set, potentially increasing the PER of the QoS flow.

Proposal 2: if discarding is done due to PSIHI, the PER calculation should be revised to reflect the actual error rate of the QoS flow.

# 5	PSER

According to the latest SA2 conclusions [23.700-60], two PDU set QoS parameters are defined to support PDU set handling:

-	PDU Set Error Rate: The PDU Set Error Rate (PSER) defines an upper bound for the rate of PDU Sets that have been processed by the sender of a link layer protocol (e.g. RLC in RAN of a 3GPP access) but that are not successfully delivered by the corresponding receiver to the upper layer (e.g. PDCP in RAN of a 3GPP access). Thus, the PSER defines an upper bound for a rate of non-congestion related packet losses. The purpose of the PSER is to allow for appropriate link layer protocol configurations (e.g. RLC and HARQ in RAN of a 3GPP access). For every 5QI the value of the PSER is the same in UL and DL. If any PDU within the PDU Set is not successfully transmitted, the PDU Set is treated as error.

-	PDU Set Delay Budget (PSDB): The PSDB defines an upper bound for the delay that a PDU Set may experience for the transfer between the UE and the N6 termination point at the UPF, i.e. time between reception of the first PDU and the successful delivery of the last arrived PDU of a PDU Set. PSDB applies to the DL PDU Set received by the UPF over the N6 interface, and to the UL PDU Set sent by the UE. For a certain 5QI the value of the PSDB is the same in UL and DL.

If legacy PDCP discarding (based on discardTimer) is triggered often for one or more of PDU sets, the PSER may exceed its target easily. Therefore, two cases can be considered:

-	If the expiry of the discardTimer is considered as congestion-related, then according to PER and PSER definition, the packet discarding does not count as packet error and no further discussion is required;

-	If the expiry is not considered as congestion –related, then there might be a need to disable PDCP discardTimer if maintaining PSER or/and PER has a higher priority.

Current timer-based discarding is working per PDCP SDU and does not consider any dependencies among different SDUs (which is the case for PDU sets). As mentioned in the definition of the PSDB, if a delivery of a PDU set should happen within a pre-defined PSDB, similar arguments can apply on the discardTimer and enhance it to account for the PDU set handling. However, as discussed in [R2-2300156], when both PDB and PSDB are enabled, PDCP discarding may apply the most stringent delay requirement, thus resulting in suboptimal use of radio resources. In particular, PDUs may be discarded due to the expiration of PDB while PSDB makes the PDU content still useful for the application. To elaborate more on this, let us consider set_duration the difference between the arrival time of the last PDU of the PDU Set and the first PDU of the same set.

-	If set_duration + PDB > PSDB, network should use PSDB as it is shorter

-	If set_duration + PDB < PSDB, network should use PDB as the last PDUs of the set have a PDB that is shorter than the PSDB of the set

However, the right value between PDB and PSDB can be known only when the last PDU of the PDU Set arrives and earlier PDUs have been already served.

Proposal 3: Further studies are required to enhance discardTimer (if needed) to handle PDU sets.

# 6	Conclusion

This document has made the following observations:

Observation 1: discarding of PDUs can be frequent for XR services.

Observation 2: when the PSIHI indicator is set, the loss of one of the PDUs of a set may be used to trigger discarding of the rest of the PDUs within that PDU set, potentially increasing the PER of the QoS flow.

And proposed the following:

Proposal 1: the discard procedures in PDCP and RLC should be enhanced to guarantee that discard will actually take place and without triggering additional delays.

Proposal 2: if discarding is done due to PSIHI, the PER calculation should be revised to reflect the actual error rate of the QoS flow.

Proposal 3: Further studies are required to enhance discardTimer (if needed) to handle PDU sets.

A corresponding text proposal related to P1 is given below.

Text Proposal

### 5.3.2	Layer 2 Enhancements

In order to enhance the scheduling of uplink resources for XR, the following improvements are envisioned:

-	New BS table(s) to reduce the quantisation errors in BSR reporting (e.g. for high bit rates);

-	Delay knowledge of buffered data, consisting of at least remaining time, and distinguishing how much data is buffered for which delay.

Editor's Note: FFS if dynamic reporting of the uplink delay (e.g. via BSR) is needed, or whether PSDB is sufficient.

Furthermore, when, for a PDU set, the number of PDUs known to either be lost or associated to discarded SDUs, exceeds a threshold (see subclause 5.1.1), all remaining PDUs of that PDU set could be discarded at the transmitter to free up radio resources. In order to ensure that discard will actually take place and without triggering additional delays in radio protocols, the discard procedures in PDCP and RLC should be enhanced.

NOTE: this means that the granularity of the discard operation at PDCP in the transmitter should be the PDU set.

Editor's Note: the discard operation could also be captured in 5.2.2.