3GPP TSG-RAN WG2 Meeting #121                                                                                   R2-2300178

Athens, Greece, Feb 27- March 3, 2023

Source:	CATT, CBN

Title:	Discussions on PTM Configuration and Mobility

Agenda Item:	8.11.2.1

Document for:	Discussion and Decision

- 1. Introduction

This contribution further discusses the open issues related to inactive multicast reception, including aspects such as PTM configuration and mobility. The detailed discussions are in Section 2, and followed by Section 3 that summarizes the conclusion.

- 2. Discussions

2.1 PTM Configuration

Issue 1: How to provide the PTM configuration for inactive multicast reception via MCCH

In previous meeting we had agreed that the mixed solution (i.e., RRC dedicated signaling + MCCH) would be used for inactive multicast reception [1]. Then based on this, we need to consider how to provide the PTM configuration for multicast via MCCH.

There are following options,

Option 1 – Use the same MCCH as that for MBS broadcast

Option 2 – Introduce a separate new MCCH logical channel for multicast

In our view, it is not necessary to use a separate MCCH channel for multicast. One may argue that separate MCCH can provide better service differentiation between broadcast and multicast.However, the same arguments have ever been raised when designing MCCH for R17 broadcast (i.e. on the multiple MCCHs for different broadcast services), but finally it is concluded to support only one MCCH in a cell. The reason is that using multiple MCCHs in one cell may increase the overhead and complexity of NG-RAN. And UE may need to monitor multiple MCCHs, which will result in the increase of power consumption. When it comes to considering MCCH for multicast, the same reasons apply.

Observation 1: The same MCCH channel for Rel-17 MBS broadcast can be used to provide the PTM configuration for inactive multicast reception, i.e., there is no strong reason to introduce a multicast-specific MCCH.

Then regarding the message to carry the PTM configuration for multicast, there are two options, i.e., introducing a new RRC message or reusing the existing MBS broadcast message (i.e. MBSBroadcastConfiguration). In our view, it is simplest to reuse the MBS broadcast message. With this, we can also reuse the legacy MCCH change notification mechanism as much as possible.

If we introduce a new RRC message, it needs more spec effort and RAN1 may also need to be involved. For example, RAN1 needs to consider how to indicate the MCCH change for multicast, i.e., defining multicast-specific DCI bit(s) or reusing the broadcast DCI bit(s). But no TU is allocated for RAN1 in Rel-18 MBS. Therefore it is suggested that,

Proposal 1: Reuse the MBS broadcast message (i.e., MBSBroadcastConfiguration) to convey the PTM configuration for inactive multicast reception.

Issue 2: How to provide MCCH configuration to UE

Regarding how to provide MCCH configuration to UE, there is a FFS on whether it can be initially provided to the UE via dedicated signaling.

- FFS whether MCCH configuration is initially provided to the UE via dedicated signalling.

For this issue we consider it separately for the agreed two scenarios.

In Rel-18, multicast reception for UEs in INACTIVE supports at least the following scenarios, with the assumption that the UE already has a valid PTM configuration: - - Scenario 1: a UE has been receiving multicast in CONNECTED, and it enters INACTIVE and continues the multicast reception. - - Scenario 2: a UE has joined a multicast session and has been directed to INACTIVE, the UE starts to receive the multicast session FFS for state changes, e.g. due to service being not provided in INACTIVE anymore etc.

- Scenario 1

Some companies think it may improve the service continuality for Scenario 1 if providing MCCH configuration via dedicated RRC signaling. Note that even UE can get the MCCH configuration via dedicated signaling, UE still needs to wait for the next repetition period to read MCCH during the MCCH window, so this in fact does not bring much additional benefit for the service continuality than reading SIB.

- Scenario 2

For Scenario 2, when UE receives the notification of session activation, UE needs to perform state transition to obtain the MCCH configuration if it is provided via dedicated signaling, and this does not align with the intention that UE starts the multicast reception in RRC_INACTIVE (i.e.,  without entering connected mode).

Observation 2: For inactive multicast reception, it is sufficient for UE to obtain the MCCH configuration from system information.

Then whether to have a new SIB to schedule MCCH needs further consideration. As we suggested early in Observation 1, the same MCCH channel for Rel-17 MBS broadcast is used to provide the PTM configuration for inactive multicast reception, it seems straightforward for Rel-18 UE to read SIB20 to get the MCCH configuration. But we still need to consider whether there is impact to MBS broadcast mechanism.

In Rel-17 MBS, the presence of SIB20 is checked during the frequency prioritization for cell reselection and MII reporting. In the following we give our analysis on the impact for these procedures:

- frequency prioritization for broadcast cell reselection

The frequency prioritization for Rel-17 broadcast cell reselection follows the principle in TS 38.304:

If the MBS broadcast capable UE is receiving or interested to receive an MBS broadcast service(s) and can only receive this MBS broadcast service(s) by camping on a frequency on which it is provided, the UE may consider that frequency to be the highest priority during the MBS broadcast session as specified in TS 38.300 [2] as long as the two following conditions are fulfilled: 1) SIB1 scheduling information of the cell reselected by the UE due to frequency prioritization for MBS contains SIB20; 2) Either: - One or more MBS FSAI(s) of that frequency is indicated in SIB21 of the serving cell and the same MBS FSAI(s) is also indicated for this MBS broadcast service in MBS User Service Description (USD) as specified in TS 26.346 [20], or - SIB21 is not provided in the serving cell and that frequency is included in the USD of this service, or - SIB21 is provided in the serving cell but does not provide the frequency mapping for the concerned service, and that frequency is included in the USD of this service.

After UE reselects to a new cell of the prioritized frequency based on Condition 2), UE also needs to check whether SIB1 has the scheduling information of SIB20 during camping on the reselected cell. As the same motivation used in LTE, Condition 1) is introduced to control congestion for a certain frequency. When the NW finds a frequency to be congested, the NW can stop providing SIB20 in the reselected cell so that UEs in the cell can spread out to other frequencies on the subsequent mobility. Also, if other UEs from a neighbor cell move to the cell of this frequency after this cell stop providing SIB20, these UEs shall also de-prioritize this frequency on the subsequent mobility. The concern may be that gNB may want to stop scheduling SIB20 for broadcast when congestion happens on the reselected cell but it cannot stop due to the existence of inactive multicast.

But in our view it is not a valid scenario. When congestion happens, the NW should disperse UEs to other frequencies as much as possible, regardless of UE receiving broadcast and/or UEs receiving inactive multicast.

- interested service reporting in MII

In MII reporting procedure of Rel-17 broadcast, the presence of SIB20 is also checked when UE determines whether to include mbs-ServiceList in the MII message. Then for the case that SIB20 is provided in the cell due to the fact that only inactive multicast is enabled in the cell (i.e., no broadcast service is provided in the cell), it shall allow RRC_CONNECTED UE interested in broadcast service to report the interested broadcast services in MII, which is not expected.

In the current MII reporting, an interested broadcast service (i.e., IE MBS- ServiceList) is included under the premise that at least one interested frequency that is included in MII is providing or is about to start this broadcast service. Thus according to the determination of the interested service in TS 38.331:

The UE shall: 1> consider an MBS service to be part of the MBS services of interest if the following conditions are met: 2> the UE is receiving or interested to receive this service via a broadcast MRB; and 2> the session of this service is ongoing or about to start; and 2> one or more MBS FSAIs in the USD for this service is included in SIB21 acquired from the PCell for a frequency belonging to the set of MBS frequencies of interest, determined according to 5.9.4.3 or SIB21 acquired from the PCell does not provide the frequency mapping for the concerned service but that frequency is included in the USD of this service. NOTE: The UE may determine whether the session is ongoing from the start and stop time indicated in the User Service Description (USD), see TS 38.300 [2] or TS 23.247 [67].

Even the current cell does not provide any ongoing broadcast service, UE shall set the interested service that is about to start (based on information in USD) to the IE MBS-ServiceList, and NW shall take the MII into account in the subsequent handover for service continuality.

Based on above analysis, we do not see any critical impact on the Rel-17 broadcast mechanism if inactive multicast UE reads MCCH configuration from SIB20. Therefore we propose,

Proposal 2: UE acquires the MCCH configuration for inactive multicast reception from SIB20.

Issue 3: The signaling design for PTM configuration in MCCH

In Rel-17, the multicast configuration in the RRCReconfiguration message has different signaling structure than the broadcast configuration in MCCH. So when it comes to the signaling design for PTM configuration of multicast in MCCH, the first thing is which structure as the baseline, i.e., Rel-17 multicast configuration or Rel-17 broadcast configuration.

Table I: The structure of PTM configuration in MCCH

| Requirements on MCCH for Rel-18 inactive multicast reception | Rel-17 multicast configuration | Rel-17 broadcast configuration |
| --- | --- | --- |
| UE should know which RNTI value is associated to which TMGI so that it can start/stop to monitor a certain RNTI value based on the MCCH change notification | UE monitors the g-RNTI(s)/g-CS-RNTI(s) configured by RRCReconfiguration and does not know the mapping between g-RNTI/g-CS-RNTI and TMGI | UE monitors the interested g-RNTI(s) in MCCH and knows the association between the g-RNTI and TMGI |
| RRC_INACTIVE UE may move between different cells so MCCH should provide all the MRB configuration for an multicast session, instead of the delta configuration by the structure of ToAddModList | The NW configures the MRB(s) which the UE needs to add/modify/release by MRB-ToAddModList-r17/ MRB-ToReleaseList-r17 | The MCCH includes the MRB(s) which the UE needs to add/modify/release by mrb-ListBroadcast-r17 |
| The CFR configuration for inactive multicast should be common for all UEs in a cell but not UE-specific configuration | CFR is configured within the dedicated BWP per UE | CFR is broadcast in SIB20 for all UEs in a cell |

Besides, to perform lossless handover in Rel-17 multicast, the target cell can reconfigure the MRB with a new MRB ID in the handover command, and thus the association between the mrb-Identity and mrb-IdentityNew is provided to UE for a known target cell. This is not suitable for common channel (e.g., MCCH) as the current cell does not know the target cell that a UE may move into.

Observation 3: The signaling structure of Rel-17 multicast configuration is designed from connected UE perspective, which is not applicable if including in MCCH,

- No mapping between G-RNTI/G-CS-RNTI and TMGI

- MRB configuration in “AddModList” style can only be used for a specific UE

- Multicast CFR is included in dedicated BWP, which is configured  only for a specific UE

- The mapping between mrb-Identity and mrb-IdentityNew is for a known target cell

The analysis in Table I show that it is more suitable to take the structure of Rel-17 broadcast configuration as baseline. Therefore we propose,

Proposal 3: For multicast PTM configuration on MCCH, take the signaling structure of Rel-17 broadcast configuration as baseline.

Issue 4: CFR configuration for inactive multicast reception

In Rel-17, multicast service is only allowed to be received in RRC_CONNECTED, and the CFR is confined within the dedicated BWP. In Rel-18, a multicast service can be received by a UE either in RRC_CONNECTED or in RRC_INACTIVE. So how to design the CFR for inactive multicast reception shall have different complexity from UE point of view.

One option is to introduce an inactive multicast specific CFR. By this way, the CFR configuration should be separately provided to UE. Furthermore, when UE changes into RRC_CONNECTED, the CFR switching should be performed, and this will cause service interruption if the connected CFR does not cover the inactive CFR.

Another Option is to use same CFR for connected reception and inactive reception. By this way the service interruption shall be eliminated because of no CFR switching during the state transition. From our view it is efficient to use same CFR for a specific multicast service, regardless it is for RRC_CONNECTED UEs or RRC_INACTIVE UEs. Besides, it also aligns well with the assumption we have made in previous meeting (i.e., ”we assume the same PDCCH/PDSCH resources (e.g. resources used for MTCH) can be used for all UEs (including UEs in CONNECTED and/or INACTIVE states) for receiving the same multicast session.”).

Therefore, it is proposed,

Proposal 4: For a certain multicast service, the same CFR configuration is used for multicast reception in both connected and inactive states.

As we proposed earlier, the same MCCH for MBS broadcast is used to provide the PTM configuration for inactive multicast reception. MCCH is sent in the broadcast CFR, thus to avoid CFR switching, it should ensure that UE can read MCCH in the multicast CFR. To achieve this purpose the NW can configure the multicast CFR that covers the broadcast CFR (Option 1). Or the broadcast CFR is also used for multicast reception (Option 2). For example, if gNB supports both broadcast and inactive multicast reception, it can provide a larger common CFR resource to satisfy the scheduling requirements for both broadcast and multicast reception. This way does not need to provide separate CFR configuration for broadcast and multicast.

Proposal 5: Discuss how to solve the co-existence issue between multicast CFR and broadcast CFR in a cell,

- Option 1: the multicast CFR overlaps with the broadcast CFR;

- Option 2: use same CFR for broadcast and multicast.

Issue 5: MCCH change notification

In legacy, MCCH change notification mechanism is used to notify the change of MCCH content due to broadcast session start, modification or stop as well as due to neighbor cell information modification.

Previously for session activation we have agreed that,

As a baseline, group paging can be used to inform Rel-18 UE(s) about the session activation (Details FFS, e.g., UE behavior when receiving such group notification).

So there is no need to additionally consider how to notify the MCCH change due to session activation/start. Then the left issue is how to notify the other MCCH change cases (e.g., PTM configuration modification of an ongoing session, session stop).

There are several solutions to consider,

Option 1 – DCI indication similar as Rel-17 broadcast

In Rel-17, a 2-bit bitmap is introduced in DCI to indicate the change of MCCH content, where the MSB indicates the start of MBS session, and the LSB indicates modification of MCCH information other than the change caused by start of new MBS service(s), e.g. modification of a configuration of an on-going MBS session(s), MBS session(s) stop or neighbor cell information modification. For the Rel-18 inactive multicast reception, we can reuse the LSB of the 2-bit bitmap to indicate the modification of MCCH due to broadcast and/or inactive multicast. This way does not cause RAN1 efforts, and also RAN1 has no TU on this WI.  As an alternative, depending on RAN1 a multicast-specific DCI can be introduced for this indication, but this way needs further efforts and considerations by RAN1.

Option 2 – per TMGI indication by group paging

Another possible way is to use group paging to indicate the change of MCCH content. Per TMGI indication for MCCH change notification can be done by group paging, but there may be issues caused by the POs of different UEs. For example, the POs of different UEs that need to be paged may spread over multiple modification periods, then how the NW guarantees that all UEs can be notified within the current modification period so that NW shall update the configuration at the beginning of next modification period.

Proposal 6: Discuss how to notify MCCH change for inactive multicast reception,

- Option 1 – DCI indication

- Option 2 – per TMGI indication by group paging

2.2 Mobility

Case 1: On-going to on-going mobility

Issue 1: how to reselect to cell providing the ongoing multicast service

Previously the following were agreed:

Multicast service continuity after cell reselection in RRC_INACTIVE state (i.e. without resuming RRC connection) will be supported (if the configuration of the new cell is available for the UE). FFS whether there are cases where the UE needs to resume the connection. FFS RAN3 impacts due to inter-gNB mobility. Upon cell reselection to neighbour cells during active multicast session, if the configuration of the session is not available for the new cell for UEs in INACTIVE, then the UE is required to resume RRC connection to get the Multicast MRB configuration.

One issue is whether it is beneficial to introduce some prioritization to the cells and/or frequencies where UE can receive multicast in RRC_INACTIVE in the cell reselection procedure.

More specifically, the following options for choosing candidate cell during cell reselection can be further considered:

Option 1 – prioritize a frequency on which the inactive multicast reception is provided

In Rel-17, there are the following mechanism for service continuity: the UE which is receiving or interested to receive MBS broadcast service(s) via PTM and can only receive these MBS broadcast service(s) via PTM while camping on the frequency providing these MBS broadcast service(s) is allowed to make this frequency highest priority when the conditions described in TS 38.304 are met; when the MBS broadcast service(s) which the UE is interested in are no longer available (after the end of the session) or the UE is no longer interested in receiving the service(s), the UE no longer prioritizes the frequency providing these MBS broadcast service(s).

Option 1 is to support frequency prioritization for inactive multicast reception, similar as the Rel-17 broadcast frequency prioritization. For the detailed mechanism to allow UE to determine a frequency providing inactive multicast reception for the concerned multicast session, there are some alternatives,

Alt 1: take Rel-17 frequency prioritization design as baseline (i.e., SIB21+USD)

Alt 2: use the dedicated frequency priority in RRC Release message

It can be discussed further on which way to go after we conclude on the supporting of frequency prioritization for inactive multicast reception.

Option 2 – Prioritize a neighbor cell(s) on which the inactive multicast reception is provided.

For Option 2, the prioritization is on a per cell basis. One straightforward way is that UE selects the candidate cells according to the mbs-NeighbourCellList in MCCH, i.e., to extend the use of Rel-17 mbs-NeighbourCellList.

Option 2 is a workable solution but it does not align well with the legacy principle for cell reselection (i.e. only prioritize a frequency during cell reselection). One may argue that in Rel-17 the mbs-NeighbourCellList in MCCH can indicate ongoing service for neighbor cells. It is worth to clarify that in Rel-17 MBS the mbs-NeighbourCellList is not used for determining the candidate cell for cell reselection, but used to allow the UE to request unicast reception of the service before moving to a cell not providing the MBS broadcast service(s) using PTM transmission.

Option 3 – Reuse the legacy cell resection procedure. i.e., no enhancement to the cell reselection

Option 3 is the simplest way and has no spec impacts but the service continuity during cell reselection cannot be ensured.

Based on discussion above, Option 1 is preferred.

Therefore, we propose that,

Proposal 7: To support service continuality during cell reselection, UE can prioritize a frequency on which inactive multicast reception is provided.

In Rel-17, NW can release UE to RRC_INACTIVE due to session deactivation/temporary no data transmission for the multicast session. When the multicast session is activated or has data transmission, group paging is used to notify UE to reconnect to the NW. For group paging monitoring, we have the following Rel-17 agreement:

RAN2 will not specify a mechanism for the UE in RRC IDLE/INACTIVE which joined a multicast session to prioritize a certain frequency for group paging monitoring.

Rel-18 shall support UE to start receiving multicast service in RRC_INACTIVE without state transition. This will cause another consideration that whether UE needs to prioritize a frequency that supports multicast reception in RRC_INACTIVE after UE is notified of session deactivation. If this enhancement is not allowed, UE may reselect to a frequency that does not support inactive multicast reception based on the current cell reselection principle. Then there shall be the unexpected case where the NW indicates UE to start receiving the multicast service in RRC_INACTIVE but UE cannot receive this multicast service in the cell of the reselected frequency. This issue can be left for further study.

Issue 2: how to acquire the PTM configuration of target cell

Previously it has been discussed on whether to provide PTM configuration of neighboring cells via RRC dedicated signaling or MCCH but not concluded. In our view, there is no need to support this as anyway UE can read MCCH on the target cell to get the PTM configuration. And it requires complex design on network interface from RAN3 perspective and increases the signaling overhead significantly. One may argue that it can reduce the latency or data loss during mobility, but seamless/lossless mobility is not required according to the WID,

Study the impact of mobility and state transition for UEs receiving multicast in RRC_INACTIVE. (Seamless/lossless mobility is not required) [RAN2, RAN3]

Therefore, it is proposed,

Proposal 8: UE acquires the PTM configuration of the target cell for inactive multicast reception by reading MCCH of the target cell.

Case 2: On-going to non-going mobility

Issue 1: how to determine if an inactive multicast service is on-going in the target cell

Another possibility is that RRC_INACTIVE UE moves from an inactive multicast on-going cell to a new cell that is not on-going for the same service.

Previously we have agreed that

Upon cell reselection to neighbour cells during active multicast session, if the configuration of the session is not available for the new cell for UEs in INACTIVE, then the UE is required to resume RRC connection to get the Multicast MRB configuration.

Since we have agreed to use MCCH to provide PTM configuration for inactive multicast reception, it is straightforward that UE resumes RRC connection if PTM configuration for inactive multicast reception is not provided in MCCH of the target cell.

Proposal 9: After cell reselection, if PTM configuration for inactive multicast reception is not provided in MCCH of the target cell, UE determines it as a non-ongoing cell and resumes RRC connection.

Issue 2: how to set the cause when UE resumes the RRC connection

There may be three scenarios when UE moves to a cell on which the inactive multicast reception is not ongoing:

- Scenario 1: UE moves out of the service area for this multicast service, i.e., the target cell does not support this multicast session

- Scenario 2: UE does not move out of the service area for this multicast service, but the target cell does not have the capability to provide the function of multicast reception in RRC_INACTIVE

- Scenario 3: UE does not move out of the service area for this multicast service, and the target cell has the capability to provide the function of multicast reception in RRC_INACTIVE, but the target cell does not open this function

For above three scenarios, according to the mentioned agreements in Issue 1 of the Mobility section, the UE behavior is to trigger RRC resume procedure. From the UE side it cannot differentiate these scenarios. But at least for scenario 3, UE needs to let gNB know that the resume procedure is triggered due to multicast reception, then gNB can switch on the inactive multicast reception and then release UE to INACTIVE state for multicast reception.

Currently there are following cause values that have been introduced:

ResumeCause ::=             ENUMERATED {emergency, highPriorityAccess, mt-Access, mo-Signalling, mo-Data, mo-VoiceCall, mo-VideoCall, mo-SMS, rna-Update, mps-PriorityAccess, mcs-PriorityAccess, spare1, spare2, spare3, spare4, spare5 }

Besides the cause value of rna-Update, other causes are mainly used to indicate the NW of unicast data access or signaling transmission for different use cases, then the NW prefers to change UE to RRC_CONNECTED based on the load condition. For example, the NW shall handle the RRC connection resume with the causes set by emergency (due to emergency service), mps-PriorityAccess (due to multimedia priority service) and mcs-PriorityAccess (due to mission critical service) with a high priority, and unless extreme load condition, the NW shall accept the access attempt to change UE to RRC_CONNECTED. If the resume procedure triggered by inactive multicast reception reuses any of above cause values, it will confuse the gNB that whether UE has requirement of unicast data access or signaling transmission, and thus gNB cannot decide to keep UE in RRC_INACTIVE or change UE to RRC_CONNECTED.

Then others may think that the current cause value of rna-Update can be reused, but it is not reliable. When UE triggers the resume procedure (e.g., due to Periodic RNA update) in the RNA, then according to the current 38.300 (Figure 9.2.2.5-2: Periodic RNA update procedure without UE context relocation), the anchor gNB may reject the request of UE context from the current gNB, thus gNB cannot switch on the inactive multicast reception without knowing the UE multicast context.

Observation 4: The legacy cause values are not suitable to indicate the multicast reception purpose to NW when triggering RRC resume procedure.

Based on Observation 3, for UE resuming RRC connection due to multicast reception, a new cause value is needed. When UE moves within RNA, UE uses this new cause value to indicate multicast reception in the RRCResumeRequest message. But another consideration is when UE reselects to a cell that is out of the RNA, it is possible that this cell is also not an on-going cell for a certain inactive multicast session. In this situation how UE set the cause value in the RRCResumeRequest message needs further consideration. In our view, the legacy cause value for RNA update can be reused to handle this scenario. When the gNB receives the cause value for RNA update, it retrieves the UE context from the anchor gNB. The anchor gNB further checks that the resumption cell is not in the previously configured RNA, and then provides the UE context to the current gNB. By this way, the current gNB knows which multicast session the UE is receiving and can further let UE continue receiving the multicast session in RRC_CONNECTED or RRC_INACTIVE based on the gNB capability.

Proposal 10: When resuming the RRC connection for multicast reception, the cause value is set as following,

- When UE moves to a cell in the RNA, the new cause value is used;

- When UE moves to a cell not in the RNA, UE sets the cause value to rna-Update.

- 3. Conclusion

This paper further discusses the open issues related to the PTM configuration and mobility for multicast reception in RRC_INACTIVE. The contributions are summarized as follows:

PTM Configuration

Issue 1: How to provide the PTM configuration for inactive multicast reception via MCCH

Observation 1: The same MCCH channel for Rel-17 MBS broadcast can be used to provide the PTM configuration for inactive multicast reception, i.e., there is no strong reason to introduce a multicast-specific MCCH.

Proposal 1: Reuse the MBS broadcast message (i.e., MBSBroadcastConfiguration) to convey the PTM configuration for inactive multicast reception.

Issue 2: How to provide MCCH configuration to UE

Observation 2: For inactive multicast reception, it is sufficient for UE to obtain the MCCH configuration from system information.

Proposal 2: UE acquires the MCCH configuration for inactive multicast reception from SIB20.

Issue 3: The signaling design for PTM configuration in MCCH

Observation 3: The signaling structure of Rel-17 multicast configuration is designed from connected UE perspective, which is not applicable if including in MCCH,

- No mapping between G-RNTI/G-CS-RNTI and TMGI

- MRB configuration in “AddModList” style can only be used for a specific UE

- Multicast CFR is included in dedicated BWP, which is configured  only for a specific UE

- The mapping between mrb-Identity and mrb-IdentityNew is for a known target cell

Proposal 3: For multicast PTM configuration on MCCH, take the signaling structure of Rel-17 broadcast configuration as baseline.

Issue 4: CFR configuration for inactive multicast reception

Proposal 4: For a certain multicast service, the same CFR configuration is used for multicast reception in both connected and inactive states.

Proposal 5: Discuss how to solve the co-existence issue between multicast CFR and broadcast CFR in a cell,

- Option 1: the multicast CFR overlaps with the broadcast CFR;

- Option 2: use same CFR for broadcast and multicast.

Issue 5: MCCH change notification

Proposal 6: Discuss how to notify MCCH change for inactive multicast reception,

- Option 1 – DCI indication

- Option 2 – per TMGI indication by group paging

Mobility

Case 1: On-going to on-going mobility

Issue 1: how to reselect to cell providing the ongoing multicast service

Proposal 7: To support service continuality during cell reselection, UE can prioritize a frequency on which inactive multicast reception is provided.

Issue 2: how to acquire the PTM configuration of target cell

Proposal 8: UE acquires the PTM configuration of the target cell for inactive multicast reception by reading MCCH of the target cell.

Case 2: On-going to non-going mobility

Issue 1: how to determine if an inactive multicast service is on-going in the target cell

Proposal 9: After cell reselection, if PTM configuration for inactive multicast reception is not provided in MCCH of the target cell, UE determines it as a non-ongoing cell and resumes RRC connection.

Issue 2: how to set the cause when UE resumes the RRC connection

Observation 4: The legacy cause values are not suitable to indicate the multicast reception purpose to NW when triggering RRC resume procedure.

Proposal 10: When resuming the RRC connection for multicast reception, the cause value is set as following,

- When UE moves to a cell in the RNA, the new cause value is used;

- When UE moves to a cell not in the RNA, UE sets the cause value to rna-Update.

- 4. Reference

[1] Report of 3GPP TSG RAN WG2 meeting #120, Online