3GPP TSG-RAN WG2 Meeting #121                                                       R2-2300199

Athens, Greece, 27th February – 3th March, 2023

Source:	CATT

Title:	Discussion on LPHAP

Agenda Item:	8.2.4

Document for:	Discussion and Decision

Introduction

During the SI phase, the Low Power High Accuracy Positioning was studied. In RAN#98-e, the new WID on Expanded and Improved NR Positioning [1] was approved. On the topic of LPHAP, the objectives of this work item related to RAN2 are:

- Specify enhancements for enabling LPHAP use-case 6 as defined in TS 22.104 including: - Extending eDRX cycle beyond 10.24s in RRC_INACTIVE state towards meeting the battery life requirement for LPHAP [RAN2, RAN3, RAN4] - Positioning-specific enhancement for eDRX cycle beyond 10.24s to be defined as part of Rel-18 WI on expanded and improved NR positioning. - NOTE: Work on this objective should be coordinated with that in Rel-18 WI on eRedCap. Towards this, the feature of extending eDRX cycle beyond 10.24s should be defined as part of Rel-18 WI on eRedCap. - NOTE: Inputs from RAN1 as necessary may be facilitated via LSs - For UL and DL+UL positioning for UEs in RRC_INACTIVE state, specify SRS configuration enhancements based on SRS positioning validity area to avoid frequent RRC connection for SRS (re)configuration [RAN2, RAN1, RAN3]. - SRS for positioning configurations in multiple cells [RAN2, RAN1]. - Note: Details including issues such as interference, timing advance, spatial relation information, pathloss reference and common SRS parameters across multiple cells can be further discussed during normative work. - Pre-configuration of one or multiple SRS for positioning configurations [RAN2, RAN3]. - SRS for positioning activation/request procedure(s) [RAN2, RAN1]. - Specify solutions for DL PRS measurements for a UE in RRC_IDLE state and reporting of the measurements in RRC_CONNECTED state [RAN2]. - Specify solutions for alignment between eDRX and PRS configurations [RAN2].

In this contribution, the above objectives will be analyzed and discussed.

Discussion

  - Extending eDRX cycle beyond 10.24s in RRC_INACTIVE state

The extend eDRX cycle beyond 10.24s in RRC_INACTIVE state is in scope of Rel-18 eRedCap WI, a large DRX value is proposed, about up to 1024 SFN. In view of the requirement of LPHAP, the evaluation was performed by RAN1 with the extended DRX cycle 20.48s and 30.72s, and the results show that the battery life requirement can be met. Obviously, it is very likely that the requirement of LPHAP can be met by the enhancement made by eRedCap WI. Hence, LPHAP can wait for the progress of eRedCap WI. And after there is progress on the extend value of eDRX cycle, the POS WI can evaluate whether the extended eDRX cycles can fulfill the battery life requirement of LPHAP.

Proposal 1: For the objective of extend eDRX cycle beyond 10.24s in RRC_INACTIVE state, RAN2 can wait for the progress of eRedCap WI.

  - SRS configuration enhancements

In Rel-17, when a positioning event is detected, UE send an event report to the LMF. The network will configure SRS for this positioning procedure. The UL positioning procedure is shown as Figure 1 [2]. The validity of the configured SRS is based on the TA validation, which is judged by the TA timer mechanism and the RSRP change solution. Additionally, after cell reselection, UE will also release the SRS configuration [3].

Figure 1: Low Power Periodic and Triggered 5GC-MT-LR Procedure with SDT (UL-only positioning).

For Rel-18 WI, with the objective of SRS configuration enhancements, the SRS is not only used for this positioning procedure or the location at present, but in a larger scope and longer time. The SRS validity criteria and configuration mechanism as defined in Rel-17 are no longer applicable.

During the SI phase, many related issues are raised on the topic of SRS configuration enhancement, e.g., SRS configuration update, entity to configure SRS, the parameters of SRS that can be pre-configured, and so on. Correspondingly, some candidate solutions were proposed. However, these solutions are too scattered to consensus. Hence, the following issues can be discussed as a starting point.

#Issue 1: How to support SRS valid in multiple cells?

Firstly, the enhanced SRS has the requirement to be valid in larger range than cell scope. The valid area need to be configured to UE together with the SRS configuration. However, before performing the positioning procedure, UE may have no valid location information. Hence the valid area can only be defined by the cell information, e.g. a list of cell or TAC.

When cell reselection occurs, UE will compare the serving cell ID with the valid area. If the serving cell ID is contained in the valid area, the SRS can be seen as valid and could be used for positioning, else the UE may need to request SRS from the network.

Proposal 2: The parameter of valid area need to be configured to UE together with the SRS configuration. FFS the format e.g. defined as a list of cells or TAC.

However, if the validity of the enhanced SRS is only defined by the parameter of valid area, the enhanced SRS may be occupied for a long time even there is no ongoing positioning service. In order to recover the UL resource timely and improve the flexibility of network resource allocation, a valid time is also need to be configured together with the enhanced SRS configuration. During the valid time, the configured SRS can be considered as valid. Once the time is expired, both UE and the network will release the SRS configuration, UE may need to request SRS from the network.

Observation 1: If the SRS validity is defined only by valid area, the SRS resources may be occupied for a long time by the UE even there is no ongoing positioning service.

Proposal 3: A valid time can be configured together with the SRS configuration to recover the UL resource timely and improve the flexibility of network resource allocation.

#Issue 2: How to pre-configure one or multiple SRS for positioning configuration

During the SI phase, many candidate solutions were proposed. Companies are inconsistent on the understanding of the concept of “pre-configure one or multiple SRS”, shown as follow:

- Case A:　Pre-configure one or multiple UE-specific SRS via dedicate signalling, the SRS is valid in an area consist of serval cells;

- Case B:　Pre-configure multiple UE-specific SRS via dedicate signalling, each SRS is valid in a different cell;

- Case C:　Pre-configure multiple common SRS via posSIB, the SRS is valid in an area consist of serval cells;

- Case D:　Pre-configure multiple common SRS via posSIB, the SRS is valid in the current cell.

These four solutions can be divided into two types. For case A and B, the SRS is UE-specific. And for case C and D, the SRS is common to multiple UE. Compare to case A, case B bring larger signalling overhead and occupy more UL resource, but with no obvious gain. Case D is a special condition of case C, i.e., when the valid area of case C is defined as a cell. Hence, case A and case C can be discussed as start point.

Proposal 4: Take the following two ways of pre-configuration of SRS as start point.

- Option A: Pre-configure one or multiple UE-specific SRS via dedicate signalling, the SRS is valid in an area

- Option B: Pre-configure multiple common SRS via posSIB, the SRS is valid in an area

#Issue 3: How to support the positioning procedure, e.g. SRS activation/request procedure

The positioning procedure will be discussed with the following two SRS configuration methods.

- Pre-configure one or multiple UE-specific SRS via dedicate signalling

In this scenario, the serving cell configures dedicated SRS for the UE. The SRS is configured together with parameters used for validity criteria, which may contain valid area and/or valid time. Once a positioning event is detected, if the SRS is judged as valid based on the validity criteria, it can be used for positioning.

Considering the SRS is UE-specific, after the event is detected, there is no need for resource coordination between RAN nodes and LMF, and there will be no interference problems. Additionally, the SRS configuration is dedicated to the UE, and the correspondence between UE and SRS configuration is known by the RAN. The network can identify the UE by detecting SRS.

Observation 2: In case of option A, The UE identity is implicitly indicated by the UE-specific SRS, i.e. it is possible for the network to identify the UE by detecting UE-specific SRS.

However, fulfill the validity criteria doesn’t mean the UE can directly transmit the dedicated SRS when a positioning event is detected. Because the network has no idea which cell the UE is camping on and when UE will have a positioning requirement. The premise that UE can send SRS directly is that all the gNB in the valid area is keep detecting the UE-specific SRS. This will bring large power consumption to the network. Hence, it would be better for UE to indicate the occurrence of the positioning event to the network firstly.

Observation 3: The NW maybe knows neither which cell the UE is camping on nor when the LCS event is triggered by UE. So gNB has to continually monitor the UE-specific SRS.

One way is keep the current event report step as specified in Rel-17, shown as step 3 in Figure 1, i.e., the event report will be sent to LMF contained in a NAS message, and RAN will be notified to detect the SRS by LMF. From the view of UE, the only difference from the procedure in Rel-17 is that the RAN doesn’t send SRS configuration to UE, shown as step 10 in Figure 1. The power consumption reduction bring by not carrying SRS configuration is limited.

Since the purpose of indicating the occurrence of the positioning event is indicates the RAN to perform SRS measurement, it is a more direct way to send the indication to RAN rather than LMF. In Rel-17, the feature combination [3] was supported, i.e., the network can configure dedicate preamble for a feature. We can take advantage of this mechanism to indicate the positioning requirement of UE.

additionalRACH-ConfigList List of feature or feature combination-specific RACH configurations, i.e. the RACH configurations configured in addition to the one configured by rach-ConfigCommon and by msgA-ConfigCommon. The network associates all possible preambles of an additional RACH configuration to one or more feature(s) or feature combination(s). The network does not configure this list to have more than 16 entries. If both rach-ConfigCommon and msgA-ConfigCommon are configured for a specific FeatureCombination, the network always provides them in the same additionalRACH-Config.

The procedure is shown in the following figure.

Figure 2. UE indicate the occurrence of the positioning to RAN via preamble

By this way, UE only need to send a dedicate UL signalling and perform SRS transmission to complete the UL positioning procedure. Compare to the legacy inactive positioning procedure, i.e., Figure 1, this method saves 1 UL message and 3 DL message. The power consumption reduction is enormously.

Proposal 5: Under the mechanism of pre-configure UE-specific SRS, the following solution can be discussed

- Before transmitting SRS, UE indicates RAN to monitor the SRS.

- After receiving the indication, RAN identifies the UE identity by monitoring the UE-specific SRS and perform SRS measurement.

Observation 4: UE only needs to send a dedicate indication and transmits SRS in proposal 5 and the power consumption reduction is enormously compared with legacy procedure.

- Pre-configured multiple SRSs via posSIB

The network configures a list of candidate SRSs to UE via system information. The SRSs is configured together with parameters used for validity criteria, which may contain a valid area and/or a valid time. When the SRS is judged to be valid based on the validity criteria, the candidate SRS can be used for positioning. To avoid two or more UE use the same SRS at the same time, a coordinate procedure is needed.

The following agreement was achieved in RAN1 110-bis meeting.

- SRS for positioning activation/request procedure(s), e.g., network activation of SRS via paging, UE request to obtain/update SRS via RACH-based procedure; - FFS: Events of invalidity of SRS configuration to trigger the UE request procedure.

Observation 5: UE request to obtain/update SRS via RACH-based procedure is recommended by RAN1 during SI.

The RACH procedure can be used for coordination, i.e., SRS activation/request. The UE can indicate its requirement to the network by the following two ways:

- Option A: via dedicate UL resource, e.g., a dedicate preamble;

- Option B: indicated in Msg3.

Before choose an SRS for the UE, the network also needs to know the UE identity.

For option A, the UE identity can be implicit indicated by the UE-specific preamble. However, this will bring restriction to the usage of preamble. Or the dedicate preamble is common for all the UE in the cell, just to indicate the positioning requirement, Msg3 is used to indicate the UE identity. The latter method is easier to implement. The procedure at UE side is shown as Figure 3(a).

Figure 3(a). UE indicate its positioning requirement to the NW via the dedicate preamble

For option B, a new bit can be introduced in Msg3 or add a new parameter of ResumeCause for RRC_INACTIVE positioning. After receiving Msg3, the network gets the UE identity and realizes that this UE asked for positioning. Then the network determines an SRS for this UE via RRCRelease message. The procedure at UE side is shown as Figure 3(b).

RRCResumeRequest ::=            SEQUENCE {

rrcResumeRequest            RRCResumeRequest-IEs

}

RRCResumeRequest-IEs ::=        SEQUENCE {

resumeIdentity                  ShortI-RNTI-Value,

resumeMAC-I                     BIT STRING (SIZE (16)),

resumeCause                     ResumeCause,

spare                           BIT STRING (SIZE (1))

}

Figure 3(b). UE indicate its positioning requirement to the NW via Msg3

It can be seen that, both option A and option B need to perform Msg3, there is no obvious difference on power consumption aspect.

Proposal 6: Under the mechanism of pre-configured multiple SRSs via posSIB, UE request to obtain/update SRS via RACH-based procedure. FFS the following solutions.

- UE request SRS from the NW via dedicate preamble.

- UE request SRS from the NW via Msg3, e.g. a new bit or a new resumeCause.

In summary, under the mechanism of pre-configure UE-specific SRS via dedicate signalling, the UE can have lower power consumption. Under the mechanism of pre-configured multiple SRSs via SI, flexible UL resource utilization and can be achieved. Both mechanisms can be supported.

  - Alignment between PRS and (e)DRX

In RAN2#120, the enhancement on alignment between PRS and DRX was discussed, the following agreement was achieved.

Agreements:

Proposal2: Alignment between DRX and PRS is beneficial from power saving point of view for LPHAP and is recommended to normative work. (14/15)

	Two directions of solutions for DRX/PRS alignments are considered: (a) PRS alignment with fixed DRX (b) DRX alignment with fixed PRS

	Solutions for the PRS/DRX alignment, e.g., LMF-based/UE-based solution, is to be discussed

	Impacts to different RRC states (RRC_INACTIVE and RRC_IDLE) is to be discussed

During the SI phase, many candidate solutions were proposed. There are two directions of DRX/PRS alignments mentioned in the agreement of RAN2#120: (a) PRS alignment with fixed DRX; (b) DRX alignment with fixed PRS.

Choose which solution is based on the DRX and PRS configuration characteristics. In this section, the PRS and DRX will be analyzed firstly.

Characteristic of PRS configuration:

At present, the PRS is requested by the LMF and configured by the RAN. And the PRS is configured to UE via system information or via LPP information. The PRS is not configuration for a certain UE, i.e., the PRS configuration is non-UE associated.

Characteristic of DRX configuration:

UE in RRC_IDLE and RRC_INACTIVE state monitors one paging occasion (PO) per DRX cycle. To determine when to monitor paging, the UE need to calculate the DRX cycle, PF and PO in turn.

For CN-initiated paging, a default cycle is broadcast in system information and a UE-specific cycle can be configured via NAS signalling. For RAN-initiated paging, a UE-specific cycle is configured via RRC signalling. UE in RRC_INACTIVE will use the shortest of the three cycles, if configured. All these DRX cycle are valued in rf32, rf64, rf128 and rf256 [3] [5].

After determine the DRX cycle, the PF and PO for paging determined by the following formulae [4]:

SFN for the PF is determined by: (SFN + PF_offset) mod T = (T div N)*(UE_ID mod N) Index (i_s), indicating the index of the PO is determined by: i_s = floor (UE_ID/N) mod Ns

It can be seen that the potential SFN which can be calculated as PF can be determined by the cell level parameters of PF_offset, N and minimum DRX cycle (i.e., rf32). Within these potential PF, the potential PO can also be determined. The PF and PO calculated by a certain UE_ID must be a subset of these potential PF and PO, i.e., these potential PF and PO contains the monitoring time of all UEs in the cell.

Figure 4. Potential PF and PO

Observation 6: All potential PF and PO of a cell are determined by the cell level parameters, e.g. PF_offset, N and minimum DRX cycle (i.e., rf32), these potential PF and PO can cover all the monitoring time of the UEs in the cell.

Analyze on potential solutions:

Take the above characteristics of PRS and DRX into consideration, it can be seen that the PRS is cell level configuration, while the PO for different UE is different. Align the PRS with a certain UE DRX or adjust the DRX of a UE to align the PRS is not applicable.

Considering the potential PF and PO of the cell only occurs on some certain times and can cover all the monitoring time of the UEs in the cell. It is possible to make alignment between the PRS and the potential PF and PO of the cell. By this way, the PRS/DRX alignment requirement for all the UE in the cell can be met.

Observation 7: By making alignment between the PRS and the potential PF and PO of the cell, the PRS/DRX alignment requirement for all the UEs in the cell can be met.

There are two directions of DRX/PRS alignments mentioned in the agreement of RAN2#120: (a) PRS alignment with fixed DRX; (b) DRX alignment with fixed PRS.

During positioning, UE need to monitor serval neighbour cells’ PRS. It is hard to coordinate the DRX parameters, e.g. cycle, PF_offset, firstPDCCH-MonitoringOccasionOfPO, to cover all the PRS. In contrast, adjust these PRS to fall within the DRX is more easily. Additionally, the DRX configuration may relate to some other business requirement, that it is not suitable for adjustment.

In summary, make PRS align with fixed DRX is a better choice.

Proposal 7: RAN2 to agree PRS alignment with fixed DRX. The DRX refer to the potential PF and PO of the cell, which can be determined by the cell level parameters, e.g. PF_offset, N and minimum DRX cycle.

Considering the PRS need to align with neighbour cells DRX, the DRX parameters, e.g. PF_offset, of neighbour cells is needed. There are two directions:

- After obtaining neighbour cells’ DRX parameters via Xn message, the serving cell generate PRS configuration.

- RAN nodes and/or AMF send the DRX parameters to LMF. The PRS configuration is coordinated and recommended by LMF and final configured by RAN.

RAN2 can further discuss these two solutions.

Proposal 8: RAN2 to discuss the specific solution to enable PRS alignment with fixed DRX with RAN3 involved.

  - Discussion on DL positioning in RRC_IDLE

In RAN2#120, the DL positioning in RRC_IDLE state was discussed, and measurement performed in IDLE and reported in CONNECTED was contained in the WID. However, the power saving benefit need to be confirmed firstly.

Proposal3: DL positioning in RRC_IDLE is recommended to normative work from R2’s perspective if power saving benefits are confirmed by R1. (15/15)

	Measurement is performed in RRC_IDLE while measurement report is sent in RRC_CONNECTED

	Feasibility of measurement report in msg5 should be evaluated with SA2/3 involved.

	Whether the CN can handle the measurement reports from the UE in RRC_CONNECTED, while the positioning measurement was performed in RRC_IDLE, can be evaluated in the WI phase with SA2 involved.

For the issue of power saving benefits, RAN2 can only give a qualitative analysis. Compared with the inactive positioning as specific in Rel-17, this mechanism may have lower power consumption when the UE stay at RRC_IDLE state. But there may be higher power consumption bring by state transition and after the UE enter RRC_CONNECED state. Whether UE can have lower power consumption may depend on the positioning cycle, i.e. how long the UE need to enter RRC_CONNECTED state to perform measurement report.

By qualitative analysis, it is hard to say whether this mechanism will bring power saving benefits. Hence, a quantitative analysis is needed, and the required positioning cycle defined for use case 6 in TS 22.104 [6] should be taken into consideration. An LS is needed to be sent to RAN1 to request evaluation.

Proposal 9: RAN2 send an LS to RAN1 to confirm whether perform measurement in IDLE and reported in CONNECTED has power saving benefits. The LS can reference to the draft LS in Annex A.

Conclusion

According to the analysis in section 2, the following observations and proposals are provided:

Extending eDRX cycle beyond 10.24s in RRC_INACTIVE state

Proposal 1: For the objective of extend eDRX cycle beyond 10.24s in RRC_INACTIVE state, RAN2 can wait for the progress of eRedCap WI.

SRS configuration enhancements

#Issue 1: How to support SRS valid in multiple cells

Proposal 2: The parameter of valid area need to be configured to UE together with the SRS configuration. FFS the format e.g. defined as a list of cells or TAC.

Observation 1: If the SRS validity is defined only by valid area, the SRS resources may be occupied for a long time by the UE even there is no ongoing positioning service.

Proposal 3: A valid time can be configured together with the SRS configuration to recover the UL resource timely and improve the flexibility of network resource allocation.

#Issue 2: How to pre-configure one or multiple SRS for positioning configuration

Proposal 4: Take the following two ways of pre-configuration of SRS as start point.

- Option A: Pre-configure one or multiple UE-specific SRS via dedicate signalling, the SRS is valid in an area

- Option B: Pre-configure multiple common SRS via posSIB, the SRS is valid in an area

#Issue 3: How to support the positioning procedure, e.g. SRS activation/request procedure

Observation 2: In case of option A, The UE identity is implicitly indicated by the UE-specific SRS, i.e. it is possible for the network to identify the UE by detecting UE-specific SRS.

Observation 3: The NW maybe knows neither which cell the UE is camping on nor when the LCS event is triggered by UE. So gNB has to continually monitor the UE-specific SRS.

Proposal 5: Under the mechanism of pre-configure UE-specific SRS, the following solution can be discussed

- Before transmitting SRS, UE indicates RAN to monitor the SRS.

- After receiving the indication, RAN identifies the UE identity by monitoring the UE-specific SRS and perform SRS measurement.

Observation 4: UE only needs to send a dedicate indication and transmits SRS in proposal 5 and the power consumption reduction is enormously compared with legacy procedure.

Observation 5: UE request to obtain/update SRS via RACH-based procedure is recommended by RAN1 during SI.

Proposal 6: Under the mechanism of pre-configured multiple SRSs via posSIB, UE request to obtain/update SRS via RACH-based procedure. FFS the following solutions.

- UE request SRS from the NW via dedicate preamble.

- UE request SRS from the NW via Msg3, e.g. a new bit or a new resumeCause.

Alignment between PRS and (e)DRX

Observation 6: All potential PF and PO of a cell are determined by the cell level parameters, e.g. PF_offset, N and minimum DRX cycle (i.e., rf32), these potential PF and PO can cover all the monitoring time of the UEs in the cell.

Observation 7: By making alignment between the PRS and the potential PF and PO of the cell, the PRS/DRX alignment requirement for all the UEs in the cell can be met.

Proposal 7: RAN2 to agree PRS alignment with fixed DRX. The DRX refer to the potential PF and PO of the cell, which can be determined by the cell level parameters, e.g. PF_offset, N and minimum DRX cycle.

Proposal 8: RAN2 to discuss the specific solution to enable PRS alignment with fixed DRX with RAN3 involved.

Discussion on DL positioning in RRC_IDLE

Proposal 9: RAN2 send an LS to RAN1 to confirm whether perform measurement in IDLE and reported in CONNECTED has power saving benefits. The LS can reference to the draft LS in Annex A.

Reference

- RP-223549 New WID on Expanded and Improved NR Positioning.

- R2-2203949 LS on Positioning in RRC_INACTIVE State.

- 3GPP TS 38.331 Radio Resource Control (RRC) protocol specification.

- TS 38.304 User Equipment (UE) procedures in Idle mode and RRC Inactive state.

- TS 24.501 Non-Access-Stratum (NAS) protocol for 5G System (5GS).

- TS 22.104 Service requirements for cyber-physical control applications in vertical domains.

- Annex A

1. Overall Description:

In RAN#98-e, the new WID on Expanded and Improved NR Positioning was approved. One of the items of this work item is: Specify solutions for DL PRS measurements for a UE in RRC_IDLE state and reporting of the measurements in RRC_CONNECTED state [RAN2].

During the SI phase, this item was discussed, and the following agreement was achieved in RAN2#120. Shown as the yellow highlight part, RAN2 is not clear whether this mechanism has power saving benefits compared with the inactive DL positioning as specified in Rel-17.

Proposal3: DL positioning in RRC_IDLE is recommended to normative work from R2’s perspective if power saving benefits are confirmed by R1. (15/15)

	Measurement is performed in RRC_IDLE while measurement report is sent in RRC_CONNECTED

	Feasibility of measurement report in msg5 should be evaluated with SA2/3 involved.

	Whether the CN can handle the measurement reports from the UE in RRC_CONNECTED, while the positioning measurement was performed in RRC_IDLE, can be evaluated in the WI phase with SA2 involved.

RAN2 kindly ask RAN1 whether PRS measurements in RRC_IDLE and reporting measurements in RRC_CONNECTED can bring power saving benefits.

2. Actions:

To WG SA2:

ACTION: 	RAN2 kindly ask RAN1 to answer the above mentioned question.

3. Date of Next TSG-RAN2 Meetings:

3GPP TSG-RAN2 Meeting #121-bis-e	             17 - 26 April 2023	     	Online