3GPP TSG-RAN WG2 Meeting #121	                                               R2-2300205

Athens, Greece, February 27th – March 3rd 2023

Source:	CATT

Title:	Enhancements for Neighbor Cell Measurements

Agenda Item:	8.6.3.1

Document for:	Discussion and Decision

Introduction

In RAN2#120 meeting, the following agreements about enhancement for neighbor measurement for IoT-NTN were made:

Agreements:

- For NB-IoT we support a trigger for neighbour cell measurements based on T-service (in the quasi-Earth fixed case) (this does not preclude anything for eMTC discussion)

Agreements:

- At least for NB-IoT NTN, for quasi-earth fixed cells, UE shall start intra/inter frequency measurement in connected mode before the t-Service if present. The exact time to start measurements in connected mode before t-Service can be left to UE implementation” (can revisit if we agree other proposal based on neighbour cell coverage)

- RAN2 will not specify the condition of stopping UE measurement before t-Service

- For earth-moving cell, the UE derives when loss of coverage of current cell happens (how to derive this information is FFS)

- For earth-moving cell, UE shall start intra/inter frequency measurements in RRC connected mode before losing coverage. The exact time to start measurements can be left to UE implementation

As above shows, for earth-moving cell, how to derive the information of when loss of coverage of current cell is FFS, and for time-based mechanism, whether neighbor cell coverage should be taken into consideration is also open issue, this contribution will focus on these 2 issues.

Discussion

Earth-moving scenario

Solutions on coverage derivation

It was agreed that the UE needs to derive when loss of coverage of current cell happens for earth-moving cell.

As for how to derive the information for loss of cell coverage, the following information is necessary:

- the real-time position of the cell-center;

- the radius of the cell or a distance threshold to the cell-center;

- UE’s position.

If the UE could get aware of the information above, the UE could derive when loss of coverage of current cell happens.

Observation 1: UE could derive when loss of coverage of current cell happens combining the UE position and the cell covering information including: radius and the real-time cell center position.

In order to help the UE to derive the coverage information of current cell, some assistance information should be provided by the network.

Regarding the real-time cell center position, the network should provide assistance information for UE due to it is changing with time. For the radius of the cell, it is easy to provide to the UE since  it is static for one cell. Hence, the network can provide the radius or one distance threshold directly to the UE.

Proposal 1: NW should provide one distance threshold and assistance information which is used to derive real-time cell center position as the reference position to help UE derive when loss of coverage of current cell happens.

As for the assistance information used to derive the real-time cell center, the following solution could be taken into consideration:

- Solution 1: Network providing the coordinate of reference location with timestamp, the moving speed of cells can be derived by ephemeris information.

In Solution 1, the UE calculates the real-time reference location based on the information including reference location with timestamp and the moving speed. One example is shown in Figure 1, the NW provides the reference location of (x1,y1), and corresponding timestamp T1. And the moving speed of the cell as (Vx,Vy) can be derived by ephemeris information. UE could get the reference location at T2 as following figure shows:

Figure 1: Calculating the real-time reference location by coordinate with timestamp

- Solution 2: The network providing UE the relative coordinate of cells center, the relative coordinate is reference to the coordinate of sub-satellite point.

In Solution 2, the coordinate of sub-satellite point can be calculated by the ephemeris of satellite. UE could get the real-time center of the cell according to the sub-satellite point and the relative position to the sub-satellite point. As illustrated in Figure 2, the sub-satellite point is the vertical projection of the satellite on the ground, which can be calculated by the ephemeris. The NW provide the relative coordinate of cell1 center is (Δx1, Δy1), and the relative coordinate of cell 2 center is (Δx2, Δy2), the UE could calculate the real-time location of sub-satellite point to be (x0,y0), and calculate the cell center of cell1 and cell2 as following figure shows.

Figure 2: Relative coordinates reference to the coordinate of sub-satellite point.

- Solution 3: The network providing UE the beam antenna angle of cells center.

In Solution 3, the beam angle is reference to the direction of sub-satellite point which is calculated by the ephemeris of satellite. With the information of beam antenna angle, UE can calculate the real-time cell center. The UE can decide whether the CHO condition is satisfied or not by comparing configured threshold and the distance between UE and cell center. This is illustrated in Figure 3, the NW provide beam antenna angle of cell1 center as (∠α, ∠β). The UE could calculate the real-time location of the sub-satellite point (x0,y0) and the height of the satellite h by the ephemeris information, then the cell center of cell1 can be calculated, e.g. (x0+tanα*h*cosβ,y0+tanα*h*sinβ) as the following figure shows.

Figure 3: Beam antenna angle reference to the direction of sub-satellite point.

All above solutions could give the assistance information to UE to determine the cell center of one earth-moving NTN cell, additionally, the network could provide the radius or a threshold to assistant the UE to determine the cell edge and decide when the loss of the coverage of the cell could happens.

For Solution 1, the network may provide different assistance information to different UEs in order to reach more accurate calculation for UE, due to the cell moving, which brings complexity for network implementation comparing with solution 2 and solution 3.  For solution 2 and solution 3 the assistance information could be unified no matter where the cell covers.

For Solution 2 and 3, the relative coordinate reference to sub-satellite point or the beam antenna angle is unchanged for earth moving cell. There is no need to update this relative information frequently. These two options make full use of the time information contained in the ephemeris, which is already supported in Rel-17.

Hence, we have a preference on Solution 2 and 3 and propose that:

Proposal 2: RAN2 to discuss the following two solutions on how to determine the real-time cell center of earth-moving cell:

- providing UE the relative coordinate of cells center, the relative coordinate is reference to the coordinate of sub-satellite point

- providing UE the beam antenna angle of cells center.

Suggestion on discussion on earth-moving cell

Considering currently, the topic about earth-moving cell is discussed under the following scenarios:

- enhancement on triggering neighbor cell measurement for connected NB-Iot NTN UE

- CHO enhancement for eMTC NTN UE

- CHO enhancement for NR NTN UE

- Triggering neighbor cell measurement for NR NTN idle/inactive UE

Considering the intention of all of the above scenarios is to help the UE derive the earth-moving cell coverage information, no matter it is used for CHO or triggering of neighbor measurement, the mains issue is on how to derive the real-time cell center position for earth-moving cell. So we suggest one unified solution could be adopted to apply to all above earth-moving scenarios. The discussion about this topic can be centralized in one agenda instead of discussing in separate agenda.

Proposal 3: To adopt one unified solution for deriving real-time cell center position of earth-moving cell to apply to following scenarios:

- enhancement on triggering neighbor cell measurement for connected NB-Iot NTN UE

- CHO enhancement for eMTC NTN UE

- CHO enhancement for NR NTN UE

- Triggering neighbor cell measurement for NR NTN idle/inactive UE

Proposal 4: The discussion on deriving real-time cell center position of earth-moving cell should be centralized in one agenda instead of discussing in separate agenda.

Neighbor cell coverage for triggering neighbor cell measurement

Even though it was agreed for quasi-earth fixed cells, UE shall start intra/inter frequency measurement in connected mode before the t-Service if present, it was still FFS on whether neighbor cell coverage should be taken into consideration. Furthermore, it also agreed, for earth-moving cells, UE shall start intra/inter frequency measurements in RRC connected mode before losing coverage. Even though the details on how to decide the time when loss coverage of current cell is different, but the theory on triggering neighbor cell measurement is same for quasi-earth fixed cells and earth-moving cells, i.e. the UE shall start intra/inter frequency measurements in RRC connected mode before losing coverage. As for whether taking the neighbor cell coverage into consideration for triggering neighbor cell measurement, the same solution could be used for quasi-earth fixed cells and earth-moving cell.

Proposal 5: Same solution should be used for quasi-earth fixed cells and earth-moving cell on the decision of whether triggering intra/inter frequency measurement in connected mode could base on neighbor cell coverage.

Considering discontinues coverage prediction is supported in R17 IoT NTN, so the UE could be aware of when the incoming satellite will cover the area where the UE locates. Before the satellite covers the area, the UE couldn’t receive any signaling from the satellite. So it is not needed to trigger the measurement on the neighbor cell too early before the cell covers the area of the UE located which will lead in unnecessary power consumption too, i.e. the neighbor cell measurement should be triggered after the neighbor cell covers the area where UE located. In order to avoid extra power consumption, too early neighbor cell measurement should be avoided.

Observation 2: It is not necessary to trigger the neighbor cell measurement before the neighbor cell covers the area where the UE locates.

It was agreed the UE shall start intra/inter frequency measurement in connected mode before losing coverage. Take the neighbor cell coverage into consideration, the UE shall also start intra/inter frequency measurement in connected mode after coverage of the neighbor cells. Combine the two condition, we propose that the connected UE should trigger the neighbor cell measurement before the end of losing coverage and after the starting serving time of the neighbor cell which is late arrival.

Proposal 6: For NB-IoT UE, the UE shall start intra/inter frequency measurement in connected mode:

- before losing coverage, or

- after the start coverage of the neighbor cell, if configured in SIB32-NB

If both configured, the late time is selected.

Conclusion

In this contribution, we give some discussion on neighbour cell measurement enhancements for IoT-NTN, the following proposal and observations are given:

Earth-moving scenario

Observation 1: UE could derive when loss of coverage of current cell happens combining the UE position and the cell covering information including: radius and the real-time cell center position

Proposal 1: NW should provide one distance threshold and assistance information which is used to derive real-time cell center position as the reference position to help UE derive when loss of coverage of current cell happens.

Proposal 2: RAN2 to discuss the following two solutions on how to determine the real-time cell center of earth-moving cell:

- providing UE the relative coordinate of cells center, the relative coordinate is reference to the coordinate of sub-satellite point

- providing UE the beam antenna angle of cells center.

Proposal 3: To adopt one unified solution for deriving real-time cell center position of earth-moving cell to apply to following scenarios:

- enhancement on triggering neighbor cell measurement for connected NB-Iot NTN UE

- CHO enhancement for eMTC NTN UE

- CHO enhancement for NR NTN UE

- Triggering neighbor cell measurement for NR NTN idle/inactive UE

Proposal 4: The discussion on deriving real-time cell center position of earth-moving cell should be centralized in one agenda instead of discussing in separate agenda.

Neighbor cell coverage for triggering neighbor cell measurement

Observation 2: It is not necessary to trigger the neighbor cell measurement before the neighbor cell covers the area where the UE locates.

Proposal 5: Same solution should be used for quasi-earth fixed cells and earth-moving cell on the decision of whether triggering intra/inter frequency measurement in connected mode could base on neighbor cell coverage.

Proposal 6: For NB-IoT UE, the UE shall start intra/inter frequency measurement in connected mode:

- before losing coverage, or

- after the start coverage of the neighbor cell, if configured in SIB32-NB

If both configured, the late time is selected.

Reference

- R2_120_meeting_report_v1