3GPP TSG-RAN WG2 Meeting #121                                                                    R2-2300207

Athens, Greece, 27th February – 3th March, 2023

Agenda Item:	8.7.3

Source:	CATT

Title:	Discussion on Network Verified UE Location

Document for:	Discussion and Decision

1. Introduction

In RAN#98-e, the new WID on NR NTN enhancements [1] was approved. On the topic of network verified UE location, the objectives of this work item is:

4.1.3 Network verified UE location Based on RAN1 conclusions of the study phase, RAN to prioritize the specification of necessary enhancements to multi-RTT to support the network verified UE location in NTN assuming a single satellite in view [RAN1, 2, 3, 4]. DL-TDoA methods for verification may be considered as lower priority and if time permits and condition in Note is satisfied. Note 1: Enhancements assume reuse of the RAT dependent positioning framework Note 2: The specification of DL-TDOA enhancements will be subject to the study of the impact of realistic UE clock drift onto DL-TDOA performance Note 3: The target accuracy for position verification purposes is as documented in clause « recommendations » of the 3GPP TR 38.882 (i.e. 10 km granularity) Note 4 : Multiple satellite in view by the UE may be considered if time allows Note 5 : The enhancements may be subject to relevant SA WGs (e.g. SA3/SA3-LI) feedbacks on the reliability of UE reports involved Note 6 : The enhancements should take into account the mirror-image ambiguity Note 7 : Network verified UE location is an optional UE feature

In this contribution, the scenario for positioning with single LEO satellite and the positioning procedure of Multi-RTT will be analysed and discussed.

2. Discussion

2.1 Scenario analysis

For RAT-dependent positioning method applied to TN, the position is calculated by the measurements from multiple TRPs at different positions. However, in case of positioning with single LEO satellite, there is only one TRP which can only be deployed on the satellite. The legacy measurement method is not applicable to this case. To solve this issue, one way is to make the LEO satellite perform measurement multiple times at different time points as different positions. This is shown as Figure 1, which takes advantage of the movement characteristic of the satellite.

Figure 1. RAT-dependent positioning supported by single LEO satellite

However, the reference signal (i.e., PRS or SRS), measurement request, etc. are configured for a single measurement for one positioning procedure according to the legacy mechanism. This is not suitable for the positioning with single LEO satellite. It is necessary to discuss how to support measurement at multiple time points in case of positioning with single LEO satellite in one positioning procedure.

Observation 1: In case of positioning with single LEO satellite, how to support measurement at multiple time points for one positioning procedure need to be discussed, considering in legacy mechanism the measurement is request at one time point for the current positioning procedure.

2.2 Solutions

As reference, the legacy Multi-RTT positioning procedure specified in TS 38.305 [2] is shown as follow.

Figure 8.10.4-1: Multi-RTT positioning procedure 0. The LMF may use the procedure in Figure 8.10.3.2.1-1 to obtain the TRP information required for Multi-RTT positioning. 1. The LMF may request the positioning capabilities of the target device using the LPP Capability Transfer procedure described in clause 8.10.3.1.1. 2. The LMF sends a NRPPa POSITIONING INFORMATION REQUEST message to the serving gNB to request UL information for the target device as described in Figure 8.10.3.2.1-2. 3. The serving gNB determines the resources available for UL-SRS and configures the target device with the UL-SRS resource sets at step 3a. 4. The serving gNB provides the UL-SRS configuration information to the LMF in a NRPPa POSITIONING INFORMATION RESPONSE message. NOTE: It is up to implementation on whether SRS configuration is provided earlier than DL-PRS configuration. 5. In the case of semi-persistent or aperiodic SRS, the LMF may request activation of UE SRS transmission by sending a NRPPa Positioning Activation Request message to the serving gNB of the target device as described in clause 8.10.3.2.3. The gNB then activates the UE SRS transmission and sends a NRPPa Positioning Activation Response message. The target device begins the UL-SRS transmission according to the time domain behavior of UL-SRS resource configuration. 6. The LMF provides the UL information to the selected gNBs in a NRPPa MEASUREMENT REQUEST message as described in clause 8.10.3.2.2. The message includes all information required to enable the gNBs/TRPs to perform the UL measurements. 7. The LMF sends a LPP Provide Assistance Data message to the target device as described in clause 8.10.3.1.2.1. The message includes any required assistance data for the target device to perform the necessary DL-PRS measurements. 8. The LMF sends a LPP Request Location Information message to request Multi-RTT measurements. 9a: The target device performs the DL-PRS measurements from all gNBs provided in the assistance data at step 7. 9b: Each gNB configured at step 6 measures the UE SRS transmissions from the target device. 10. The target device reports the DL-PRS measurements for Multi-RTT to the LMF in a LPP Provide Location Information message. 11. Each gNB reports the UE SRS measurements to the LMF in a NRPPa Measurement Response message as described in clause 8.10.3.2.2. 12. The LMF sends a NRPPa POSITIONING DEACTIVATION message to the serving gNB as described in clause 8.10.3.2.3. 13. The LMF determines the RTTs from the UE and gNB Rx-Tx time difference measurements for each gNB for which corresponding UL and DL measurements were provided at steps 10 and 11 and calculates the position of the target device.

In this section, some issues for the positioning supported by single LEO satellite will be discussed.

- Entity to determine the time points for performing measurement

For positioning supported by single LEO satellite, to support measurements at multiple time points for one positioning procedure, we need to discuss which entity to determine these multiple time points.

In case of location verification, the positioning request is raised by AMF and sent to LMF, which may contain a positioning requirement, e.g. accuracy or the time limitation. With the positioning requirement, LMF can determine the measurement requirement, e.g. to achieve the positioning accuracy of 5km, the network need to perform 5 times of measurement and the interval between two times of measurement need up to 10s. Then, there are two options to determine the exactly measurement time points:

- Option1: The measurement time points are chosen by LMF;

- Option2: The measurement time points are chosen by RAN;

Considering the interaction between RAN and LMF belong to RAN3 scope, this issue can be left to RAN3.

Proposal 1: For the issue of which entity to make decision on the measurement time points is left to RAN3.

- How to support measurement at multiple time points for one positioning procedure

In case of positioning with single LEO satellite, to support measure at multiple time points for one positioning procedure, there are two options:

- Option 1: The network triggers the measurement timely at each selected time point.

- Option 2: The network triggers the measurement once, and contains the configuration of multiple time points.

For option 1, the network could send the measurement configuration at each determined time point to trigger the UE to perform measurement accordingly. The network can configure the reference signal and measurement more flexible than option 2, especially when handover occurs during the positioning procedure. For example, by option 1, after cell changes, the network can configure SRS for UE in the new cell again. While by option 2, coordination between cells and LMF is needed.

For option 2, the network sends the measurement configuration for multiple time points to indicate when the UE shall perform measurement. The positioning procedure can be performed with less signalling overhead than option 1 since the configuration of reference signal and measurements at multiple time points can be sent to UE in one configuration procedure.

With these pros and cons, RAN2 to further discusses these two options.

Proposal 2: RAN2 to discuss the following two methods for supporting measurement at multiple time points for one positioning procedure.

- Option 1: The network triggers the measurement timely at each selected time point. ;

- Option 2: The network triggers the measurement once, and contains the configuration of multiple time points.

3. Conclusion

In this contribution, we initially discussed the UE location verification for NR NTN. Based on the discussion, we provided the following observations and proposals:

Scenario analysis

Observation 1: In case of positioning with single LEO satellite, how to support measurement at multiple time points for one positioning procedure need to be discussed, considering in legacy mechanism the measurement is request at one time point for the current positioning procedure.

Solutions

Proposal 1: For the issue of which entity to make decision on the measurement time points is left to RAN3.

Proposal 2: RAN2 to discuss the following two methods for supporting measurement at multiple time points for one positioning procedure.

- Option 1: The network triggers the measurement timely at each selected time point. ;

- Option 2: The network triggers the measurement once, and contains the configuration of multiple time points.

4. References

- RP-223534 Revised WID: NR NTN (Non-Terrestrial Networks) enhancements.

- TS 38.805 Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN.