3GPP TSG-RAN WG2 Meeting #121	R2-2300216

# Athens, Greece, 27th February – 3rd March 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 3787 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to Application layer measurement reporting procedure and alignment of terminologies |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_QoE-Core |  |  |  |  |  | Date: |  |  | 2023-02-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Clause 5.7.16.2: There is an issue in the current description of the application layer measurement reporting procedure. Currently, the mandatory present field measConfigAppLayerId in the MeasurementReportAppLayer message is only set to the value of the measConfigAppLayerId received together with application layer measurement report information (i.e. encapsulated QoE measurement report). However, there may be cases in which the UE initiates the reporting procedure to send RAN-visible QoE report(s) and/or session start/stop information. In those cases it is not specified to which value the measConfigAppLayerId should be set by the UE. - In the RAN3 reply LS R3-226778 (=R2-2300030), RAN3 clarified the motivation for specifying that both RAN visible QoE reports and the encapsulated QoE reports shall be sent by the UE in the same MeasurementReportAppLayer message when the RAN visible QoE reporting periodicity is not explicitly configured, except in the case of RAN overload. This requirement has been already specified in stage 2 but is missing in stage 3. - In stage 3 the following terminologies are used for QoE measurement reports: - The term “application layer measurement report” is used to refer to an “application layer measurement report container” and “RAN visible application layer measurement report”. - The term “application layer measurement report container” is used to refer to an encapsulated QoE measurement report - The term “RAN visible application layer measurement report” is used to refer to a RAN-visible QoE measurement report. However, in clause 5.7.16.2 and in the field description of measReportAppLayerContainer the incorrect term “application layer measurement report” is used with regards to reporting of an encapsulated QoE measurement report. Instead, it should say “application layer measurement report container”. - In clause 6.2.2 the field description of ran-VisibleMeasurements is incorrect since the field contains a RAN visible application layer measurement report and not the configuration of RAN visible application layer measurement parameters. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In clause 5.7.16.2 the condition for setting measConfigAppLayerId in the MeasurementReportAppLayer message has been corrected as follows: 2> set the measConfigAppLayerId in the MeasurementReportAppLayer message to the value of the measConfigAppLayerId received from upper layers; - In clause 5.7.16.2 a new note has been added to capture the requirement for the UE to send both RAN visible QoE reports and the encapsulated QoE reports in the same MeasurementReportAppLayer message when the RAN visible QoE reporting periodicity is not explicitly configured, except in the case of RAN overload. - In clause 5.7.16.2 and in the field description of measReportAppLayerContainer the incorrect term “application layer measurement report” has been replaced by “application layer measurement report container”. - In clause 6.2.2 the field description of ran-VisibleMeasurements has been corrected as follows: The field contains the RAN visible application layer measurement report. Impact analysis Impacted 5G architecture options: NR SA, NE-DC, NR-DC Impacted functionality: Application layer measurement reporting Inter-operability: - There are no interoperability issues. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - To change 1): The UE does not know how to set the measConfigAppLayerId when only RAN-visible QoE report(s) and/or session start/stop information are sent in the MeasurementReportAppLayer message. - To change 2): The requirement for the UE to send both RAN visible QoE reports and the encapsulated QoE reports in the same MeasurementReportAppLayer message when the RAN visible QoE reporting periodicity is not explicitly configured, except in the case of RAN overload, is missing in stage 3. - To change 3): Incorrect term is used for reporting of encapsulated QoE measurement reports. - To change 4): Incorrect field description of ran-VisibleMeasurements is specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7.16.2, 6.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of changes

### 5.7.16	Application layer measurement reporting

#### 5.7.16.1	General

Figure 5.7.16.1-1: Application layer measurement reporting

The purpose of this procedure is to send application layer measurement reports to the network.

#### 5.7.16.2	Initiation

A UE capable of application layer measurement reporting in RRC_CONNECTED may initiate the procedure when configured with application layer measurement, i.e. when appLayerMeasConfig and SRB4 have been configured by the network.

Upon initiating the procedure, the UE shall:

1>	for each measConfigAppLayerId received from upper layers:

2>	if the UE AS has received application layer measurement report container from upper layers which has not been transmitted; and

2>	if the application layer measurement reporting has not been suspended for the measConfigAppLayerId associated with the application layer measurement report container according to clause 5.3.5.13d:

3>	set the measReportAppLayerContainer in the MeasurementReportAppLayer message to the received value in the application layer measurement report container;

2>	if session start or stop information has been received from upper layers for the measConfigAppLayerId:

3>	set the appLayerSessionStatus in the MeasurementReportAppLayer message to the received value of session start or stop information;

2>	if RAN visible application layer measurement report has been received from upper layers:

3>	for each appLayerBufferLevel value in the received RAN visible application layer measurement report:

4>	set the appLayerBufferLevel values in the appLayerBufferLevelList in the MeasurementReportAppLayer message to the buffer level values received from the upper layer in the order with the first appLayerBufferLevel value set to the newest received buffer level value, the second appLayerBufferLevel value set to the second newest received buffer level value, and so on until all the buffer level values received from the upper layer have been assigned or the maximum number of values have been set according to appLayerBufferLevel, if configured;

3>	set the playoutDelayForMediaStartup in the MeasurementReportAppLayer message to the received value of playout delay for media startup in the RAN visible application layer measurement report, if any;

3>	for each PDU session ID value indicated in the received RAN visible application layer measurement report, if any:

4>	set the PDU-SessionID field in the pdu-SessionIdList in the MeasurementReportAppLayer message to the indicated PDU session ID value;

2>	set the measConfigAppLayerId in the MeasurementReportAppLayer message to the value of the measConfigAppLayerId received from upper layers;

1>	if the encoded RRC message is larger than the maximum supported size of one PDCP SDU specified in TS 38.323 [5]:

2>	if the RRC message segmentation is enabled based on the field rrc-SegAllowed received in appLayerMeasConfig:

3>	initiate the UL message segment transfer procedure as specified in clause 5.7.7;

2>	else:

3>	discard the RRC message;

1>	else:

2>	submit the MeasurementReportAppLayer message to lower layers for transmission upon which the procedure ends.

NOTE:	If the UE AS has received from upper layers both RAN visible application layer measurement report and application layer measurement report container for a measConfigAppLayerId, and application layer measurement reporting has not been suspended for the measConfigAppLayerId, then the UE sends both application layer measurement reports in the same MeasurementReportAppLayer message when ran-VisiblePeriodicity has not been explicitly configured.

Next change

### 6.2.2	Message definitions

<Text omitted>

#### –	MeasurementReportAppLayer

The MeasurementReportAppLayer message is used for sending application layer measurement report.

Signalling radio bearer: SRB4

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

MeasurementReportAppLayer message

-- ASN1START

-- TAG-MEASUREMENTREPORTAPPLAYER-START

MeasurementReportAppLayer-r17 ::=     SEQUENCE {

criticalExtensions                    CHOICE {

measurementReportAppLayer-r17     MeasurementReportAppLayer-r17-IEs,

criticalExtensionsFuture              SEQUENCE {}

}

}

MeasurementReportAppLayer-r17-IEs ::=   SEQUENCE {

measurementReportAppLayerList-r17       MeasurementReportAppLayerList-r17,

lateNonCriticalExtension                OCTET STRING                                                           OPTIONAL,

nonCriticalExtension                    SEQUENCE{}                                                             OPTIONAL

}

MeasurementReportAppLayerList-r17 ::= SEQUENCE (SIZE (1..maxNrofAppLayerMeas-r17)) OF MeasReportAppLayer-r17

MeasReportAppLayer-r17 ::=        SEQUENCE {

measConfigAppLayerId-r17              MeasConfigAppLayerId-r17,

measReportAppLayerContainer-r17       OCTET STRING                                                             OPTIONAL,

appLayerSessionStatus-r17             ENUMERATED {started, stopped}                                            OPTIONAL,

ran-VisibleMeasurements-r17           RAN-VisibleMeasurements-r17                                              OPTIONAL

}

RAN-VisibleMeasurements-r17 ::=       SEQUENCE {

appLayerBufferLevelList-r17           SEQUENCE (SIZE (1..8)) OF AppLayerBufferLevel-r17                        OPTIONAL,

playoutDelayForMediaStartup-r17       INTEGER (0..30000)                                                       OPTIONAL,

pdu-SessionIdList-r17                 SEQUENCE (SIZE (1..maxNrofPDU-Sessions-r17)) OF PDU-SessionID            OPTIONAL,

...

}

AppLayerBufferLevel-r17 ::= INTEGER (0..30000)

-- TAG-MEASUREMENTREPORTAPPLAYER-STOP

-- ASN1STOP

MeasReportAppLayer field descriptions

appLayerSessionStatus Indicates that an application layer measurement session in the application layer starts or ends.

measReportAppLayerContainer The field contains the application layer measurement report container, see Annex L (normative) in TS 26.247 [68], clause 16.5 in TS 26.114 [69] and TS 26.118 [70].

ran-VisibleMeasurements The field contains the RAN visible application layer measurement report.

RAN-VisibleMeasurements field descriptions

appLayerBufferLevelList The field indicates a list of application layer buffer levels, and each AppLayerBufferLevel indicates the application layer buffer level in ms. Value 0 corresponds to 0ms, value 1 corresponds to 10ms, value 2 corresponds to 20 ms and so on. If the buffer level is larger than the maximum value of 30000 (5 minutes), the UE reports 30000.

playoutDelayForMediaStartup Indicates the application layer playout delay for media start-up in ms. Value 0 corresponds to 0ms, value 1 corresponds to 1ms, value 2 corresponds to 2 ms and so on. If the playout delay for media start-up is larger than the maximum value of 30000ms, the UE reports 30000.

pdu-SessionIdList Contains the identity of the PDU session, or the identities of the PDU sessions, used for application data flows subject to the RAN visible application layer measurements.

End of changes