3GPP TSG-RAN WG2 Meeting #121                                                        R2-2300222

Athens, Greece, 27th Feb– 3rd Mar, 2023

Source:              CATT

Title:	             PDU set and data burst information

Agenda Item:	8.5.2.1

Document for:	Discussion and Decision

# Introduction

This contribution addresses the latest agreements from SA2#154 Ad Hoc on the PDU set and data burst information provided by CN to RAN and discusses:

- the associated UL parameters, including whether jitter is applicable to XR traffic in UL

- the related impacts on RAN

# Discussion

# DL parameters summary and UL counterparts

After SA2 concluded the SI on XR [1] the following DL parameters are supported for characterizing a DL flow carrying XR traffic:

Semi-static configuration via control plane (AF/SMF) at QoS flow level:

- PDU related: legacy 5QI (FFS if new 5QI(s) to be defined in normative phase), Flow Bit Rates, Packet Loss Rate, etc

- Burst related (in TSCAI):

  - Periodicity (including non-integer values)

  - Traffic jitter

- PDU Set related (in NGAP):

  - PSDB

  - PSER

  - PSIHI

Dynamic configuration via user plane (GTP-U header):

- Burst related: End of Data Burst indication in the header of the last PDU of the Data Burst

- PDU Set related:

  - PDU Set Sequence Number

  - End PDU of the PDU Set

  - PDU SN within a PDU Set

  - PDU Set Size in bytes

  - PDU Set Importance

Regarding the UL flows, SA2 concluded the following note in [1]:

NOTE 1: Further PDU Set handling for Uplink will be studied and led by RAN WGs. SA WG2 can align with RAN's progress and decision for Uplink, if any.

We discuss the impact of the above parameters in RAN in Section 2.2, but considering the RAN2 agreements achieved during the SI (copied below) it seems obvious that the same set of parameters will be needed in UL to allow RAN providing the same QoS treatments as in DL.

1: From RAN2 viewpoint, the following information would be useful for PDU set handling in UL and DL: Semi-static information (from CN to RAN): At least PSER and PSDB. Dynamic information: At least identifying which PDU belongs to which data burst/PDU set is also needed, including means to determine at least PDU set boundaries.

Proposal 1: The same set of semi-static and dynamic XR QoS parameters shall be supported for UL XR QoS flows and PDU Sets as for DL XR QoS flows and PDU Sets.

Then, how UE identifies the UL PDU Sets and bursts can be left to UE implementation or can be based on instruction from SMF (as for DL). We suggest reporting these RAN requirements for UL QoS parameters and user plane identification to SA2 and consult them on the feasibility of the latter option.

Proposal 2: RAN2 consults SA2 on the feasibility for SMF to instruct UEs how to identify UL PDU Sets and bursts.

Similarly, for control plane semi-static parameters, the question is whether they should be determined by UE implementation or provided by CN. It must be noted that SA2 already concluded that the XR burst periodicity is provided at PDU Session Establishment/Modification via an NGAP Message (by re-using/extending the TSCAI/TSCAC) for both DL and UL QoS flows [1].

Observation 1: SA2 already concluded that the XR burst periodicity of a QoS flow is provided at QoS flow level in TSCAI/TSCAC for both DL and UL QoS flows.

The jitter information associated with each above periodicity is also provided in the same way but for DL bursts only. Then for the UL jitter, in the last RAN2 meeting it was not clear to companies whether the XR traffic is expected to experience some jitter on bursts arrival times in UE buffer in UL as well. Therefore, it has become an official question that the Chair added to this agenda item:

Including discussion on whether jitter is applicable to XR traffic in UL

The jitter issue was also discussed in a reply LS from SA4 to SA2 [2]:

Q4: SA2 discussed the possibility to receive a jitter range associated with Data Burst periodicity by AF/AS to 5GS. SA2 would like SA4 whether it is feasible for the AF/AS to provide such jitter range to 5GS.

SA4 response:

-        Assuming the jitter as being the variance on the release of the Coded Picture Buffer or the RTP output buffer

o   the application server cannot recognize the jitter range happened in the intermediate path between the application server and the UE within 5GS. The application server can only recognize the jitter range at the server side.

o   Such a jitter may vary during operation depending on cloud encoder load, content complexity, and many other factors. Obviously, it will be the attempt of a good video encoder to minimize the delay of encoding for each frame and make sure that real-time encoding can be maintained. However, SA4 does neither have a study nor any requirements on the performance of video encoders in real-time XR services.

Clearly the jitter being discussed by SA4 above is the jitter at the encoder output or RTP egress buffer of the application and does not consider any travel path on any network interface. In other words this is the jitter of an XR (video) flow at the UE input. Which confirms jitter is applicable to XR traffic in UL. Moreover, to be more quantitative, published studies [3] have categorized the range of such jitter on known encoders implemented on a given HW and showing, for example, a jitter on encoded frames in the range 5ms for an NVIDIA NVENC H.264 encoder. This is further evidence that UL jitter exists for frames arrival at the UE input.

Proposal 3: Same as in DL, jitter is applicable to XR traffic bursts in UL.

The next question is: which entity provides this parameter?

Option 1: Same as for DL, the UL jitter is provided by UPF to SMF, and SMF can refine the UL jitter by deleting the estimated jitter between the UPF and RAN. But estimating the jitter between UPF and RAN is tricky and removing it from the total jitter at UPF is even more tricky.

Option 2: The AS/AF outside of the N6 interface knows which encoder is used by the UE and could derive what is the expected jitter from such encoder. However, this would require that the AS/AF knows the exact complete encoder configuration (many parameters for a given encoding norm), and an encoder performance may also vary from a hardware implementation to another. In addition, this solution is based on a technical assumption beyond 3GPP.

Option 3: If the UE is able to identify the packet at the end of a burst (similar to CN for DL), then the UE could measure the inter-frame inter-arrival (IFI) at its input itself, and perform jitter statistics measurements. Then UE reports this to gNB as UAI. This approach is the simplest because it measures the jitter directly. We suggest taking this solution as baseline.

Proposal 4: The jitter on arrival times of UL XR video bursts is measured by the UE and reported to gNB as UAI.

Then, for the PDU Set semi-static parameters, i.e. PSDB, PSER, PSIHI, similar to the identification criteria of the UL XR PDU Sets and bursts, it can be left to UE implementation or can be provided by SMF (as for DL). It can be noted, for example, that regarding PSDB, [1] captures the following: PSDB applies to the DL PDU Set received by the UPF over the N6 interface, and to the UL PDU Set sent by the UE. For a certain 5QI the value of the PSDB is the same in UL and DL. Therefore, in its simplest option, the current DL parameter could also apply to UL PDU Sets of the associated UL QoS flow. Anyways, we suggest expressing RAN2’s need for these same UL PDU Set parameters to SA2 and consult them on the feasibility of CN providing them, as in DL.

Proposal 5: RAN2 consults SA2 on the feasibility for CN to provide the parameters PSDB, PSER, PSIHI, for UL PDU Sets, as for DL PDU Sets.

# How to use the burst and PDU Set parameters in RAN?

# Burst parameters

XR burst level new parameters are the periodicities, the jitter and the end of burst indication. The periodicities and associated jitter can be used by gNB for configuring e.g. DRX, CGs and SPS configurations. So the impact is reduced to gNB implementation and should not have any additional specification impact on top of those already being discussed in RAN e.g. non-integer DRX periodicity support and enhanced configured grants (RAN1).

Proposal 6: The new bursts level parameters (periodicity, jitter and end of burst indication), can be used by gNB for configuring e.g. DRX, CG and SPS configurations. No further specification impact is foreseen.

# PDU set parameters

#### PSDB, PSIHI and importance

We discuss together these three parameters because they all impact the discarding of PDU Sets. Their definition is given below, copied from [1]:

PDU Set Delay Budget: The PSDB defines an upper bound for the delay that a PDU Set may experience for the transfer between the UE and the N6 termination point at the UPF, i.e. time between reception of the first PDU and the successful delivery of the last arrived PDU of a PDU Set. PSDB applies to the DL PDU Set received by the UPF over the N6 interface, and to the UL PDU Set sent by the UE. For a certain 5QI the value of the PSDB is the same in UL and DL.

PSIHI: Whether all PDUs are needed for the usage of PDU Set by application layer (PDU Set Integrated Indication).

PDU Set Importance: This parameter is used to identify the importance of a PDU Set within a QoS flow. RAN may use it for PDU Set level packet discarding in presence of congestion.

In addition, in their reply LS on PDU set handling [4], SA2 clarified the following:

SA2 also agrees to define PDU Set importance that is conveyed on per-PDU Set basis. All the PDU Sets within one QoS flow should apply the same PSER, PSDB and PSIHI. The PDU Set importance of the different PDU Sets within one QoS flow can be different

A first question is whether PSDB, defined at PDU Set level, can coexist with the legacy 5QI’s PDB, at PDU level. No such limitation was mentioned by SA2 so far, but we understand this is being discussed in SA2. In our contribution in AI 8.5.2.3 discussing PDU discard [5], we propose explicitly asking SA2 about it, but in the rest of this document, we assume, as a baseline, that they can coexist.

RAN2 started discussing the impact of PDSB on RAN and concluded some preliminary discard timer agreements:

- RAN2#119bis: 1. For UE transmitter, the PDCP discard should be performed per PDU set basis. 2. For UE transmitter, The PDCP discard is managed per SDU for PDU set, the PDCP entity discards all PDCP SDUs associated with the PDU set. - RAN2#120: RAN2 to support timer-based discarding of UL transmit side of PDCP PDU/SDUs of a PDU set. FFS how this is modelled in PDCP specification, can be discussed in WI phase.

However, in their reply LS on PDU Set handling [8], SA4 wrote:

With regards to the PSDB, the SA4 assumes the PDU Set reception will happen within the PSDB target. However, the delivery of late PDU Sets may still be useful in some cases.

We understand the above as PSDB could be configured to the UE for other purpose but the PDCP discard timer. And indeed, it can be only used by the UE to compute the remaining time for a PDU Set and report it to gNB (already agreed to support in RAN2) to assist UL scheduler. To distinguish both cases, we require another configuration parameter controlling the PDCP PDU Set discard timer for a given UL flow e.g. discardOutdatedPDU-Set. RAN would discard the PDU Sets exceeding the PSDB only when discardOutdatedPDU-Set is set. The same applies to DL.

Proposal 7: RAN2 expresses the need for a new parameter, discardOutdatedPDU-Set, to SA2/SA4, to control whether to discard or not PDU Sets exceeding the PSDB.

Now, regarding the PDU Set importance, SA2 suggest that RAN may use it for PDU Set level packet discarding in presence of congestion. In our view, the simplest interpretation of this requirement is that the above PDCP discarding of PDU Sets would apply selectively on some PDU Sets only. That is, in presence of congestion, it is expected that most PDU Sets exceed their PSDB, however, discarding the PDU Sets of lower importance would be sufficient to release the congestion. In order to do that, a threshold on the PDU set importance, PDUsetImportanceDiscardThreshold, is needed to allow the UE to perform the selective discarding of PDU Sets.

Proposal 8: RAN2 expresses the need for a new parameter, PDUsetImportanceDiscardThreshold, to SA2, which sets the importance value below which a UE (/gNB) discards an UL (/DL) PDU Set that exceeded the PSDB, if discardOutdatedPDU-Set is configured.

Finally, the PSIHI also controls the PDU Sets discarding, but, while PSDB is only about delay budget, PSIHI is more generic and also covers PDUs lost for other reasons, e.g. max number of HARQ ReTx reached in RLC UM, or exceeded PDB.

Since PDCP discard in DL is fully under gNB control/implementation, all above has no impact on RAN specification in DL. So we can focus on the UL. To summarize, we generated the below table capturing the UE behavior on UL PDUs and PDU Sets based on the various parameters configuration.

Table 1: UE behaviour on UL PDUs and PDU Sets based on PSDB, PSIHI, and importance parameters

|  | PSDB not configured | PSDB configured |
| --- | --- | --- |
| PSIHI = 0 | Legacy PDCP discard timer applies at PDU level | New PDCP discard timer applies at PDU set level. If discardOutdatedPDU-Set is configured, all PDUs of a PDU Set are discarded whenever the above PDCP timer expires and the PDU Set importance is lower than PDUsetImportanceDiscardThreshold. Individual PDU PDCP timer operates as in legacy, if configured. |
| PSIHI = 1 | All PDUs of a PDU set are discarded whenever: - Any PDU is lost due to max nb of HARQ ReTx reached in RLC UM - The (individual legacy) PDCP timer of any PDU expires | New PDCP discard timer applies at PDU set level. If discardOutdatedPDU-Set is configured, all PDUs of a PDU set are discarded whenever the above PDCP timer expires and the PDU Set importance is lower than PDUsetImportanceDiscardThreshold. Individual PDU PDCP timer operates as in legacy, if configured. In addition to the above PDU-set PDCP timer, all PDUs of a PDU set are discarded whenever any PDU is lost due to either max nb of HARQ ReTx reached in RLC UM or the (individual legacy) PDCP timer of any PDU expires, if configured. |

Proposal 9: For DL QoS flows, PSDB, PSIHI and importance parameters impact the gNB implementation but have no specification impact.

Proposal 10: Capture in the TR the above UE behavior table on UL PDU Set discarding.

Further considering the importance parameter, it can also indicate a priority level to be used as a PDU-Set-specific priority in LCP. Indeed, in the simplest mapping option where all PDU Sets of a DRB are mapped, as in legacy, onto the same LCH [9], all PDUs (hence PDU Sets) will be treated with the same priority, which is the priority of the LCH. In such case, the PDU Set importance can provide a priority differentiation of the head-of-line (HOL) PDU set bidding with other LCHs (e.g. eMBB). Note this is NOT to prioritize different PDU set types with respect to each other within an XR QoS flow. As illustrated in Figure 1 this LCP priority differentiation between low importance PDU sets and high importance PDU sets of the same LCH, helps, together with the selective PSDB-based PDU Set discard discussed above, in solving the congestion issue when carrying PDU sets of different importance in the same QoS flow / DRB / LCH, while keeping in-order delivery naturally. More details are elaborated in [10].

Figure 1: PDU Set importance used as a HOL PDU Set specific priority wrt other LCHs

Proposal 11: When PDU Sets of different importance are multiplexed in the same LCH, the PDU Set importance can be used as a PDU-Set-specific priority providing, on top of the LCH priority, a priority differentiation of the head-of-line (HOL) PDU set competing with other LCHs.

#### PSER

The PSER definition is given below, copied from [1]:

- PDU Set Error Rate: The PDU Set Error Rate (PSER) defines an upper bound for the rate of PDU Sets that have been processed by the sender of a link layer protocol (e.g. RLC in RAN of a 3GPP access) but that are not successfully delivered by the corresponding receiver to the upper layer (e.g. PDCP in RAN of a 3GPP access). Thus, the PSER defines an upper bound for a rate of non-congestion related packet losses. The purpose of the PSER is to allow for appropriate link layer protocol configurations (e.g. RLC and HARQ in RAN of a 3GPP access). For every 5QI the value of the PSER is the same in UL and DL. If any PDU within the PDU Set is not successfully transmitted, the PDU Set is treated as error.

NOTE 2:	In this release, a PDU set is considered as successfully delivered when all PDUs of a PDU Set are delivered successfully.

Specifically on this parameter, in their reply LS on PDU set handling [1], SA2 asked RAN2:

SA2 defined a new QoS parameter PDU Set Error Rate (PSER) and kindly asks RAN2 to provide feedback on this new QoS parameter in relation to its intended purpose i.e. appropriate link layer protocol configurations.

Surprisingly, RAN1 is not copied in this LS although we think RAN1 should be involved in assessing the use and need of this parameter. In legacy, the BLER target is derived from the QFI’s max packet loss rate and we assume the PSER could similarly be involved for setting/adjusting the target BLER over Uu. We note though that in DL, gNB has all information to measure/monitor the PSER and set/adapt the DL BLER accordingly. But in UL, RAN2 agreed no in-band marking in support of PDU Set identification over Uu. In other words, the gNB has no clue about PDU Sets it receives in UL, it only sees PDUs. Therefore gNB cannot track the PSER in UL. Hence, UE should maintain the UL PSER measurement and feedback this information (based on PSER threshold) to gNB that can make use of it for adapting the UL BLER accordingly, or any other action. Alternately, the UE could autonomously trigger PDCP duplication in UL upon reporting a PSER above a threshold.

Observation 2: In DL, gNB has all information to measure/monitor the actual PSER and set/adapt the DL BLER to meet the target PSER. No specification impact is foreseen.

Proposal 12: UE should maintain the UL PSER measurement and feedback this information (based on PSER threshold) to gNB.

Proposal 13: RAN2 discusses UE autonomously triggering PDCP duplication upon reporting PSER above a threshold.

# Conclusion

According to the analysis in section 2, we have the following observations and proposals:

Proposal 1: The same set of semi-static and dynamic XR QoS parameters shall be supported for UL XR QoS flows and PDU Sets as for DL XR QoS flows and PDU Sets.

Proposal 2: RAN2 consults SA2 on the feasibility for SMF to instruct UEs how to identify UL PDU Sets and bursts.

Observation 1: SA2 already concluded that the XR burst periodicity of a QoS flow is provided at QoS flow level in TSCAI/TSCAC for both DL and UL QoS flows.

Proposal 3: Same as in DL, jitter is applicable to XR traffic bursts in UL.

Proposal 4: The jitter on arrival times of UL XR video bursts is measured by the UE and reported to gNB as UAI.

Proposal 5: RAN2 consults SA2 on the feasibility for CN to provide the parameters PSDB, PSER, PSIHI, for UL PDU Sets, as for DL PDU Sets.

Proposal 6: The new bursts level parameters (periodicity, jitter and end of burst indication), can be used by gNB for configuring e.g. DRX, CG and SPS configurations. No further specification impact is foreseen.

Proposal 7: RAN2 expresses the need for a new parameter, discardOutdatedPDU-Set, to SA2/SA4, to control whether to discard or not PDU Sets exceeding the PSDB.

Proposal 8: RAN2 expresses the need for a new parameter, PDUsetImportanceDiscardThreshold, to SA2, which sets the importance value below which a UE (/gNB) discards an UL (/DL) PDU Set that exceeded the PSDB, if discardOutdatedPDU-Set is configured.

Proposal 9: For DL QoS flows, PSDB, PSIHI and importance parameters impact the gNB implementation but have no specification impact.

Proposal 10: Capture in the TR the above UE behavior table on UL PDU Set discarding.

Proposal 11: When PDU Sets of different importance are multiplexed in the same LCH, the PDU Set importance can be used as a PDU-Set-specific priority providing, on top of the LCH priority, a priority differentiation of the head-of-line (HOL) PDU set competing with other LCHs.

Observation 2: In DL, gNB has all information to measure/monitor the actual PSER and set/adapt the DL BLER to meet the target PSER. No specification impact is foreseen.

Proposal 12: UE should maintain the UL PSER measurement and feedback this information (based on PSER threshold) to gNB.

Proposal 13: RAN2 discusses UE autonomously triggering PDCP duplication upon reporting PSER above a threshold.

# References

- TR 23.700-60 v18.0.0, Study on XR (Extended Reality) and media services (Rel-18)

- S4aV220921, Reply LS on further details on XR traffic, SA4

- M. Lecci, M. Drago, A. Zanella, M. Zorzi, An Open Framework for Analyzing and Modeling XR Network Traffic, IEEE Access, Sept 2021

- S2-2301378, Reply LS on PDU Set Handling

- R2-2300224, On PDU Discarding, CATT

- S2-2301472, Support of PDU Set based handling (23.501)

- S2-2301380, AF Session procedures update to support PDU Set QoS Parameters (23.502)

- S4aR230035, Reply LS on PDU Set Handling (reply to R2-2213351 LS on PDU Set Handling)

- R2-230225, Protocol stack impacts from serving an XR QoS flow, CATT

- R2-230223, Discussion on the PDU Prioritization, CATT