3GPP TSG-RAN WG2 Meeting #121                                                        R2-2300224

Athens, Greece, 27th Feb– 3rd Mar, 2023

Source:              CATT

Title:	             On PDU Discarding

Agenda Item:	8.5.2.3

Document for:	Discussion and Decision

# Introduction

XR-specific PDU discarding has been discussed in both RAN2 and SA2. Two aspects have been considered: timer-based discarding and PDU Set discarding due to PDU Set QoS parameters. This contribution will discuss the models and related issues of the two aspects.

# Discussion

## Timer-based discarding

RAN2 agreed “RAN2 to support timer-based discarding of UL transmit side of PDCP PDU/SDUs of a PDU set. FFS how this is modelled in PDCP specification, can be discussed in WI phase.” in RAN2#120 meeting and captured it in TR 38.835 [1].

We identify below issues on timer-based discarding which can be discussed in SID:

- Issue 1: Can RAN make use of the PSDB as currently defined?

- Issue 2: Which PDU arrival assumption to take into consideration for the PDU Set discard timer design?

- Issue 3: Whether the PDU Set discard timer can be configured with the legacy PDCP discardTimer together?

- Issue 4: UE behavior regarding the PDU Set discard timer: how to apply the PDU Set discard timer? Should it be impacted by the other time information, i.e., the delay information reported for timer-based scheduling?

- Issue 5: Which layer(s) should perform the discarding: Should PDCP layer perform the discarding only, or other layers, such as RLC, MAC can be involved?

Issue 1: Can RAN make use of the PSDB as currently defined by SA2?

In SA2 TR [2], the PSDB is introduced as follows:

- PDU Set Delay Budget: The PSDB defines an upper bound for the delay that a PDU Set may experience for the transfer between the UE and the N6 termination point at the UPF, i.e. time between reception of the first PDU and the successful delivery of the last arrived PDU of a PDU Set. PSDB applies to the DL PDU Set received by the UPF over the N6 interface, and to the UL PDU Set sent by the UE. For a certain 5QI the value of the PSDB is the same in UL and DL. NOTE 3: To enable support for PSDB, it is assumed that there is a maximum duration threshold for inter arrival time between PDUs and first arrived PDU within the PDU Set as per SLA or pre-configuration. How to handle the case of maximum duration threshold is not met needs to be discussed in normative phase.

As can be seen, the PSDB is currently defined between the UE and the UPF, similar to the legacy 5QI packet delay budget (PDB). However, the PSDB can only be useful to RAN if it reflects the maximum delay budget over the air interface. In legacy specification, different notes in the 5QI table (Table 5.7.4-1 of TS 23.501) already characterize the CN-PDB to be subtracted from the 5QI PDB to get the packet delay budget that applies to the radio interface. So we expect the same should be done for the PSDB.

Proposal 1: Similar to the 5QI PDB, SA2 should provide a mean for RAN to convert the PSDB into the equivalent delay budget over the air-interface.

In the rest of the document, we consider PSDB as the PDU Set delay budget over the air-interface and we call the timer introduced by PSDB “PDU Set discard timer”.

Issue 2: Which PDU arrival assumption to take into consideration for the PDU Set discard timer design?

There are two models of PDU Set arrival as Figure 1. In model 1, the PDUs in a PDU Set arrive at the PDCP layer simultaneously. In model 2, the PDUs in a PDU set could arrive at the PDCP layer at different times.

Figure 1 Two models of PDU Set arrival

First we note that model 1 is a special case of model 2. Then, as can be seen from above SA2 TR extract, Note 3 clearly assumes that PDUs of a PDU Set may arrive at different times at the UPF in DL and at the UE in UL. Therefore we suggest adopting model 2 for designing the PSDB timer.

Proposal 2: The PDU Set PDCP discard timer is designed taking into consideration that PDUs of a PDU Set may arrive at different times.

Issue 3: Whether PDU Set discard timer can be configured with PDCP discardTimer together?

Basically, the gNB can set the legacy PDCP discardTimer optionally based on UL PDB indicated from CN and configure it to the UE. For XR service, PSDB is introduced and it could be used for timer-based discarding [1]. When both PDCP discardTimer and PDU Set discard Timer are configured, there are two cases:

- If one PDCP discardTimer expires and the PDU Set discard timer is running, whether to discard the PDU Set depends on the PSIHI value. That is, PDU Set should be discarded when one PDU is lost due to the expiry of its PDCP discardTimer, if PSIHI is set. Then PDU Set discard timer is useless since the PDU Set has been discarded. Thus, the PDCP discardTimer increases the risk of PDU Set discarding. We are not sure if this is a reasonable case.

- If PDU Set discard timer expires and any PDCP discardTimer is running, since the PDU Set is discarded, the remaining PDCP discardTimer should be stopped (invalid). In this case, only PDU Set discard timer takes effect.

Observation 1: When PSIHI is set, PDCP discardTimer increases the risk of PDU Set discarding and then PSDB discard timer could be useless.

Proposal 3: RAN2 asks SA2 if 1) the legacy PDB and the PSDB can be configured together 2) if yes, whether both combinations exist: PSDB > PDB and PDB > PSDB.

Issue 4: UE behavior regarding the PDU Set discard timer

When PDU Set discard timer is configured, the PDUs in a PDU Set should be treated as a whole. Therefore, and as a consequence of proposal #2, the most straightforward model is a single timer that is started upon reception of the 1st PDU of a PDU Set.

Proposal 4: The new PDU Set discard timer is modeled as one PDCP timer per PDU Set that is started upon reception of the 1st PDU of a PDU Set.

RAN2 agreed to introduce delay information consisting of at least “remaining time” which is reported from UE to gNB. We need to confirm if the other time information, i.e., the delay information could be applied to the timer-based discarding, and then, if the gNB should schedule the packets exceeding the “remaining time”.

The purpose of delay information is to trigger time-aware scheduling in gNB side. UE should calculate the delay information based on PSDB. But if this delay information would also be used by the UE for PDU Set discarding, the function would be redundant with the PDU Set discard timer.

Proposal 5: Confirm that the delay information reported from UE to gNB has no impact on the timer-based discarding.

Issue 5: Which layer(s) should perform the discarding

When the PDU Set discard timer expires, PDCP layer can discard the PDCP SDUs not associated with PDCP SNs directly. If the PDCP SDU has been associated with a PDCP SN and delivered to lower layers, should RLC and MAC perform the discarding?

In current specification, PDCP and RLC shall perform the packet discarding as below. And MAC will not perform the discarding. These rules are also applicable for the PDU Set discarding and then the specification change is not needed.

TS 83.323 [4] ## 5.3 SDU discard When the discardTimer expires for a PDCP SDU, or the successful delivery of a PDCP SDU is confirmed by PDCP status report, the transmitting PDCP entity shall discard the PDCP SDU along with the corresponding PDCP Data PDU. If the corresponding PDCP Data PDU has already been submitted to lower layers, the discard is indicated to lower layers. For SRBs, when upper layers request a PDCP SDU discard, the PDCP entity shall discard all stored PDCP SDUs and PDCP PDUs. NOTE: Discarding a PDCP SDU already associated with a PDCP SN causes a SN gap in the transmitted PDCP Data PDUs, which increases PDCP reordering delay in the receiving PDCP entity. It is up to UE implementation how to minimize SN gap after SDU discard. TS 38.322 [6] ## 5.4 SDU discard procedures When indicated from upper layer (e.g. PDCP) to discard a particular RLC SDU, the transmitting side of an AM RLC entity or the transmitting UM RLC entity shall discard the indicated RLC SDU, if neither the RLC SDU nor a segment thereof has been submitted to the lower layers. The transmitting side of an AM RLC entity shall not introduce an RLC SN gap when discarding an RLC SDU.

Proposal 6: PDCP layer and RLC layer shall perform the discarding of PDUs of a PDU Set in the same way as the legacy single packet discarding. No specification change is needed.

## PDU Set discarding due to PDU Set QoS parameters

There are two PDU Set parameters that may impact the PDU Set discarding: PDU Set Importance and PSII (which is called PSIHI in the latest SA2 LS [3]). According to the SA2 LS [3], PSIHI is per QoS flow and PDU Set Importance is per PDU Set.

PDU Set Importance is included in the GTP-U header in DL and maintained by UE itself in UL. In DL, gNB may use the PDU Set Importance for PDU Set level packet discarding in presence of congestion as stated in SA2  [3]. This mechanism can be extended to UL. In UL, gNB could indicate to UE that the PDU Set discarding based on PDU Set Importance is allowed, and then UE performs PDU Set discarding based on the PDU Set Importance maintained by the UE.

Proposal 7: RAN2 consider PDU Set discarding based on PDU Set Importance in UL which should be configured by gNB.

PSIHI is semi-static information provided via control plane (NGAP) for both DL and UL. Although SA4 provided several examples of PDU Set integrated handling in [7], SA2 agreed PSIHI for PDU Set integrated handling only. PDU Set integrated handling is to enhance the scheduling efficiency and promote user experience. It should be applied in both DL and UL. In UL, the PDU Set discarding due to PSIHI should be enabled by gNB to avoid misunderstanding.

Proposal 8: The gNB could enable PDU Set discarding in UL based on the received PSIHI from CN via NGAP.

# Conclusion

According to the analysis in section 2, we provides below observations and proposals. TP on 38.835 is provided in section 4.

Proposal 1: Similar to the 5QI PDB, SA2 should provide a mean for RAN to convert the PSDB into the equivalent delay budget over the air-interface.

Proposal 2: The PDU Set PDCP discard timer is designed taking into consideration that PDUs of a PDU Set may arrive at different times.

Observation 1: When PSIHI is set, PDCP discardTimer increases the risk of PDU Set discarding and then PSDB discard timer could be useless.

Proposal 3: RAN2 asks SA2 if 1) the legacy PDB and the PSDB can be configured together 2) if yes, whether both combinations exist: PSDB > PDB and PDB > PSDB.

Proposal 4: The new PDU Set discard timer is modeled as one PDCP timer per PDU Set that is started upon reception of the 1st PDU of a PDU Set.

Proposal 5: Confirm that the delay information reported from UE to gNB has no impact on the timer-based discarding.

Proposal 6: PDCP layer and RLC layer shall perform the discarding of PDUs of a PDU Set in the same way as the legacy single packet discarding. No specification change is needed.

Proposal 7: RAN2 consider PDU Set discarding based on PDU Set Importance in UL which should be configured by gNB.

Proposal 8: The gNB could enable PDU Set discarding in UL based on the received PSIHI from CN via NGAP.

# References

- TR 38.835  NR; Study on XR enhancements for NR v1.0.0

- TR23.700-60, Study on XR (Extended Reality) and media services. V18.0.0

- TS 38.413, NG-RAN; NG Application Protocol (NGAP), V17.6.0

- S2-2301378, Reply LS on PDU Set Handling

- TS 38.323, Packet Data Convergence Protocol (PDCP) specification, V17.3.0

- S4aR230035, Reply LS on PDU Set Handling (reply to R2-2213351 LS on PDU Set Handling)

- TS 38.322, Radio Link Control (RLC) protocol specification, V17.2.0

- R2-2206969, LS Reply on QoS support with PDU Set granularity