GPP TSG-RAN WG2 Meeting #121		R2-2300226

Athens, Greece, 27th Feb– 3rd Mar, 2023

Source:	CATT

Title:	DRX enhancements for XR Power Saving

Agenda Item:	8.5.3

Document for:	Discussion and Decision

# Introduction

In the RAN plenary 98-e, the following objective was captured in the new R18 XR WID [1] for XR-specific power saving:

Specify the enhancements related to power saving: - DRX support of XR frame rates corresponding to non-integer periodicities (through at least semi-static mechanisms e.g. RRC signalling) (RAN2).

In this document, we compare the various solutions proposed during the SI to address the WID’s objective (non-integer XR frame periodicities) and provide our preference.

# Discussion

The frame rate of most XR services is not integer. A typical 60fps, for example, has a period of 16.66ms, which does not match any integer DRX period. It results that, after a certain amount of time, the on-duration of the DRX period falls into an unexpected period, during which XR traffic cannot be received immediately. Several solutions have been put forward to solve the issue that can be classified into three main alternatives.

Alt1: Pre-configuring multiple start offsets

The network pre-configures multiple DRX start offsets in the DRX cycle, for example (0ms, 16ms, 33ms) can be configured in a DRX cycle of 50ms for 60fps. The UE adjusts DRX start offsets accordingly.

Alt2: Pre-configuring different DRX cycles in one round

Alternately, use different DRX cycles in one round, e.g 16ms, 16ms and 17ms in a round for a period of 16.66ms.

These schemes are used to align DRX on-duration and XR service cycles.

However, these solutions have some disadvantages. In order to match different non-integer DRX cycles, it would require a large number of configurations. For example: For XR frame rate with 90fps, 9 DRX start offsets, e.g., (0ms, 11ms, 22ms, 33ms, 44ms, 55ms, 66ms, 77ms, 89ms) need to be configured in a DRX cycle of 100ms.

Moreover, a general comment associated with this multi-configuration switching or dynamic parameters adjustment is that it can be shown to address (more or less efficiently) the current frame rates under study, but it is not guaranteed to work for any kind of non-integer periodicity. Hence such approach is not really future-proof.

Observation 1: Both pre-configuring multiple start offsets and pre-configuring different DRX cycles in one round requires a large number of configurations to match different non-integer DRX cycles.

Observation 2: Both pre-configuring multiple start offsets and pre-configuring different DRX cycles in one round cannot guarantee to work for any kind of non-integer periodicity. It is not really future-proof.

Alt3: Introducing non-integer DRX cycles and floor operations

The current DRX formulas to start drx-onDurationTimer are:

- For long DRX cycle: [(SFN × 10) + subframe number] modulo (drx-LongCycle) = drx-StartOffset , where SFN is the current System Frame Number, and the subframe number is the current subframe number.

- For short DRX cycle: [(SFN × 10) + subframe number] modulo (drx-ShortCycle) = (drx-StartOffset) modulo (drx-ShortCycle) .

The DRX cycle mismatch issue can be solved by enhancing the DRX formula so that it can support non-integer DRX cycles [2]-[8]. However, directly using a non-integer for modulo operation may cause rounding errors, which is the main argument of companies not supporting this approach: different UE implementations of the non-integer formula may not guarantee that the gNB and the UE are always aligned. Therefore, in order to solve this, a common approach proposed in [2][5][6][7][8] is to introduce floor operation in the formula. More specifically, [5] proposes to replace the legacy formula involving modulo operations with an equivalent formula only involving floor operations (based on the equivalence A modulo B = A – [ B × floor (A / B) ]), which results in the following equivalent DRX formula:

floor [n – [drx-LongCycle × floor (n / drx-LongCycle) ] ] = drx-StartOffset

where n = [(SFN × 10) + subframe number]

Similarly, the short DRX cycle formula becomes:

floor [n – [drx-ShortCycle × floor (n / drx-ShortCycle) ] ] =

floor [drx-StartOffset – [drx-ShortCycle × floor (drx-StartOffset / drx-ShortCycle) ] ]

where n = [(SFN × 10) + subframe number]

However, the worry is that floor operations may not solve the issue of possible different UE implementations of the division n/drx-LongCycle if drx-LongCycle can take any non-integer value. Therefore, [2] and [5] also propose that drx-LongCycle and drx-ShortCycle are configured as rationale numbers. Indeed, if the RRC parameter drx-LongCycle is configured in the form a / b where a and b are RRC configured as separate integer values, the operation n / drx-LongCycle simplifies to n * b / a which reduces to an Euclidian division of two positive integers, N/D, for which quotient Q and remainder R always exist and are unique. Its simplest implementation is the greatest common divisor algorithm using only subtractions and comparisons:

R:=N; Q:=0; while R ≥ D do R:=R-D; Q:=Q+1; end

The resulting quotient Q directly yields the floor result in the above formulas, which is evidence that such representation definitely solves the potential rounding errors from different UE implementations.

Observation 3: Replacing the legacy DRX formula involving modulo operation with its equivalent floor-based expression and configuring drx-LongCycle and drx-ShortCycle in rationale numbers solves the potential rounding ambiguity issue from different implementations.

Proposal 1: drx-LongCycle and drx-ShortCycle are configured in rationale numbers.

Proposal 2: The legacy DRX formula involving modulo operation is replaced with its equivalent expression involving floor operations instead.

# Conclusion

According to the analysis in section 2, we have the following observations and proposals:

Observation 1: Both pre-configuring multiple start offsets and pre-configuring different DRX cycles in one round requires a large number of configurations to match different non-integer DRX cycles.

Observation 2: Both pre-configuring multiple start offsets and pre-configuring different DRX cycles in one round cannot guarantee to work for any kind of non-integer periodicity. It is not really future-proof.

Observation 3: Replacing the legacy DRX formula involving modulo operation with its equivalent floor-based expression and configuring drx-LongCycle and drx-ShortCycle in rationale numbers solves the potential rounding ambiguity issue from different implementations.

Proposal 1: drx-LongCycle and drx-ShortCycle are configured in rationale numbers.

Proposal 2: The legacy DRX formula involving modulo operation is replaced with its equivalent expression involving floor operations instead.

# Reference

- RP-223502, New WID on XR Enhancements for NR, Nokia, Qualcomm, RAN#98-e, December 12-16, 2022

- TR 38.835 v1.0.0, NR; Study on XR enhancements for NR (Release 18)

- R2-22111809, DRX enhancements for XR, Qualcomm

- R2-2211529, DRX enhancements for XR, ZTE Corporation, Sanechips

- R2-2211715, DRX Enhancements for XR, Apple

- R2-2211860, C-DRX enhancements for XR, MediaTek Inc., RAN2#120

- R2-2212040, Discussion of DRX enhancement, Lenovo, RAN2#120

- R2-2212237, Candidate solutions on C-DRX enhancement, NEC

- R2-2212332, DRX Enhancement for XR, Google Inc.