3GPP TSG-RAN WG2 Meeting #121	R2-2300230

Athens, Greece, 27th February – 3rd March, 2023

Source: 	Huawei, HiSilicon

Title: 	Discussion on cell DTX/DRX

Agenda Item:	8.3.2

Document for:	Discussion and decision

# Introduction

In RAN2#120 meeting, the cell DTX/DRX was discussed with the following agreements [1]:

Agreements

1 	Clarify previous agreement to: periodic cell DTX/DRX pattern is configured by UE-specific RRC.   Periodic cell DTX/DRX can be activated/deactivated by L1/L2 signalling and UE-specific RRC signaling.

2 	Capture in TR 38.864 that both UE specific and common L1/L2 signalling can be considered for at least activating/deactivating the cell DTX/DRX pattern, per the agreement in 119b-e.

3	Cell DTX and Cell DRX modes can be configured and operated separately (e.g. one RRC configuration set for DL and the other set for UL).  Cell DTX/DRX can also be configured and operated together.

4	It is up to NW whether legacy UEs can access cells with Cell DTX/DRX

5	Cell DTX/DRX can be configured per serving cell and can be applicable for different cells in CA.  No additional RAN2 impacts or enhancements are foreseen.

6	Whether to support multiple Cell DTX/DRX configurations can be discussed later in the normative phase.

7	At least the following parameters can be configured per cell DTX/DRX configuration: periodicity, start slot/offset, on duration.  Details related to UE behaviour can be discussed during WI phase

In the approved 3GPP TR 38.864, the higher layer procedures of cell DTX/DRX have the following description [2]:

Cell DTX/DRX is applied to at least UEs in RRC_CONNECTED state. A periodic Cell DTX/DRX (i.e., active and non-active periods) can be configured by gNB via UE-specific RRC signalling per serving cell. Below examples on Cell DTX/DRX behaviour during non-active periods are assumed to be possible options, and the UE behaviour/impact will be studied: - Example 1: gNB is expected to turn off all transmission and reception for data traffic and reference signal during Cell DTX/DRX non-active periods. - Example 2: gNB is expected to turn off its transmission/reception only for data traffic during Cell DTX/DRX non-active periods (i.e., gNB will still transmit/receive reference signals) - Example 3: gNB is expected to turn off its dynamic data transmission/reception during Cell DTX/DRX non-active periods (i.e., gNB is expected to still perform transmission/reception in periodic resources, including SPS, CG-PUSCH, SR, RACH, and SRS). - Example 4: gNB is expected to only transmit reference signals (e.g., CSI-RS for measurement). The study focus on UE behavior when at any point in time the cell activates a single DTX/DRX configuration. It is up to NW whether legacy UEs can access cells with Cell DTX/DRX. The Cell DTX/DRX mode can be activated/de-activated via dynamic L1/L2 signalling and UE-specific RRC signaling. Both UE specific and common L1/L2 signalling can be considered for activating/deactivating the Cell DTX/DRX mode. Cell DTX and Cell DRX modes can be configured and operated separately (e.g., one RRC configuration set for DL and another for UL). Cell DTX/DRX can also be configured and operated together. At least the following parameters can be configured per Cell DTX/DRX configuration: periodicity, start slot/offset, on duration. Details related to UE behaviour can be discussed during WI phase. Whether to support multiple Cell DTX/DRX configurations can be discussed later in the WI phase. It is beneficial to align UE DRX with Cell DTX and DRX alignment among multiple UEs. The alignment mechanism can be discussed during the WI phase. From RAN2 perspective, Cell DTX/DRX is feasible.

The objectives of the work item on cell DTX/DRX after the RAN#98-e meeting are the following [3]:

2. Specify enhancement on cell DTX/DRX mechanism including the alignment of cell DTX/DRX and UE DRX in RRC_CONNECTED mode, and inter-node information exchange on cell DTX/DRX [RAN2, RAN1, RAN3] • Note: No change for SSB transmission due to cell DTX/DRX. • Note: The impact to IDLE/INACTIVE UEs due to the above enhancement should be avoided.

In this paper, we discuss the details of the cell DTX/DRX mechanism.

# Discussion

## 2.1 Cell and UE behaviours

Based on the four Examples mentioned in TR 38.864, our understanding of the cell and UE behaviours during Cell DTX/DRX inactive periods and UE impacts are summarised in Table I and Table II.

Table I Cell DTX Example 1-4

| Cell DTX | NW behaviour in cell inactive period | UE behaviour in cell inactive period | UE impacts |
| --- | --- | --- | --- |
| Example 1 | No DL transmission. | Stop monitoring PDCCH and SPS traffic. Stop SSB or CSI based measurement, synchronization and beam management. UE cannot receive SIB. | High for connected, idle and inactive UEs. |
| Example 2 | No TX of data traffic (including dynamic, semi-persistent). The common signals (i.e. SIB) and all reference signals are still transmitted. | Stop monitoring PDCCH and SPS traffic. | Small for connected UEs. No impact on idle and inactive UEs. |
| Example 3 | No TX of dynamic data traffic. The semi-persistent data traffic, common signals and reference signals are still transmitted. | Stop monitoring PDCCH. | Very Small for connected UEs. No impact on idle and inactive UEs. |
| Example 4 | No TX of data traffic and common signals. The reference signals are still transmitted. | Stop monitoring PDCCH and SPS. UE cannot receive SIB. | Small for connected, medium for idle and inactive UEs. |

Table II Cell DRX Example 1-4

| Cell DRX | NW behaviour in cell inactive period | UE behaviour in cell inactive period | UE impacts |
| --- | --- | --- | --- |
| Example 1 | No UL reception. | Stop SR/PUCCH, UL dynamic data traffic, CG-PUSCH, SRS, and RACH. | High for connected, idle and inactive UEs. |
| Example 2 | No RX of data traffic (including dynamic and semi-persistent). The RACH and all reference signals are still received. | Stop SR/PUCCH, UL dynamic data traffic and CG-PUSCH. | Small for connected UEs. No impact on idle and inactive UEs. |
| Example 3 | No RX of dynamic data traffic. The semi-persistent data traffic, RACH and reference signals are still received. | Stop SR/PUCCH and UL dynamic data traffic. | Very Small for connected UEs. No impact on idle and inactive UEs. |

Although Example 1 stops TX/RX of more signals in the gNB, it has higher impacts on idle and inactive UEs than other Examples. Without the SSB and common signals, e.g. SIB, RACH, the idle and inactive UEs cannot discover, measure, synchronize, and access to the cell during the inactive periods. In our understanding Example 4 has no impact on the UL but maintains only reference signals in the DL having impact on idle and inactive UEs. To avoid impacts on IDLE/INACTIVE UEs and not to change SSB transmission, as mentioned in the WID [3], Example 1 and Example 4 should be excluded.

Observation 1: Concerning our above analysis Example 1 and Example 4 from the TR are not compliant with the WID in terms of IDLE/INACTIVE UE impacts and should be excluded.

Example 3 has the smallest impact on UEs, but it also has the smallest network energy saving gain compared with the existing mechanisms, i.e. UE C-DRX and intelligent network scheduling. To achieve higher NES gain than the existing mechanisms, the Cell DTX/DRX solution should stop more signals than dynamic data transmission and reception. We think CG/SPS should not be transmitted/received, otherwise there is limited gain in comparison with the existing UE C-DRX. In Example 2, the gNB will stop dynamic and semi-persistent data transmission and reception but still transmit/receive reference signals and common signals during the Cell DTX/DRX inactive period. In our view, Example 2 has small impacts on the UE side and can improve NES gains which should be prioritized.

Observation 2: Concerning our above analysis Example 2 from the TR, when maintaining all common (i.e. SIB) and reference signals, has the best balance of NES gain while having very small impact on IDLE/INACTIVE UEs.

Proposal 1: To avoid impacts on IDLE/INACTIVE UEs and not to change SSB transmission, as stated in the WID, prioritize Example 2 (gNB turns off RX/TX of data traffic but no change of the RX/TX of RACH, common signals and all reference signals) for NES gain improvement.

For Example 2, the connected UE should proactively stop dynamic and SPS transmission during the Cell DTX inactive period, and stop dynamic and CG-PUSCH transmission and SR during the Cell DRX inactive period, to avoid transmission/reception failure. The UE could also stop monitoring PDCCH during the Cell DTX inactive period, when the gNB will not send anything on the PDCCH, to save UE energy.

Proposal 2: The UE stops dynamic, CG/SPS transmission and SR during Example 2 Cell DTX/DRX inactive periods, and also stops monitoring PDCCH during the Cell DTX inactive period for UE energy saving purposes.

Based on Example 2, if the transmission/reception of UE-specific reference signals (i.e. CSI-RS and SRS), can be further stopped during the Cell DTX/DRX inactive period, higher NES gain can be achieved. The UE impacts of no TX/RX of UE-specific reference signals should be evaluated by RAN1. If the UE impacts are acceptable, we should consider stopping UE-specific reference signals during the Cell DTX/DRX inactive period.

Observation 3: Higher NES gain can be achieved if UE-specific reference signals can be stopped during Cell DTX/DRX inactive period, which is in RAN1 scope.

Proposal 3: If found feasible by RAN1, UE-specific reference signals can be stopped during the Cell DTX/DRX inactive period for higher NES gains.

## 2.2 Cell DTX/DRX (de)activation

A periodic Cell DTX/DRX can be configured to the UEs by the gNB via UE-specific RRC signalling. The (de)activation of the cell DTX/DRX configuration for UEs should support two solutions [2]:

- (de)activation by UE-specific RRC signalling

The configuration and activation of the Cell DTX/DRX can be carried in one RRC message which can reduce signalling overhead and simplify the network and UE behaviours. When a new UE enters a Cell DTX/DRX cell, directly configuring and activating the Cell DTX/DRX configuration by one RRC message is efficient.

However, if the gNB wants to dynamically (de)activate Cell DTX/DRX, the (de)activation by RRC is slower and less efficient than by DCI.

- (de)activation by UE-specific or cell-level L1/L2 signalling

Compared to RRC signalling, (de)activation by L1/L2 signalling is faster which can support a more real-time (de)activation of Cell DTX/DRX. DCI as a physical layer signalling can be more dynamic than MAC CE. DCI can also be UE-specific or cell-level according to the existing mechanisms.

Cell DTX/DRX is a cell-level technique, (de)activation by cell-level DCI can save signalling overhead but is limited to the scenario that (de)activates all the existing UEs. For a new UE entering a DTX/DRX cell, configuration by RRC and then activation by the cell-level DCI is inefficient, and the cell-level DCI may mislead the UEs already camping on that cell. In our view, regardless whether the cell-level signalling for (de)activation Cell DTX/DRX is supported, at least one kind of UE-specific signalling for (de)activating Cell DTX/DRX should be supported for new UEs entering the cell. For that purpose it can be the UE-specific RRC signalling.

In summary, (de)activation by UE-specific RRC signalling is simple and direct but not dynamic enough, which makes it suitable for the slow changing scenario, e.g. the traffic load changes slowly and Cell DTX/DRX does not change frequently. On the other hand (de)activation by DCI signalling can support dynamic scenarios, with fast and frequent Cell DTX/DRX changes. The (de)activation by cell-level DCI signalling can also save signalling overhead in some cases whereas activation by UE-specific RRC signalling is more suitable for new UEs entering a cell with DTX/DRX enabled.

Proposal 4: Cell DTX/DRX (de)activation is supported by both UE-specific RRC signalling and cell-level DCI.

## 2.3 Cell DTX/DRX configuration

Single or multiple cell DTX/DRX configuration:

When considering single or multiple cell DTX/DRX configurations on the same serving cell, we can conclude that the multiple cell DTX/DRX configuration is more flexible but also more complex. The multiple cell DTX/DRX scenario adds configuration switching to the list of procedures. We suggest to start from a single cell DTX/DRX configuration for simplicity. When the cell DTX/DRX technique with a single configuration is more mature, we may further consider the multiple cell DTX/DRX configurations.

Proposal 5-1: Start from a single Cell DTX/DRX configuration.

Separate or joint cell DTX/DRX configuration:

As mentioned in the agreements of the RAN2#120 meeting [1], Cell DTX/DRX can be configured separately and together. Therefore, at least the separated Cell DTX and Cell DRX configurations should be supported. One RRC configuration set can include Cell DTX parameters, e.g. periodicity, start slot/offset, on duration; and another RRC configuration set can include that of Cell DRX. One RRC message can carry both Cell DTX and Cell DRX configuration sets, thus supporting a joint configuration. It is worth noting that the joint Cell DTX/DRX configuration is most efficient in the scenario when parameters of Cell DTX and Cell DRX are the same, and there is only one set of parameters for the whole cell. That being said we prefer to have the possibility of separate cell DRX and DTX configuration and (de)activation for NW flexibility, while leaving to the implementation whether the configurations are aligned.

Proposal 5-2: Start from separate single cell DTX and single cell DRX configuration which can be used together.

Proposal 5-3: For the maximisation of gNB NES gains there should be only one set of parameters in a single cell.

## 2.4 Alignment between UE DRX and cell DTX

When a cell has activated cell DTX and the connected UE has activated UE DRX, there are three alignment possibilities between the cell DTX and the UE DRX:

- No alignment: UE C-DRX on duration has no overlap with the cell DTX active period.

- Partial alignment: part of UE C-DRX on duration overlaps with the cell DTX active period.

- Alignment: all the UE C-DRX on duration overlap with the cell DTX active period.

Since there will be no DL traffic during the cell DTX inactive period, no PDCCH transmission will take place during this time. If there is no alignment, the UE cannot be successfully scheduled. If the alignment is only partial, the PDCCH monitoring is wasting energy during the part of UE C-DRX on duration which does not overlap with the cell DRX active period. Consequentially, the alignment between UE C-DRX and cell DTX/DRX is necessary and beneficial for power consumption and transmission reliability reasons. When Cell DTX is activated, the UE should adopt a UE DRX aligned with the Cell DTX. It should be assured that the active period of cell DTX is fully overlapping the on duration of UE C-DRX for UE power efficiency.

The alignment of UE C-DRX with a certain Cell DTX does not result in a on/off pattern exactly the same as the on/off pattern of the Cell DTX. As shown in Fig.1, the on/off pattern of an aligned UE DRX could vary, e.g. have different duration, periodicity, or offset, but still fall within the cell DTX active time. Various UEs can have different DRX patterns, according to traffic characteristics and network scheduling requirements, and still be aligned with a certain Cell DTX

Fig.1 Different UE DRX patterns aligned with a certain Cell DTX (if Cell DTX is activated).

Proposal 6-1: Alignment between UE C-DRX and cell DTX/DRX is necessary and beneficial for power consumption and transmission reliability reasons.

Proposal 6-2: If Cell DTX is activated, an aligned UE C-DRX configuration with Cell DTX means that the on-duration of C-DRX falls within NW DTX active time, and the aligned C-DRX of different UEs can vary in offset, periodicity or on-duration.

# Conclusion

In this paper, we discussed the details of the Cell DTX/DRX mechanism, and made the following proposals:

Proposal 1: To avoid impacts on IDLE/INACTIVE UEs and not to change SSB transmission, as stated in the WID, prioritize Example 2 (gNB turns off RX/TX of data traffic but no change of the RX/TX of RACH, common signals and all reference signals) for NES gain improvement.

Proposal 2: The UE stops dynamic, CG/SPS transmission and SR during Example 2 Cell DTX/DRX inactive periods, and also stops monitoring PDCCH during the Cell DTX inactive period for UE energy saving purposes.

Proposal 3: If found feasible by RAN1, UE-specific reference signals can be stopped during the Cell DTX/DRX inactive period for higher NES gains.

Proposal 4: Cell DTX/DRX (de)activation is supported by both UE-specific RRC signalling and cell-level DCI.

Proposal 5-1: Start from a single Cell DTX/DRX configuration.

Proposal 5-2: Start from separate single cell DTX and single cell DRX configuration which can be used together.

Proposal 5-3: For the maximisation of gNB NES gains there should be only one set of parameters in a single cell.

Proposal 6-1: Alignment between UE C-DRX and cell DTX/DRX is necessary and beneficial for power consumption and transmission reliability reasons.

Proposal 6-2: If Cell DTX is activated, an aligned UE C-DRX configuration with Cell DTX means that the on-duration of C-DRX falls within NW DTX active time, and the aligned C-DRX of different UEs can vary in offset, periodicity or on-duration.

# References

- R2-2213352 Report from Session on NES, UAV, Small Data, Rel-15-17 UP, Rel-17 Small Data, IIoT/URLLC, and RACH partitioning

- 3GPP TR 38.864 V1.0.0

- RP-223540 New WID Netw_Energy_NR