3GPP TSG-RAN WG2 Meeting #121	R2-2300256

27 Feburary – 3 March, 2023, Athens, GR

Agenda item:	8.5.4

Source:	Dell Technologies

Title:	Dynamic BSR formulation and reporting for XR

WID/SID:	FS_NR_XR_enh

Document for:	Discussion / Decision

# Introduction

Based on the Rel-18 study item description ‘’NR enhancements for XR‘’, RAN2 is to study potential scheduling and resource allocation improvements for XR services as follows.

- Objectives on XR-specific capacity improvements (RAN1, RAN2): - Study mechanisms that provide more efficient resource allocation and scheduling for XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc…). Focus is on the following mechanisms: - SPS and CG enhancements. - Dynamic scheduling/grant enhancements.

During the last RAN2 meeting (RAN2#120), the following discussions are reported on the major directions of technical improvements to be considered for BSR formulation and reporting procedures.

- RAN2 thinks we need one or more additional BSR table(s) for XR. FFS whether these are static (=specified) or dynamic (e.g. generated, differs according to some RRC parameter), can be discussed in WI phase. - RAN2 will introduce data volume information associated with delay information (e.g. remaining time) in a MAC CE. FFS if this is extension of BSR or new format. FFS how to do that (e.g. what exactly is reported) and how to ensure this information is up-to-date e.g. considering UL scheduling delay. - RAN2 needs to discuss additional BSR triggering conditions to allow timely availability of buffer status information at gNB. This can be discussed in WI phase.

Therefore, this contribution presents our views on BSR feedback for XR services. And therefore, it presents our proposals on dynamic BSR formulation and reporting procedures, where they can be considered as BSR tables being dynamically constructed at devices, driven by expected application data volume and network BSR configurations.

# Introduction

Buffer status report (BSR) is vital for dynamic uplink scheduling. With BSR reports, UEs signal serving cells with an indication of the currently buffered uplink traffic size. Per [1], there are two BSR tables defined as short and long BSR reports. A short BSR report is 5 bit long while the long BSR is 8 bit long. As shown by the below Table 6.1.3.1-2 of the long BSR report, each BSR index is associated wit a predefined uplink traffic size interval in the uplink buffers. The larger the BSR index, the larger the range of the buffered traffic size.

In the current specifications, the BSR quantization follows an exponential function as follows:

Bk = Bmin (1+p) k, with p = (Bmax / Bmin)1 / (N-1) – 1

where  Bmin  and  Bmax  are the minimum and maximum possible buffer size which a UE can report. As can be observed, the quantization precision reduces for the larger buffer size intervals. Accordingly, the network will not be exactly aware of how much traffic is being buffered at the reporting device with a considerable standard deviation over the larger buffered traffic sizes. Accordingly, this leads the network to either schedule and allocate too much or too little uplink resource for the actual buffered uplink traffic size. Therefore, this results in extended buffering delays for potential latency critical XR traffic or a degraded spectral efficiency due to the resource over scheduling.

Existing BSR tables and reporting criteria were not problematic for eMBB and URLLC traffic since,

- For eMBB traffic: traffic size is large; however, there is not a tight latency budget as XR traffic, accordingly, it is reasonable for the network to schedule uplink resource as of multiple times corresponding to several BSR transmissions.

- For URLLC traffic: traffic size is typically very small which falls in the high BSR precision quantization area, and hence, the existing BSR tables are suitable for offering a good reporting accuracy of URLLC payload.

Although, for the large XR traffic size, there is a tight latency budget, i.e., packet delay budget (PDB) associated with the XR payload. Accordingly, current BSR tables and reporting procedures need to be further optimized to offer a further faster and more fine grained quantization precision.

Observation 1: Current BSR tables and reporting criteria are suitable for conventional eMBB services since those are not tied with a limited radio latency budget. Thus, the larger quantization errors of the larger buffer range are reduced in time by the multiple gNB scheduling instants.

Observation 2: Current BSR tables and reporting criteria should be further optimized to match the XR-specific traffic profile characteristics and latency targets. Thus, a more precise BSR quantization and reporting procedures are needed.

Proposal 1: RAN2 to specify new BSR formulation, reporting and triggering procedures that fit the XR traffic characteristics.

Table 6.1.3.1-2: Buffer size levels (in bytes) for 8-bit Buffer Size field

| Index | BS value | Index | BS value | Index | BS value | Index | BS value |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 64 | ≤ 560 | 128 | ≤ 31342 | 192 | ≤ 1754595 |
| 1 | ≤ 10 | 65 | ≤ 597 | 129 | ≤ 33376 | 193 | ≤ 1868488 |
| 2 | ≤ 11 | 66 | ≤ 635 | 130 | ≤ 35543 | 194 | ≤ 1989774 |
| 3 | ≤ 12 | 67 | ≤ 677 | 131 | ≤ 37850 | 195 | ≤ 2118933 |
| 4 | ≤ 13 | 68 | ≤ 720 | 132 | ≤ 40307 | 196 | ≤ 2256475 |
| 5 | ≤ 14 | 69 | ≤ 767 | 133 | ≤ 42923 | 197 | ≤ 2402946 |
| 6 | ≤ 15 | 70 | ≤ 817 | 134 | ≤ 45709 | 198 | ≤ 2558924 |
| 7 | ≤ 16 | 71 | ≤ 870 | 135 | ≤ 48676 | 199 | ≤ 2725027 |
| 8 | ≤ 17 | 72 | ≤ 926 | 136 | ≤ 51836 | 200 | ≤ 2901912 |
| 9 | ≤ 18 | 73 | ≤ 987 | 137 | ≤ 55200 | 201 | ≤ 3090279 |
| 10 | ≤ 19 | 74 | ≤ 1051 | 138 | ≤ 58784 | 202 | ≤ 3290873 |
| 11 | ≤ 20 | 75 | ≤ 1119 | 139 | ≤ 62599 | 203 | ≤ 3504487 |
| 12 | ≤ 22 | 76 | ≤ 1191 | 140 | ≤ 66663 | 204 | ≤ 3731968 |
| 13 | ≤ 23 | 77 | ≤ 1269 | 141 | ≤ 70990 | 205 | ≤ 3974215 |
| 14 | ≤ 25 | 78 | ≤ 1351 | 142 | ≤ 75598 | 206 | ≤ 4232186 |
| 15 | ≤ 26 | 79 | ≤ 1439 | 143 | ≤ 80505 | 207 | ≤ 4506902 |
| 16 | ≤ 28 | 80 | ≤ 1532 | 144 | ≤ 85730 | 208 | ≤ 4799451 |
| 17 | ≤ 30 | 81 | ≤ 1631 | 145 | ≤ 91295 | 209 | ≤ 5110989 |
| 18 | ≤ 32 | 82 | ≤ 1737 | 146 | ≤ 97221 | 210 | ≤ 5442750 |
| 19 | ≤ 34 | 83 | ≤ 1850 | 147 | ≤ 103532 | 211 | ≤ 5796046 |
| 20 | ≤ 36 | 84 | ≤ 1970 | 148 | ≤ 110252 | 212 | ≤ 6172275 |
| 21 | ≤ 38 | 85 | ≤ 2098 | 149 | ≤ 117409 | 213 | ≤ 6572925 |
| 22 | ≤ 40 | 86 | ≤ 2234 | 150 | ≤ 125030 | 214 | ≤ 6999582 |
| 23 | ≤ 43 | 87 | ≤ 2379 | 151 | ≤ 133146 | 215 | ≤ 7453933 |
| 24 | ≤ 46 | 88 | ≤ 2533 | 152 | ≤ 141789 | 216 | ≤ 7937777 |
| 25 | ≤ 49 | 89 | ≤ 2698 | 153 | ≤ 150992 | 217 | ≤ 8453028 |
| 26 | ≤ 52 | 90 | ≤ 2873 | 154 | ≤ 160793 | 218 | ≤ 9001725 |
| 27 | ≤ 55 | 91 | ≤ 3059 | 155 | ≤ 171231 | 219 | ≤ 9586039 |
| 28 | ≤ 59 | 92 | ≤ 3258 | 156 | ≤ 182345 | 220 | ≤ 10208280 |
| 29 | ≤ 62 | 93 | ≤ 3469 | 157 | ≤ 194182 | 221 | ≤ 10870913 |
| 30 | ≤ 66 | 94 | ≤ 3694 | 158 | ≤ 206786 | 222 | ≤ 11576557 |
| 31 | ≤ 71 | 95 | ≤ 3934 | 159 | ≤ 220209 | 223 | ≤ 12328006 |
| 32 | ≤ 75 | 96 | ≤ 4189 | 160 | ≤ 234503 | 224 | ≤ 13128233 |
| 33 | ≤ 80 | 97 | ≤ 4461 | 161 | ≤ 249725 | 225 | ≤ 13980403 |
| 34 | ≤ 85 | 98 | ≤ 4751 | 162 | ≤ 265935 | 226 | ≤ 14887889 |
| 35 | ≤ 91 | 99 | ≤ 5059 | 163 | ≤ 283197 | 227 | ≤ 15854280 |
| 36 | ≤ 97 | 100 | ≤ 5387 | 164 | ≤ 301579 | 228 | ≤ 16883401 |
| 37 | ≤ 103 | 101 | ≤ 5737 | 165 | ≤ 321155 | 229 | ≤ 17979324 |
| 38 | ≤ 110 | 102 | ≤ 6109 | 166 | ≤ 342002 | 230 | ≤ 19146385 |
| 39 | ≤ 117 | 103 | ≤ 6506 | 167 | ≤ 364202 | 231 | ≤ 20389201 |
| 40 | ≤ 124 | 104 | ≤ 6928 | 168 | ≤ 387842 | 232 | ≤ 21712690 |
| 41 | ≤ 132 | 105 | ≤ 7378 | 169 | ≤ 413018 | 233 | ≤ 23122088 |
| 42 | ≤ 141 | 106 | ≤ 7857 | 170 | ≤ 439827 | 234 | ≤ 24622972 |
| 43 | ≤ 150 | 107 | ≤ 8367 | 171 | ≤ 468377 | 235 | ≤ 26221280 |
| 44 | ≤ 160 | 108 | ≤ 8910 | 172 | ≤ 498780 | 236 | ≤ 27923336 |
| 45 | ≤ 170 | 109 | ≤ 9488 | 173 | ≤ 531156 | 237 | ≤ 29735875 |
| 46 | ≤ 181 | 110 | ≤ 10104 | 174 | ≤ 565634 | 238 | ≤ 31666069 |
| 47 | ≤ 193 | 111 | ≤ 10760 | 175 | ≤ 602350 | 239 | ≤ 33721553 |
| 48 | ≤ 205 | 112 | ≤ 11458 | 176 | ≤ 641449 | 240 | ≤ 35910462 |
| 49 | ≤ 218 | 113 | ≤ 12202 | 177 | ≤ 683087 | 241 | ≤ 38241455 |
| 50 | ≤ 233 | 114 | ≤ 12994 | 178 | ≤ 727427 | 242 | ≤ 40723756 |
| 51 | ≤ 248 | 115 | ≤ 13838 | 179 | ≤ 774645 | 243 | ≤ 43367187 |
| 52 | ≤ 264 | 116 | ≤ 14736 | 180 | ≤ 824928 | 244 | ≤ 46182206 |
| 53 | ≤ 281 | 117 | ≤ 15692 | 181 | ≤ 878475 | 245 | ≤ 49179951 |
| 54 | ≤ 299 | 118 | ≤ 16711 | 182 | ≤ 935498 | 246 | ≤ 52372284 |
| 55 | ≤ 318 | 119 | ≤ 17795 | 183 | ≤ 996222 | 247 | ≤ 55771835 |
| 56 | ≤ 339 | 120 | ≤ 18951 | 184 | ≤ 1060888 | 248 | ≤ 59392055 |
| 57 | ≤ 361 | 121 | ≤ 20181 | 185 | ≤ 1129752 | 249 | ≤ 63247269 |
| 58 | ≤ 384 | 122 | ≤ 21491 | 186 | ≤ 1203085 | 250 | ≤ 67352729 |
| 59 | ≤ 409 | 123 | ≤ 22885 | 187 | ≤ 1281179 | 251 | ≤ 71724679 |
| 60 | ≤ 436 | 124 | ≤ 24371 | 188 | ≤ 1364342 | 252 | ≤ 76380419 |
| 61 | ≤ 464 | 125 | ≤ 25953 | 189 | ≤ 1452903 | 253 | ≤ 81338368 |
| 62 | ≤ 494 | 126 | ≤ 27638 | 190 | ≤ 1547213 | 254 | > 81338368 |
| 63 | ≤ 526 | 127 | ≤ 29431 | 191 | ≤ 1647644 | 255 | Reserved |

On the new BSR formulation and reporting procedures, two directions can be considered as follows.

- Define and specify one or more new BSR tables for XR applications: this is the straightforward option, with the least standardization effort. New BSR tables are predefined to cover the XR traffic volumes of various XR applications. The new BSR tables shall still adopt predefined BSR quantization precisions over the various buffered traffic volumes, similar to existing 5-bit and 8-bit BSR tables, respectively.

- Define and specify dynamic BSR parameters for dynamic BSR table construction and feedback: The key advantage of this option is the ability to place the BSR reporting precision within the range of the buffered traffic volume that is expected from the XR application itself. Thus, with the same average BSR reporting overhead (e.g., 5 or 8 bits), the BSR reporting precision is enhanced, matching the traffic profile of each XR application. However, this option requires further specification efforts for determining and specifying the dynamic BSR parametrization and corresponding signaling updates.

We support option 2 for dynamic BSR parametrization and dynamic BSR reporting, respectively. Clearly with option 2, the forward as well as backward use-case compatibility are both maintained. That is, for future use cases of different traffic profiles and scheduling requirements, new BSR tables may not be needed since the dynamic BSR parametrization and dynamic table construction can be sufficient with little standardization efforts of the parameter value updates. Thus, with option 2, the BSR quantization precision can be dynamically matched to the most likely traffic volume range of each XR application, offering application-driven BSR reporting precision.

Proposal 2: We recommend defining dynamic BSR parametrization for dynamic BSR table formulation and reporting at each XR device, where the dynamically constructed BSR tables have various BSR reporting precisions which match the XR application traffic volume.

# BSR Table enhancements

## Introduce dynamically constructed BSR tables for XR traffic

As adopting a fixed and precise BSR quantization step seems to be a simple solution, with a minimal standardization effort, for enhancing the BSR quantization precision, it may significantly increase the BSR reporting overhead, especially for UEs that may not require such high BSR precision over the entire range of buffered traffic volume, e.g., URLLC UEs over high traffic volumes or eMBB UEs over low traffic volumes.

Observation 3: For XR uplink video uploading, the expected traffic size is large and thus, the higher BSR indices are to be used for most of the time. With lower BSR granularity over the higher traffic size ranges, the gNB needs several BSR reports to fully schedule existing uplink traffic in the buffer, extending the exhibited traffic buffering latency. This problem is likely not applicable for XR uplink traffic of pose and/or control updates.

Therefore, we propose a new BSR table format, which can be dynamically constructed at UEs based on a configured set of quantization steps versus the various BSR traffic size intervals. As depicted in Fig. 1, the entire buffered traffic volume of the BSR table is divided into traffic sub-ranges. For each traffic sub-range, a certain BSR step size is adopted. Therefore, the network can dynamically place a higher precision (i.e., a smaller BSR step size) only over the most likely experienced buffering traffic volume of each XR use-case, while adopting a looser BSR precision (i.e., with a larger BSR step size) over the other buffering traffic ranges. This avoids specifying hard-coded and use-case-specific BSR tables of predetermined BSR precision. With the proposed dynamic BSR table formatting, the reporting overhead can still be fixed, similar to existing 5 and 8 bit BSR tables. This is achievable by placing more BSR feedback levels (and therefore overhead) only over the traffic volume of interest, while compensating the added overhead by relaxing the BSR precision over the other buffered traffic ranges, leading to always fixed BSR reporting overhead.

The expected buffering traffic volume information, with which the traffic sub-ranges are determined, can be known at the network as:

- Explicit assistance signalling from UEs: devices signal the serving RAN nodes with an indication of the expected buffering traffic volume.

- Implicitly based on latest BSR reports from devices: network dynamically changes the BSR parametrization, and therefore, constructed BSR table, based on the latest received BSR reports from devices. Thus, the network may select a BSR parameters group that offers the best BSR reporting precision over the buffering traffic range which the latest BSR reports from the device have indicated.

Figure 1: Dynamic BSR table constriction and precision.

Observation 4: Expected buffering traffic volume information is essential at the network side for dynamic BSR precision adaptation. The network can be aware of this information whether by explicit device assistance signalling or from previous BSR reports during the active session.

Proposal 3: RAN2 to study and specify whether new signalling procedures are needed for carrying expected buffering traffic volume information from devices to the network.

As shown by Fig. 2, several BSR aggregate traffic size ranges are defined, where for each, a specifically configured quantization step size is used. This gives the network the control over which buffered traffic size range to assign a higher BSR quantization precision and which to relax the BSR quantization step. Based on the configured step size group and respective traffic size range set, UEs dynamically construct the corresponding BSR table and hence, similarly to conventional BSR reporting, UEs report the BSR index which corresponds to the actual traffic size interval.

Figure 2: Dynamic BSR table format.

In principle, the proposed BSR table format can be suitable for various types of traffic profiles. The network can still trade-off the BSR reporting overhead versus the desired BSR reporting precision. For example, for critical XR traffic, the network configures XR devices with a BSR step group that dictates a precise and fine-grained quantization step over the larger buffered traffic size ranges while adopting a relaxed, i.e., larger, step size for the lower traffic sizes. Hence, the BSR overhead can dynamically controlled while still offering precise BSR reporting over the traffic size that is most likely to be always experienced.

Proposal 4: For generality of BSR tables, RAN2 to kindly study that BSR step sizes and corresponding buffer size intervals can be dynamically configured and signalled from gNBs such as the appropriate BSR table is dynamically constructed at the UEs.

Proposal 5: Network configures devices with a BSR step size group, which indicates multiple BSR quantization steps, each corresponding to a certain buffered traffic size range. UEs accordingly dynamically construct the dynamic BSR tables.

# BSR reporting enhancements

Existing BSR can be triggered based on three triggering criteria as follows:

- Regular BSR: BSR reporting is triggered when a new and higher-priority PDU is available in the buffer and retxBSR-Timer is expired.

- Periodic BSR: BSR reporting is triggered when periodicBSR-Timer is expired.

- Padding BSR: BSR reporting is triggered when the padding bits are equal or larger than the size of the BSR MAC CE, and where the BSR can be piggybacked within an allocated uplink resource set.

Even if we assumed that XR traffic is always considered as higher priority than other traffic in the uplink buffers, still, multiple BSR transmissions may be needed to carry information on the actual available traffic in the buffers, which may jeopardize the radio latency target. Hence, new triggering criteria are needed for XR BSR reporting.

Proposal 6: new immediate triggering criteria of BSR reporting are needed to capture the case of higher priority XR traffic arrival in the buffer without waiting for the triggering timers to expire.

In one simple solution, the longer BSR format, i.e., 8-bit BSR table, is only triggered when there are data flows of multiple logical channels (LCH) in the buffers. When a single LCH is active, the short BSR report is triggered instead. Since the longer BSR format offers a more precise BSR accuracy and extended BSR traffic size ranges, we suggest triggering the longer BSR format even when there is a single active LCH in the buffer, for XR traffic.

Proposal 7: Change the long BSR reporting condition to be always reported even in case of a single LCH -presence.

The XR traffic is characterized by a target tight PDB, unlike eMBB traffic. However, existing BSR reports only include information of the buffered traffic size, regardless of the timing or latency limitations associated with this buffered traffic. So, in one option, a BSR report is triggered when the XR traffic buffering delay exceeds a predefined threshold. Or, in another option, a BSR reporting is triggered when the remaining PDB of the available buffered payload is below a threshold. This way, the reporting trigger of the BSR is dynamically tied to the critical delay targets of XR traffic.

Proposal 8: Consider buffered packet timing or latency information as triggering conditions for BSR reporting. In particular, UE triggers a BSR transmission when there is an XR PDU set available in the buffer and for which the buffering delay exceeds a predefined threshold.

Proposal 9: RAN2 to study and agree on potential latency/delay criteria for triggering BSR reporting. Examples include remaining PDB of an XR PDU set available in the buffer.

# Conclusions

This contribution has presented Dell’s views on enhancing the existing BSR reporting procedures, for XR capacity enhancements. The following observations and proposals are made:

Observation 1: Current BSR tables and reporting criteria are suitable for conventional eMBB services since those are not tied with a limited radio latency budget. Thus, the larger quantization errors of the larger buffer range are reduced in time by the multiple gNB scheduling instants.

Observation 2: Current BSR tables and reporting criteria should be further optimized to match the XR-specific traffic profile characteristics and latency targets. Thus, a more precise BSR quantization and reporting procedures are needed.

Proposal 1: RAN2 to specify new BSR formulation, reporting and triggering procedures that fit the XR traffic characteristics.

Proposal 2: We recommend defining dynamic BSR parametrization for dynamic BSR table formulation and reporting at each XR device, where the dynamically constructed BSR tables have various BSR reporting precisions which match the XR application traffic volume.

Observation 3: For XR uplink video uploading, the expected traffic size is large and thus, the higher BSR indices are to be used for most of the time. With lower BSR granularity over the higher traffic size ranges, the gNB needs several BSR reports to fully schedule existing uplink traffic in the buffer, extending the exhibited traffic buffering latency. This problem is likely not applicable for XR uplink traffic of pose and/or control updates.

Observation 4: Expected buffering traffic volume information is essential at the network side for dynamic BSR precision adaptation. The network can be aware of this information whether by explicit device assistance signalling or from previous BSR reports during the active session.

Proposal 3: RAN2 to study and specify whether new signalling procedures are needed for carrying expected buffering traffic volume information from devices to the network.

Proposal 4: For generality of BSR tables, RAN2 to kindly study that BSR step sizes and corresponding buffer size intervals can be dynamically configured and signalled from gNBs such as the appropriate BSR table is dynamically constructed at the UEs.

Proposal 5: Network configures devices with a BSR step size group, which indicates multiple BSR quantization steps, each corresponding to a certain buffered traffic size range. UEs accordingly dynamically construct the dynamic BSR tables.

Proposal 6: new immediate triggering criteria of BSR reporting are needed to capture the case of higher priority XR traffic arrival in the buffer without waiting for the triggering timers to expire.

Proposal 7: Change the long BSR reporting condition to be always reported even in case of a single LCH -presence.

Proposal 8: Consider buffered packet timing or latency information as triggering conditions for BSR reporting. In particular, UE triggers a BSR transmission when there is an XR PDU set available in the buffer and for which the buffering delay exceeds a predefined threshold.

Proposal 9: RAN2 to study and agree on potential latency/delay criteria for triggering BSR reporting. Examples include remaining PDB of an XR PDU set available in the buffer.

# References

- 3GPP TS 38.321: "NR; Medium Access Control (MAC); Protocol specification".