3GPP TSG-RAN WG2 Meeting #121	 R2-2300264

Athens GR, Feb 27- March 3, 2023

Agenda Item:	8.6.3.1

Souce:	MediaTek Inc.

Title:	Enhancements on neighbour cell measurement

Document for:		Discussion and Agreement

# 1 Introduction

RAN4 indicated RAN2 in LS[1] to ask for neighbour cell ephemeris: Similar as NR NTN, the mobility and measurement requirements for IoT NTN apply provided that valid information for the neighbour/target cell is made available to the UE.

RAN2#120 has made the following agreement:

Agreements:

- RAN2 doesn’t introduce neighbour cell ephemeris in Rel-17 IoT-NTN, neither for eMTC not for NB-IoT. RAN2 agrees to support this in Rel-18, with details FFS.

RP#98e has revised the Rel-18 WID [2], a new Work item was added.

The following mobility enhancements objectives are listed.

-	Support signalling in system information of neighbour cell ephemeris, for eMTC and NB-IoT [RAN2]

This contribution will discuss the aspects of neighbour cell ephemeris.

# 2 Discussion

In Rel-17, there are 3 different format of neighbour cell ephemeris defined in the specification. SSV and orbital parameters are used in SIB31 for serving cell UL compensation. The TLE format is used in SIB32 for predict the discontinuous coverage. The TLE format has a long validity duration, but the accuracy degrades a lot over time. Thus, TLE format is not suitable for neighbour cell ephemeris in the purpose of neighbour cell measurement. Therefore, the neighbour cell ephemeris can be put in the SIB31 in SSV and orbital format.

Proposal 1: Add the optional parameter of neighbour cell ephemeris in SIB31 in SSV and orbital format.

Frequency of the neighbour cell is required by UE to perform neighbour cell measurement, but the cell id list is not mandatory. Common TA as a part of the satellite assistant information is required to predict the timing error. The epoch time and the UL sync validity duration are also required to calculate the validity duration of the satellite assistant information. But the k-offset and k-mac are not required as data transmission is not expected between UE and neighbour cell.

Proposal 2: Other IEs are needed together with the neighbour cell ephemeris:

- The frequency and optional cell id list

- Common TA

- UL sync validity duration

- Epoch time

# 3 Conclusions

Proposal 1: Add the optional parameter of neighbour cell ephemeris in SIB31 in SSV and orbital format.

Proposal 2: Other IEs are needed together with the neighbour cell ephemeris:

- The frequency and optional cell id list

- Common TA

- UL sync validity duration

- Epoch time

# 4 References

- R4-2217265, RAN4, LS on information for neighbor/target cell in IoT NTN,RAN4#104-bis-e, Oct 2022.

- RP-223519, MediaTek, WID on IoT NTN Enhancements, RAN#98e, Dec 2022.