3GPP TSG RAN WG2 Meeting #121	R2-2300284

Athens, Greece

Feb. 27th – Mar. 3rd, 2023

Agenda item:	8.11.2.2

Source:	TD Tech, Chengdu TD Tech

Title:	Common signalling for multicast reception in RRC_INACTIVE state

Document for:	Discussion and Decision

- Introduction

During the RAN2#120 meeting, the following agreement on multicast reception in RRC_INACTIVE state was made for the 3GPP R18 WI “Enhancements of NR Multicast and Broadcast services”.

We will have a mixed approach and we start with the following:

    - When NW configures UE to continue the multicast reception in INACTIVE state, NW provides the PTM configuration for the activated multicast session via the RRC dedicated signalling, at least for the serving cell (FFS other cases).

    - MCCH is used in case there is a need to indicate a PTM configuration in case there is a need for change in PTM config or during mobility beyond serving cell / gNB. FFS session status change and other indications.

    - We assume that the UE can only receive multicast service after it joined the session.

    - FFS whether MCCH configuration is initially provided to the UE via dedicated signalling.

Based on the above agreement, how to send the common signalling for multicast reception in RRC_INHACTIVE state and how to send MCCH change notification are discussed with the related proposals suggested in the contribution.

- Common signalling for multicast reception in RRC_INACTIVE state

The common signalling for multicast reception in RRC_INACTIVE state includes:

- Multicast session activation notification

- RRC state switch from RRC_INACTIVE state to RRC_CONNECTED state

- Multicast session deactivation notification

- Multicast session release notification

- PTM configuration informations of a multicast session in each neighbour cell

- Configuration for UE to evaluate the reception quality of a multicast session

How to send the above common signalling is discussed in the section.

In the last RAN2 meetings, group paging was agreed to send multicast session activation notification. When a multicast session is activated, gNB may let some UEs receive the multicast session in RRC_CONNECTED state and the other UEs receive the multicast session in RRC_INACTIVE state. Therefore the following information needs to be sent with group paging.

- TMGI of activated multicast session: used to indicate to UE which multicast session is activated

- RRC_INACTIVE state indicator (optional)

This item is used to indicate to UE the activated multicast session can be received in RRC_INACTIVE state.

- PTM configuration of activated multicast session (optional)

If the PTM configuration of the activated multicast is not available in UE or updated upon the multicast session activation, this item is used to send the PTM configuration of the activated multicast session to UE.

- UE ID list for multicast reception in RRC_CONNECTED state or UE ID list for multicast reception in RRC_INACTIVE state (optional)

UE ID list for multicast reception in RRC_CONNECTED/ RRC_INACTIVE state is used to indicate which UEs need to switch to RRC_CONNECTED/ RRC_INACTIVE state for multicast reception. The remaining UEs need to switch to RRC_INACTIVE/ RRC_CONNECTED state.

- PRACH configuration information (optional): used by UE to initiate the random access procedure for multicast reception in RRC_CONNECTED state.

For RRC state switch from RRC_INACTIVE state to RRC_CONNECTED state, MCCH can be used. In detail, the following information can be sent on MCCH to make the related UE switch from RRC_INACTIVE state to RRC_CONNECTED state.

- TMGI of multicast session: used to indicate to UE which multicast session needs RRC state switch

- RRC_CONNECTED state indicator: used to indicate some/all UEs need to switch to RRC_CONNECTED state

- PTM configuration of multicast session

If the PTM configuration of the related multicast session is updated upon RRC state switch, this item is used to send the updated PTM configuration to UE.

- UE ID list for multicast reception in RRC_CONNECTED state or UE ID list for multicast reception in RRC_INACTIVE state (optional)

- PRACH configuration information (optional)

For multicast session deactivation notification, MCCH can be used. In detail, the following information can be sent on MCCH for multicast session deactivation notification.

- TMGI of deactivated multicast session: used to indicate to UE which multicast session is deactivated

- Deactivation indicator: used to indicate the related multicast session is deactivated.

- PTM configuration of multicast session (optional)

This item is used to send to UE the PTM configuration of the deactivated multicast session which can be used by UE when the deactivated multicast session is re-activated.

For multicast session release notification, MCCH can be used. In detail, the following information can be sent on MCCH to notify UE in RRC_INACTIVE state to release the related multicast session. For UE in RRC_CONNECTED state, the dedicated signalling can be used to send multicast session release notification.

- TMGI of released multicast session: used to indicate to UE which multicast session is released

- Release indicator: used to indicate the related multicast session is released.

For a multicast session in RRC_INACTIVE state, in order to support service continuity during UE mobility, the PTM configuration information of the multicast session in each neighbour cell can be provided to UE on MCCH. When UE in RRC_INACTIVE state acquires MCCH, UE can obtain the PTM configuration information of a joined multicast session in each neighbour cell and then use the PTM configuration information in a neighbour cell to continue receiving the joined multicast session in the neighbour cell without RRC state switch.

For the multicast session received by UE in RRC_INACTVE state, the configuration for UE to evaluate the reception quality of the multicast session can be sent to UE on MCCH. The configuration can include one item or several items of the following information:

      - Expected BLER of multicast session

If the calculated BLER of a multicast session by UE is larger than the expected BLER of the multicast session, UE concludes the reception quality can’t satisfy the QoS requirement of the multicast session.

      - RSRP/RSRQ threshold of multicast session: for a multicast session, if the calculated RSRP/RSRQ is larger than the given RSRP/RSRQ threshold, UE concludes the reception quality can’t satisfy the QoS requirement.

      - Duration for reception quality evaluation: indicate to UE a time interval for the evaluation of the BLER/RSRP/RSRQ of the multicast session

      - Permission indicator: indicate to UE whether or not UE is permitted to initiate a random access procedure for receiving the multicast session in RRC_CONNECTED state.

If UE finds the reception quality of a multicast session can’t satisfy the QoS requirement of the multicast session and UE is permitted to initiate a random access random procedure, it can initiate a random access procedure for receiving the multicast session in RRC_CONNECTED state.

Based on the above discussion, the following proposals are suggested.

Proposal 1: Group paging carries the following information for multicast session activation notification.

- TMGI of activated multicast session

- RRC_INACTIVE state indicator (optional)

- PTM configuration of activated multicast session (optional)

- UE ID list for multicast reception in RRC_CONNECTED state or UE ID list for multicast reception in RRC_INACTIVE state (optional)

- PRACH configuration information (optional)

Proposal 2: MCCH is used to notify UE of RRC state switch from RRC_INACTIVE state to RRC_CONNECTED state.

Proposal 3: MCCH is used to send multicast session deactivation notification.

Proposal 4: MCCH is used to send multicast session release notification.

Proposal 5: MCCH is used to send the PTM configuration of a multicast session in each neighbour cell.

Proposal 6: MCCH is used to send the configuration for UE to evaluate the reception quality of a multicast session.

Proposal 7: If permitted, UE can initiate a random access procedure for receiving a multicast session in RRC_CONNECTED state if UE concludes the reception quality of the multicast session can’t satisfy the QoS requirement of the multicast session.

- MCCH change notification

When the MCCH for indicating the PTM configuration of a multicast session in RRC_INACTIVE state is a cell specific MCCH, MCCH change notification can be sent under the following cases:

- When the PTM configuration of a multicast session is changed

- When the PTM configuration of a multicast session in a neighbour cell is changed

- When a multicast session needs some/all UEs to switch from RRC_INACTIVE state to RRC_CONNECTED state

- When a multicast session is deactivated or released

- When the configuration for UE to evaluate the reception quality of a multicast session is changed

When the MCCH for indicating the PTM configuration of a multicast session in RRC_INACTIVE state is a session specific MCCH, there’s no need to send MCCH change notification due to the following reasons:

- A session specific MCCH is configured per G-RNTI/G-CS-RNTI.

- For a G-RNTI/G-CS-RNTI, the session specific MCCH configured for the G-RNTI/G-CS-RNTI and the multicast sessions mapped onto the G-RNTI/G-CS-RNTI are scheduled by the G-RNTI/G-CSR-RNTI.

- UE monitors the G-RNTI/G-CS-RNTI for both the session specific MCCH and corresponding multicast sessions according to the corresponding DRX mode.

- UE has no need to monitor an extra RNTI for the session specific MCCH, thus there’s no need to send MCCH change notification for the session specific MCCH.

When the MCCH for indicating the PTM configuration of a multicast session in RRC_INACTIVE state is a cell specific MCCH, the cell specific MCCH is used to send the PTM configuration and common signalling of each multicast session in RRC_INACTIVE. The cell specific MCCH is different from the MCCH of broadcast sessions in the following aspects.

- UE needs to obtain the PTM configurations of all broadcast sessions and then select some broadcast sessions to receive according to its capability. Therefore, the MCCH of broadcast sessions is sent with a modification period and a repetition period. When UE acquires the MCCH, it can obtain the PTM configurations of all broadcast sessions.

- UE can join several multicast sessions. UE just needs to obtain the PTM configurations of the joined multicast sessions.

For the cell specific MCCH of multicast sessions, if the MCCH is configured via SIB, the content of the MCCH is transmitted periodically. Different multicast sessions may have different QoS requirements. If only one modification period and only one repetition period is configured for all multicast sessions, the modification period and repetition period must take smaller values to satisfy the QoS requirements of different multicast sessions, which means more radio resource consumption by the MCCH and more power consumption in UE for monitoring the MCCH.

In order to reduce the radio resource consumption by the MCCH and the power consumption in UE for the MCCH, a modification period, a repetition period, M>=1 transmission periods and M>=1 transmission times can be introduced for the MCCH. The modification period is denoted by Pm, the repetition period is denoted by Pr, the m-th transmission period is denoted by T(m) and the m-th transmission times is denoted by C(m), where Pm=2kPr, T(m)=2k(m)Pm, C(m)<=Pm/Pr, K>=0 is an integer, K(m)>=0 is an integer and m=1,….,M.

For a multicast session using the m-th transmission period T(m) and the m-th transmission times C(m), the PTM configuration information and common signalling of the multicast session are ONLY sent on the MCCH in the first C(m) repetition periods of the first modification period within each transmission period of T(m) ms long. The MCCH change notification for the multicast session is ONLY transmitted in the first C(m) repetition periods of the first modification period within the transmission period where the MCCH change notification for the multicast session is triggered.

For a UE, gNB can allocate a same transmission period and a same transmission times for the multicast sessions joined by the UE. The UE only needs to monitor the MCCH according to the allocated transmission period and the allocated transmission times instead of monitoring the MCCH according to the modification period and the repetition period.

An example with three transmission periods and three transmission times is illustrated in Figure1. In Figure 1, the following information is shown:

- The modification period is Pm=80ms, the repetition period is Pr=20ms and Pm/Pr=4.

- The services using T(1)=80ms and C(1)=4 have their configuration information and common signalling transmitted in the first C(1)=4 repetition periods of the first modification period within each transmission period of T(1)=80ms long. That is, among the four modification periods shown in Figure 1, the information for the services using T(1)=80ms and C(1)=4 is sent on the MCCH in each repetition period of the first/second/third/fourth medication period.

- The services using T(2)=160ms and C(2)=4 have their configuration information and common signalling transmitted in the first C(2)=4 repetition periods of the first modification period within each transmission period of T(2)=160ms long. That is, among the four modification periods shown in Figure 1, the information for the services using T(2)=160ms and C(2)=4 is sent on the MCCH in each repetition period of the first/third modification period.

- The services using T(3)=320ms and C(3)=2 have their configuration information and common signalling transmitted in the first C(3)=2 repetition periods of the first modification period within each transmission period of T(3)=320ms long. That is, among the four modification periods shown in Figure 1, the information for the services using T(3)=320ms and C(1)=2 is sent on the MCCH in the first C(3)=2 repetition periods of the first modification period.

- The PTM configuration of some service using T(3)=320ms and C(3)=2 is changed in the transmission period shown in Figure 1, the MCCH change notification is sent in the first C(3)=2 repetition periods of the first modification period within the transmission period.

Based on the above discussion, the following proposals are suggested.

Proposal 8: When the MCCH for indicating the PTM configuration of a multicast session in RRC_INACTIVE state is a cell specific MCCH, MCCH change notification can be sent under the following cases:

- When the PTM configuration of a multicast session is changed

- When the PTM configuration of a multicast session in a neighbour cell is changed

- When a multicast session needs some/all UEs to switch from RRC_INACTIVE state to RRC_CONNECTED state

- When a multicast session is deactivated or released

- When the configuration for UE to evaluate the reception quality of a multicast session is changed

Proposal 9: When the MCCH for indicating the PTM configuration of a multicast session in RRC_INACTIVE state is a cell specific MCCH and configured via SIB, a modification period Pm, a repetition period Pr, M transmission periods T(m), m=1=1,….,M and M transmission times C(m), m=1,……,M can be introduced. For a multicast session using the m-th transmission period T(m) and the m-th transmission times C(m), the PTM configuration information and common signalling of the multicast session is sent in the first C(m) repetition periods of the first modification period within each transmission period of T(m) ms long, where Pm=2kPr, T(m)=2k(m)Pm, C(m)<=Pm/Pr, K>=0 is an integer, K(m)>=0 is an integer, m=1,….,M, and M>=1.

- Conclusion

Based on the discussion in the former sections, the following proposals are suggested.

Proposal 1: Group paging carries the following information for multicast session activation notification.

- TMGI of activated multicast session

- RRC_INACTIVE state indicator (optional)

- PTM configuration of activated multicast session (optional)

- UE ID list for multicast reception in RRC_CONNECTED state or UE ID list for multicast reception in RRC_INACTIVE state (optional)

- PRACH configuration information (optional)

Proposal 2: MCCH is used to notify UE of RRC state switch from RRC_INACTIVE state to RRC_CONNECTED state.

Proposal 3: MCCH is used to send multicast session deactivation notification.

Proposal 4: MCCH is used to send multicast session release notification.

Proposal 5: MCCH is used to send the PTM configuration of a multicast session in each neighbour cell.

Proposal 6: MCCH is used to send the configuration for UE to evaluate the reception quality of a multicast session.

Proposal 7: If permitted, UE can initiate a random access procedure for receiving a multicast session in RRC_CONNECTED state if UE concludes the reception quality of the multicast session can’t satisfy the QoS requirement of the multicast session.

Proposal 8: When the MCCH for indicating the PTM configuration of a multicast session in RRC_INACTIVE state is a cell specific MCCH, MCCH change notification can be sent under the following cases:

- When the PTM configuration of a multicast session is changed

- When the PTM configuration of a multicast session in a neighbour cell is changed

- When a multicast session needs some/all UEs to switch from RRC_INACTIVE state to RRC_CONNECTED state

- When a multicast session is deactivated or released

- When the configuration for UE to evaluate the reception quality of a multicast session is changed

Proposal 9: When the MCCH for indicating the PTM configuration of a multicast session in RRC_INACTIVE state is a cell specific MCCH and configured via SIB, a modification period Pm, a repetition period Pr, M transmission periods T(m), m=1=1,….,M and M transmission times C(m), m=1,……,M can be introduced. For a multicast session using the m-th transmission period T(m) and the m-th transmission times C(m), the PTM configuration information and common signalling of the multicast session is sent in the first C(m) repetition periods of the first modification period within each transmission period of T(m) ms long, where Pm=2kPr, T(m)=2k(m)Pm, C(m)<=Pm/Pr, K>=0 is an integer, K(m)>=0 is an integer, m=1,….,M, and M>=1.

- References

- RP-221458, Revised WID: Enhancements of NR Multicast and Broadcast Services, RAN Meeting#96-e

- R2-2211271, Analysis of options for sending PTM configuration, RAN2#120, TD Tech, Chengdu TD Tech,

- R2-2206990, A new MCCH transmission method, RAN2#119-e, Chengdu TD Tech, TD Tech