3GPP TSG-RAN WG2 Meeting #121	R2-2300286

Athens, Greece, Feb 27 - Mar 03, 2023

Agenda Item:	8.11.2.1

Source:	MediaTek Inc.

Title:	Discuss on PTM configuration for multicast in RRC INACTIVE

Document for:	Discussion, Decision

Introduction

In the last meeting, RAN2 made the following agreement to introduce a mixed approach for multicast reception in RRC INACTIVE:

We will have a mixed approach and we start with the following: - When NW configures UE to continue the multicast reception in INACTIVE state, NW provides the PTM configuration for the activated multicast session via the RRC dedicated signalling, at least for the serving cell (FFS other cases). - MCCH is used in case there is a need to indicate a PTM configuration in case there is a need for change in PTM config or during mobility beyond serving cell / gNB. FFS session status change and other indications. - We assume that the UE can only receive multicast service after it joined the session. FFS whether MCCH configuration is initially provided to the UE via dedicated signalling.

In this contribution, we analyse the pros and cons for the parts of two options, then provides our views on the detailed design for PTM configuration delivery.

Discussion

Initialization of multicast reception in RRC INACTIVE

Authorization

In the last meeting’s agreement, we assume the UE can only receive multicast service after it joined the session:

… - 3. We assume that the UE can only receive multicast service after it joined the session.

In our understanding, the authorization procedure makes the network aware of UE which receives multicast service and makes sure only authorized UE can receive the corresponding multicast service. This is aligned with the design principle of multicast and also legacy behaviour. Therefore, we support the assumption and make the following proposal:

Proposal 1: UE shall join in the multicast session before receiving multicast in RRC INACTIVE.

The time of PTM configuration delivery

In the LS to SA2, we assume the RRC_INACTIVE UE can receive MBS data without going back to CONNECTED when session start, by “pre-configuring” the PTM configuration to UE.

Q5 When MBS Session is activated and MBS data allowed to be received in RRC_INACTIVE state, is it possible that the RRC_INACTIVE UE receives MBS data without going back to RRC connected state? … - It is possible that the RRC_INACTIVE UE receives MBS data without going back to RRC_CONNECTED state when the MBS session is being activated provided the UE has already joined the multicast session and the UE has valid MRB configuration. - …

Observation 1: According to the LS to SA2, RAN2 assumes that UE may receive multicast in INACTIVE state without going back to CONNECTED when session start, if the PTM configuration is pre-configured to UE.

Therefore, we suggest clarifying the following scenario for PTM configuration delivery before we further design the signalling:

Proposal 2: The PTM configuration can be configured to UE before the session start, and UE stored the configuration until session start. When session start, UE can receive multicast in INACTIVE state by restoring the configuration without going back to RRC_CONNECTED.

If proposal 2 is agreed, the PTM configuration and the information of whether UE can receive multicast in RRC_INACTIVE should also be provided to UE when UE join in the multicast session, to avoid additional signalling interaction. This prevents high signalling load where multiple UEs request PTM configuration simultaneously when session start.

Figure 1: Different time of PTM configuration is delivered

Proposal 3: At least the following information can be provided to UE when UE join in the multicast session:

- PTM configuration (for serving cell)

- Whether UE can receive multicast in RRC_INACTIVE

Dedicated signalling

The PTM configuration need to be delivered to UE when UE joins in the multicast service. In some scenarios, the multicast MCCH may not have the information for the updated MTCH information, or the multicast MCCH do not have the information of neighbour cell with the same service, or UE misses the multicast MCCH period (due to handover or configuration update). In those cases, the PTM configuration may also need to be delivered to UE by dedicated signalling.

No matter UE is in RRC CONNECTED state or RRC INACTIVE state, RRCRelease message with suspendconfig can be used to indicate PTM configuration. For RRC CONNECTED state, RRCRelease message (with suspendconfig) can be used to indicate both PTM configuration and state transition indication. For RRC INACTIVE state, RRCRelease message (with suspendconfig) can be used to indicate PTM configuration without letting UE fully resume to RRC CONNECTED. We think this is a general way for initial PTM configuration delivery.

Proposal 4: When network configures UE to receive multicast in INACTIVE state, RRCRelease message with suspendconfig can be used to deliver the PTM configuration.

MBS multicast configuration and multicast MCCH

RAN2 also agreed on the MCCH when PTM configuration is changed. This change of PTM configuration may be due to UE mobility or configuration update.

… - MCCH is used in case there is a need to indicate a PTM configuration in case there is a need for change in PTM config or during mobility beyond serving cell / gNB. FFS session status change and other indications. …

Firstly, we want to unify the terminology for the signalling and logical channel for multicast in RRC INACTIVE, since the term “MCCH” is used for broadcast in Rel-17.

Proposal 5: The term “MBS multicast configuration” is used for the message which indicates the configuration of MBS multicast session used for RRC INACTIVE.

Proposal 6: The term “multicast MCCH” is used for the logical channel providing MBS multicast configuration information for multicast reception in RRC INACTIVE.

Initial config of MBS multicast configuration

In Rel-17 broadcast, the parameters needed for the reception of MCCH are provided via system information. However, considering the security concerns and the authorization rules mentioned in section 2.1, we suggest that the same information for multicast MCCH is provided to UE via dedicated signalling (when initializing). This information can be merged into the initial multicast configuration for RRC INACTIVE to simplify the signalling interaction.

Proposal 7: When network configures UE to receive multicast in INACTIVE state, the MBS multicast configuration (i.e., the information carried in multicast MCCH) is provided to UE by RRC message. The RRC message can be RRCReconfiguration and/or RRCRelease(with suspend), which used for UEs in CONNECTED and INACTIVE state.

The content of MBS multicast configuration

Just as the legacy MCCH for broadcast, the multicast MCCH may provide the MTCH for multicast and associated information for multicast session (e.g., session ID, associated G-RNTI mapping, MRB config and neighbouring cells information), so that UE can acquire the MBS multicast configuration on MCCH.

However, considering the principle for multicast, the MBS multicast configuration should be provided to UE per service and only contains the configuration for certain service which has been authorized to UE. For one certain multicast session, the MBS multicast configuration should be shared to all UEs receiving the same multicast session in the cell. This is aligned with the legacy design and also saves resource efficiency.

Proposal 8: The MBS multicast configuration (i.e., the information carried in multicast MCCH) provides the MTCH and associated information for certain multicast session (per service).

Proposal 9: The MBS multicast configuration for a certain multicast session is common to all UEs which receive the same multicast session in the cell.

Proposal 10: UE can only receive MBS multicast configuration for the multicast session which UE has joined in. FFS detailed signalling design for multicast MCCH and MBS multicast configuration.

Configuration update during mobility

If the multicast MCCH is initially provided by dedicated signalling (rather than SIB), the information needed for the reception of MCCH for neighbouring cells providing the same multicast session should be also provided in MBS multicast configuration, so that MCCH for neighbour cell can be received by UE after UE moves to that cell in RRC INACTIVE. After UE moves to another cell, UE receives the same multicast session and updates MBS multicast configuration for that session from the new cell.

Proposal 11: The MBS multicast configuration (i.e., the information carried in multicast MCCH) provides the information needed for the reception of MCCH for neighbouring cells with the same multicast session

Proposal 12: UE can receive the same multicast session and update the MBS multicast configuration by the information of MBS multicast configuration from the previous cell after mobility.

Configuration update by network

If the configuration for multicast session is updated by the network, the legacy MCCH mechanism can be used to announce the change of the content for multicast MCCH. The network notifies UE about the change when MCCH modification period, UE then applies the new MBS multicast configuration in the updated multicast MCCH.

It should be noted that the UE should only be notified when the MBS multicast configuration of the session in which the UE has joined in is changed.

Proposal 13: The legacy mechanism (MCCH modification period, notification mechanism) can be used to notify UE about the change of the MBS multicast configuration in multicast MCCH. UE should only be notified when the MBS multicast configuration of the session in which the UE has joined in is changed.

CFR design discussion

In the previous meeting, regarding the physical resource for UE to receive multicast in RRC INACTIVE state, the following assumption/agreement was achieved:

- The following is taken as baseline: we assume the same PDCCH/PDSCH resources (e.g. resources used for MTCH) can be used for all UEs (including UEs in CONNECTED and/or INACTIVE states) for receiving the same multicast session. Different configuration/resources are not precluded as well. FFS what exactly can be common and what not (e.g. HARQ, SPS etc.) and what is needed in addition (to legacy PTM config).

For the UE receiving multicast in RRC connected state, the CFR is configured within the UE’s dedicated BWP as agreed in RAN1#104-e meeting:

Agreement in RAN1#104-e: From RAN1 perspective, the CFR (common frequency resource) for multicast of RRC-CONNECTED UEs, which is confined within the frequency resource of a dedicated unicast BWP and using the same numerology (SCS and CP), includes the following configurations: · Starting PRB and the number of PRBs · One PDSCH-config for MBS (i.e., separate from the PDSCH-Config of the dedicated unicast BWP) · One PDCCH-config for MBS (i.e., separate from the PDCCH-Config of the dedicated unicast BWP) · SPS-config(s) for MBS (i.e., separate from the SPS-Config of the dedicated unicast BWP) · FFS: Other configurations and details including whether signaling of starting PRB and the length of PRBs is needed when CFR is equal to the unicast BWP · FFS: Whether a unified CFR design is also used for broadcast reception for RRC_IDLE/INACTIVE and RRC_CONNECTED · FFS: Whether Coreset(s) for CFR in addition to existing Coresets in UE dedicated BWP is needed · Note: The terminology of CFR is only aiming for RAN1 discussion, and the detailed signaling design is up to RAN2 Note: This agreement does not negate any previous agreements made on CFR

However, for UE receiving multicast in RRC INACTIVIE state, whether the similar mechanism is reused is not clear/discussed, especially considering that the UE’s dedicated BWP in different RRC state can be different. For multicast reception in RRC INACTIVE, The CFR design/configuration is more crucial, and the corresponding discussion shall be prioritized. Considering the CFR design is RAN1’s expertise, we suggest to sending an LS to ask RAN1 for their views on the CFR design.

Proposal 14: Send an LS to RAN1 to ask for their views on CFR design for UE receiving the multicast in RRC INACTIVE state.

Conclusion

In this contribution, the following proposals are made:

Proposal 1: UE shall join in the multicast session before receiving multicast in RRC INACTIVE.

Proposal 2: The PTM configuration can be configured to UE before the session start, and UE stored the configuration until session start. When session start, UE can receive multicast in INACTIVE state by restoring the configuration without going back to RRC_CONNECTED.

Proposal 3: At least the following information can be provided to UE when UE join in the multicast session:

      - PTM configuration (for serving cell)

      - Whether UE can receive multicast in RRC_INACTIVE

Proposal 4: When network configures UE to receive multicast in INACTIVE state, RRCRelease message with suspendconfig can be used to deliver the PTM configuration.

Proposal 5: The term “MBS multicast configuration” is used for the message which indicates the configuration of MBS multicast session used for RRC INACTIVE.

Proposal 6: The term “multicast MCCH” is used for the logical channel providing MBS multicast configuration information for multicast reception in RRC INACTIVE.

Proposal 7: When network configures UE to receive multicast in INACTIVE state, the MBS multicast configuration (i.e., the information carried in multicast MCCH) is provided to UE by RRC message. The RRC message can be RRCReconfiguration and/or RRCRelease(with suspend), which used for UEs in CONNECTED and INACTIVE state.

Proposal 8: The MBS multicast configuration (i.e., the information carried in multicast MCCH) provides the MTCH and associated information for certain multicast session (per service).

Proposal 9: The MBS multicast configuration for a certain multicast session is common to all UEs which receive the same multicast session in the cell.

Proposal 10: UE can only receive MBS multicast configuration for the multicast session which UE has joined in. FFS detailed signalling design for multicast MCCH and MBS multicast configuration.

Proposal 11: The MBS multicast configuration (i.e., the information carried in multicast MCCH) provides the information needed for the reception of MCCH for neighbouring cells with the same multicast session

Proposal 12: UE can receive the same multicast session and update the MBS multicast configuration by the information of MBS multicast configuration from the previous cell after mobility.

Proposal 13: The legacy mechanism (MCCH modification period, notification mechanism) can be used to notify UE about the change of the MBS multicast configuration in multicast MCCH. UE should only be notified when the MBS multicast configuration of the session in which the UE has joined in is changed.

Proposal 14: Send an LS to RAN1 to ask for their views on CFR design for UE receiving the multicast in RRC INACTIVE state.

References

RP-213568 R18 MBS WID: Enhancements of NR Multicast and Broadcast Services

R2_120 MBS session notes (Dawid) 2022-11-18 1010_rm