3GPP TSG-RAN WG2 Meeting #121                                                       R2-2300293

Athens, GR, 27 Feb - 3 Mar 2023

Source:	CATT

Title:	Consideration on Inter-RAT Signaling Based Logged MDT Override Protection

Agenda Item:	8.13.3

Document for:	Discussion and Decision

Introduction

In RAN2#119bis-e meeting, signalling based logged MDT override protection in inter-RAT scenario was discussed and the following agreements were achieved [1].

=> The scenario when the UE is configured with NR Signaling-based logged MDT measurement configuration and reselects to E-UTRAN is excluded in R18 scope. => email discussion on solution direction UE capability based v.s. UE notification => RAN2 will investigate UE and NW impacts due to EUTRA MDT configuration override protection in inter-RAT scenario realized by simultaneous LTE and NR configuration in the UE. => FFS if the extension of the LTE LoggedMeasurementConfiguration (with Logged MDT type indication) is needed. => FFS Cross-RAT reporting for Logged MDT results (i.e. UE reports E-UTRAN logged MDT results in NR) is whether supported in R18. => Intra-EUTRA case will not be considered.

In this contribution, we will continue to discuss signalling based logged MDT override protection in inter-RAT scenario in section 2.

Discussion

- Solutions for inter-RAT signaling based logged MDT override protection

In RAN2#119bis-e meeting, solutions to address the inter-RAT scenario in R18 WID for signaling based logged MDT override protection had been discussed based on offline discussion [2], but no consensus was reached. The solutions were discussed in [2] mainly focus on the following:

Solution 1: Override protection by simultaneous LTE and NR configuration;

Solution 2: Override protection by cross-RAT signaling but no cross-RAT reporting;

Solution 3: Override protection by cross-RAT reporting;

For solution 1, if UE supports simultaneous LTE and NR logged MDT configuration and its reporting by allocating separate memory, when the UE reselects to NR cell from LTE cell, the UE and NR network does not need to know whether the MDT configuration is configured by LTE cell or not, less spec impact will be introduced. However, solution 1 breaks the legacy principles, i.e. UE keeps only a single MDT configuration (NR or LTE MDT configuration) and results in more memory at UE side. In addition, if solution 1 is supported, maybe more extra evaluation and consideration (e.g. UE capability) will be needed, which will introduce more complexity.

Solution 2 seems to be a simpler way which only needs to introduce a signaling based logged MDT indicator in E-UTRAN logged MDT measurement configuration, the UE could report the available indicator to NR network if there is E-UTRAN signaling based logged MDT measurement configuration or unretrieved report in UE as current R17 mechanism. The NR network itself will determine to override the LTE signaling based logged MDT configuration or measurement results if the NR network needs to configure the NR signaling based logged MDT configuration.

For solution 3, it will allow the inter-RAT logged MDT measurement results reporting which will change the current mechanism and introduce some complexity. Especially, we maybe need to discuss with RAN3 before making the final decision as the inter-RAT logged MDT measurement results need to be transferred in the interface, e.g. transfer from NR node to LTE node.

Based on the above considerations, we prefer solution 2 to achieve a simpler solution.

Proposal 1: RAN2 to agree override protection for LTE Logged MDT by cross-RAT signaling but no cross-RAT reporting when the UE moves to NR from LTE.

The following is the impacts from solution 2:

- Whether to extend the LTE LoggedMeasurementConfiguration

In RAN2#119bis-e meeting, if the extension of the LTE LoggedMeasurementConfiguration (with Logged MDT type indication) is needed was discussed but still is FFS now. If we go with solution 2, the extension of the LTE LoggedMeasurementConfiguration (with Logged MDT type indication) will be needed since the UE needs to know the MDT type to provide assistance information to NR network when the UE moves to NR cell from LTE cell.

Proposal 2: Extend the LTE LoggedMeasurementConfiguration to include Logged MDT type indication information.

- Assistance information reporting

If solution 2 is supported, the UE needs to send assistance information to NR network if the UE has LTE signaling based logged MDT configuration or unretrieved report available when the UE moves to NR cell from LTE cell. Similar as R17 mechanism, the UE can send assistance information by UL complete message with the following two methods:

Method 1: re-use the existing sigLogMeasConfigAvailable-r17 field;

Method 2: introduce a new field, e.g. sigLogMeasConfigAvailableLTE-r18;

Considering that the intention of the assistance information reporting is avoiding NR network sends management based logged MDT configuration to UE, the NR network doesn't need to know which MDT type the UE has stored. Therefore, the existing sigLogMeasConfigAvailable-r17 field is enough to achieve it.

We prefer Method 1 to achieve assistance information reporting.

Proposal 3: Re-use the existing sigLogMeasConfigAvailable-r17 field to achieve assistance information reporting for inter-RAT signaling based logged MDT override protection.

Conclusion

According to the discussion in section 2, we propose:

Proposal 1: RAN2 to agree override protection for LTE Logged MDT by cross-RAT signaling but no cross-RAT reporting when the UE moves to NR from LTE.

Proposal 2: Extend the LTE LoggedMeasurementConfiguration to include Logged MDT type indication information.

Proposal 3: Re-use the existing sigLogMeasConfigAvailable-r17 field to achieve assistance information reporting for inter-RAT signaling based logged MDT override protection.

Reference

- Draft_R2_119bis-e_meeting_report_v2

- R2-2210996 [801][R18 SON_MDT] MDT override solution direction (Nokia)