3GPP TSG-RAN WG2 Meeting #121	R2-2300327

Athens, Greece, February 27th – March 3rd, 2023

Agenda item:		8.16.2

Source:		Futurewei Technologies

Title:		Discussion on AI/ML model transfer and delivery

Document for:		Discussion and Decision

# Introduction

Model transfer and delivery was discussed during meeting #120 [1] and the group concluded that

For model transfer/delivery for AI/ML models (for the target use cases of this SI), RAN2 to study CP-based, UP-based solutions.

The topic has continuously been discussed after the meeting, in long email discussions ([Post120][053][AIML18]). The goal was to collect pros/cons on model transfer/delivery. It could also be used to collect comments on different architectural assumptions.

# Discussions

In this discussion we try to answer the following questions, in the scope of RAN1 use cases of AI/ML.

- What would be the common assumptions for this discussion?

- Which solution is better for model transfer/delivery, control plane based or use plane based?

- Which entity is better for model transfer/delivery, gNB or 5GC?

- Are the better solutions use case dependent, or conclusions can be generalized?

## High-Level Assumptions

To facilitate the discussion and put everyone on the same foundation, we would like to assume that the entity that transfers or delivers the models (e.g., gNB or CN) is also the entity that hosts, stores, and manages the models. We would also like to assume that the entity that does the model transfer/delivery initiates/triggers the transmission.

Proposal 1: Propose to have the common assumptions when discussing model transfer and delivery.

- The entity that does the model transfer/delivery (e.g., gNB or CN) is also the entity that hosts, stores, and manages the models.

- The entity that does the model transfer/delivery (e.g., gNB or CN) initiates/triggers the transmission; it does not relay the transfer/delivery for other entities (e.g., OAM), which should be considered as other cases.

## Control Plane Solution vs User Plane Solution

In RAN2 meeting 120 and the post-meeting email discussions, both CP-based and UP-based solutions have been discussed, covering basic signaling/message flows and pros/cons of each solution type. Here we try to compare both solutions and provide analysis to see which method is better for RAN1 use cases. The major pros and cons of each solution are compared in Table 1.

Table. 1 Comparison between Control Plane solutions and User Plan solutions

| Items Compared | Control Plane Solution | User Plane Solution |
| --- | --- | --- |
| Management of the model | gNB/CN hosts the models and manages the life cycle of the models (LCM) | gNB/CN hosts the models but the mechanism of managing the models at user plane was not clear. |
| Model Size | Challenging to support large size models due to CP message segmentation | Can support large size models |
| Simultaneous Multi-model Transfer and Delivery | Challenging | Capable |
| Latency | Shorter | Longer |
| Robustness | Strong | Less strong than the CP |
| Control Plane Traffic | Heavy, and can block the transmission of other important signaling messages unless a new SRB type is introduced. | Light, no signaling blockage issue. |
| Handover Impact | Maintain model transfer/delivery continuity during handover is challenging, especially at gNB. | Better handling of model transfer/delivery during mobility. |
| Failure Impact | Solution not clear when RLF causes interruption of model transfer/delivery | Model transmission continuity won’t be interrupted based on current mechanism |
| Spec Impact | RRC segmentation to support large model; may need to define new SRB; may impact spec of SA2/CT1. | Impact to SA2/CT1 is unclear for UP-based solution. |

From Table 1 we can see CP-based solutions have some major issues making them not suitable for AI/ML model transfer and delivery, at least for RAN1 use cases.

- The biggest issue is that the use of CP will unavoidably introduce heavy traffic to the control plane, as the size of model files could go beyond 100s of MBs. In fact, in our view, model files should not be treated as control signals. The use of CP to transmit model files has potential negative impact of blocking other important control messages in the transmission queue. More importantly, this issue has no easy fix.

- The second big issue is that the CP/RRC signalling was not originally designed to transmit large data such as model files. This will cause data segmentation issue of RRC signalling and needs to be addressed if this approach is used.

- There are other serious issues associated with this approach, such as maintaining model transfer/delivery continuity during handover, as well as lacking the flexibility of transfer/deliver multiple models files simultaneously.

On the other hand, comparing to CP-based solutions, the major drawback of the UP-based solutions is longer latency. However, if the source of transfer/delivery is gNB, the latency is not much longer than the CP/gNB solution. The issues associated with the CP-based solutions are not issues of the UP-based solutions, or at least much less serious.

Observation 1: CP-based solution has more serious issues and they are much harder to solve, such as the CP blockage issue which is inherited with the approach. User plane solution is more suitable for model transfer/delivery for RAN1 use cases such as CSI feedback and beam management.

Proposal 2: Prioritize UP-based solutions for model transfer/delivery for RAN1 use cases.

## gNB vs CN

Once we have found out that the UP is the better place for model transfer/delivery for RAN1 use cases, the questions we would like to answer are,

- Where is the best place to host model depository and manage them?

- Is the answer use case dependent?

We break these questions into different aspects and list the answers in Table 2.

Table. 2 Comparison between gNB and CN as model depository and managing entity

| Aspects Compared | gNB |  | CN |  |
| --- | --- | --- | --- | --- |
|  | CP | UP | CP | UP |
| Better place to store the models? | No. gNB’s major function is to provide access. |  | Yes. But only good for storage, not for management. |  |
| Better place for LCM? | Yes. | No (needs a way to make management from the UP possible) | No. Not for air interface related use cases, e.g., CSI feedback and BM. |  |
| Better for large model? | Worst | Best | Better | Best |
| Better mobility support? | No | Yes | Yes | Yes |
| Control Plane overhead | High | Low | High | Low |
| Control Plane Blockage | Yes | No | Yes | No |
| Able to initiate the transfer/delivery? | Yes | No | Yes | No |
| Better place for training? | Yes: Easy to get training data. No: gNB’s major function is to provide access. This is extra burden. |  | No. Exposure of RAN measurements to CN is limited. |  |
| Transmission latency | Short |  | Longer |  |
| Architecture Issues? | No, but needs minor enhancement | Yes, there is no UP termination at gNB. | No |  |

Comparing the two entities for model transfer and delivery, the answer is not that straightforward. Given the disadvantages we listed above for CP-based solutions, here we mainly compare them on the UP aspects. Some of the key aspects to consider include

- gNB is not the best place for storing the models but it is a better place than CN for model LCM; having the CN managing the models for RAN1 air interface related use cases just does not make sense. In addition, although managing AI/ML models at the CP and transfer/deliver them through UP will need some signalling between the two, it is doable by enhancing the standard.

- Although training the models at gNB increases the burden of gNB, it is the right place to get training data as the use cases of concern are for the PHY layer; the training data will come from the PHY layer, which resides at gNB. For other use cases, such as positioning, the better training place could be at the CN as CN has the positioning data available for training. In this case, CN would be the better place to host the models.

- The UP/gNB-based solution does have an obvious disadvantage; the current protocol stack does not support UP data terminates at gNB, instead, it will terminate at UPF at the CN. However, this is not an issue unsolvable, given that starting from 5G, the Service Based Architecture (SBA) is there for us to utilize. We will cover this at the next section.

- For other aspects, gNB and CN may have similar performance using the UP.

Observation 2: Whether models should be stored and managed at gNB or CN (and get transferred/delivered from there) depends on the use cases the models are applied for. For air interface related use cases, such as CSI feedback and beam management, gNB could be a better place. For positioning and other use cases where data is readily available at the CN, CN may be a better place.

Proposal 3: Prioritize UP based solution at gNB for CSI feedback and beam management use cases.

# Conclusions

For the discussion of AI/ML model transfer/delivery, the followings are observed and proposed:

Proposal 1: Propose to have the common assumptions when discussing model transfer and delivery.

- The entity that does the model transfer/delivery (e.g., gNB or CN) is also the entity that hosts, stores, and manages the models.

- The entity that does the model transfer/delivery (e.g., gNB or CN) initiates/triggers the transmission; it does not relay the transfer/delivery for other entities (e.g., OAM), which should be considered as other cases.

Observation 1: CP-based solution has more serious issues and they are much harder to solve, such as the CP blockage issue which is inherited with the approach. User plane solution is more suitable for model transfer/delivery for RAN1 use cases such as CSI feedback and beam management.

Proposal 2: Prioritize UP-based solutions for model transfer/delivery for RAN1 use cases.

Observation 2: Whether models should be stored and managed at gNB or CN (and get transferred/delivered from there) depends on the use cases the models are applied for. For air interface related use cases, such as CSI feedback and beam management, gNB could be a better place. For positioning and other use cases where data is readily available at the CN, CN may be a better place.

Proposal 3: Prioritize UP based solution at gNB for CSI feedback and beam management use cases.

# References

[R2-2212226], Discussion on AIML methods, Huawei/HiSilicon, contribution to RAN2 meeting #120