3GPP TSG-RAN2 Meeting #121	R2-2300336

Athens, Greece, Feb 27- Mar 3, 2023

Agenda Item:	8.11.2.2

Work Item:	NR_MBS_enh-Core

Source: 	Qualcomm Incorporated

Title: 	Notifications and RRC state transitions for multicast reception

in RRC_INACTIVE

Document for:	Discussion/Decision

# Introduction

Rel-17 MBS added support of multicast services reception by UEs in CM_CONNECTED/ RRC_CONNECTED state only. One of the objectives of Rel-18 WI on enhancements of NR MBS is as follows (highlighting added), see RP-221458:

This Work Item is to further enhance the NR Multicast/Broadcast functions based on Rel-17 MBS. The objectives for Rel-18 include: - Specify support of multicast reception by UEs in RRC_INACTIVE state [RAN2, RAN3] - PTM configuration for UEs receiving multicast in RRC_INACTIVE state [RAN2] - Study the impact of mobility and state transition for UEs receiving multicast in RRC_INACTIVE. (Seamless/lossless mobility is not required) [RAN2, RAN3] - <<skip>> Note: collaboration with SA2 is expected in due course for the above objectives.

RAN2#119e discussed the options to provide Multicast configuration for INACTIVE state, and agreed the following:

For PTM configuration delivery, RAN2 further investigates the following solutions:

- Option 1: Dedicated signalling

- Option 2: Solution based on SIB+MCCH

- We do not preclude some “mix” of the options

During RAN2#119bis-e, several agreements were made applicable to both options. However, there was no conclusion on which option to choose.

RAN2#120 further discussed the options and agreed the following:

We will have a mixed approach and we start with the following:

    - When NW configures UE to continue the multicast reception in INACTIVE state, NW provides the PTM configuration for the activated multicast session via the RRC dedicated signalling, at least for the serving cell (FFS other cases).

    - MCCH is used in case there is a need to indicate a PTM configuration in case there is a need for change in PTM config or during mobility beyond serving cell / gNB. FFS session status change and other indications.

    - We assume that the UE can only receive multicast service after it joined the session.

    - FFS whether MCCH configuration is initially provided to the UE via dedicated signalling.

In this contribution, we provide further views on the outstanding issues for multicast reception in RRC_INACTIVE state particularly on the notifications to the UE and RRC state transitions.

# RRC State Transitions

## Transition from RRC_CONNECTED to RRC_INACTIVE

RAN2 previously agreed the following:

In RAN2#119e

It is up to gNB to decide whether a multicast session may be received by UE(s) in INACTIVE. FFS what information gNB may be provided to form such decision (related to SA2 discussion).

It is supported that gNB transmit one multicast session to both UEs in CONNECTED and INACTIVE in the same cell. FFS how the gNB configures this.

It is assumed the network can choose which UEs receive in RRC INACTIVE and which in RRC Connected and can move UEs between the states for Multicast service reception.

In RAN2#119bis-e

Dedicated RRC signalling (i.e. RRC release message with suspendConfig) is used for switching a multicast receiving UE from RRC_CONNECTED to RRC_INACTIVE and continue multicast reception (details FFS).

Regarding the “FFS what information gNB may be provided to form such decision (related to SA2 discussion)”, there are two aspects of this:

- Information provided from 5GC to gNB

- Information provided from UE to gNB

Regarding information provided from 5GC to gNB, we understand SA2 has made some conclusions and there is ongoing discussion in RAN3 on what information needs to be passed between CN and RAN. Without going to those details, we can leave it up to RAN3 and SA2.

Leave it up to RAN3 and SA2 for information provided from 5GC to gNB to facilitate gNB to decide whether a multicast session may be received by UE(s) in RRC_INACTIVE.

Regarding information provided from UE to gNB, in previous RAN2 meetings, some companies proposed UE to provide its preference whether to receive a multicast service in RRC_CONNECTED vs RRC_INACTIVE state. In our view decision on whether a UE should receive multicast service in RRC_INACTIVE state is totally up to RAN decision as long as the UE is capable of supporting the feature.

UE indicates capability on whether it supports MBS multicast reception in RRC_INACTIVE.

Other than the UE capability indication, UE does not provide additional indication to RAN about its preference to receive multicast in RRC_INACTIVE state (i.e. it is up to network decision).

## Transition from RRC_INACTIVE to RRC_CONNECTED

RAN2#119bis-e reached several agreements regarding usage of group paging:

For both option 1 and option 2, as a baseline, group paging can be used to switch UEs receiving multicast from RRC_INACTIVE to RRC_CONNECTED, and UEs continue the multicast reception in CONNECTED. FFS if there is any potential issue if Rel-17 group paging is reused. FFS if there are other cases when UE triggers resume. FFS if MCCH can also be used in case of option 2.

Rel-18 UE in INACTIVE can be informed when the session is activated (Details FFS).

As a baseline, group paging can be used to inform Rel-18 UE(s) about the session activation (Details FFS, e.g., UE behavior when receiving such group notification).

If a UE is in RRC_INACTIVE and is configured to receive a multicast session in RRC_INACTIVE, the UE may be notified when the multicast session is deactivated. FFS how (e.g., informed via group paging, MCCH, or other ways).

FFS how UE determines whether it can receive the multicast session in RRC_INACTIVE or not when the session is activated, taking into account the following solutions (can further update the descriptions if needed, and several solutions may be needed, some solutions may apply only for certain configuration options)

- 1. When the multicast session is activated, UE can receive the multicast session in RRC_INACTIVE if the PTM configuration used in RRC_INACTIVE for the session is available to the UE and the UE has joined the session already (e.g., configuration provided to UE via dedicated RRC signaling or via MCCH), otherwise it goes back to RRC_CONNECTED to receive the multicast session.

- 2. When the multicast session is activated, UE is indicated by group paging whether it can receive the multicast session in RRC_INACTIVE or not (detailed signaling FFS).

- 3. UE is configured "whether it can receive the multicast session in RRC_INACTIVE" by dedicated signaling before UE is released. When the multicast session is activated, UE stays in RRC_INACTIVE or resumes RRC connection accordingly (detailed signaling FFS).

If option 1 is supported for PTM configuration

- As a baseline, group paging may be used to inform the UE when network changes the PTM configurations, and UE upon reception triggers RRC connection resume procedure to obtain the updated configurations (details of group paging can be FFS).

### Multicast session activation while UE is in RRC_INACTIVE

Based on the discussion during RAN2#119bis-e and agreements shown above, there are at least the following scenarios which the group paging should be able to unambiguously indicate to the multicast UE in RRC_INACTIVE (which has already joined the multicast session and has valid PTM configurations):

- The multicast session was not activated and now it is activated:

  - All applicable UEs continue to receive the service in RRC_INACTIVE, or

  - All applicable UEs move to RRC_CONNECTED to receive the service, or

  - Specific UEs move to RRC_CONNECTED to receive the service (in accordance with the agreement in RAN2#119e)

- The multicast session was already activated but was in temporary data inactivity, now the inactivity is ended:

  - All applicable UEs continue to receive the service in RRC_INACTIVE, or

  - All applicable UEs move to RRC_CONNECTED to receive the service, or

  - Specific UEs move to RRC_CONNECTED to receive the service (in accordance with the agreement in RAN2#119e)

To fulfil these different scenarios, a single flag per TMGI should be enough in group paging to indicate whether all the UEs receiving the service with a given TMGI should stay in RRC_INACTIVE and continue to receive the service, or all the UEs receiving the service with a given TMGI should move to RRC_CONNECTED. However, to move a selected subset of UEs (i.e. 1c and 2c above), the specific UEs need to be notified, which can be done using legacy paging.

A single flag per TMGI should be enough in group paging to indicate whether all the UEs receiving the service with a given TMGI should stay in RRC_INACTIVE and continue to receive the service, or all the UEs receiving the service with a given TMGI should move to RRC_CONNECTED.

Legacy UE-specific paging can be (re)used to move specific UE(s) in RRC_INACTIVE to RRC_CONNECTED.

Therefore, we propose the following:

Reuse Rel-17 group paging to indicate activation (or end of inactivity) of a service. Upon such indication, UEs in RRC_INACTIVE that have not joined the multicast session but interested to receive it need to move to RRC_CONNECTED (i.e. similar to Rel-17).

Add a per-TMGI flag in Rel-18 group paging to indicate whether all UEs (that have joined the multicast session and have valid PTM configuration) continue to receive the service associated with the TMGI in RRC_INACTIVE or should move to RRC_CONNECTED.

UE-specific paging (i.e. PagingRecordList) can be (re)used to move specific UE(s) to RRC_CONNECTED. This overrides the per-TMGI flag in the group paging for the specific UE(s). FFS whether new paging cause is needed.

### Multicast session deactivation while UE is in RRC_INACTIVE

In Rel-17, upon receiving multicast deactivation indication from 5GC, RAN will release all radio resources and inform UEs by releasing MRB configuration via dedicated RRC signaling procedure.

In case of Rel-18, for RRC_INACTIVE UEs, it should be possible for gNB to indicate the multicast session deactivation to multicast RRC_INACTIVE capable UEs by using either Group Paging method or by using MCCH based method.

Both group paging and MCCH-based options are possible to indicate deactivation of multicast session while UE is in RRC_INACTIVE.

RAN2 previously agreed that MCCH is used to indicate PTM configuration change while the UE is in RRC_INACTIVE. Similarly, deactivation of the session can be indicated by MCCH.

For UEs receiving multicast in RRC_INACTIVE, deactivation of the multicast session is indicated by MCCH.

# Summary

Based on the discussion above, we have following observations:

Observation 1.	A single flag per TMGI should be enough in group paging to indicate whether all the UEs receiving the service with a given TMGI should stay in RRC_INACTIVE and continue to receive the service, or all the UEs receiving the service with a given TMGI should move to RRC_CONNECTED.

Observation 2.	Legacy UE-specific paging can be (re)used to move specific UE(s) in RRC_INACTIVE to RRC_CONNECTED.

Observation 3.	Both group paging and MCCH-based options are possible to indicate deactivation of multicast session while UE is in RRC_INACTIVE.

Based on these observations and discussion above, we have the following proposals:

Proposal 1.	Leave it up to RAN3 and SA2 for information provided from 5GC to gNB to facilitate gNB to decide whether a multicast session may be received by UE(s) in RRC_INACTIVE.

Proposal 2.	UE indicates capability on whether it supports MBS multicast reception in RRC_INACTIVE.

Proposal 3.	Other than the UE capability indication, UE does not provide additional indication to RAN about its preference to receive multicast in RRC_INACTIVE state (i.e. it is up to network decision).

Proposal 4.	Reuse Rel-17 group paging to indicate activation (or end of inactivity) of a service. Upon such indication, UEs in RRC_INACTIVE that have not joined the multicast session but interested to receive it need to move to RRC_CONNECTED (i.e. similar to Rel-17).

Proposal 5.	Add a per-TMGI flag in Rel-18 group paging to indicate whether all UEs (that have joined the multicast session and have valid PTM configuration) continue to receive the service associated with the TMGI in RRC_INACTIVE or should move to RRC_CONNECTED.

Proposal 6.	UE-specific paging (i.e. PagingRecordList) can be (re)used to move specific UE(s) to RRC_CONNECTED. This overrides the per-TMGI flag in the group paging for the specific UE(s). FFS whether new paging cause is needed.

Proposal 7.	For UEs receiving multicast in RRC_INACTIVE, deactivation of the multicast session is indicated by MCCH.