3GPP TSG-RAN WG2 #121 	R2-2300348

Athens, Greece, 27th February – 3rd March, 2023

Title:		Signalling design for the side control information

Source:	Huawei, HiSilicon

Agenda item:	8.1.2

Document for:	Discussion

- Introduction

According to Rel-18 NCR WID [1], the following side control information for controlling the NCR-Fwd is supported from gNB to NCR:

- Beamforming - UL-DL TDD operation - ON-OFF information

For the side control information of beamforming, RAN1 has the following agreements [2]:

- The semi-static beam indication for backhaul link is supported as: - If the beam indication framework in Rel-15 is used for NCR-MT - The DL beam is indicated by MAC CE to select one of TCI state ID from the RRC-configured list of beams for C-link - The UL beam is indicated by SRI on C-link via MAC CE. - If the beam indication framework in Rel-17 is used for NCR-MT - The DL and UL beam are indicated by MAC CE to select one of TCI state ID from the RRC-configured list of beams for C-link - The following pre-defined rules are applied to determine the beam for backhaul link: - In the time domain resource with simultaneous downlink reception or uplink transmission in C-link and backhaul link, the beam of backhaul link is the same as the beam of C-link regardless whether there is beam indicated by the dedicated signal for backhaul link. - In the time domain resource without simultaneous downlink reception or uplink transmission in C-link and backhaul link, if the NCR does not support capability with the new signalling for backhaul beam indication or if no beam is indicated for backhaul link by the dedicated signal, - When Rel-15/16 beam indication framework is used for C-link, - The beam determined by QCL assumption for CORESET with the lowest ID and spatial relationship for PUCCH with lowest PUCCH resource ID in the C-link is applied for the DL and UL of backhaul link, respectively. - When Rel-17 beam indication framework (i.e., unified TCI framework) is used for C-link, the indicated unified TCI for C-link DL and UL is applied for the DL and UL of backhaul link, respectively. Otherwise, the beam indicated by the dedicated signalling is applied for backhaul link. - For each periodic beam indication for access link, one RRC signalling is used including the information defined by the following: - A list of X (1≤X≤Xmax) forwarding resource, each is defined as {Beam index, time resource} - FFS: The value of Xmax, other details Each time resource is defined by {Starting slot defined as the slot offset in one period, starting symbol defined by symbol offset within the slot, duration defined by the number of symbols} with dedicated field. - The periodicity is configured as part of the RRC signaling for periodic beam indication - The same periodicity is assumed for all time resource(s) in one periodic beam indication. - The reference SCS is configured as part of the RRC signaling for periodic beam indication - The same reference SCS is assumed for all time resource(s) in one periodic beam indication. - For each aperiodic beam indication for access link, one DCI is used with the information defined by Option-1: - Lmax fields are used to indicate the beam information and each field refers to one beam index ; - Note: The bitwidth of this field is determined by the number of beams used for access link. - Tmax fields to indicate the time resource; - Note: A list of time resource is pre-defined by RRC signalling. The bitwidth of this field for time resource indication is determined by the length of list. - FFS: The value of Tmax - Down-select between Tmax=1 or Tmax=Lmax. - FFS: How to define the association between time indication and beam indication Each time resource is defined by {Starting slot defined as the slot offset, starting symbol defined by symbol offset within the slot, duration defined by the number of symbols} with dedicated field.

According to RAN1 agreements, the semi-static beam for backhaul link, the periodic beam for access link, and the aperiodic beam for access link are indicated by MAC CE, RRC, and DCI, respectively. In this paper, we will discuss the RRC and MAC related design to support the side control information of beamforming, and give a TP for TS 38.321.

- Discussion

2.1 Backhaul link/C-link beam indication

For the backhaul link beam indication, currently RAN1 only agreed to support the semi-static indication. When there is simultaneous transmission in backhaul link and C-link, the beam used by backhaul link is same as the beam used by C-link. And when there isn’t simultaneous transmission in backhaul link and C-link, the dedicated backhaul beam indication can be used to select one beam from the RRC configured C-link beams, which is carried by the MAC CE. The C-link beams of NCR-MT are indicated by RRC signaling based on the legacy mechanism, e.g., via the ServingCellConfig in RRCReconfiguration.

The RAN1 agreement also states that the format of beam indication varies for the UL/DL directions and Rel-15/Rel-17 frameworks. For DL, the TCI state ID is used for both Rel-15 and Rel-17; for UL, the SRI is used for Rel-15 and the TCI state ID is used for Rel-17. In this case, there is no need to define 4 MAC CEs for this purpose, since the Rel-15/Rel-17 frameworks are not simultaneously used by an NCR at a time. In our view, one MAC CE for the Rel-15/Rel-17 UL backhaul beam and one MAC CE for the Rel-15/Rel-17 DL backhaul beam are sufficient. The Rel-15/Rel-17 framework information is indicated by the RRC configuration as follows:

- The NCR-MT reports its capability of beam management to the gNB.

- For the NCR-MT supporting Rel-17 features, the gNB can send RRC configurations to it with Rel-17 IEs or Rel-15 IEs. The NCR-MT can infer whether the beam indication is based on Rel-17 or Rel-15 by the content in RRC configurations. If the Rel-17 IEs are configured, the NCR-MT should use the format of Rel-17 based beam indication. Otherwise, the format of Rel-15 based beam indication is used.

- For the NCR-MT only supporting Rel-15 features, the gNB just sends Rel-15 RRC configurations to it.

In summary, the NCR-MT knows whether the beam is indicated using Rel-15 or Rel-17 framework based on the RRC configuration, and knows whether the beam indication is for DL or UL transmission by the eLCID. In this case, the MAC CE may just include the field of TCI state ID or SRI. This field gives the indication of a specific beam. According to RAN1 agreements and TS 38.331[3],

- for Rel-15 DL beam , it should be a TCI state ID which needs 7 bits;

- for Rel-15 UL beam, it should be an SRI which needs 4 bits.

- for Rel-17 DL beam, it should be a TCI state ID which needs 7 bits;

- for Rel-17 UL beam, it should be a TCI state ID which needs 6 bits;

Therefore, the DL beam ID field length is set as 7 bits; the UL beam ID field length is set as 6 bits, and for SRI, 4 bits are used and the remaining 2 bits are reserved. The formats of MAC CEs are given as follows:

(a) downlink                                       (b) uplink

Figure 1. NCR backhaul link beam indication MAC CEs.

We give the TP of 38.321 in the Appendix, including the formats and the handling of the MAC CEs.

Proposal 1: Adopt the NCR backhaul link beam indication MAC CEs in Figure 1 and the corresponding TP of 38.321 in Appendix.

2.2 Access link beam indication

According to RAN1 agreements, the access link beam indication includes periodic beam indication and aperiodic beam indication. The aperiodic beam is indicated by DCI which is in RAN1 scope. In this part, we mainly focus on the RRC signaling for the periodic beam indication.

For the periodic access beam indication, both single beam index per indication and multiple beams per indication are supported. The RRC signaling should include a list of X (1≤X≤Xmax) forwarding resource as stated by the RAN1 agreement. And each forwarding resource is defined as a combination of beam index and time resource. As for the relationship between the beam index and the physical beam information, it is pre-configured on the gNB and the NCR by OAM. Besides, the periodicity and the reference SCS should also be included to indicate the period and slot length. In summary, we propose:

Proposal 2: The structure of RRC signaling for the periodic access beam indication should be designed as {{Beam index, Time resource}1, …, {Beam index, Time resource}X, periodicity, SCS}.

3.	Conclusion

This paper mainly discusses the RRC and MAC related parts for supporting the beam indication. We give the MAC CE design for the semi-static backhaul link beam indication and RRC signaling design for the periodic access beam indication.

Proposal 1: Adopt the NCR backhaul link beam indication MAC CE in Figure 1 and the corresponding TP of 38.321 in Appendix.

Proposal 2: The structure of RRC signaling for the periodic access beam indication should be designed as {{Beam index, Time resource}1, …, {Beam index, Time resource}X, periodicity, SCS}.

4.	References

- RP-222673, “New WID on NR network-controlled repeaters”, ZTE, Sanechips, RAN#97e, September 12-16, 2022.

- Chair’s notes, TSG-RAN WG1 Meeting #111, Toulouse, France, November 2022.

- 3GPP TS 38.331: “Radio Resource Control (RRC) protocol specification”.

5. Appendix: TP for TS 38.321 BL CR

---------------------------------------------------Start of change---------------------------------------------------

5.18	Handling of MAC CEs

UNCHANGED PART OMITTED

5.18.xx	Downlink backhaul beam indication for NCR

NCR Downlink Backhaul Beam Indication MAC CE is used by a gNB to indicate to an NCR the beam to be used for the downlink backhaul transmission.

Upon reception of an NCR Downlink Backhaul Beam Indication MAC CE, the NCR shall:

-	indicate to lower layers the information regarding the NCR Downlink Backhaul Beam Indication MAC CE.

5.18.xx	Uplink backhaul beam indication for NCR

NCR Uplink Backhaul Beam Indication MAC CE is used by a gNB to indicate to an NCR the beam to be used for the downlink backhaul transmission.

Upon reception of an NCR Uplink Backhaul Beam Indication MAC CE, the NCR shall:

-	indicate to lower layers the information regarding the NCR Uplink Backhaul Beam Indication MAC CE.

------------------------------------------------------Next change---------------------------------------------------

6.1	Protocol Data Units

UNCHANGED PART OMITTED

6.1.3.xx	NCR Downlink Backhaul Beam Indication MAC CE

The NCR Downlink Backhaul Beam Indication MAC CE is identified by MAC subheader with eLCID as specified in Table 6.2.1-1b. It has a fixed size and consists of a single octet defined as follows (Figure 6.1.3.xx-1):

-	Downlink TCI State ID: This field contains TCI-StateId (comprising all 7 bits), as specified in TS 38.331 [5], of a TCI State, which is used as the downlink beam indication for backhaul link transmission.

Figure 6.1.3.xx-1: NCR Downlink Backhaul Beam Indication MAC CE

6.1.3.xx	NCR Uplink Backhaul Beam Indication MAC CE

The NCR Uplink Backhaul Beam Indication MAC CE is identified by MAC subheader with eLCID as specified in Table 6.2.1-1b. It has a fixed size and consists of a single octet defined as follows (Figure 6.1.3.xx-1):

-	Uplink TCI State ID or SRI: This field contains TCI-UL-State-Id (comprising all 6 bits) of a TCI State, which is used as the uplink beam indication for backhaul link transmission, if the ul-TCI-StateList is configured, as specified in TS 38.331 [5]; otherwise, this field contains an SRI (contained in the 4 rightmost bits) and 2 reserved bits.

Figure 6.1.3.xx-1: NCR Uplink Backhaul Beam Indication MAC CE

------------------------------------------------------Next change---------------------------------------------------

6.2	Formats and parameters

6.2.1	MAC subheader for DL-SCH and UL-SCH

UNCHANGED PART OMITTED

Table 6.2.1-1b Values of one-octet eLCID for DL-SCH

| Codepoint | Index | LCID values |
| --- | --- | --- |
| 0 to 224 | 64 to 288 | Reserved |
| 225 | 289 | NCR Downlink Backhaul Beam Indication MAC CE |
| 226 | 290 | NCR Uplink Backhaul Beam Indication MAC CE |
| 227 | 291 | Serving Cell Set based SRS TCI State Indication MAC CE |
| 228 | 292 | SP/AP SRS TCI State Indication MAC CE |
| 229 | 293 | BFD-RS Indication MAC CE |
| 230 | 294 | Differential Koffset |
| 231 | 295 | Enhanced SCell Activation/Deactivation MAC CE with one octet Ci field |
| 232 | 296 | Enhanced SCell Activation/Deactivation MAC CE with four octet Ci field |
| 233 | 297 | Unified TCI States Activation/Deactivation MAC CE |
| 234 | 298 | PUCCH Power Control Set Update for multiple TRP PUCCH repetition MAC CE |
| 235 | 299 | PUCCH spatial relation Activation/Deactivation for multiple TRP PUCCH repetition MAC CE |
| 236 | 300 | Enhanced TCI States Indication for UE-specific PDCCH |
| 237 | 301 | Positioning Measurement Gap Activation/Deactivation Command |
| 238 | 302 | PPW Activation/Deactivation Command |
| 239 | 303 | DL Tx Power Adjustment |
| 240 | 304 | Timing Case Indication |
| 241 | 305 | Child IAB-DU Restricted Beam Indication |
| 242 | 306 | Case-7 Timing advance offset |
| 243 | 307 | Provided Guard Symbols for Case-6 timing |
| 244 | 308 | Provided Guard Symbols for Case-7 timing |
| 245 | 309 | Serving Cell Set based SRS Spatial Relation Indication |
| 246 | 310 | PUSCH Pathloss Reference RS Update |
| 247 | 311 | SRS Pathloss Reference RS Update |
| 248 | 312 | Enhanced SP/AP SRS Spatial Relation Indication |
| 249 | 313 | Enhanced PUCCH Spatial Relation Activation/Deactivation |
| 250 | 314 | Enhanced TCI States Activation/Deactivation for UE-specific PDSCH |
| 251 | 315 | Duplication RLC Activation/Deactivation |
| 252 | 316 | Absolute Timing Advance Command |
| 253 | 317 | SP Positioning SRS Activation/Deactivation |
| 254 | 318 | Provided Guard Symbols |
| 255 | 319 | Timing Delta |

-------------------------------------------------------End of change------------------------------------------------