3GPP TSG RAN WG2 #121	R2-2300353

Athens, Greece, February-March, 2023

Agenda Item:	8.14.2

Source:	ZTE Corporation, Sanechips

Title:	Further discussion on QoE measurement in IDLE and INACTIVE

Document for:	Discussion and Decision

Introduction

The new WID of enhancement on NR QoE management and optimizations for diverse services was approved in RAN#96[1]. In which, the following objective is included:

... - Support for new service type, such as AR, MR, MBS and other new service type defined or to be supported by SA4. Support RAN-visible parameters for the additional service types, and the existing service if needed, and the coordination with SA4 is needed [RAN3, RAN2]. - Specify the new service and the existing service defined or to be supported by SA4, combined with high mobility scenarios, e.g., High Speed Trains. - Specify for QoE measurement configuration and collection in RRC_INACTIVE and RRC_IDLE states for MBS, at least for broadcast service [RAN3, RAN2]. - Specify the mechanism to support the alignment of the existing radio related measurement and QoE reporting.

In RAN2#119 e-meeting, based on the contributions, the following agreements have been achieved:

- No need to send LS to SA4 since RAN3 is already sending an LS. - FFS if there is a new explicit indicator or new service type used for MBS QoE configuration in RRC_IDLE/RRC_INACTIVE. Wait for RAN3 progress and SA4 LS reply to RAN3. - The gNB can send the QoE configuration for MBS broadcast service to UE by RRC message in RRC_CONNECTED via dedicated signalling. The UE stores the configuration for QoE and performs the application layer measurement for MBS broadcast service. - FFS if configuration can be done in IDLE/INACTIVE states. - FFS how does gNB determine which UEs can be configured with MBS QoE measurements. - The baseline principles for QoE measurement collection for MBS services in RRC_INACTIVE and RRC_IDLE states are: - 1)The UE is configured with IDLE/INACTIVE QoE via RRC. - 2)The UE buffers the QoE reports generated while in RRC IDLE/INACTIVE state. - 3)FFS if UE can setup/resume RRC connection just for QoE reporting, or whether the QoE reports are sent to the network when the UE moves to RRC CONNECTED state due to other reasons. - When the UE moves to RRC_CONNECTED state, the UE sends the QoE measurements availability indication to the gNB. - For buffering of QoE reports generated in RRC IDLE/INACTIVE state, RAN2 should discuss at least the minimal memory size requirement. FFS if AS layer is responsible for storing the QoE reports (as in Rel-17).

Based on the discussion in RAN2#120 meeting, following agreements on reference timing delivery have been achieved:

- Ask SA4 if we can use application layer information for QoE measurements in IDLE/INACTIVE the Rel-18 area scope given that the needed information requires cell knowledge. - For buffering of QoE reports generated in RRC IDLE/INACTIVE state, RAN2 will make some assumptions on the minimal memory size requirement and the buffering layer. We can indicate these to SA4/SA5 to see if they think those assumptions are realistic. - Ask SA4/5 on how network would handle reports based on when they were collected, and whether it matters how “old” they are. - UE can be configured to do QoE measurements for MBS broadcast in all RRC states. - As a baseline, UE does not tigger RRC Resume – RRC Setup just for the sake of reporting QoE. FFS whether there are cases where we deviate from this baseline.

In this contribution, we will firstly discuss the issues of QoE configuration and reporting in RRC_IDLE/INACTIVE, and then our proposals will be given.

Discussion

QoE configuration in RRC_IDLE/INACTIVE

Based on the previous meeting, RAN2 has agreed that UE can be configured to do QoE measurement for MBS broadcast in all RRC states. It is possible that the QoE configuration received can be resued for idle/inactive states, for this case and explicit indication can be used by the gNB to indicate that the QoE configuration information of the CONNECTED state is also applicable to the QoE of the Non-CONNECTED state. Since currently NW configures the QoE configuration to UE via RRCReconfiguration and RRCResume messages, the above mentioned indication can be included in the same message to inform UE whether the received configuration is applicable to idle/inactive or not.

Proposal 1a: An indicating is introduced in QoE configuration to indicate if currently configured QoE configuration information is applicable to INACTIVE/IDLE states .

In the above method, after the core network analyzes the QoE measurement report of CONNECTED state, it may identify different requirement for idle/inactive QoE for MBS service, and would like to update/modify QoE configuration when releasing UE to idle/inactive state. For this purpose, it is proposed to use the RRCRelease to modify  the QoE configuration information of the CONNECTED state to make it suitable for the QoE of the Non-CONNECTED states.

Proposal 1b: The gNB can update the QoE configuration information of the Non-CONNECTED state by RRCRelease when releasing UE from CONNECTED state.

Furthermore, as for the question leftover from the RAN2#119 meeting: how does gNB determine which UEs can be configured with MBS QoE measurements. Since not all UEs can have the capability to support MBS service, let alone the QoE data collection for this service type, NW needs to be aware of the capability so that it can provide configuration accordingly. Similar to other service type, e.g. (VR service type), the new service type (MBS) needs to be signalled in the UE capability information (e.g., the QoE-Parameters of UE Capability Information) to indicate whether UE supports logging QoE measurement for MBS type. For the above capability information, some companies believe that UE needs to indicate MBS measurements in the CONNECTED state and the Non-CONNECTED state, respectively. In our understanding the main intention to support logging of QoE in idle/inactive is to allow continuously logging the broadcast service QoE data without interruption, to have separate capability seems unnecessary for this purpose. For UE supporting QoE for MBS service, it shall support this feature for all RRC states. A simple capability indication which contains information that supports MBS measurements in both CONNECTED and Non-CONNECTED states has been able to meet service requirements.

Observation 1: UE capability information should be needed since NW need to configure UE with the corresponding configuration.

Proposal 2: Introduce a UE capability information in the QoE-Parameters to indicate that MBS measurement is supported in CONNECTED and Non-CONNECTED states.

QoE Reporting in RRC_IDLE/INACTIVE

Based on the conclusions of the previous meeting, the UE can buffer the QoE reports generated in IDLE/INACTIVE states. As a baseline, UE can not tigger RRC Resume-RRC Setup just for the sake of reporting QoE. Therefore, in Non-CONNECTED state, the QoE measurement report processing for the MBS service includes the following methods:

Option 1: UE reporting the QoE measurement reporting stored in UE when entering CONNECTED state.

Option 2: UE reporting QoE measurement information by small data transmission (SDT) when in Non- CONNECTED state.

In the case of QoE measurement including legacy QoE and RVQoE, the QoE measurement reporting of MBS service in Non-CONNECTED state needs to be reported based on the service requirement analysis.

For legacy QoE, its real-time requirement is not high, and it does not need RAN real-time processing. The gNB only needs to forward it to MCE after receiving the measurement reporting. Therefore, in the Non-CONNECTED state, the measurement reporting stored in UE can be uploaded after the UE enters the CONNECTED state. In other words, legacy QoE reports use the method of Option 1. For buffering of QoE reports generated in RRC IDLE/INACTIVE state, RAN2 has made some assumptions on the minimal memory size requirement and the buffering layer, and sent LS to SA4/SA5.

Observation 2: For legacy QoE, the QoE reporting is stored in the Non-CONNECTED UE, and is reported when the UE enters the CONNECTED state.

For RVQoE, its real-time requirement is higher, and gNB may want to do some real-time optimization in scheduling according to the received RVQoE measurement reports. Therefore, in the Non-CONNECTED state, UE may need to upload the QoE measurement reporting to gNB in time. Based on the real-time requirements, Option 2 can meet the above requirements. However, considering that SA4/5 have a more authoritative understanding of the availability of QoE reporting, we think it needs to be further discussed after receiving the LS’s [2] reply from SA2.

Proposal 3: The study of RVQoE reporting method can be postponed until a reply LS from SA4/5 is received.

Conclusion

In the above sections, the following observations and proposals were made:

Observation 1: UE capability information should be needed since NW need to configure UE with the corresponding configuration.

Observation 2: For legacy QoE, the QoE reporting is stored in the Non-CONNECTED UE, and is reported when the UE enters the CONNECTED state.

Proposal 1a: An indicating is introduced in QoE configuration to indicate if currently configured QoE configuration information is applicable to INACTIVE/IDLE states .

Proposal 1b: The gNB can update the QoE configuration information of the Non-CONNECTED state by RRCRelease when releasing UE from CONNECTED state.

Proposal 2: Introduce a UE capability information in the QoE-Parameters to indicate that MBS measurement is supported in CONNECTED and Non-CONNECTED states.

Proposal 3: The study of RVQoE reporting method can be postponed until a reply LS from SA4/5 is received.

# References

- RP-221803, Enhancement on NR QoE management and optimizations for diverse services, June, 2022

- R2-2213054, LS on QoE measurements in RRC IDLE/INACTIVE states, November,2022