3GPP TSG RAN WG2 #121	R2-2300356

Athens, Greece, February-March, 2023

Agenda Item:	8.14.5

Source:	ZTE Corporation, Sanechips

Title:	Discussion on Rel-18 other QoE enhancement

Document for:	Discussion and Decision

Introduction

The new WID of enhancement on NR QoE management and optimizations for diverse services was approved in RAN#96[1]. In which, the following objective is included:

... - Support for new service type, such as AR, MR, MBS and other new service type defined or to be supported by SA4. Support RAN-visible parameters for the additional service types, and the existing service if needed, and the coordination with SA4 is needed [RAN3, RAN2]. - Specify the new service and the existing service defined or to be supported by SA4, combined with high mobility scenarios, e.g., High Speed Trains. - Support the continuity of legacy QoE measurement job for streaming and MTSI service during intra-5GC inter-RAT handover process [RAN2, RAN3].

In this contribution, we will discuss the issues of new service types and the continuity of legacy QoE measurement. Then we’ll give our proposals.

Discussion

New service types

In Rel 18, considering some new supported service types (e.g., AR, MR and MBS) in the QoE measurement, it is necessary to carry the new service types when performing the QoE measurement. But in SA4's reply LS to RAN3, RAN3 respectfully asks SA4 the status and plans to define QoE metrics for AR, MR and MBS in Rel-18. Therefore, we think we need to wait for RAN3’s conclusion on this topic.

Proposal 1: RAN2 waits for RAN3’s conclusion about the new service types.

Continuity of legacy QoE measurement

In the intra-5GC inter-RAT handover process, the QoE measurement functions supported by LTE and NR are different, and there are the following problems to ensure continuity:

-For handover from source gNB to target ng-eNB, more than one QoE measurement collection jobs may have been activated for a service type (e.g. streaming or MTSI) in the source gNB, but the target ng-eNB can only support one QoE measurement collection job for the service type.

-For handover from source ng-eNB to target gNB, how does the target gNB configure measConfigAppLayerID for the source ng-eNB is also a problem that needs to be analyzed?

In our understanding, the above problems may be easily resolved if the QoE measurement function supported by LTE can be enhanced. However, impact to LTE specs have been removed from the WID in RAN plenary, which means RAN2 need more time to discuss if there are any solution involving only NR that can resolve this issues in R18, while the solution may be complicated and inefficient. Further, considering that there are many important topics that need to be discussed in the current WID while the time budget is limited, perhaps RAN2 can consider lower the priority of this topic and postpone the discussion.

Proposal 2: In R18, RAN2 deprioritize the study of continuity of legacy QoE measurements during intra-5GC inter-RAT handover.

Conclusion

In previous sections, the following observations and proposals were made:

Proposal 1: RAN2 waits for RAN3’s conclusion about the new service types.

Proposal 2: In R18, RAN2 deprioritize the study of continuity of legacy QoE measurements during intra-5GC inter-RAT handover.

# References

- RP-221803, Enhancement on NR QoE management and optimizations for diverse services, June, 2022