3GPP TSG-RAN WG2 Meeting #121                                     R2-2300365

Athens, Greece, Feb 27 – Mar 03, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.306 | CR | 0854 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on NR NTN UE capability for enhanced measurement requirements for cell reselection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions-Core |  |  |  |  |  | Date: |  |  | 2023-02-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In RAN4 feature list R4-2215143, 25-4 is for enhanced RRM requirements for measurements in IDLE and INACTIVE modes. It is stated that “If UE does not support the capability, legacy TN non-HST measurement requirements are applied for both LEO and GEO”. And RAN2 has captured this 25-4 in TS 38.306 according to the description in RAN4 feature list. In RAN2#121, a RAN4 LS is received in R4-2220427, which clarifies that if the NTN UE does not support this feature, the NTN modified versions are to be applied instead of the legacy terrestrial requirements. The modifications to accommodate the specificities of NTN scenarios include: - the introduction of a scaling factor (Kmulti_SMTC) dependent on the number of satellites associated to the neighbor cells; - introduction of requirements considerations regarding the parameter t-service; As a follow-up handling, the corresponding correction should be made in the field description of UE capability for enhanced RRM requirements for measurements in IDLE and INACTIVE modes. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that “If UE does not support this feature, legacy TN non-HST measurement requirements with the NTN specific modifications as specified in TS 38.133 [5] are applied for both LEO and GEO” in the field description of UE capability for enhanced RRM requirements for measurements in IDLE and INACTIVE modes. Impact analysis: Impacted functionality: - Cell reselection in NR NTN Inter-operability issues: - Since this change only impacts UE, there are no interoperability issues. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is unclear what measurement requirements should be applied if enhanced RRM requirements for measurements in IDLE and INACTIVE modes is not supported by NTN capable UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

## 5.6	RRM measurement features

Definitions for feature

High speed inter-frequency IDLE/INACTIVE measurements It is optional for UE to support high speed inter-frequency measurements in RRC_IDLE/RRC_INACTIVE as specified in TS 38.133 [5].

Location-based measurement initiation It is optional for the UE to support location based RRM measurements of neighbour cells in RRC_IDLE/RRC_INACTIVE as specified in TS 38.304 [21].

Relaxed measurement It is optional for UE to support relaxed RRM measurements of neighbour cells in RRC_IDLE/RRC_INACTIVE as specified in TS 38.304 [21].

Rel-17 relaxed measurement for RRC_IDLE/RRC_INACTIVE It is optional for RedCap UE to support Rel-17 relaxed RRM measurements of neighbour cells in RRC_IDLE/RRC_INACTIVE as specified in TS 38.304 [21].

Enhanced RRM requirements for measurements in IDLE and INACTIVE modes It is optional for UE to support enhanced RRM requirements for measurements for NTN bands (FR1 only and FDD only) in RRC_IDLE/RRC_INACTIVE as specified in TS 38.133 [5]. If UE does not support this feature, legacy TN non-HST measurement requirements with the NTN specific modifications as specified in TS 38.133 [5] are applied for both LEO and GEO.

Time-based measurement initiation It is optional for the UE to support time based RRM measurements of neighbour cells in RRC_IDLE/RRC_INACTIVE as specified in TS 38.304 [21].

End of change