3GPP TSG-RAN WG2 Meeting #121	                                     R2-2300366

Athens, Greece, Feb 27 – Mar 03, 2023

Agenda item: 	8.6.3.1

Source:	Intel Corporation

Title: 	Discussion on neighbour cell measurements in IoT NTN

Document for:	Discussion and Decision

# Introduction

The Rel-18 IoT NTN WI was approved in [1], and one objective is to enhance neighbour cell measurements and corresponding measurement triggering before RLF.

And in RAN2#119, the following agreements were achieved for neighbour cell measurements:

Agreements:

- IoT NTN can use the mechanism for neighbour cell measurements in connected mode (specified in Rel-17 for NB-IoT). FFS if any enhancements are needed (e.g. triggers) for both NB-IoT and eMTC.

- RAN2 to continue working on a new time-based trigger for triggering intra and inter frequency measurements in connected mode, e.g. the serving cell is going to stop covering the current area, for both earth-moving and earth-fixed cell (FFS on distance-based trigger)

- Measurement results reporting is not supported in Rel-18 NB-IoT NTN.

In RAN2#120, further agreements were made in this aspect.

Agreements:

- For NB-IoT we support a trigger for neighbour cell measurements based on T-service (in the quasi-Earth fixed case) (this does not preclude anything for eMTC discussion)

Agreements:

- At least for NB-IoT NTN, for quasi-earth fixed cells, UE shall start intra/inter frequency measurement in connected mode before the t-Service if present. The exact time to start measurements in connected mode before t-Service can be left to UE implementation” (can revisit if we agree other proposal based on neighbour cell coverage)

- RAN2 will not specify the condition of stopping UE measurement before t-Service

- For earth-moving cell, the UE derives when loss of coverage of current cell happens (how to derive this information is FFS)

- For earth-moving cell, UE shall start intra/inter frequency measurements in RRC connected mode before losing coverage. The exact time to start measurements can be left to UE implementation

In this paper, we further provide our views on the neighbour cell measurements in IoT NTN.

# Discussion

## 2.1 Focusing on NB-IoT NTN

In Rel-17 NB-IoT, the following agreements were made for neighbour cell measurements:

RAN2#112-e agreements: The solution includes reduction of the time between declaration of RLF and the start of the random-access procedure (points C and D) RAN2#114-e agreements: The criteria to start measurements is based on a combination of serving cell quality threshold (option b) and variance of the serving cell quality (option c) RAN2#115-e agreements: The conditions where the UE is required to perform measurements are specified. No requirement on when to stop measurements is needed. The configuration of the criteria for starting the measurements is provided via broadcast signalling.

The motivation of the support of neighbour cell measurements and corresponding measurement triggering before RLF is to shorten the time between declaration of RLF and the start of the random-access procedure as mentioned by agreements above, and this enhancement is to start neighbour cell measurements early before the potential RLF. In this case when a RLF really happens, the UE can skip the cell search phase, and instead the UE can select a target cell based on the measurement results previously acquired in connected mode, and perform RRC re-establishment directly. This enhancement is used to achieve fast RRC re-establishment, and it could be reused in NB-IoT NTN scenario.

Since in legacy NB-IoT, neighbour cell measurement is not supported in connected mode, in Rel-17 a specific event is specified to trigger neighbour cell measurement (i.e., when the NRSRP value is lower than a threshold). When the entering condition of this event is met, it’s likely that a RLF may happen soon, then neighbour cell measurement can be performed to identify potential qualified target cells. But in legacy eMTC, neighbour cell measurement in connected mode is already supported and supposed to be “always-on”, or the s-measure can be configured to enable neighbour cell measurements. It’s useless to introduce a new event to trigger neighbour cell measurement as the eMTC UE is supposed to perform measurements all the time according to measurement configuration. Even if the RLF happens, based on available neighbour cell measurement results, for an eMTC UE it still can be up to UE implementation to select a target cell for fast RRC re-establishment. So, we only need to consider “how to trigger neighbour cell measurements” for NB-IoT NTN, not for eMTC NTN, because an eMTC UE can skip the cell research during RRC re-establishment depending on existing neighbour cell measurement results.

In RAN#98, a discussion was made regarding if the WID needs to be updated to clarify this enhancement is only for NB-IoT NTN, not for eMTC NTN. The proposed clarification is as below:

Clarification 2: The objective text is not intended to include applying the NB-IoT measurement triggering before RLF mechanism and its enhancements to eMTC.

Although this is the majority view to go for this clarification, but it was not endorsed finally. But RAN2 chair made a clarification on the RAN2 discussion as follows:

FYI: RAN2 chair instructions, for RAN2, may be related e.g. to further work on this objective (dep on company input to RAN2): There should be gain for a common/reasonable case for the functionality to be introduced, i.e. please don’t use an unlikely corner case in order to motivate a certain solution.

To progress more in this enhancement, a feasible RAN2 way forward could be focusing on NB-IoT NTN first, and if there is time within the WI and RAN2 can be convinced that there is a gain to also support measurement triggering before RLF for eMTC NTN, RAN2 can consider to copy-paste agreed enhancements (if any for NB-IoT NTN) to eMTC NTN if time allows.

Proposal 1: regarding measurement triggering before RLF, RAN2 first works on NB-IoT NTN (i.e. eMTC NTN is de-prioritized). I.e., If there is time within the WI and the benefit id justified to also support measurement triggering before RLF in eMTC NTN, RAN2 can consider introducing this feature to eMTC NTN.

## 2.2 Optimization beyond R17 NB-IoT

In Rel-17 NB-IoT, there was no time to discuss further optimization for neighbour cell measurements. For example, the following agreements were made:

RAN2#115-e agreements: Prioritisation of carriers/cells to measure is left to the UE implementation. Provision of information regarding which cells/carriers to be considered is not supported. It is up to UE implementation to choose and prioritize carrier/cell list for measurement.

Above agreement means that for the execution of intra/inter-frequency neighbour cell measurements, there is no prioritisation related assistance information provided for UE. But in IoT NTN, especially in case of discontinuous coverage, there may be only one or two satellites deployed for collecting data from NB-IoT UEs worldwide. And it is possible that network provides the information of next upcoming cells (maybe a UE specific smaller group of neighbour cells compared to the list of neighbour cells broadcast in system information for all UEs) to connected UE for potential RRC re-establishment, so the UE may prioritize the measurements on these upcoming cells. In this way, it can save UE power consumption to focus on more likely neighbour cell measurements before RLF. Similarly, in case of Quasi-earth-fixed cell, it’s also beneficial to provide the information of upcoming cells to shorten the search delay of new target cell.

Proposal 2: For Quasi-earth-fixed cell in NB IoT NTN, network provides the information of next upcoming cells, e.g., a group of cell IDs, to make UE focus on more likely neighbour cells for RRC re-establishment and save UE power consumption.

## 2.3 Optimization for earth-moving cell

In case of earth-moving cell, in Rel-18 NR NTN the following agreements were made for connected mode.

Agreements:

- RAN2 confirms that at least for the moving cell case the next serving cells can be largely predicted in NTN (at least for UEs not at the cell edge) thanks to the existence of predefined satellite orbits and negligible UE’s mobility in comparison to satellite’s motion (we can further discuss at the next meeting whether this applies to idle mode UEs as well)

- In Earth-moving cell, the reference location and distance threshold of serving cell are provided by network for UE to estimate when the serving cell stops providing coverage at the present UE location. FFS how the reference location and/or distance threshold are provided to the UE

For NR NTN, satellite orbital parameters, the reference location and distance threshold of a serving cell are agreed for UE to identify when the serving cell stops providing coverage at the present UE location .

Since there is an overlap between NR NTN and IoT NTN (i.e., same issue on the prediction of cell coverage is discussed in both NR NTN and IoT NTN), at first RAN2 can confirm the agreements above can also be applied to IoT NTN, and then satellite orbital parameters, the reference location and distance threshold of a neighbour cell can also be provided to UE to predict the next serving cells in earth-moving cell. In this case, when neighbour cell measurement is triggered is based on the coverage prediction of current serving cell (e.g., some time before the cell stop time), and a UE only measures on these candidate cells based on the coverage prediction of next upcoming cells. And both prediction mentioned above are based on satellite orbital parameters, the reference location and distance threshold of serving cell and neighbours.

Proposal 3: For earth-moving cell in NB IoT NTN, network provides satellite orbital parameters, the reference location and distance threshold of a serving cell, to enable the prediction of when the serving cell stops providing coverage at the present UE location, and the neighbour cell measurement is triggered some time before this cell stop time.

Proposal 4: For earth-moving cell in NB IoT NTN, network provides satellite orbital parameters, the reference location and distance threshold of a neighbour cell, to enable the prediction of next serving cells at UE side, and when neighbour cell measurement is triggered, UE only measures on these candidate cells.

# Conclusion

In this paper, we discuss the neighbour cell measurements in IoT NTN, and we have the following proposals:

Proposal 1: regarding measurement triggering before RLF, RAN2 first works on NB-IoT NTN (i.e. eMTC NTN is de-prioritized). I.e., If there is time within the WI and the benefit id justified to also support measurement triggering before RLF in eMTC NTN, RAN2 can consider introducing this feature to eMTC NTN.

Proposal 2: For Quasi-earth-fixed cell in NB IoT NTN, network provides the information of next upcoming cells, e.g., a group of cell IDs, to make UE focus on more likely neighbour cells for RRC re-establishment and save UE power consumption.

Proposal 3: For earth-moving cell in NB IoT NTN, network provides satellite orbital parameters, the reference location and distance threshold of a serving cell, to enable the prediction of when the serving cell stops providing coverage at the present UE location, and the neighbour cell measurement is triggered some time before this stop time.

Proposal 4: For earth-moving cell in NB IoT NTN, network provides satellite orbital parameters, the reference location and distance threshold of a neighbour cell, to enable the prediction of next serving cells at UE side, and when neighbour cell measurement is triggered, UE only measures on these candidate cells.

# References

[1] RP-221806		Revised WID on IoT NTN enhancements	MediaTek Inc.