3GPP TSG-RAN WG2 Meeting #121                                            　    　　　　        　　 	　　R2-2300378

Athens, Greece, Feb 27th – Mar 3rd 2023

Source: 	KDDI Corporation

Title:	Considerations on Cell DTX/DRX

Tdoc Type:	Discussion

Agenda Item:	8.3.2 DTX/DRX mechanism

Document for:	Discussion and decision

Release:	Rel-18

Introduction

In the last RAN#98-e, Network energy savings for NR, was approved as a new WID with the following objectives.

The objectives of the work item are the following:

1.	Specify SSB-less SCell operation for inter-band CA for FR1 and co-located cells, if found feasible by RAN4 study, where a UE measures SSB transmitted on PCell or another SCell for an SCell’s time/frequency synchronization (including downlink AGC), and L1/L3 measurements, including potential enhancement on SCell activation procedures if necessary [RAN4, RAN2]

2.	Specify enhancement on cell DTX/DRX mechanism including the alignment of cell DTX/DRX and UE DRX in RRC_CONNECTED mode, and inter-node information exchange on cell DTX/DRX [RAN2, RAN1, RAN3]

•	Note: No change for SSB transmission due to cell DTX/DRX.

•	Note: The impact to IDLE/INACTIVE UEs due to the above enhancement should be avoided.

3.	Specify the following techniques in spatial and power domains

•	Specify necessary enhancements on CSI and beam management related procedures including measurement and report, and signaling to enable efficient adaptation of spatial elements (e.g. antenna ports, active transceiver chains) [RAN1, RAN2]

•	Specify necessary enhancements on CSI related procedures including measurement and report, and signaling to enable efficient adaptation of power offset values between PDSCH and CSI-RS [RAN1, RAN2]

•	Note: Above objectives are only for UE specific channels/signals

•	Note: Legacy UE CSI/CSI-RS capabilities applies when considering total number of CSI reports and requirements

4.	Specify mechanism(s) to prevent legacy UEs camping on cells adopting the Rel-18 NES techniques, if necessary [RAN2]

5.	Specify CHO procedure enhancement(s) in case source/target cell is in NES mode [RAN2]

6.	Specify inter-node beam activation and enhancements on restricting paging in a limited area [RAN3].

7.	Specify the corresponding RRM/RF core requirements, if necessary, for the above features [RAN4]

In this contribution, we would like to share our considerations regarding cell DTX/DRX.

Discussion

2.1 Cell DTX/DRX

The following cell DTX/DRX behaviour during non-active periods are captured as possible options in the TR.

-	Example 1: gNB is expected to turn off all transmission and reception for data traffic and reference signal during Cell DTX/DRX non-active periods.

-	Example 2: gNB is expected to turn off its transmission/reception only for data traffic during Cell DTX/DRX non-active periods (i.e., gNB will still transmit/receive reference signals)

-	Example 3: gNB is expected to turn off its dynamic data transmission/reception during Cell DTX/DRX non-active periods (i.e., gNB is expected to still perform transmission/reception in periodic resources, including SPS, CG-PUSCH, SR, RACH, and SRS).

-    Example 4: gNB is expected to only transmit reference signals (e.g., CSI-RS for measurement).

Considering the note that mentioned no change for SSB transmission due to cell DTX/DRX in the WID, our understanding is that the case that SSB used as a reference signal for UE measurement can be excluded.  However, the particular channel / reference signal that will be turned off needs further discussion and decision. How to generate the signal, mapping these signals to the physical resource blocks, or how to indicate them to the UE, etc. are within RAN1 scope. For RAN2, the basic parameters of cell DTX/DRX were discussed in RAN2#120 and captured in the TR that the cell DTX/DRX can be configured with periodicity, start slot/offset, on duration, however, whether the cell DTX/DRX active time is fixed or changeable depending on the arrival of data has not been discussed or agreed. Therefore, the feasible patterns of cell DTX/DRX, the parameters that are needed for cell DTX/DRX, or the possibility to reuse the existing UE C-DRX mechanism for cell DTX/DRX, etc. needed to be discussed and decided.

Proposal 1: RAN2 discuss the cell DTX/DRX framework and related parameters.

2.2.1 Cell DRX

UE C-DRX was designed to save UE power consumption by configuring UE to enter inactive, or sleep mode in the DRX off period and to monitor PDCCH in DRX during the on-duration period. The UE C-DRX is characterized by parameters, e.g. on-duration, inactivity-timer, retransmission-timer, cycle and active-time, drx cycle startoffset. Similar as the UE C-DRX, the parameters, periodicity, start slot/offset, on duration that were captured in the TR can be described as below and indicated to the UE.

Table 1. Parameters for cell DRX

Figure 1. cell DRX cycle

In terms of the granularity of these 3 parameters, slot order, or sub frame order, or radio frame order can be considered. A granularity of milliseconds or sub milliseconds which is used for the UE C-DRX is also one of the options. In order to apply these parameters to different scenarios, multiple values for each can be enumerated, similar to the UE C-DRX.  In terms of the applied scenario, TDD or FDD, a unified mechanism that can be applied to both TDD and FDD can be discussed in RAN2.

Proposal 2: RAN2 agree, cell-drx cycle startoffset, cell-drx on duration and cell-drx cycle should be indicated to the UEs, so that the UEs know when to start transmitting uplink data within the cell DRX cycle.

Proposal 3: RAN2 discuss the granularity of cell-drx cycle startoffset, cell-drx on duration, as well as, cell-drx cycle in slot order, or sub frame order, or radio frame order, or sub milliseconds/milliseconds order.

Proposal 4: RAN2 agree that multiple values are enumerated for parameters, at least, cell-drx cycle startoffset, cell-drx on duration, as well as cell-drx cycle.

Proposal 5: RAN2 discuss a unified cell DRX mechanism that can be applied to both TDD and FDD

There are parameters, such as drx-inacitvity-timers, dtx-retransmissiontimer, etc are specified for UE C-DRX. For example, the drx inactivity timer for UE C-DRX is specified to avoid increased latency in case of consecutive data for the UE after a reception of PDCCH. If there is no further data for the UE until the drx inactivity timer expires, the UE will return back to sleep. If we imply the similar mechanism for cellDRX, things will become complicated. If cell-drx-inactivity-timer is not specified, which means the cell-drx on duration is fixed, it will cause increased UE transmission latency because the UE has to wait until the next cell-drx cycle to transmit its data. On the other hand, if the cell-drx-inactivity-timer is specified, as the cell receives data from multiple UEs and the data generated from different UEs is random within a cell drx cycle due to the users’ custom and behavior, questions, such as, whether to restart the cell-drx inactivity-timer every time when the cell receives a uplink data, or to inform other UEs and how to inform multiple UEs that the cell will be on drx on mode without increasing signaling overhead, etc. needs to be discussed. From our perspective, a simpler mechanism is preferable.

Figure 2 scenarios about cell-drx-inactivity-timer

Proposal 6: RAN2 discuss whether cell-drx-inactivity-timer is needed for cell drx cycle.

2.2.2 Cell DTX

Similar to cell DRX, we think that cell DTX pattern can also be characterized by at least the following parameters. During the cell-dtx on duration, we can assume the cell configure downlink signals/channels as usual without any specification change. During the cell-dtx off period, questions, such as, which signal/channel, CSI-RS or PDCCH or PDSCH, etc. is going to be impacted; is it necessary to reconfigure these signal/channels; how and when to reconfigure these signal/channels if it is needed, etc. have to be discussed. Although these are more within the scope of RAN1, an initial discussion from RAN2 can help to trigger RAN1 discussion earlier and have more time to discuss details about the UE behavior during the cell-dtx off period.

Table 2 Parameters for cell DTX

Figure 3 cell DTX cycle

Proposal 7: RAN2 agree cell-dtx cycle startoffset, cell-dtx on duration and cell-dtx cycle should be indicated to the UEs, so that the UEs know when to start receiving downlink data within the cell DTX cycle.

Proposal 8: RAN2 discuss whether to reconfigure any signal/channel, such as, CSI-RS, PDCCH, PDSCH etc. when the cell enters cell-dtx off period after the cell-dtx on duration.

2.2.3 regarding the cell DTX/DRX configuration

As written in the TR, cell DTX/DRX is expected to be applied to the UEs in RRC_CONNECTED state. Therefore, it is a UE specific configuration and RRCreconfiguration can be used to convey this message to the UEs. Cell DTX/DRX is a mechanism to define when and how the UE can monitor or decode or send some signal or data. It is more related to Layer1/layer2 UE actions, therefore, the cell DTX/DRX configuration can be set under the mac-cell configuration, which is similar to the configuration method used for UE C-DRX.

Proposal 9: cell DTX/DRX setting can be configured to the UE via RRC reconfiguration, similar as UE C-DRX.

Conclusion

We introduce the following proposals.

Proposal 1: RAN2 discuss the cell DTX/DRX framework and related parameters.

Proposal 2: RAN2 agree, cell-drx cycle startoffset, cell-drx on duration and cell-drx cycle should be indicated to the UEs, so that the UEs know when to start transmitting uplink data within the cell DRX cycle.

Proposal 3: RAN2 discuss the granularity of cell-drx cycle startoffset, cell-drx on duration, as well as, cell-drx cycle in slot order, or sub frame order, or radio frame order, or sub milliseconds/milliseconds order.

Proposal 4: RAN2 agree that multiple values are enumerated for parameters, at least, cell-drx cycle startoffset, cell-drx on duration, as well as cell-drx cycle.

Proposal 5: RAN2 discuss a unified cell DRX mechanism that can be applied to both TDD and FDD

Proposal 6: RAN2 discuss whether cell-drx-inactivity-timer is needed for cell drx cycle.

Proposal 7: RAN2 agree cell-dtx cycle startoffset, cell-dtx on duration and cell-dtx cycle should be indicated to the UEs, so that the UEs know when to start receiving downlink data within the cell DTX cycle.

Proposal 8: RAN2 discuss whether to reconfigure any signal/channel, such as, CSI-RS, PDCCH, PDSCH etc. when the cell enters cell-dtx off period after the cell-dtx on duration.

Proposal 9: cell DTX/DRX setting can be configured to the UE via RRC reconfiguration, similar as UE C-DRX.

Reference

[1] TR 38.864  Study on network energy savings for NR

[2] TS 38.300-h30 NR; NR and NG-RAN Overall Description;Stage 2

[3] TS 38.321-h30 NR; Medium Access Control (MAC) protocol specification

[4] TS 38.331-h30 NR; Radio Resource Control (RRC) protocol specification