3GPP TSG-RAN WG2 Meeting #121	                                     R2-2300402

Athens, Greece, Feb 27 – Mar 03, 2023

Agenda item: 	8.4.2.2

Source:	Intel Corporation

Title: 	Discussion on LTM RRC model

Document for:	Discussion and Decision

# Introduction

In RAN2#119e meeting, RAN2 down select the following on RRC modeling:

A L1/L2 inter-cell mobility candidate (target) configuration is received within an RRC message before the L1/L2 dynamic switch is triggered.

RAN2 continues the discussion on the RRC models by focusing on Model 1 and Model 2 and stage-3 details.

- a.	Model 1: One RRCReconfiguration message (or FFS RRCReconfiguration IEs) for each candidate target configuration

- b.	Model 2: One CellGroupConfig IE (FFS additional IEs) for each candidate target configuration

Last RAN2 #120 meeting, some agreements were made for RRC modelling for LTM as follows:

P1	RAN2 to confirm that the CellGroupConfig IE is (mandatory) needed within an LTM candidate cell configuration.

P3	The RadioBearerConfig IE can be optionally supported in an LTM candidate configuration

P5	The MeasConfig IE can be optionally supported in an LTM candidate configuration.

P8	The OtherConfig IE is not required to be part of the LTM candidate cell configuration.

P9	The LTM candidate cell configuration should be designed as a To AddMod/ToRelease structure.

P10	The LTM candidate cell configuration ASN.1 structure comprises at least a CellGroupConfig IE and a configuration ID.

On Delta Configuration

A UE stores the reference configuration as a separate configuration.

The reference configuration is managed separately

In this paper, we present the high-level change required for model 1 (RRCReconfiguration) for consideration.

# Discussion

During last RAN2 #120 meeting, two RRC models were left on the table for LTM. They are:

- Model 1: list of RRCReconfiguration in one RRCReconfiguration (like CHO)

- Model 2: list of cellGroupConfig in one RRCReconfiguration message

Below, we show the ASN to implement model 1 followed by the procedure changes required for model 1.

RRCReconfiguration message

RRCReconfiguration ::=                  SEQUENCE {

rrc-TransactionIdentifier               RRC-TransactionIdentifier,

criticalExtensions                      CHOICE {

rrcReconfiguration                      RRCReconfiguration-IEs,

criticalExtensionsFuture                SEQUENCE {}

}

}

RRCReconfiguration-IEs ::=              SEQUENCE {

radioBearerConfig                       RadioBearerConfig                                                      OPTIONAL, -- Need M

secondaryCellGroup                      OCTET STRING (CONTAINING CellGroupConfig)                              OPTIONAL, -- Cond SCG

measConfig                              MeasConfig                                                             OPTIONAL, -- Need M

lateNonCriticalExtension                OCTET STRING                                                           OPTIONAL,

nonCriticalExtension                    RRCReconfiguration-v1530-IEs                                           OPTIONAL

}

/*** skip unrelated parts***/

RRCReconfiguration-v1700-IEs ::=        SEQUENCE {

otherConfig-v1700                       OtherConfig-v1700                                              OPTIONAL, -- Need M

sl-L2RelayUE-Config-r17                 SetupRelease { SL-L2RelayUE-Config-r17 }                       OPTIONAL, -- Need M

sl-L2RemoteUE-Config-r17                SetupRelease { SL-L2RemoteUE-Config-r17 }                      OPTIONAL, -- Need M

dedicatedPagingDelivery-r17             OCTET STRING (CONTAINING Paging)                               OPTIONAL, -- Cond PagingRelay

needForGapNCSG-ConfigNR-r17             SetupRelease {NeedForGapNCSG-ConfigNR-r17}                     OPTIONAL, -- Need M

needForGapNCSG-ConfigEUTRA-r17          SetupRelease {NeedForGapNCSG-ConfigEUTRA-r17}                  OPTIONAL, -- Need M

musim-GapConfig-r17                     SetupRelease {MUSIM-GapConfig-r17}                             OPTIONAL, -- Need M

ul-GapFR2-Config-r17                    SetupRelease { UL-GapFR2-Config-r17 }                          OPTIONAL, -- Need M

scg-State-r17                           ENUMERATED { deactivated }                                     OPTIONAL, -- Need N

appLayerMeasConfig-r17                  AppLayerMeasConfig-r17                                         OPTIONAL, -- Need M

ue-TxTEG-RequestUL-TDOA-Config-r17      SetupRelease {UE-TxTEG-RequestUL-TDOA-Config-r17}              OPTIONAL,  -- Need M

nonCriticalExtension                    RRCReconfiguration-v18xx-IEs                                                    OPTIONAL

}

RRCReconfiguration-v18xx-IEs ::=    SEQUENCE {

ltm_CellGroupReconfig                   LTM_CellGroupReconfig-r18                                      OPTIONAL, -- Need M

nonCriticalExtension                    SEQUENCE {}                                           		OPTIONAL

}

The example IE LTM_CellGroupReconfig is used to add, modify and release the configuration of LTM candidate cells reconfiguration.

LTM_CellGroupReconfig information element

-- ASN1START

-- TAG-LTM_CELLGROUPRECONFIG-START

LTM_CellGroupReconfig-r18 ::=   SEQUENCE {

ltm_cellGroupReconfigToRemoveList-r18         LTM_CellGroupReconfigToRemoveList-r18   OPTIONAL,   -- Need N

ltm_cellGroupReconfigToAddModList-r18         LTM_CellGroupReconfigToAddModList-r18   OPTIONAL,   -- Need N

...

}

LTM_CellGroupReconfigToRemoveList-r18 ::=     SEQUENCE (SIZE (1.. maxNrofLTM_Cells-r18)) OF LTM_CellReconfigId-r18

-- TAG- LTM_CELLGROUPRECONFIG -STOP

-- ASN1STOP

The example IE LTM_CellGroupReconfigToAddModList concerns a list of LTM cell group reconfigurations to add or modify, with for each entry the ltm_CellReconfigId and the associated ltmCellReconfig.

LTM_CellGroupReconfigToAddModList information element

-- ASN1START

-- TAG-LTM_CELLGROUPRECONFIGTOADDMODLIST-START

LTM_CellGroupReconfigToAddModList-r18::= SEQUENCE (SIZE (1.. maxNrofLTM_Cells-r18)) OF LTE_CellReconfigToAddMod-r18

LTE_CellConfigToAddMod-r18 ::=     SEQUENCE {

ltmCellReconfigId-r18            LTM_CellReConfigId-r18,

ltmCellReconfig-r18              OCTET STRING (CONTAINING RRCReconfiguration)          OPTIONAL,

...,

}

-- TAG-LTM_CELLGROUPRECONFIGTOADDMODLIST-STOP

-- ASN1STOP

Below is the example procedure text for model 1:

#### 5.3.5.3	Reception of an RRCReconfiguration by the UE

The UE shall perform the following actions upon reception of the RRCReconfiguration, or upon execution of the conditional reconfiguration (CHO, CPA or CPC), or upon execution of LTM:

1>	if the RRCReconfiguration is applied due to a conditional reconfiguration execution upon cell selection performed while timer T311 was running, as defined in 5.3.7.3:

2>	remove all the entries within VarConditionalReconfig, if any;

[text omitted]

#### 5.3.5.x	LTM Cell Group Reconfiguration

##### 5.3.5.x.1	General

The network configures the UE with one or more candidate target SpCells in the LTM cell group reconfiguration. The UE applies the LTM cell group reconfiguration associated with one of the target SpCells indicated by network via MAC CE. The network provides the configuration parameters for the target SpCell in the ltmCelllReconfig IE.

The UE performs the following actions based on a received LTM_CellGroupReconfig IE:

1>	if the LTM_CellGroupReconfig contains the ltm_cellGroupReconfigToRemoveList:

2>	perform LTM cell group reconfiguration removal procedure as specified in 5.3.5.x.2;

1>	if the LTM_CellGroupReconfig contains the ltm_cellGroupReconfigToAddModList:

2>	perform conditional reconfiguration addition/modification as specified in 5.3.5.x.3;

##### 5.3.5.x.2	LTM Cell Group Reconfiguration Removal

The UE shall:

1>	for each ltmCellReconfigId value included in the ltm_cellGroupReconfigToRemoveList that is part of the current UE LTM Cell Group Reconfiguration in VarLtmCellReconfig:

2>	remove the entry with the matching ltmCellReconfigId from the VarLtmCellReconfig;

NOTE:	The UE does not consider the message as erroneous if the ltm_cellGroupReconfigToRemoveList includes any ltmCellReconfigId value that is not part of the current UE configuration.

##### 5.3.5.x.3	LTM Cell Group Reconfiguration Addition/Modification

For each ltmCellReconfigId received in the ltm_cellGroupReconfigToAddModList IE the UE shall:

1>	if an entry with the matching ltmCellReconfigId exists in the ltm_cellGroupReconfigToAddModList within the VarLtmCellReconfig:

2>	if the entry in ltm_cellGroupReconfigToAddModList includes an ltmCellReconfig;

3>	replace ltmCellReconfig within the VarLtmCellReconfig with the value received for this ltmCellReconfigId;

1>	else:

2>	add a new entry for this ltmCellReconfigId within the VarLtmCellReconfig;

##### 5.3.5.x.4	LTM Cell Group Reconfiguration execution

The UE shall:

1>	if lower layers indicate LTM Cell group switching:

2> apply the stored ltmCellReconfig of the candidate cell index indicated by the lower layer and perform the actions as specified in 5.3.5.3;

In the last RAN2, it is agreed that CellGroupConfig is mandatory as follow:

“P1 RAN2 to confirm that the CellGroupConfig IE is (mandatory) needed within an LTM candidate cell configuration.”

In addition, RAN2 also agreed that RRM configuration and radio bearer configuration may also be needed. In the following, we discuss these 2 points further to justify why they are needed.

RRM configuration

In intra-DU case, measurement configuration may remain the same since when a UE is served by source cell or a target cell (one cell among current candidate cells), it’s likely to have the same neighbour cells. For inter-DU case, this may be different as DU may be far apart with different neighbour cells or implemented by different vendors and the corresponding strategy of measurement configuration may be very different. Therefore, measurement reconfiguration may be needed at least for inter-DU case. If it is needed, CU should prepare the RRM configuration, as it’s not in DU scope.

Observation 1: measurement reconfiguration may be needed at least for inter-DU case.

L2 reset for radio bearer configuration

In our companion paper [2], we have comparison of how to perform L2 reset by comparing the different options below:

- Option 1: The UE determines whether the switch is intra DU or inter DU and the follows different rule or configuration for these two cases which controls whether to reset or not reset. Determination could be based on configuration (e.g. of a DU ID, cell group id etc)

  - Option 1a: indicate if MAC layer (as well as other protocol layer) needs to be reset or indicate if it is intra-DU or inter DU cell switch.

  - Option 1b: indicate DU identifier

- Option 2: The UE receives command to reset or not reset by MAC CE

Our view is that Option 1b using the DU identifier value to indicate whether the L2 reset is the better option.  Since the L2 reset involves also PDCP recovery, the DU identifier should not be included in the CellGroupConfig.

Observation 2:  Since the L2 reset involves also PDCP recovery, the DU identifier should not be included in the CellGroupConfig.

From the above discussion, there may be other configurations such as RRM and BWP indication be part of candidate cell configuration of a candidate cell that cannot be included cellGroupConfig IE, i.e., cell-group-level target cell configuration is not sufficient. Therefore, we think it is required and more flexible to use RRCReconfiguration to configure target candidate cells. Regarding the concern on signalling overhead, If delta configuration is supported, signaling overhead will not be an issue.

Proposal 1: RAN2 to agree to use Model 1: One RRCReconfiguration message for each candidate target configuration RRCReconfiguration to configure target candidate cells.

Finally, one remaining FFS how the UE determine the BWPs (for DL and UL) to be used upon the execution of L1/L2 inter-cell mobility. We think it can re-use the firstActiveUplinkBWP-Id and firstActiveDownlinkBWP-Id IE and follow legacy procedure. Network can configure these two BWP-Id in the pre-configuration for the UE. If it is not configured, the UE will use initial BWP to access the cell. Alternatively, network can indicate the BWP used in MAC CE. But it will save MAC CE size if it can be pre-configured in RRC.

Proposal 2: network can provide the firstActiveUplinkBWP-Id and firstActiveDownlinkBWP-Id in the pre-configuration of the candidate cell for the UE to determine the BWPs to be used upon LTM.

Proposal 3: if the firstActiveUplinkBWP-Id and firstActiveDownlinkBWP-Id is absent, the UE follow legacy procedure to determine the BWPs to be used upon LTM.

# Conclusion

Observation 1: measurement reconfiguration may be needed at least for inter-DU case.

Observation 2:  Since the L2 reset involves also PDCP recovery, the DU identifier should not be included in the CellGroupConfig.

Proposal 1: RAN2 to agree to use Model 1: One RRCReconfiguration message for each candidate target configuration RRCReconfiguration to configure target candidate cells.

Proposal 2: network can provide the firstActiveUplinkBWP-Id and firstActiveDownlinkBWP-Id in the pre-configuration of the candidate cell for the UE to determine the BWPs to be used upon LTM.

Proposal 3: if the firstActiveUplinkBWP-Id and firstActiveDownlinkBWP-Id is absent, the UE follow legacy procedure to determine the BWPs to be used upon LTM.

# References

[1] RP-213565		New WID on Further NR mobility enhancements		MediaTek

[2] R2-2300403		Discussion on LTM cell switch				Intel