3GPP TSG RAN WG2 Meeting #121                                                         R2-2300405

# Athens, Greece, 27th February - 3rd March 2023

Agenda item:	8.19.2

Source:	Intel Corporation

Title:	RAN2 impact to support eDRX in RRC_INACTIVE above 10.24sec

Document for:	Discussion and decision

# Introduction

This document discusses RAN2 impacts to support extended DRX (eDRX) in RRC_INACTIVE above 10.24 sec as captured in Rel-18 RedCap WID [1] as part of power saving/energy efficiency enhancements considering discussions and conclusions made by SA2 as captured in [2]-[7].

# Discussion

## SA2 agreements relevant for RAN2 discussion

The following points can be highlighted for RAN2 consideration based on SA2 inputs provided via LS [2][4], conclusions of TR 23.700-68 [5] and the agreed CRs on eDRX to TS 23.501 / 23.502 [6][7]:

Handling of MT traffic buffering and paging trigger for eDRX in RRC_INACTIVE

- When UE in RRC_INACTIVE is configured with eDRX cycle ≤ 10.24sec (Rel-17 behaviour)

  - RAN can buffer DL packets up to the duration of the eDRX cycle chosen by NG-RAN

- When UE in RRC_INACTIVE is configured with eDRX cycle > 10.24sec (Rel-18 new behaviour)

  - NG-RAN may send an indication to the AMF if the AMF supports the indication for the CN to handle MT communication.

    - It is left up to NG-RAN when to send this indication to AMF

    - If this indication is not sent to CN when UE in RRC_INACTIVE is configured with eDRX > 10.sec, CN cannot apply the high latency communication functionality, other NFs will not be aware of the UE reachability, certain high latency communication related services provided to the AF via NEF would not be available, NAS message delivery might fail and downlink data in RAN might be lost.

  - When CN handles MT for UE in RRC_INACTIVE with eDRX>10.24sec:

    - CN considers the UE in “CM_CONNECTED with RRC Inactive and eDRX >10.24sec“

    - From CN PoV, all MT data handling, high latency communication, between AMF, SMF, UPF, SMSF and any other NF follow same signalling flows as for a UE in CM-IDLE.

    - CN is also notified after the UE transitions back to RRC_CONNECTED (i.e. when UE resumes from RRC_INACTIVE with eDRX > 10.24sec to RRC_CONNECTED.

SA2 has enabled a mechanism for RAN to be able to enable handling of the MT traffic buffering and paging trigger in CN when UE is in RRC_INACTIVE with eDRX cycle value > 10.24 sec (referred in this discussion as “MT handling in CN for RAN paging” for simplicity).

Whether to use “MT handling in CN for RAN paging” is left up to RAN decision. However, if this is not used, CN cannot guarantee certain functionality (i.e. “CN cannot apply the high latency communication functionality, other NFs will not be aware of the UE reachability, certain high latency communication related services provided to the AF via NEF would not be available, NAS message delivery might fail and downlink data in RAN might be lost”).

When “MT handling in CN for RAN paging” is used, CN considers the UE in “CM_CONNECTED with RRC Inactive and eDRX >10.24sec”, buffers MT traffic, triggers paging to RAN (which would be in charge to trigger RAN-paging mechanism) and should be notified when UE resumes back to RRC_CONNECTED.  And CN is also notified when UE subsequently goes to RRC_INACTIVE.

## Configuration of eDRX > 10.24sec in RRC_INACTIVE

The following summarizes the key points on how eDRX in INACTIVE was defined in Rel-17:

- RAN can configure a UE with eDRX ≤ 10.24sec in RRC_INACTIVE by including ran-ExtendedPagingCycle-r17 in RRCRelease message. However this is only possible when eDRX in RRC_IDLE is also configured and the value of eDRX cycle in RRC_INACTIVE is shorter or equal to the value of eDRX cycle in RRC_IDLE.

- RAN indicates in SIB1 whether UEs in RRC_INACTIVE can use this feature (via eDRX-AllowedInactive-r17) and hyper SFN (hyperSFN-r17) is also broadcasted.

Similar operation could be reused for eDRX > 10.24sec with the difference that RAN might also get input on the eDRX to be used from CN. Although it might be good to discuss some details that could be reuse Rel-17 signaling or are specific to Rel-18 feature:

- How is UE configured with eDRX > 10.24sec and actual values/range to be supported. It is straight forward to define a new IE within suspendConfig to indicate whether UE is configured with eDRX > 10.24sec e.g. ExtendedPagingCycleAbove1024.  The actual values and range can be further discussed considering the associated reachability delay that it would introduce. The largest value possible is determined by H-SFN range, i.e. 174.76minutes (2.91hours). A UE needs to be able to be reachable via CN paging, therefore the values should match with the ones allowed by eDRX in RRC_IDLE. TS 38.304 defines “TeDRX_CN : UE-specific eDRX cycle in Hyper-frames, (T eDRX_CN  = 2, …, 1024 Hyper-frames) configured by upper layers” and TS 38.413 defines “NR Paging eDRX Cycle” with the possible values ”hfquarter, hfhalf, hf1, hf2, hf4, hf8, hf16, hf32, hf64, hf128, hf256, hf512, hf1024”. Therefore, the simplest approach could be to use the same values/range for eDRX in RRC_INACTIVE and RRC_IDLE.

- How does the network indicate to Rel-18 UEs in RRC_INACTIVE when this feature can be used. If all cells within a RNA have the same support of eDRX, this kind of indication would not be needed. On the other hand, for Rel-17, it was finally agreed due to network concerns on actual deployments to define an parameter to indicate when a UE in INACTIVE supporting eDRX ≤ 10.24sec is allowed or not to use the feature in a given cell, i.e. eDRX-AllowedInactive-r17.

eDRX-AllowedInactive

The presence of this field indicates that extended DRX for RAN paging is allowed in the cell for UEs in RRC_INACTIVE. The UE shall stop using extended DRX for RAN paging in RRC_INACTIVE if eDRX-AllowedInactive is not present.

For Rel-18, a new indication does not seem needed unless there are similar deployment concerns that not all the cells may be able to support this feature uniformly. For example, Rel-18 eDRX > 10.24sec in RRC_INACTIVE introduces a larger reachability delay and not all networks may be able to support it e.g. due to the required buffering.

When RAN configures a UE with eDRX above 10.24sec, RAN could inform CN as it is discussed in first section. This topic could be left up to RAN3.

- A new IE is defined in RRCRelease to optionally configure a UE with eDRX > 10.24sec in RRC_INACITVE as part of suspendConfig. The values and maximum range for eDRX > 10.24sec in RRC_INACTIVE are the same as for eDRX in RRC_IDLE.

- To discuss whether some form of indication is needed in SIB1 to indicate when RAN enables the usage of Rel-18 eDRX > 10.24sec in RRC_INACTIVE (e.g. reusing eDRX-AllowedInactive-r17 or a new one).

## SDT and eDRX > 10.24sec in RRC_INACTIVE

From SA2 discussion, it seems that SA2 has not discussed how would eDRX > 10.24sec for RRC_INACTIVE work if UE were also configured to use SDT. From NAS specification side, it seems the only reference to SDT is in CT1 TS 24.501 where the following note is captured:

NOTE 2:  If the UE supports Small Data Transmission (SDT) (see 3GPP TS 38.300 [27]), the following applies:

- if the UE due to pending uplink NAS messages or user data packets is requesting the lower layers to transition to RRC_CONNECTED state, but has not received a response from the lower layers, the UE can send the pending NAS messages or user data packets to the lower layers, and can receive multiple downlink NAS messages or multiple downlink user data packets from the lower layers while the UE remains in 5GMM-CONNECTED mode with RRC inactive indication over 3GPP access (i.e., without transitioning to 5GMM-CONNECTED mode). When the NAS layer triggers the transmission of pending uplink NAS messages or user data packets, and if the SDT is ongoing, the NAS layer will receive the response from the lower layers only after the SDT session has completed or failed;

- the NAS layer is not aware of the classification of NAS messages or the user data packets as belonging to the SDT session at the lower layers; and the setting of access category and the RRC establishment cause indicated to the lower layers when sending the pending uplink user data packets while the UE remains in 5GMM-CONNECTED

- mode with RRC inactive indication, is left to implementation.

SA2 has not discussed any interaction/handling of eDRX > 10.24sec and SDT for a UE in RRC_INACTIVE. It is important to keep in mind that SDT is a “transparent” feature to NAS and currently only a general NOTE is captured in TS 24.501.

Based on SA2 modeling explained in previous section, if a UE in RRC_INACTIVE were configured with SDT and eDRX > 10.24sec, the following operations could be considered:

- CN handles MT buffering. Current description on SA2 agreed CRs [6][7] does not seem to properly support SDT operation as CN is only notified when UE enters into “RRC_INACTIVE with eDRX > 10.24sec” and when goes back to “RRC_CONNECTED”.  As CN needs to be informed when the UE is reachable for eDRX >10.24s, SDT seems only feasible if additional NG-AP signaling is allowed when SDT starts/ends. However, this not only breaks the Rel-17 agreement that SDT is transparent to the CN but could also be complex in terms of modelling as UE is still in INACTIVE during SDT (i.e., CN will need to be informed that UE is reachable even though it is still in INACTIVE).  This also seems to diminish the benefit of reducing signaling and latency when using SDT. The extra signaling when moving from RRC_CONNECTED to RRC_INACTIVE and viceverse is a drawback also identified by SA2 as captured in TR 23.700-68 [5]. In summary, while it might be possible to support SDT with eDRX>10.24s, the current SA2 specs do not support it and  it might be good to check with SA2/CT whether CN could handle SDT data while UE is in INACTIVE with eDRX > 10.24sec. This would include both initiation of SDT due to MO or MT SDT data.

- RAN handles MT buffering. This seems allowed by current description on SA2 agreed CRs [6][7] as decision is left to RAN but it could introduce some issues on CN side as Observation 2 summarizes. The following were some of the concerns or drawback identified by SA2 and captured in TR TR 23.700-68 [5]:

  - The Downlink Data Delivery Status cannot be supported (unless the buffering is done in the SMF).

  - AMF is unaware of that the UE is in RRC_INACTIVE state and may not be reachable for a DL NAS message. This may result in that the DL NAS message is sent towards the UE at a timing when the UE will not monitor POs until the NAS retransmission timer expires.

  - UE would need to establish a new RRC connection at some point as network would release the UE into IDLE due to the NAS retransmission failure timer.

  - When the UE detects that it is outside the RNA it will perform RNA update due to mobility and the new RAN node will try to fetch the UE context based on the I-RNTI. If there is no Xn and the RAN node is unable to fetch the UE context the RAN node will reject the RRC Resume (RNAU) and the UE is moved to RRC-Idle. RAN does not inform the AMF about this. UE would need to establish a new RRC connection as network triggers a fallback from resume to setup. Although this is an issue that can exist for any legacy RRC_INACTIVE operation (depending on network deployment).

  - The buffered DL data in the old RAN node will be lost if nothing is done to forward the data to the new RAN node. Some of the proposed solutions suggest to leave the handling of the buffered DL data is done based on implementation, other ones to specify CN assisted UE context/DL data forwarding from the old RAN node to the new RAN node.

  - The buffering capacity of RAN would need to be extended (beyond legacy i.e. 5.12 or 10.24sec)

SA2 captured in NAS specification that both modes of operation (mode 1 and mode 2) are allowed considering that RAN3 requested to SA2 in LS [3] that final decision on which mode of operation is preferred to handle the MT buffering should be left to network implementation:

“On the triggering criteria for gNB sending a CN based MT communication handling request to CN, it is decided by RAN3 to leave it to network implementation”

In summary, SA2 indeed weighed the PROS/CONS from CN on how to support eDRX > 10.24sec for a UE in RRC_INACTIVE but does not seem to have considered SDT in that discussion. We suggest discussing whether SDT needs to be supported with eDRX > 10.24sec and if so, whether there is anything to consider on the design of eDRX in NAS and/or AS layer.

If specification allows network to configure SDT and eDRX > 10.24sec at the same time to a UE in RRC_INACTIVE, there are two possible modes of operation: mode (1) CN handles the MT buffering and mode (2) RAN handles the MT-buffering.  Both options have issues that need to be addressed in SA2 and RAN2.

Based on this, we see two possible options in RAN2:

- Specification precludes that a UE to be configured with eDRX >10.24sec and SDT at the same time. This option might be desirable by UE if there are concerns or implications on the NAS handling or NAS/AS interactions.

- Specification allows that a UE to be configured with eDRX >10.24sec and SDT at the same time.

- It is left up to network implementation the appropriate handling when/whether eDRX >10.24sec and SDT are configured to a UE at the same time. This might be sufficient if the concern is only in network side; e.g., network and operators might be able to take final decision on whether the concerns listed above are acceptable or can be avoided via their network and/or UE configurations.

- Specification explicitly allows that a UE is configured with eDRX >10.24sec and SDT at the same time and possible concerns are addressed or minimized. For example:

- For mode 1) CN handles MT buffering, RAN2 could check with SA2/CT1 whether additional CN/NAS signaling can be allowed to allow SDT operation.

- For mode 2) RAN handles MT buffering, the list of concerns in RAN/SA side could be discussed and see which ones need to be addressed by specification. For example, concern (a)  does not seem feasible to support based on SA2 discussion; for concerns (b) and (c), it might be sufficient if RAN precludes the usage of SRB2 for SDT or if NAS enable new signaling to be aware when SDT is ongoing; and, for concerns (d) to (f), it might be sufficient to relay on proper network configuration/implementation or to discuss whether some simple enhancement could be enabled in the network side.

While SA2 specifications and Rel-18 CRs do not seem to support a combination of eDRX > 10.24sec and SDT for a UE in RRC_INACTIVE, it may be possible to support it in certain deployment scenarios, and/or may require some updates to SA2 specifications.

- To discuss whether network can configure SDT and eDRX > 10.24sec at the same time to a UE in RRC_INACTIVE, considering option 1) specification precludes this scenario, or option 2) specification allows this. If this scenario is supported from RAN2 point of view, inform SA2/CT1/RAN3 for any feedback.

## Paging for UE in RRC_INACTIVE with eDRX > 10.24sec

Paging in extended DRX mechanism (defined in section 7.4 of TS 38.304) and Discontinuous Reception for paging (defined in section 7.1 of TS 38.304) is reused as baseline when a UE in RRC_INACTIVE with eDRX > 1.24sec. From this, RAN2 could start discussing the required changes/updates. For example:

- In section 7.4, the calculation of the PTW, HF is equally applicable for eDRX > 10.24sec in RRC_INACTIVE.

- In section 7.2, the new conditions and how T is calculated would need to be included when eDRX is > 10.24sec in RRC_INACTIVE.

- Paging in extended DRX mechanism (defined in section 7.4 of TS 38.304) and Discontinuous Reception for paging (defined in section 7.1 of TS 38.304) is reused as baseline when a UE in RRC_INACTIVE with eDRX > 1.24sec. FFS required updates, for example discuss the new condition/calculation for T in section 7.1 of TS 38.304.

## SI notification for a UE in RRC_INACTIVE with eDRX > 10.24sec

In Rel-17, a new indication is defined in the Short Message to indicate of a BCCH modification other than SIB6, SIB7 and SIB8 (systemInfoModification-eDRX). Its definition explains that this indication applies only to UEs using eDRX cycle longer than the BCCH. Therefore, current definition is already equally applicable for this new feature. It is important to understand that this was already accounting for UEs in RRC_IDLE that may also be configured with eDRX >10.24sec. In summary, it can be reused with no major specification change. The procedural text on 38.331 would need to also allow UEs in RRC_INACTIVE to also receive this indication:

1>	if the UE is configured with an RRC_INACTIVE or RRC_IDLE eDRX cycle longer than the modification period and the systemInfoModification-eDRX bit of Short Message is set:

2>	apply the SI acquisition procedure as defined in clause 5.2.2.3 from the start of the next eDRX acquisition period boundary.

- Legacy systemInfoModification-eDRX indication in Short message is applicable for Rel-18 UEs in RRC_INACTIVE configured with eDRX > 10.24sec.

## UE capabilities for eDRX > 10.24sec in RRC_INACTIVE

In Rel-17, a new optional capability is defined for eDRX in RRC_INACTIVE ≤ 10.24sec (i.e., extendedDRX-CycleInactive-r17). For similar reasons, a new capability would be required for eDRX in RRC_INACTIVE > 10.24sec.

- A new optional capability with signaling and per UE is defined for Rel-18 UEs in RRC_INACTIVE configured with eDRX > 10.24sec.

# Conclusion

The observations captured are the following:

Observation 1.	SA2 has enabled a mechanism for RAN to be able to enable handling of the MT traffic buffering and paging trigger in CN when UE is in RRC_INACTIVE with eDRX cycle value > 10.24 sec (referred in this discussion as “MT handling in CN for RAN paging” for simplicity).

Observation 2.	Whether to use “MT handling in CN for RAN paging” is left up to RAN decision. However, if this is not used, CN cannot guarantee certain functionality (i.e. “CN cannot apply the high latency communication functionality, other NFs will not be aware of the UE reachability, certain high latency communication related services provided to the AF via NEF would not be available, NAS message delivery might fail and downlink data in RAN might be lost”).

Observation 3.	When “MT handling in CN for RAN paging” is used, CN considers the UE in “CM_CONNECTED with RRC Inactive and eDRX >10.24sec”, buffers MT traffic, triggers paging to RAN (which would be in charge to trigger RAN-paging mechanism) and should be notified when UE resumes back to RRC_CONNECTED.  And CN is also notified when UE subsequently goes to RRC_INACTIVE.

Observation 4.	SA2 has not discussed any interaction/handling of eDRX > 10.24sec and SDT for a UE in RRC_INACTIVE. It is important to keep in mind that SDT is a “transparent” feature to NAS and currently only a general NOTE is captured in TS 24.501.

Observation 5.	If specification allows network to configure SDT and eDRX > 10.24sec at the same time to a UE in RRC_INACTIVE, there are two possible modes of operation: mode (1) CN handles the MT buffering and mode (2) RAN handles the MT-buffering.  Both options have issues that need to be addressed in SA2 and RAN2.

Observation 6.	While SA2 specifications and Rel-18 CRs do not seem to support a combination of eDRX > 10.24sec and SDT for a UE in RRC_INACTIVE, it may be possible to support it in certain deployment scenarios, and/or may require some updates to SA2 specifications.

The proposals captured are the following:

Proposal 1.	A new IE is defined in RRCRelease to optionally configure a UE with eDRX > 10.24sec in RRC_INACITVE as part of suspendConfig. The values and maximum range for eDRX > 10.24sec in RRC_INACTIVE are the same as for eDRX in RRC_IDLE.

Proposal 2.	To discuss whether some form of indication is needed in SIB1 to indicate when RAN enables the usage of Rel-18 eDRX > 10.24sec in RRC_INACTIVE (e.g. reusing eDRX-AllowedInactive-r17 or a new one).

Proposal 3.	To discuss whether network can configure SDT and eDRX > 10.24sec at the same time to a UE in RRC_INACTIVE, considering option 1) specification precludes this scenario, or option 2) specification allows this. If this scenario is supported from RAN2 point of view, inform SA2/CT1/RAN3 for any feedback.

Proposal 4.	Paging in extended DRX mechanism (defined in section 7.4 of TS 38.304) and Discontinuous Reception for paging (defined in section 7.1 of TS 38.304) is reused as baseline when a UE in RRC_INACTIVE with eDRX > 1.24sec. FFS required updates, for example discuss the new condition/calculation for T in section 7.1 of TS 38.304.

Proposal 5.	Legacy systemInfoModification-eDRX indication in Short message is applicable for Rel-18 UEs in RRC_INACTIVE configured with eDRX > 10.24sec.

Proposal 6.	A new optional capability with signaling and per UE is defined for Rel-18 UEs in RRC_INACTIVE configured with eDRX > 10.24sec.

# Reference

- RP-223544, Enhanced support of reduced capability NR devices, Rel-18 WID, December 2022.

- R2-2211136 / S2-2209958, LS On long eDRX support for RRC_INACTIVE, From: SA2, To: RAN2, RAN3, October 2022.

- R3-226776, Reply LS on long eDRX support for RRC_INACTIVE, From: RAN3, To: SA2, Cc: RAN2, November 2022.

- S2-2301858, Reply LS on long eDRX support for RRC_INACTIVE, From: SA2, To: RAN2, RAN3, January 2023.

- TR 23.700-68, Study on support of reduced capability NR devices, SA2, v18.1.0, December 2022.

- S2-2209583 / S2-2301477, eDRX CRs to TS 23.501 including CR# 3705 (October, 2022) and CR# 4081 (January, 2023).

- S2-2209675 / S2-23101859 / S2-23101448, eDRX CRs to TS 23.502 including CR# 3555 (October, 2022), CR# 3685 (January, 2023) and CR# 3822 (January, 2023).