3GPP TSG RAN WG2 Meeting #121                                                         R2-2300406

# Athens, Greece, 27th February - 3rd March 2023

Agenda item:	8.19.3

Source:	Intel Corporation

Title:	RAN2 impacts to support UEs with Baseband Bandwidth Reduction

Document for:	Discussion and decision

# Introduction

This document discusses RAN2 impacts to support UEs with Baseband (BB) Bandwidth (BW) reduction as captured in Rel-18 RedCap WID [1] as part of the UE complexity/cost reduction in FR1:

UE BB bandwidth reduction [RAN1, RAN2, RAN4]

- 5 MHz BB bandwidth only for PDSCH (for both unicast and broadcast) and PUSCH, with 20 MHz RF bandwidth for UL and DL

- The other physical channels and signals are still allowed to use a BWP up to the 20 MHz maximum UE RF+BB bandwidth.

- Support additional separate early indication(s) [RAN1, RAN2]

Both 15 kHz SCS and 30 kHz SCS are supported.

Aim to define at most one Rel-18 RedCap UE type for further UE complexity reduction.

The existing UE capability framework is used, and changes to capability signalling are specified only if necessary. By default, all UE capabilities applicable to a Rel-17 RedCap UE are applicable unless otherwise specified.

# Discussion

## Summary on RAN1 agreements relevant for RAN2 discussion

The following summarizes RAN1 agreements to help identifying related topics of interest for RAN2 to discuss:

- R18 RedCap UE need to still receive (i.e. store) 20MHz (i.e. RF is not reduced) and only the max. number of PRBs to decode/transmit per slot is reduced

  - R18 RedCap UE can share same/separate initial BWP specific to R17 RedCap UEs.

    - NOTE-1: FFS on need to support separate initial BWP specific to R18 RedCap UEs

  - Post-FFS buffer size is still 20MHz for both broadcast/unicast (same as legacy)

  - For PUSCH (unicast) and PDSCH (unicast/broadcast), the maximum number of PRBs that the UE can decode per slot is reduced:

    - for 15kHz - To 25 PRB per slot (or also per hop for PUSCH)

    - for 30kHz - To X per slot (or also per hop for PUSCH).

      - NOTE-2: FFS if X is 11 or 12

  - Depending on the PHY channel (further details in next sections)

    - Network might be allowed to provide more number of PRBs (#PRBs) than the maximum number of PRBs (max_#PRBs) that the UE can process per slot

      - When network provides #PRBs > max_#PRBs, the understanding is that UE’s time to decode is relaxed

        - NOTE-3: FFS whether this time is or not specified for each case

    - The different PRBs per slow might be allowed only continuous or also distributed (i.e., no contiguous) depending on the specific case

  - Mode of operations on the BW used by the #PRBs allocated to the UE in a slot (or hope for PUSCH)

    - Mode A) BW occupied by the #PRBs in one slot can be larger than 5MHz (same as in legacy, i.e., the total #PRBs in one slot can be larger than the max_#PRB)

      - SIB1 and Other System Information (OSI)

        - Legacy SIB(s) can be used

      - Paging BW

      - RAR

        - If time between RAR reception and Msg3 transmission is more than [NT,1 + NT,2 + 0.5 + X ms]

          - UE can receive RAR and transmits Msg3

          - NOTE-5: X is FFS

        - Else,

          - UE behavior is up to implementation

          - NOTE-6: if Early Indication (EI) in Msg1 is used to differentiate R18 RedCap UE, network can avoid this scenario

    - Mode B) BW occupied by the #PRBs in one slot can be larger than 5MHz but the total #PRBs in one slot is smaller or equal to max_#PRBs (therefore, those PRBs can be non-contiguous, i.e., distributed)

      - PDSCH (unicast)

    - Mode C) BW occupied by the #PRBs in one slot is smaller or equal to 5MHz

      - Msg.3

      - PUSCH (unicast)

The following points might be important to highlight from above summary of RAN1 agreements:

For SIB1/OSI, paging and RAR, network can provide a greater number of PRBs (#PRBs) than the maximum number of PRBs (max_#PRBs) that the UE can process per slot with the understanding that UE’s time to decode this is relaxed.

For SIB1/OSI and paging, RAN1 preference seems not to specify the time that UE may take to decode the PRBs sent in one slot as there is no immediate respond from UE to the network required.

For RAR, RAN1 plans to specify the minimum time required between RAR and Msg.3 for UE to guarantee that UE can receive RAR and transmit Msg.3. If this is not met, UE behaviour is left up to implementation although this case can be avoided by the network if early identification in Msg.1 is used.

For unicast (PDSCH, Msg.3 and PUSCH), the total number of PRBs per slot is smaller or equal to the maximum number supported. For PDSCH, those PRBs can be allocated anywhere in the total BW (i.e. PRBs can be distributed) and for Msg.3 and PUSCH, those PRBs need to be allocated within a 5MHz BW.

Considering RAN1 agreements so far it seems like Rel-18 RedCap will not be backwards compatible to R17 RedCap (i.e., Rel-17 RedCap network cannot support R18 RedCap UE without implementing R18 specific changes). Depending on the final design, it may be possible that Rel-18 RedCap network is able to handle Rel-17 RedCap UEs even if R17 RedCap is not supported.

Rel-18 RedCap does not seem backwards compatible to R17 RedCap (i.e., Rel-17 RedCap network cannot support R18 RedCap UE without implementing R18 specific changes).

From above summary of RAN1 agreements the following captures RAN1 inter-related topic that in our understanding RAN2 could start discussing (where different color font is used to corresponding details above):

- Topic 1) UE capability update required for Rel-18 UE supporting baseband (BB) bandwidth (BW) reduction.

- Topic 2) Initial BWP for Rel-18 RedCap UEs (e.g. operation and configuration)

- Topic 3) Additional separate early indication(s) including Msg.1/Msg.A PRACH and/or Msg.3/MsgA PUSCH.

- Topic 4) Layer 2 buffer size for DL/UL considering the reduction on the number of PRBs and the BWP decoded. This topic is discussed in our companion document [2] as it is inter-related to the discussion on data rate discussion (which is handled in this other document [2]).

There will be configurations that might require new definition or update but for this we suggest RAN2 wait for further progress on RAN1 side before discussing this topic.

Other RAN2-centric topic that might require some initial discussion is the following:

- Topic 5) Access control for Rel-18 RedCap UEs (e.g. cell barring)

On summary. we suggest RAN2 to start with the identification of the key areas of discussion.

- For Rel-18 UEs supporting Baseband (BB) Bandwidth (BW) reduction, RAN2 discusses expected behaviour and foreseen RAN2 impacts (if any) at least in the following topics: 1) UE capabilities, 2) initial BWP specific to RedCap, 3) additional/separate early indication (EI), 4) Layer 2 buffer size and 5) access control.

## UE capability impact

Rel-18 WID [1] captures that “at most one Rel-18 RedCap UE type for further UE complexity reduction”. From RAN1 discussion and conclusions, it seems straight forward to agree that at least this new capability should help the network to differentiate UEs with reduced BB BW. For Rel-17, the maximum FR1 RedCap UE bandwidth is 20 MHz (which is mandatory for Rel-17 RedCap UE. Therefore supportOfRedCap-r17 would not be applicable for Rel-18 RedCap UE.

- A new UE capability is defined for network know whether UE supports Baseband Bandwidth Reduction.

  - Baseband Bandwidth Reduction is defined as per UE feature with optional with capability signal, ‘FR1 only’ to the need to Diff. FR1/FR2 (as this feature is defined for FR1), and ‘No’ to the need to Diff. TDD/FDD.

RAN2 should discuss whether all UE capabilities from Rel-17 RedCap UE are or not applicable to Rel-18 RedCap UE keeping in mind that Rel-18 WID [1] captures that by default, all UE capabilities applicable to a Rel-17 RedCap UE are applicable unless otherwise specified. RAN2 feature list defined for Rel-17 RedCap is summarized below:

| Feature group | Components | Field name in TS 38.331 | Parent IE in TS 38.331 |
| --- | --- | --- | --- |
| Support 16 DRBs | Defines whether the RedCap UE supports 16 DRBs as specified in TS 38.331 [2]. | supportOf16DRB-RedCap-r17 | RedCapParameters-r17 |
| Support 18 bit length of PDCP sequence number | Defines whether the RedCap UE supports 18 bit length of PDCP sequence number as specified in TS 38.331 [2]. | longSN-RedCap-r17 | PDCP-Parameters |
| Support AM DRB with 18 bit length of RLC sequence number | Defines whether the RedCap UE supports AM DRB with 18 bit length of RLC sequence number as specified in TS 38.331 [2]. | am-WithLongSN-RedCap-r17 | RLC-Parameters |
| Support of RRM relaxation in RRC_CONNECTED | Defines whether the RedCap UE supports Rel-17 relaxed RRM measurements in RRC_CONNECTED as specified in TS 38.331 [2]. | rrm-RelaxationRRC-ConnectedRedCap-r17 | MeasAndMobParametersCommon |
| Support of extended DRX in RRC_INACTIVE | Defines whether a UE (RedCap or Non-RedCap) supports the extended DRX in RRC_INACTIVE with values of 256, 512 and 1024 radio frames as specified in TS 38.331 [2]. The UE may indicate support for extended DRX in RRC_INACTIVE only if it supports extended DRX in RRC_IDLE. | extendedDRX-CycleInactive-r17 | MAC-ParametersCommon |

RAN1 feature list defined for Rel-17 RedCap is summarized below:

| Feature group | Components | Field name TS 38.331 | Parent IE TS 38.331 |
| --- | --- | --- | --- |
| RedCap UE | 1. Maximum FR1 RedCap UE bandwidth is 20 MHz. 2. Maximum FR2 RedCap UE bandwidth is 100 MHz. 3. Early indication of RedCap UE in Msg.1 for 4-step RACH 4. Separate initial UL BWP for RedCap UEs - It includes the configuration(s) needed for RedCap UE to perform random access - Enabling/disabling of frequency hopping for common PUCCH resources 5. Separate initial DL BWP for RedCap UEs - It includes CSS/CORESET for random access - For separate initial DL BWP used for paging, CD-SSB is included - For separate initial DL BWP only used for RACH, SSB may or may not be included - For separate initial DL BWP used in connected mode as BWP#0 configuration option 1, CD-SSB is included 6. 1 UE-specific RRC configured DL BWP per carrier 7. 1 UE-specific RRC configured UL BWP per carrier 8. RRC reconfiguration of any parameters related to BWP 9. UE-specific RRC configured DL BWP with CD-SSB or NCD-SSB 10. NCD-SSB based measurements in RRC-configured DL BWP | supportOfRedCap-r17 | RedCapParameters-r17 |
| RRC-configured DL BWP without CD-SSB or NCD-SSB | RRC-configured DL BWP without CD-SSB or NCD-SSB | bwp-WithoutCD-SSB-OrNCD-SSB-RedCap-r17 | BandNR |
| Half-duplex FDD operation type A for RedCap UE | 1. Half-duplex FDD operation (instead of full-duplex FDD operation) type A for RedCap UE | halfDuplexFDD-TypeA-RedCap-r17 | BandNR |

- RAN2-centric Rel-17 RedCap UE capabilities are applicable to Rel-18 RedCap UE (which includes supportOf16DRB-RedCap-r17, longSN-RedCap-r17, am-WithLongSN-RedCap-r17, rrm-RelaxationRRC-ConnectedRedCap-r17, extendedDRX-CycleInactive-r17).

  - Discuss whether any of these Rel-17 RedCap capabilities is not applicable or requires any update for Rel-18.

- Ask RAN1 on RAN1 centric features for Rel-18 RedCap including the applicability (or not) of Rel-17 RedCap features (which include supportOfRedCap-r17, bwp-WithoutCD-SSB-OrNCD-SSB-RedCap-r17, halfDuplexFDD-TypeA-RedCap-r17).

## Access control impact

Rel-17 RedCap allows the network to indicate if Half-Duplex FDD RedCap UEs (halfDuplexRedCapAllowed-r17) is supported as well as whether the cell is barred or not for a RedCap UE with 1 Rx branch (cellBarredRedCap1Rx) or for RedCap UE with 2 Rx branch (cellBarredRedCap2Rx). This access control defined specific to Rel-17 RedCap can be included in SIB1 and can be used by UE to implicitly know whether the cell supports/allows or not RedCap UEs.

Access control specific to Rel-17 RedCap UEs indicates the UE whether a cell supports and allows the access of Rel-17 RedCap UEs that are half duplex, or have 1 or 2 Rx branches.

Like Rel-17 RedCap, it seems beneficial that network can control the access of Rel-18 RedCap UEs. This new indication to barrer Rel-18 RedCap UEs could be specified as part of legacy RedCap-ConfigCommonSIB-r17.

RedCap-ConfigCommonSIB-r17 ::= SEQUENCE {

halfDuplexRedCapAllowed-r17    ENUMERATED {true}               OPTIONAL,  -- Need R

cellBarredRedCap-r17           SEQUENCE {

cellBarredRedCap1Rx-r17        ENUMERATED {barred, notBarred},

cellBarredRedCap2Rx-r17        ENUMERATED {barred, notBarred}

}                   OPTIONAL,  -- Need R

...

}

- Network indicates the UE whether Rel-18 RedCap UEs are or not allowed in a given cell. This new indication is defined within RedCap-ConfigCommonSIB-r17. FFS on the actual information or granularity.

Further discussion may be needed to understand whether the indications that limit the access of Rel-17 RedCap UEs (i.e. halfDuplexRedCapAllowed-r17, cellBarredRedCap1Rx, cellBarredRedCap2Rx) could be re-used/applied also for Rel-18 RedCap UEs and a different flag indication is defined to indicate whether the barring also applies or not to Rel-18 RedCap UEs. Alternatively Rel-18 RedCap specific indications may need to be defined for half duplex, 1 Rx and 2Rx).

This section refers to Rel-18 RedCap UEs in general without indicating whether differentiation between UE supporting BB BW reduction and UE supporting data rate reduction is required. We suggest leaving this FFS until further conclusion/progress is done on how these two features work/interact with each other.

- Confirm that network wants to be able to indicate its support/barred control for Rel-18 RedCap UEs that are half duplex and with 1 or 2 RX branch (i.e., there are different barring for each as it is done for Rel-17 RedCap UEs).

  - If Proposal 6 is agreed, to discuss whether option (1) Rel-17 RedCap barring signaling can be re-used/applied (halfDuplexRedCapAllowed-r17, cellBarredRedCap1Rx-r17, cellBarredRedCap2Rx-r17) for Rel-18 RedCap UEs (in which case a new barring flag would also be defined to indicate whether barring applies to Rel-18 RedCap UEs) or option (2) new Rel-18 barring specific parameters are defined to Rel-18 RedCap UEs for half duplex and with 1 RX or 2 RX branch.

Rel-17 RedCap also defined a parameter in SIB1 to control intra-frequency cell (re)selection depending on whether the cell is or not barred for this UEs.

intraFreqReselectionRedCap

Controls cell selection/reselection to intra-frequency cells for RedCap UEs when this cell is barred, or treated as barred by the RedCap UE, as specified in TS 38.304 [20]. If not present, a RedCap UE treats the cell as barred, i.e.,the UE considers that the cell does not support RedCap.

For rel-18, it would be discussed whether same parameter could also apply for Rel-18 RedCap UEs or different one is desirable for

- Confirm that network wants to control intra-frequency cell (re)selection depending on whether the cell is or not barred for Rel-18 RedCap UEs. If so, to discuss whether to reuse Rel-17 RedCap indication (intraFreqReselectionRedCap-r17) or a new one specific to Rel-18 RedCap UE is preferable.

## Initial BWP for Rel-18 RedCap UE

RAN1 already agreed that Rel-18 RedCap UE can share same/separate initial BWP specific to Rel-17 RedCap UEs but further options/details were left FFS and RAN2 point of view seems helpful.

RAN1 already agreed that RedCap specific initial BWP can be used by Rel-18 RedCap UE as well as legacy Rel-17 RedCap UEs.

One open question is whether network needs to be able to configure the usage of RedCap-specific initial BWP distinguish whether it is applicable Rel-17 RedCap UEs and/or Rel-18 RedCap UEs. This would mean that network could configure the usage of the RedCap-specific initial BWP to (1) only Rel-17 RedCap (as specified in Rel-17), (2) only Rel-18 RedCap and (3) both Rel-17 and Rel-18 RedCap UEs. Case (3) seems the simplest approach however legacy signaling already allows network to indicate whether Rel-17 RedCap is or not allowed/barred as explained in previous section (by the parameters defined as part of RedCap-ConfigCommonSIB-r17). Therefore it might be also easy to enable the option for the network to configure either case if this seem helpful from network and operator’s point of view.

Another open question is whether an additional/separate Rel-18 RedCap-specific BWP is or not needed. This topic is currently under discussion by RAN1, therefore we suggest waiting for RAN1 conclusion on it. On summary, if additional/separate R18 RedCap specific BWP is needed, then case 1 and case 2 above are valid, i.e. network will configure R17 redcap specific BWP for R17 RedCap UEs, and R18 RedCap specific BWP for R18 RedCap UEs.  However if RAN1 agree that it is not needed, then case 3 is valid, i.e. same BWP is applied for both R17 and R18 UEs.

- Wait for RAN1 conclusion on whether an additional/separate Rel-18 RedCap-specific BWP is or not needed, i.e., no need to discuss this topic in RAN2 for now.

  - Discuss whether from RAN2 point of view, network needs to be able to configure the usage of RedCap specific initial BWP between (1) only Rel-17 RedCap (as specified in Rel-17), (2) only Rel-18 RedCap and (3) both Rel-17 and Rel-18 RedCap UEs.

## Early Indication for Rel-18 RedCap UE

Rel-18 WID [1] captures to “support additional separate early indication(s)”. This means that some form of early identification is enabled for Rel-18 RedCap, i.e. network should distinguish these UEs by Msg.1 and/or Msg.3.

- Confirm that some form of early identification is enabled for Rel-18 RedCap. I.e., network should be able to distinguish Rel-18 RedCap UEs by Msg.1 and/or Msg.3.

RAN1 and RAN2 input would be required to determine whether early identification (EI) via Msg1 is or not required for Rel-18 RedCap UEs. During Rel-17 RedCap discussion, it was explained that EI Msg1 would be helpful for the network to be able to differentiate UEs that may have 1 or 2 Rx antennas. This justification would be equally applicable to Rel-18 RedCap UEs. In addition, there is a Rel-18 RedCap specific/new motivation to justify the need of EI Msg.1: this can help network to know when to relax the time allocation between RAR and Msg.2 if RAR were larger than 5MHz. However, limiting the size of RAR might not be as critical e.g., if network is already to differentiate that this UE as a RedCap UE (regardless on whether it is a Rel-17 or Rel-18 UE). From RAN2 point of view, it does not seem essential to differentiate EI Msg1 between Rel-17 and Rel-18 RedCap UEs, although final decision should be left up to RAN1.

- Early indication (EI) via Msg.1 is helpful from RAN2 point of view for Rel-18 RedCap.  EI Msg.1 specific to Rel-18 RedCap is not essential; reusing EI. Msg.1 of Rel-17 RedCap is sufficient. Final decision/details to be concluded by RAN1.

On other hand, if separate Msg.1 for Rel-18 RedCap UE were not preferable or used by the network, it is needed to use Msg.3 differentiation specific to Rel-18 RedCap UE. Due to the BB BW limitation and the relaxation on the decoding time, it seems critical that Rel-18 RedCap UEs are differentiated when performing unicast data transmission. During Rel-17 RedCap WI, the preference was to allocate new LCID, this can be a similar approach that could be considered to differentiate Rel-18 RedCap at Msg.3. Another alternative could be via RRC.

- Early indication via Msg.3 (e.g. defining new LCID) is enabled for network to differentiate Rel-18 RedCap UEs from other kind of UEs (including Rel-17 RedCap UEs) considering their limitation in the number of PRBs.

# Conclusion

The observations captured are the following:

Observation 1.	For SIB1/OSI, paging and RAR, network can provide a greater number of PRBs (#PRBs) than the maximum number of PRBs (max_#PRBs) that the UE can process per slot with the understanding that UE’s time to decode this is relaxed.

Observation 2.	For SIB1/OSI and paging, RAN1 preference seems not to specify the time that UE may take to decode the PRBs sent in one slot as there is no immediate respond from UE to the network required.

Observation 3.	For RAR, RAN1 plans to specify the minimum time required between RAR and Msg.3 for UE to guarantee that UE can receive RAR and transmit Msg.3. If this is not met, UE behaviour is left up to implementation although this case can be avoided by the network if early identification in Msg.1 is used.

Observation 4.	For unicast (PDSCH, Msg.3 and PUSCH), the total number of PRBs per slot is smaller or equal to the maximum number supported. For PDSCH, those PRBs can be allocated anywhere in the total BW (i.e. PRBs can be distributed) and for Msg.3 and PUSCH, those PRBs need to be allocated within a 5MHz BW.

Observation 5.	Rel-18 RedCap does not seem backwards compatible to R17 RedCap (i.e., Rel-17 RedCap network cannot support R18 RedCap UE without implementing R18 specific changes).

Observation 6.	Access control specific to Rel-17 RedCap UEs indicates the UE whether a cell supports and allows the access of Rel-17 RedCap UEs that are half duplex, or have 1 or 2 Rx branches.

Observation 7.	RAN1 already agreed that RedCap specific initial BWP can be used by Rel-18 RedCap UE as well as legacy Rel-17 RedCap UEs.

The proposals captured are the following:

Proposal 1.	For Rel-18 UEs supporting Baseband (BB) Bandwidth (BW) reduction, RAN2 discusses expected behaviour and foreseen RAN2 impacts (if any) at least in the following topics: 1) UE capabilities, 2) initial BWP specific to RedCap, 3) additional/separate early indication (EI), 4) Layer 2 buffer size and 5) access control.

Proposal 2.	A new UE capability is defined for network know whether UE supports Baseband Bandwidth Reduction.

Proposal 2.1.	Baseband Bandwidth Reduction is defined as per UE feature with optional with capability signal, ‘FR1 only’ to the need to Diff. FR1/FR2 (as this feature is defined for FR1), and ‘No’ to the need to Diff. TDD/FDD.

Proposal 3.	RAN2-centric Rel-17 RedCap UE capabilities are applicable to Rel-18 RedCap UE (which includes supportOf16DRB-RedCap-r17, longSN-RedCap-r17, am-WithLongSN-RedCap-r17, rrm-RelaxationRRC-ConnectedRedCap-r17, extendedDRX-CycleInactive-r17).

Proposal 3.1.	Discuss whether any of these Rel-17 RedCap capabilities is not applicable or requires any update for Rel-18.

Proposal 4.	Ask RAN1 on RAN1 centric features for Rel-18 RedCap including the applicability (or not) of Rel-17 RedCap features (which include supportOfRedCap-r17, bwp-WithoutCD-SSB-OrNCD-SSB-RedCap-r17, halfDuplexFDD-TypeA-RedCap-r17).

Proposal 5.	Network indicates the UE whether Rel-18 RedCap UEs are or not allowed in a given cell. This new indication is defined within RedCap-ConfigCommonSIB-r17. FFS on the actual information or granularity.

Proposal 6.	Confirm that network wants to be able to indicate its support/barred control for Rel-18 RedCap UEs that are half duplex and with 1 or 2 RX branch (i.e., there are different barring for each as it is done for Rel-17 RedCap UEs).

Proposal 6.1.	If Proposal 6 is agreed, to discuss whether option (1) Rel-17 RedCap barring signaling can be re-used/applied (halfDuplexRedCapAllowed-r17, cellBarredRedCap1Rx-r17, cellBarredRedCap2Rx-r17) for Rel-18 RedCap UEs (in which case a new barring flag would also be defined to indicate whether barring applies to Rel-18 RedCap UEs) or option (2) new Rel-18 barring specific parameters are defined to Rel-18 RedCap UEs for half duplex and with 1 RX or 2 RX branch.

Proposal 7.	Confirm that network wants to control intra-frequency cell (re)selection depending on whether the cell is or not barred for Rel-18 RedCap UEs. If so, to discuss whether to reuse Rel-17 RedCap indication (intraFreqReselectionRedCap-r17) or a new one specific to Rel-18 RedCap UE is preferable.

Proposal 8.	Wait for RAN1 conclusion on whether an additional/separate Rel-18 RedCap-specific BWP is or not needed, i.e., no need to discuss this topic in RAN2 for now.

Proposal 8.1.	Discuss whether from RAN2 point of view, network needs to be able to configure the usage of RedCap specific initial BWP between (1) only Rel-17 RedCap (as specified in Rel-17), (2) only Rel-18 RedCap and (3) both Rel-17 and Rel-18 RedCap UEs.

Proposal 9.	Confirm that some form of early identification is enabled for Rel-18 RedCap. I.e., network should be able to distinguish Rel-18 RedCap UEs by Msg.1 and/or Msg.3.

Proposal 10.	Early indication (EI) via Msg.1 is helpful from RAN2 point of view for Rel-18 RedCap.  EI Msg.1 specific to Rel-18 RedCap is not essential; reusing EI. Msg.1 of Rel-17 RedCap is sufficient. Final decision/details to be concluded by RAN1.

Proposal 11.	Early indication via Msg.3 (e.g. defining new LCID) is enabled for network to differentiate Rel-18 RedCap UEs from other kind of UEs (including Rel-17 RedCap UEs) considering their limitation in the number of PRBs.

# Reference

- RP-223544, Enhanced support of reduced capability NR devices, Rel-18 WID, December 2022.

- R2-2300407, RAN2 impacts to support UEs with Data Rate Reduction, Intel Corporation, February 2023.