3GPP TSG RAN WG2 Meeting #121                                                         R2-2300407

# Athens, Greece, 27th February - 3rd March 2023

Agenda item:	8.19.3

Source:	Intel Corporation

Title:	RAN2 impacts to support UEs with Data Rate Reduction

Document for:	Discussion and decision

# Introduction

This document discusses RAN2 impacts to support UEs with peak data rate reduction as captured in Rel-18 RedCap WID [1] as part of the UE complexity/cost reduction in FR1:

UE peak data rate reduction [RAN1, RAN2, RAN4]

- Relaxation of the constraint (vLayers·Qm·f ≥ 4) for peak data rate reduction

- The relaxed constraint is, e.g., 1 (instead of 4).

- The parameters (vLayers·Qm·f) can be as in Rel-17 RedCap.

Aim to define at most one Rel-18 RedCap UE type for further UE complexity reduction.

Check in RAN#99 regarding:

- Whether UE peak data rate reduction for UE is limited only with UE BB bandwidth reduction or standalone

RAN2 should also take into consideration the following agreements already made by RAN1 on this topic:

- UE peak data rate reduction is supported at least as an add-on to UE BB bandwidth reduction,

  - The constraint vLayers·Qm·f ≥ 4 is relaxed to vLayers·Qm·f ≥ X.

  - FFS: the value of X

- If UE peak data rate reduction is supported as a standalone feature,

  - The constraint vLayers·Qm·f ≥ 4 is relaxed to vLayers·Qm·f ≥ Y.

  - FFS: the value of Y

  - Note: Whether this option is supported will be decided in RAN plenary.

- The minimum DL peak rate target (for FD-FDD) is [10] Mbps based on peak data rate calculation according to 38.306.

  - The same value for [10] Mbps is used for DL and UL

# Discussion

## UE capability impact

RAN2 should already confirm the specification impact to TS 38.306/38.331 from RAN1 highlighted above in yellow. I.e., UE’s capability should allow a UE supporting Baseband (BB) Bandwidth (BW) reduction to also indicate UE’s support of UE peak data rate reduction to a minimum peak data rate (for FD-FDD) of 10 Mbps for DL and UL.

- A new UE capability for UE Data Rate Reduction is defined as a per UE feature with optional, with capability signal, ‘FR1 only’ to the need to Diff. FR1/FR2 (as this feature is only defined for FR1), ‘No’ to the need to Diff. TDD/FDD and with a pre-requisite that UE shall all support Baseband Bandwidth Reduction capability. This new capability allows the UE to support a minimum peak data rate (for FD-FDD) of 10 Mbps for DL and UL.

RAN2 should wait for RAN guidance on whether the UE peak data rate reduction capability can be supported on its own, i.e., without supporting BB BW reduction.

- RAN2 wait for RAN guidance on whether UE peak data rate reduction feature can be supported standalone (i.e., without supporting BB BW reduction).

## Maximum data rate for DL/UL

To enable this reduction of the data rate, RAN1 is discussing to relax the value of X in vLayers·Qm·f ≥ X. From RAN2 point of view, this would require an update to section 4 on “Supported max data rate for DL/UL” in TS 38.306. For example as shown below:

For single carrier NR SA operation, the UE (except a UE supporting data rate reduction) shall support a data rate for the carrier that is no smaller than the data rate computed using the above formula, with J=1 CC and component vLayers(j)⋅Qmj⋅fj is no smaller than 4. For UE supporting data rate reduction in single carrier NR SA operation, the UE shall support a data rate for the carrier that is no smaller than the data rate computed using the above formula, with J=1 CC and component vLayers(j)⋅Qmj⋅fj is no smaller than X.

NOTE 3: As an example, the value 4 in the component above can correspond to vLayers(j)=1, Qmj= 4 and fj=1.

For further reference on RAN1 ongoing discussion, the intention is to find a proper product X equal to (vLayers·Qm·f) that allows us to get 10Mbps peak data rate. I.e. a Rel-18 RedCap UE would need a combination of (layer, modulation, scaling factor) that allows obtaining the target 10Mbps. For example, the proper product of (layer, modulation, scaling factor) will be around 3 (e.g., 3.2, 3.4, or other values) and how to obtain [3] is not are critical, one possible example can be (one layer, 16qam, scaling factor 0.75).

Section on the support of maximum data rate in DL/UL in TS 38.306 would need to be updated based on RAN1 agreement on the value X for which the constraint vLayers·Qm·f ≥ 4 is relaxed (i.e., vLayers·Qm·f ≥ X) while getting 10 Mbps peak data rate.

Rel-18 RedCap UE would indicate a combination of (layer, modulation, scaling factor) that allows obtaining the target 10Mbps and no smaller than X by using existing capability signaling. i.e. there is no impact to UE capability signaling associated with layer, modulation order and scaling factor.

# Conclusion

The observations captured are the following:

Observation 1.	Section on the support of maximum data rate in DL/UL in TS 38.306 would need to be updated based on RAN1 agreement on the value X for which the constraint vLayers·Qm·f ≥ 4 is relaxed (i.e., vLayers·Qm·f ≥ X) while getting 10 Mbps peak data rate.

Observation 2.	Rel-18 RedCap UE would indicate a combination of (layer, modulation, scaling factor) that allows obtaining the target 10Mbps and no smaller than X by using existing capability signaling. i.e. there is no impact to UE capability signaling associated with layer, modulation order and scaling factor.

The proposals captured are the following:

Proposal 1.	A new UE capability for UE Data Rate Reduction is defined as a per UE feature with optional, with capability signal, ‘FR1 only’ to the need to Diff. FR1/FR2 (as this feature is only defined for FR1), ‘No’ to the need to Diff. TDD/FDD and with a pre-requisite that UE shall all support Baseband Bandwidth Reduction capability. This new capability allows the UE to support a minimum peak data rate (for FD-FDD) of 10 Mbps for DL and UL.

Proposal 2.	RAN2 wait for RAN guidance on whether UE peak data rate reduction feature can be supported standalone (i.e., without supporting BB BW reduction).

# Reference

- RP-223544, Enhanced support of reduced capability NR devices, Rel-18 WID, December 2022.

- R2-2300406, RAN2 impacts to support UEs with Baseband Bandwidth Reduction, Intel Corporation, February 2023.