3GPP TSG-RAN WG2 Meeting #121	                                     R2-2300408

Athens, Greece, Feb 27 – Mar 03, 2023

Agenda item: 	8.4.2.1

Source:	Intel Corporation

Title: 	Discussion on the early TA acquisition

Document for:	Discussion and Decision

# Introduction

In RAN2#120, the baseline stage-2 running CR was endorsed for further update [1], and early DL/UL synchronization is added as step 4 of LTM. And in RAN1#111, an LS on RAN1 agreements for L1/L2-based inter-cell mobility was sent to RAN2 [2], which contains more information about the details of early UL synchronization, i.e., early TA acquisition of candidate cells.

In this paper, we further discuss the follow-up RAN2 work for early TA acquisition.

# Discussion

## 2.1 Background information

The baseline procedure of LTM is illustrated as below [1]. In step 2, the gNB transmits an RRCReconfiguration message to the UE including the configuration of one or multiple LTM candidate target cells.  And in step 4, the UE may perform DL synchronization and TA acquisition with candidate target cell(s) before receiving the LTM cell switch command.

Figure 1: Signaling procedure for LTM

In our companion paper [3], we provide a draft TP for stage-2 running CR to capture some high-level description of early TA acquisition based on RAN1 agreements. And the whole procedure of early TA acquisition can be split into 3 steps, i.e., configuration, execution, and TA updating, as illustrated in Figure 2.

Figure 2: Signaling procedure for early TA acquisition

Based on current RAN1 agreements, the “execution phase” is still under discussion, i.e., FFS on whether RAR is used for providing TA value to UE. If RAR is needed, UE can obtain the TA from RAR, which means it’s before the reception of cell switch command; if RAR is not needed, TA value of candidate cell is indicated in cell switch command, which means UE doesn’t get TA value before LTM execution. This FFS also impacts the discussion on TA updating, i.e., it’s not clear whether the UE needs to get involved in TA maintenance work, or it’s all up to network implementation to handle. Therefore, RAN2 still needs to wait for further RAN1 input before discussing RAN2 for TA updating.

Currently RAN2 can discuss the “configuration” of early TA acquisition, which is in RAN2 scope. And there are two discussion points to address in this aspect as follows:

- How to configure CFRA resource of candidate cells.

- How to handle the resource allocation of preamble for a candidate cell.

## 2.2 How to configure CFRA resource of candidate cells

The configuration of early TA acquisition is provided to UE from source cell prior to the transmission of PDCCH order. As explained in Table 1, the PDCCH order itself contains some configuration of preamble, such as preamble index, SSB index and the RACH occasion index. And in early TA acquisition case, the PDCCH order also contains the indication of candidate cell, e.g., a candidate configuration index of a candidate cell. But PDCCH order alone is not sufficient, other necessary PRACH configurations have to be provided by RRC message, e.g., cell specific RACH parameters (IE RACH-ConfigCommon).

Table 1. DCI format 1_0 used for PDCCH order

| DCI fields | Meaning |
| --- | --- |
| Identifier for DCI formats | The value of this bit field is always set to 1, indicating a DL DCI format |
| Frequency domain resource assignment | If the CRC of the DCI format 1_0 is scrambled by C-RNTI and the "Frequency domain resource assignment" field are of all ones, the DCI format 1_0 is for random access procedure initiated by a PDCCH order |
| Random Access Preamble index | This field contains 6 bits. It indicates which Random access preamble to use in case of Contention Free Random Access(CFRA) or the value 000000 in the case of Contention based Random Access(CBRA) procedure |
| UL/SUL indicator | ‘0’ for the non-supplementary uplink, and ‘1’ for the supplementary uplink |
| SS/PBCH index | If the value of the "Random Access Preamble index" is not all zeros, this field indicates the SS/PBCH that shall be used to determine the RACH occasion for the PRACH transmission; otherwise, this field is reserved. |
| PRACH Mask index | If the value of the "Random Access Preamble index" is not all zeros, this field indicates the RACH occasion associated with the SS/PBCH indicated by "SS/PBCH index" for the PRACH transmission; otherwise, this field is reserved |
| Reserved bits | 10 bits |

There are two options to provide RACH parameters of a candidate cell for early TA acquisition.

Option 1: RACH configuration is included in the candidate cell RRC model.

In current RRC model design, a RRCReconfiguration Message or an IE CellGroupConfig can be used for a candidate cell. Based on current ASN.1 structure, the common RACH configuration parameters are already available within IE ReconfigurationWithSync. Furthermore, there is a field rach-ConfigDedicated within IE ReconfigurationWithSync to provide CFRA configuration.

If we follow this way, i.e., CFRA resource is configured in RRC message for a candidate cell, PDCCH order only needs to indicate which candidate cell is triggered for early TA acquisition. Or if CFRA resource is not configured in RRC message, which means the common RACH configuration is provided by RRC, and dedicated preamble configuration for CFRA (and also the candidate cell ID for early TA acquisition) is provided by PDCCH order only.

Option 2: separate configuration, i.e., not a part of LTM candidate cell RRC model.

Although option 1 can work, we still see two drawbacks of it. The first one is UE has to decode the candidate cell’s configuration before it’s selected as the target cell, which may lead to some unnecessary decoding operation at UE side. The second one is option 1 includes the RACH configuration for early TA as part of the LTM candidate cell configuration, thus the reconfiguration can’t be done without touching the LTM candidate cell configuration.

To avoid unnecessary decoding (e.g. the UE may partially decode just the RACH configuration for early TA acquisition) and make this procedure more flexible (e.g., the preamble configuration can be generated/updated later separately), the RACH configuration for early TA acquisition of a candidate cell can be a separate configuration as illustrated in Figure 3. Similar to option 1, this separate configuration can only include common RACH configuration parameters, then PDCCH order provides the remaining preamble configuration, or, this separate configuration can include all CFRA configuration parameters, then the PDCCH order only contains the indication of candidate cell ID (candidate configuration index).

Figure 3: separate RACH configuration for early TA acquisition

Proposal 1: define a separate RRC RACH configuration of candidate cell for early TA acquisition, i.e., not a part of LTM candidate cell RRC model.

## 2.3 How to handle the resource allocation of preamble for a candidate cell

The preamble resource is configured by the combination of SSB index, PRACH mask index and preamble index as illustrated in Figure 4. Since one SSB index could map to multiple PRACH occasions, e.g., ssb-perRACH-Occasion is oneFourth and PRACH Mask Index is 0, this means all 4 PRACH occasions can be used for preamble transmission. And in each PRACH occasion, the preamble with the configured preamble index can be transmitted by UE. In total, there are 4 preamble resources that can be used by UE.

Figure 4: the configuration of Preamble resources

Preamble resource for early TA acquisition towards a candidate cell is allocated by this candidate cell. And there are two major options of preamble resource allocation.

Option 1: preamble resource is UE specific.

If CFRA resource is configured in RRC, this resource is UE specific and can’t be used by other UEs.

Option 2: preamble resource can be shared by multiple UEs.

If only common RACH configuration is provided in RRC, and PDCCH order indicates the other CFRA configuration, it can enable shared preamble resource by multiple UEs. E.g., the UE measures SSB beams of candidate cells and reports L1/L3 measurement results to source cell, then based on measurement report source cell selects a candidate cell, a PRACH occasion and a preamble according to a SSB index (e.g., this SSB is with the highest RSRP of this candidate cell). Then source cell indicates the preamble resource to UE by PDCCH order.

From UE point of view, when the source cell triggers the PRACH via PDCCH order, the UE will use it for preamble transmission. Then the UE will release it, i.e., the UE doesn’t occupy this preamble resource all the time, and this preamble resource can be used by other UEs later. From network perspective, a candidate cell needs to allocate several preamble resources to all UEs in a source cell, and source cell makes sure to trigger ONE UE at a time for early TA acquisition, i.e., only one UE uses one specific preamble resource. In this way, it can still save preamble resources by reusing preambles among multiple UEs for each candidate cell.

Proposal 2: To enable shared preamble resource by multiple UEs, CFRA configuration does not need to be provided in the RRC RACH configuration for early TA as it can be indicated in the PDCCH order.

# Conclusion

In this paper, we discuss the configuration and resource allocation of early TA acquisition, and we propose:

Proposal 1: define a separate RRC RACH configuration of candidate cell for early TA acquisition, i.e., not a part of LTM candidate cell RRC model.

Proposal 2: To enable shared preamble resource by multiple UEs, CFRA configuration does not need to be provided in the RACH configuration for early TA as it can be indicated in the PDCCH order.

# References

[1] R2-2213332		38.300 running CR for introduction of NR further mobility enhancements,	MediaTek Inc.

[2] R1-2212948		LS on RAN1 agreements for L1/L2-based inter-cell mobility, RAN1

[3] R2-2300400		Procedure descriptions of LTM, Intel corporation